3GPP TSG-RAN5 Meeting #93-e	R5-217104r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2594 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test Case 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 6 , SS transmits RRC configuration message to establish DRB1 and SRB2 . In the current Prose ,Table 8.1.2.1.1.3.3-1A: CellGroupConfig is listing mac-LogicalChannelConfig as NotPresent which is contrary to the specification TS 38.508 , Table 4.6.3-148: RLC-BearerConfig & Table 4.6.3-66: LogicalChannelConfig. A conformant UE will reject the reconfiguration message without Logical channel configuration for SRB and DRB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to fill LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5s217104r1: Voided Tables 8.1.2.1.1.3.3-2 and 8.1.2.1.1.3.3-2A. Also updated Table 8.1.2.1.1.3.3-4, where IE mac-LogicalChannelConfig is set to LogicalChannelConfig according to 38.508-1[4], table 4.6.3-66. Introduced Steps 12A1 and 12A2 to perform the Open UE Test Loop procedure, and introdced Steps 12A3 and 12A4 to introduce the Deactivate Test Mode procedure because having this active at this stage may cause unspecified behaviour once the PDU Session is established at Step 12C. Corrected the Table numbers for Step 11 (CellGroupConfig). This is an editorial change |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.2.1.1	RRC reconfiguration / DRB / SRB / Establishment / Modification / Release / Success

8.1.2.1.1.1	Test Purpose (TP)

(1)

with { UE having completed the RRC connection establishment procedure}

ensure that {

when { SS sends in sequence a SecurityModeCommand and an RRCReconfiguration message to establish a DRB }

then { UE establishes the initial security configuration in accordance with the received securityConfigSMC included in SecurityModeCommand and successfully completes the reconfiguration }

}

(2)

with { UE in NR RRC_CONNECTED state without SRB2 }

ensure that {

when { SS sends an RRCReconfiguration message including SRB2 configuration }

then { UE successfully establishes the signalling radio bearer }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCReconfiguration message to reconfigure the current UE configuration of SRB and DRB }

then { UE reconfigures the data and signalling radio bearers and sends an RRCReconfigurationComplete message }

}

(4)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCReconfiguration message including a drb-ToReleaseList }

then { for each drb-Identity release the PDCP entity and RLC entity and logical channel and indicate release of the DRB(s) to upper layers }

}

8.1.2.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in:  TS 38.331, clauses 5.3.4.3, 5.3.5.6.3, 5.3.5.6.4, and 5.3.5.6.5. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.4.3]

The UE shall:

1>	derive the KgNB key, as specified in TS 33.501 [11];

1>	derive the KRRCint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

1>	request lower layers to verify the integrity protection of the SecurityModeCommand message, using the algorithm indicated by the integrityProtAlgorithm as included in the SecurityModeCommand message and the KRRCint key;

1>	if the SecurityModeCommand message passes the integrity protection check:

2>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	derive the KUPint key associated with the integrityProtAlgorithm indicated in the SecurityModeCommand message, as specified in TS 33.501 [11];

2>	configure lower layers to apply SRB integrity protection using the indicated algorithm and the KRRCint key immediately, i.e. integrity protection shall be applied to all subsequent messages received and sent by the UE, including the SecurityModeComplete message;

[TS 38.331, clause 5.3.5.6.3]

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModList that is not part of the current UE configuration (SRB establishment or reconfiguration from E-UTRA PDCP to NR PDCP):

2>	establish a PDCP entity;

2>	if AS security has been activated:

3>	if target RAT of handover is E-UTRA/5GC, or;

3>	if the UE is only connected to E-UTRA/5GC:

4>	configure the PDCP entity with the security algorithms and keys (KRRCenc and KRRCint) configured/derived as specified in TS 36.331 [10];

3>	else:

4>	configure the PDCP entity with the security algorithms according to securityConfig and apply the keys (KRRCenc and KRRCint) associated with the master key (KeNB/ KgNB) or secondary key (S-KgNB) as indicated in keyToUse, if applicable;

2>	if the current UE configuration as configured by E-UTRA in TS 36.331 [10] includes an SRB identified with the same srb-Identity value:

3>	associate the E-UTRA RLC entity and DCCH of this SRB with the NR PDCP entity;

3>	release the E-UTRA PDCP entity of this SRB;

2>	if the pdcp-Config is included:

3>	configure the PDCP entity in accordance with the received pdcp-Config;

2>	else:

3>	configure the PDCP entity in accordance with the default configuration defined in 9.2.1 for the corresponding SRB;

1>	for each srb-Identity value included in the srb-ToAddModList that is part of the current UE configuration:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC, or;

3>	if the UE is only connected to E-UTRA/5GC:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key configured/derived as specified in TS 36.331 [10], i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key configured/derived as specified in TS 36.331 [10], i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

3>	else:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB), as indicated in keyToUse , i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KeNB/KgNB) or secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

2>	else, if the discardOnPDCP is set:

3>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

2>	if the pdcp-Config is included:

3>	reconfigure the PDCP entity in accordance with the received pdcp-Config.

[TS 38.331, clause 5.3.5.6.4]

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration; or

1>	for each drb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the drb-Identity;

2>	if SDAP entity associated with this DRB is configured:

3>	indicate the release of the DRB to SDAP entity associated with this DRB (TS 37.324 [24], clause 5.3.3);

2>	if the UE is operating in EN-DC:

3>	if a new bearer is not added either with NR or E-UTRA with same eps-BearerIdentity:

4>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

[TS 38.331, clause 5.3.5.6.5]

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

2>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

3>	if target RAT of handover is E-UTRA/5GC, or;

3>	if the UE is only connected to E-UTRA/5GC:

4>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10];

3>	else:

4>	configure the PDCP entity with the ciphering algorithms according to securityConfig and apply the KUPenc key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

2>	if the PDCP entity of this DRB is configured with integrityProtection:

3>	configure the PDCP entity with the integrity algorithms according to securityConfig and apply the KUPint key associated with the master (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

2>	if an sdap-Config is included:

3>	if an SDAP entity with the received pdu-Session does not exist:

4>	establish an SDAP entity as specified in TS 37.324 [24] clause 5.1.1;

4>	If an SDAP entity with the received pdu-Session did not exist prior to receiving this reconfiguration:

5>	indicate the establishment of the user plane resources for the pdu-Session to upper layers;

3>	configure the SDAP entity in accordance with the received sdap-Config as specified in TS 37.324 [24] and associate the DRB with the SDAP entity;

2>	if the UE is operating in EN-DC:

3>	if the DRB was configured with the same eps-BearerIdentity either by NR or E-UTRA prior to receiving this reconfiguration:

4>	associate the established DRB with the corresponding eps-BearerIdentity;

3>	else:

4>	indicate the establishment of the DRB(s) and the eps-BearerIdentity of the established DRB(s) to upper layers;

8.1.2.1.1.3	Test description

8.1.2.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None

Preamble:

-	If pc_IP_Ping is set to TRUE then the UE is in 5GS state 1N-A, PDU SESSION ACTIVE according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-1

-	Else, the UE is in 5GS state 1N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.2.1.1.3.2	Test procedure sequence

Table 8.1.2.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. | <-- | NR RRC: Paging | - | - |
| 2 | UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | SS transmits an RRCSetup message | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete | - | - |
| 5 | SS transmits a SecurityModeCommand message to activate AS security. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. | <-- | NR RRC: RRCReconfiguration | - | - |
| 7 | Check: Does the UE transmit a SecurityModeComplete message? | --> | NR RRC: SecurityModeComplete | 1 | P |
| 8 | Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? | --> | NR RRC: RRCReconfigurationComplete | 1,2 | P |
| 9 | Void |  |  | - | - |
| 10 | Void |  |  | - | - |
| 11 | The SS transmits an RRCReconfiguration message to modify SRB and DRB configuration. | <-- | NR RRC: RRCReconfiguration | - | - |
| 12 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | 3 | P |
| 12A | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? | - | - | 3 | - |
| 12A1 | The SS transmits an OPEN UE TEST LOOP Message. | <-- | OPEN UE TEST LOOP | - | - |
| 12A2 | The UE transmits an OPEN UE TEST LOOP COMPLETE Message. | --> | OPEN UE TEST LOOP COMPLETE | - | - |
| 12A3 | The SS transmits DEACTIVATE TEST MODE message. | <-- | DEACTIVATE TEST MODE | - | - |
| 12A4 | The UE transmits DEACTIVATE TEST MODE COMPLETE message. | --> | DEACTIVATE TEST MODE COMPLETE | - | - |
| 12B | Cause the UE to request establishment of PDU session.(Note 1) | - | - | - | - |
| 12C | The PDU session establishment procedure as specified in TS 38.508-1 [4] subclause 4.5A.2 takes place. | - | - | - | - |
| 13 | The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMMAND | - | - |
| 14 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | 4 | P |
| 16 | The UE transmits an ULInformationTransfer message and a UL NAS TRASPORT containing PDU SESSION RELEASE COMPLETE. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMPLETE | - | - |
| NOTE 1: The request to establish a PDU session may be performed by MMI or AT Command. |  |  |  |  |  |

8.1.2.1.1.3.3	Specific message contents

Table 8.1.2.1.1.3.3-1: RRCReconfiguration (Step 6, Table 8.1.2.1.1.3.2-1)

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.2.1.1.3.3-1A: CellGroupConfig (Table 8.1.2.1.1.3.3-1)

| Derivation Path: TS 38.508-1, table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { |  | 2 entries |  |  |
| RLC-Bearer-Config[1] SEQUENCE { |  |  | entry 1 |  |
| logicalChannelIdentity |  | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |  |
| drb-Identity |  | DRB-Identity with condition DRB1 |  |  |
| } |  |  |  |  |
| reestablishRLC |  | Not present |  |  |
| RLC-Config |  | Not present |  |  |
| mac-LogicalChannelConfig |  | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |  |
| RLC-Bearer-Config[2] SEQUENCE { |  |  | entry 2 |  |
| logicalChannelIdentity |  | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |  |
| srb-Identity |  | SRB-Identity with condition SRB2 |  |  |
| } |  |  |  |  |
| reestablishRLC |  | Not present |  |  |
| RLC-Config |  | Not present |  |  |
| mac-LogicalChannelConfig |  | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| mac-CellGroupConfig |  | Not present |  |  |
| physicalCellGroupConfig |  | Not present |  |  |
| spCellConfig SEQUENCE |  | Not present |  |  |
| } |  |  |  |  |

Table 8.1.2.1.1.3.3-2:  Void

Table 8.1.2.1.1.3.3-2A:  Void

Table 8.1.2.1.1.3.3-3: RRCReconfiguration (Step 11, Table 8.1.2.1.1.3.2-1)

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.2.1.1.3.3-4: CellGroupConfig (Table 8.1.2.1.1.3. 3-3)

| Derivation Path: TS 38.508-1, table 4.6. 3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { |  | 2 entries |  |  |
| RLC-Bearer-Config[1] SEQUENCE { |  |  | entry 1 |  |
| logicalChannelIdentity |  | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |  |
| srb-Identity |  | SRB-Identity with condition SRB2 |  |  |
| } |  |  |  |  |
| reestablishRLC |  | true |  |  |
| RLC-Config |  | Not present |  |  |
| mac-LogicalChannelConfig |  | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |  |
| RLC-Bearer-Config[2] SEQUENCE { |  |  | entry 2 |  |
| logicalChannelIdentity |  | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |  |
| drb-Identity |  | DRB-Identity with condition DRB1 |  |  |
| } |  |  |  |  |
| reestablishRLC |  | true |  |  |
| RLC-Config |  | Not present |  |  |
| mac-LogicalChannelConfig |  | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| mac-CellGroupConfig |  | Not present |  |  |
| physicalCellGroupConfig |  | Not present |  |  |
| spCellConfig SEQUENCE |  | Not present |  |  |
| } |  |  |  |  |

Table 8.1.2.1.1.3.3-5: RRCReconfiguration (Step 13, Table 8.1.2.1.1.3.2-1)

| Derivation Path: TS 38.508-1, table 4.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.2.1.1.3.3-6: RadioBearerConfig (Table 8.1.2.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToReleaseList | 1 | DRB1 |  |
| } |  |  |  |

Table 8.1.2.1.1.3.3-7: CellGroupConfig (Table 8.1.2.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | 1 |  |  |
| rlc-BearerToAddModList | Not Present |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity { | 1 entry |  |  |
| logicalChannelIdentity[1] | LogicalChannelIdentity | entry 1 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not Present |  |  |
| tag-Config | Not Present |  |  |
| phr-Config | Not Present |  |  |
| physicalCellGroupConfig | Not Present |  |  |
| rlf-TimersAndConstants | Not Present |  |  |
| } |  |  |  |