3GPP TSG-RAN5 Meeting #93-e	R5-217108r3

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2595 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR-DC testcase 7.1.1.11.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Qualcomm, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The PHR configuration defined in Table 7.1.1.11.1.3.3-2 is not clear on which Cell this configuration needs to be applied. Since the test purpose is to check reporting of PHR on both PCell and PSCell, it is proposed to add PHR configuration on PCell and PSCell - NR Cell10 power at T0 is set to off, but it is used in intial preamble procedure to add SCG Cell - UE can only report PHR on NRCell 10 when there is uplink grant configured. - Change of power levels will trigger PHR reporting on specific cells and therefore needs to be clarified in prose T0 T1 = PCell PHR reporting T1 T2 = PCell PHR reporting T2 T3 = PSCell PHR reporting T3 T4 = PSCell PHR reporting - FR1 OTA requirements specified in 38.508-1 clause 6.2.2.2.3 only allows Serving cell power level or Non-suitable "Off" power level.Therefore the current test case does not align with OTA requirement specified in 38.508-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added specific message contents for RRC Reconfiguration message to add PHR reporting on PCell and PSCell and Added Step 3A - Changed power of NR Cell10 to -82 - Added clarification to Step 1 to configure uplink grant on PCell and PSCell - Added clarification in the test procedure on which cell the PHR reporting shall be expected. - Modified the test sequence to allow the test case to be exceuted in FR1+FR2 envoirment. - Added a note to clarify the limitation in OTA envoirnment and phr-Tx-PowerFactorChange for PCell is not tested |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R3 : update to use the term “OTA” |  |  |  |  |  |  |  |  |

#### 7.1.1.11	NR Dual Connectivity

##### 7.1.1.11.1	DC power headroom reporting / PSCell activation and DL pathloss change reporting

7.1.1.11.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state on Pcell }

ensure that {

when { PSCell is added }

then { UE transmits a Power Headroom Report for the PCell and PSCell }

}

(2)

with { UE in RRC_CONNECTED state with PSCell and with Power headroom reporting for phr-Tx-PowerFactorChange }

ensure that {

when { the DL Pathloss has changed more than phr-Tx-PowerFactorChange dB and phr-ProhibitTimer is running }

then { UE does not transmit a MAC PDU containing Power Headroom MAC Control Element }

}

(3)

with { UE in RRC_CONNECTED state with PSCell and with Power headroom reporting for phr-Tx-PowerFactorChange }

ensure that {

when { the phr-ProhibitTimer expires and power headroom report is triggered due to DL Pathloss change  }

then { UE transmits a MAC PDU containing Power Headroom MAC Control Element for the Pcell and PSCell }

}

7.1.1.11.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clause 5.4.6

[TS 38.321, clause 5.4.6]

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one activated Serving Cell of any MAC entity which is used as a pathloss reference since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

NOTE 1:	The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between.

-	phr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink;

-	addition of the PSCell (i.e. PSCell is newly added or changed);

-	phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

NOTE 2:	The MAC entity should avoid triggering a PHR when the required power backoff due to power management decreases only temporarily (e.g. for up to a few tens of milliseconds) and it should avoid reflecting such temporary decrease in the values of PCMAX,f,c/PH when a PHR is triggered by other triggering conditions.

If the MAC entity has UL resources allocated for a new transmission the MAC entity shall:

1>	if it is the first UL resource allocated for a new transmission since the last MAC reset:

2>	start phr-PeriodicTimer;

1>	if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled; and

1>	if the allocated UL resources can accommodate the MAC CE for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in clause 5.4.3.1:

2>	if multiplePHR with value true is configured:

3>	for each activated Serving Cell with configured uplink associated with any MAC entity:

4>	obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell and clause 5.1.1.2 of TS 36.213 [17] for E-UTRA Serving Cell;

4>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

4>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

5>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

3>	if phr-Type2OtherCell with value true is configured:

4>	if the other MAC entity is E-UTRA MAC entity:

5>	obtain the value of the Type 2 power headroom for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity);

5>	if phr-ModeOtherCG is set to real by upper layers:

6>	obtain the value for the corresponding PCMAX,f,c field for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity) from the physical layer.

3>	instruct the Multiplexing and Assembly procedure to generate and transmit the Multiple Entry PHR MAC CE as defined in clause 6.1.3.9 based on the values reported by the physical layer.

2>	else (i.e. Single Entry PHR format is used):

3>	obtain the value of the Type 1 power headroom from the physical layer for the corresponding uplink carrier of the PCell;

3>	obtain the value for the corresponding PCMAX,f,c field from the physical layer;

3>	instruct the Multiplexing and Assembly procedure to generate and transmit the Single Entry PHR MAC CE as defined in clause 6.1.3.8 based on the values reported by the physical layer.

2>	start or restart phr-PeriodicTimer;

2>	start or restart phr-ProhibitTimer;

2>	cancel all triggered PHR(s).

7.1.1.11.1.3	Test description

7.1.1.11.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and NR Cell 10 is the PSCell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in all cells.

UE:

-	None.

Preamble:

-	The UE is in state NR RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC), Test Mode (On) associated with UE test loop mode A  configured on NR Cell 1 according to TS 38.508-1 [4], clause 4.5.4.

7.1.1.11.1.3.2	Test procedure sequence

Table 7.1.1.11.1.3.2-0: Cell configuration power level changes over time for Conducted test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/SCS | -82 | -82 |  |
| T1 | Cell-specific RS EPRE | dBm/SCS | -89 | -82 |  |
| T2 | Cell-specific RS EPRE | dBm/SCS | -82 | -82 |  |
| T3 | Cell-specific RS EPRE | dBm/SCS | -82 | -89 |  |
| T4 | Cell-specific RS EPRE | dBm/SCS | -82 | -82 |  |

Table 7.1.1.11.1.3.2-0A: Cell configuration power level changes over time for OTA test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/SCS | -82 | -82 |  |
| T1 | Cell-specific RS EPRE | dBm/SCS | n/a | n/a |  |
| T2 | Cell-specific RS EPRE | dBm/SCS | n/a | n/a |  |
| T3 | Cell-specific RS EPRE | dBm/SCS | -82 | -91 |  |
| T4 | Cell-specific RS EPRE | dBm/SCS | -82 | -82 |  |

Table 7.1.1.11.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits UL grant on PCell and PSCell to the UE at every 10ms in PDCCH occasion. | <-- | - | - | - |
| 2 | SS transmits NR RRCReconfiguration message to configure to specific Power Headroom parameters for NR Cell | <-- | RRCReconfiguration | - | - |
| 3 | Check: Does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PCell ? (Note 1) | --> | MAC PDU | 1 | P |
| 3A | Check: Does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PSCell? (Note 1) | --> | MAC PDU | 1 | P |
| 4 | The UE transmits an NR RRCReconfigurationComplete message including nr-SCG-Response (Note 1) | --> | RRCReconfigurationComplete | - | - |
| 5 | Wait for T= 10% of prohibitPHR-Timer from step 3 | - | - | - | - |
| - | EXCEPTION : Steps 6 to 12 shall be executed depending on PSCell Configuration. (Note 3) | - | - | - | - |
| 6 | IF PSCell is configured as FR1 THEN Reduce SS power level for NR PCell so as to cause a DL_Pathloss change at UE by 5dB, row T1 of Table 7.1.1.11.1.3.2-0. | - | - | - | - |
| 7 | Check: For 80% of prohibitPHR-Timer since step 3, does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PCell? | --> | MAC PDU | 2 | F |
| 8 | Check: After prohibitPHR-Timer after step 3, does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PCell? | --> | MAC PDU | 3 | P |
| 9 | Increase SS power level for NR PCell so as to cause a DL_Pathloss change at UE by 5dB, row T2 of Table 7.1.1.11.1.3.2-0/0A. | - | - | - | - |
| 10 | Check: For 80% of prohibitPHR-Timer since step 8, does the UE transmit a MAC PDU containing Power Headroom MAC Control Element on PCell? | --> | MAC PDU | 2 | F |
| 11 | Check: After prohibitPHR-Timer after step 8, does the UE transmit a MAC PDU containing Power Headroom MAC Control Element on PCell? | --> | MAC PDU | 3 | P |
| 12 | Wait for T= 10% of prohibitPHR-Timer from step 11 | - | - | - | - |
| 13 | Reduce SS power level for NR PSCell so as to cause a DL_Pathloss change at UE by 5dB, row T3 of Table 7.1.1.11.1.3.2-0/0A. | - | - | - | - |
| 14 | IF PSCell is configured as FR2 THEN Check: For 80% of prohibitPHR-Timer since step 3A, does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE? | --> | MAC PDU | 2 | F |
| 15 | Check: Does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PSCell? | --> | MAC PDU | 3 | P |
| 16 | Increase SS power level for NR PSCell so as to cause a DL_Pathloss change at UE by 5dB, row T4 of Table 7.1.1.11.1.3.2-0/0A. | - | - | - | - |
| 17 | Check: For 80% of prohibitPHR-Timer since step 15, does the UE transmit a MAC PDU containing Power Headroom MAC Control Element on PSCell? | --> | MAC PDU | 2 | F |
| 18 | Check: After prohibitPHR-Timer after step 15, does the UE transmit a MAC PDU containing Power Headroom MAC Control Element on PSCell? | --> | MAC PDU | 3 | P |
| 19 | The SS transmits an NR RRCReconfiguration message to disable Power Headroom reporting | <-- | RRCReconfiguration | - | - |
| 20 | The UE transmits an NR RRCReconfigurationComplete message to confirm the disabling of Power Headroom parameters | --> | RRCReconfigurationComplete | - | - |
| Note 1: Steps 3 and 4 can happen in any order. Note 2: Void. Note 3: Steps 6 to 12 are excluded when executed with FR1+FR2 band combination due to limitation in FR1 OTA requirements specified in 38.508-1 [4] clause 6.2.2.2.3. phr-Tx-PowerFactorChange for PCell is not tested due to this limitation |  |  |  |  |  |

7.1.1.11.1.3.3	Specific Message Contents

Table 7.1.1.11.1.3.3-1: RRCReconfiguration (step 2, Table 7.1.1.11.1.3.2-1)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-phr |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration-SCG-phr | OCTET STRING (CONTAINING RRCReconfiguration) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.11.1.3.3-1A: RRCReconfiguration-SCG-phr (step 2, Table 7.1.1.11.1.3.2-1)

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
| secondaryCellGroup |  | CellGroupConfig-phr |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.11.1.3.3-2: CellGroupConfig-phr (step 2, Table 7.1.1.11.1.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| phr-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | infinity |  |  |
| phr-ProhibitTimer | sf500 |  |  |
| phr-Tx-PowerFactorChange | dB3 |  |  |
| multiplePHR | true |  |  |
| dummy | false |  |  |
| phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |