3GPP TSG-RAN5 Meeting # 92-e	R5-217182r1

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0266 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce and update PICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo and Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduce and update PICS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PICS for voiceOverNR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature will not be adequately tested on UE and hence Interroperability cannot be guaranteed Revision 1: Title alignment with 3GU. Revision 2: Removed pc_NR_5GC_EmergencyServices (R5-217990 clause 5.2.2) Updated comments of pc_voiceOverNR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Applicable to Rel-15 and later |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### A.4.3.7	General Capabilities

Table A.4.3.7-1: UE General Capabilities

| Item |  | UE General Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support UL transmission via either MCG path or SCG path for the split SRB as specified in TS 37.340[20] |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_splitSRB_WithOneUL_Path |  | No |  |  |  |  |  |
| 2 |  | Support UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340[20] |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_splitDRB_withUL_Both_MCG_SCG |  | Yes |  |  |  |  |  |
| 3 |  | Support direct SRB between the SN and the UE as specified in TS 37.340[20] |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_srb3 |  | Yes |  |  |  |  |  |
| 4 |  | Support of reflective QoS |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_as_ReflectiveQoS |  | No |  |  |  |  |  |
| 5 |  | Support of NAS reflective QoS |  | 24.501, 6.2.5.1.4.1, 9.11.4.1 |  | Rel-15 |  | pc_nas_ReflectiveQoS |  | No |  |  |  |  |  |
| 6 |  | Support of SMS over NAS |  | 24.501, 5.5.1.2 |  | Rel-15 |  | pc_sms_over_NAS |  | No |  |  |  |  |  |
| 7 |  | Support of CMAS message on NR |  | 38.331, 5.2.2.2.2 |  | Rel-15 |  | pc_CMAS_NR |  | No |  |  |  |  |  |
| 8 |  | Support of ETWS message on NR |  | 38.331, 5.2.2.2.2 |  | Rel-15 |  | pc_ETWS_NR |  | No |  |  |  |  |  |
| 9 |  | The UE supports additional UE-requested PDU establishment |  | 24.501, 6.4.1.5 |  | Rel-15 |  | pc_Additional_PDU_establishment |  | No |  |  |  | pc_ExpectedNoOfPDUSessionsAtRegistration +1 |  |
| 10 |  | The UE includes the SM PDU DN request container IE in the PDU SESSION ESTABLISHMENT REQUEST message |  | 24.501, 6.4.1.2 |  | Rel-15 |  | pc_SM_PDU_DN_RequestContainer |  | No |  |  |  |  |  |
| 11 |  | Support of emergency services fallback in NR connected to 5GCN |  | 24.501 |  | Rel-15 |  | pc_NR_5GC_EmergencyService_fallback |  | No |  |  |  |  |  |
| 12 |  | Support of EPS fallback |  | 24.501, |  | Rel-15 |  | pc_EPS_fallback |  | No |  |  |  |  |  |
| 13 |  | Support of UE requested PDU session modification |  | 24.501, 6.4.2.2 |  | Rel-15 |  | pc_MO_PDU_Session_Modification |  | Yes |  |  |  |  |  |
| 14 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | Support of voiceFallbackIndication |  | 38.306, 4.2.13 |  | Rel-16 |  | pc_voiceFallbackIndication |  | No |  |  |  |  |  |
| 16 |  | Support provision of referenceTimeInfo |  | 38.306, 4.2.2 |  | Rel-16 |  | pc_referenceTimeProvision_r16 |  | No |  |  |  | specifically for TSC (time sensitive communication) services |  |
| 17 |  | Support of RACS |  | 24.501, 9.11.3.1 |  | Rel-16 |  | pc_5GC_RACS |  | No |  |  |  |  |  |
| 18 |  | Support of RRC message Segmentation in the UL |  | 38.306, 5.4 |  | Rel-16 |  | pc_NR_UL_Segmentation |  | No |  |  |  | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |  |
| 19 |  | Support of RRC_INACTIVE as specified in TS 38.331 [9]. |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_inactiveState |  | Yes |  |  |  |  |  |
| 20 |  | Support of UE local release when the security check is successful but SOR Transparent container indicates ACK has been NOT requested |  | 23.122 clause C.2 |  | Rel-15 |  | pc_SOR_ACKNotReqLocalRel |  | No |  |  |  |  |  |
| 21 |  | Support of RRC connection release with deprioritisation |  | 38.306, 5.3 |  | Rel-15 |  | pc_NR_RRC_Release_With_Deprioritisation |  | No |  |  |  |  |  |
| 22 |  | Support of RRC connection establishment failure with temporary offset |  | 38.306, 5.3 |  | Rel-15 |  | pc_NR_RRC_ConEstFail_With_TempOffset |  | No |  |  |  |  |  |
| 23 |  | Support of Closed Access Group |  | 24.501, 9.11.3.1 |  | Rel-16 |  | pc_CAG |  | No |  |  |  |  |  |
| 24 |  | Support of Stand-alone Non-Public Network |  | 23.501, 3.1 |  | Rel-16 |  | pc_SNPN |  | No |  |  |  |  |  |
| 25 |  | Support of test function SET UL MESSAGE for using a preconfigured UE capability container over NR |  | 38.509, 5.9 |  | Rel-16 |  | pc_Set_UE_Cap_Info_NR |  | No |  |  |  |  |  |
| 26 |  | Support of network slice-specific authentication and authorization |  | 24.501, 9.11.3.1 |  | Rel-16 |  | pc_5GC_NSSAA |  | No |  |  |  |  |  |
| 27 |  | Support of EAP-AKA’ as EAP method for network slice-specific authentication and authorization |  | 24.501, 5.4.7 |  | Rel-16 |  | pc_5GC_NSSAA_EAP_AKA_Prime |  | No |  |  |  |  |  |
| 28 |  | Support reduced control plane latency as defined in TS 38.331 [9] |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_reducedCP_Latency |  | No |  |  |  |  |  |
| 29 |  | Support of release preference assistance information |  | 38.306, 4.2.2 |  | Rel-16 |  | pc_releasePreference_r16 |  | No |  |  |  |  |  |
| XX |  | Support IMS voice over NR |  | 38.306, 4.2.13 |  | Rel-15 |  | pc_voiceOverNR |  | No |  |  |  | A UE supporting IMS voice over NR shall support: - IMS emergency call over NR, and - IMS voice over E-UTRA/EPC if it supports E-UTRA/EPC. |  |