3GPP TSG-RAN5 Meeting #93-e	R5-217227r1

# Electronic Meeting, 8th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1538 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM Test Case 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy in RAN4 [6] core-spec, but not added in RAN5 TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Test Case 6.7.9.3 and the message contents for tables H.3.6A-1 and H.3.6A-2 is came from Huawei’s CR: R5-217214. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 6.7.9.3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.9.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR 1530 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Editor’s note is added - Test procedure & messages contents is updated - The SSB index: 0 is remvoed from message contents. - “TT” is added in Test requirement - The source of messages contetns for tables H.3.6A-1 and H.3.6A-2 is added. |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	MU & TT needs further study

-	Test Procedure is incomplete

-	Message content is incomplete

-	Test applicability is not added in TS38.522 yet

6.7.9.3.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

6.7.9.3.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.9.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.3.

6.7.9.3.4	Test description

6.7.9.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.3.4.1-1.

Table 6.7.9.3.4.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.3.4.1-2.

Table 6.7.9.3.4.1-2: Initial conditions for CSI-RS based CMR and CSI-IM based IMR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.9.3.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based CMR and CSI-IM based IMR measurements. The UE is configured to perform RLM and BFD measurement based on the CSI-RS. The connection setup is done according to the settings in Annex C.1.1.

2.	Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based CMR and CSI-IM measurements. The UE is configured to perform RLM, BFD and L1-SINR measurement based on the CSI-RSs. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

6.7.9.3.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.3.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.3.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.3.5-1. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

3.	The UE shall start sending L1-SINR report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS #0 and CSI-RS #1 in the periodic L1-SINR reports. If the value for both CSI-RSs is within the limits in Table 6.7.9.3.5-2 and Table 6.7.9.3.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.9.3.5-1 as appropriate and repeat steps 3-5.

6.7.9.3.4.3	Message contents

Same message content as in subclause 6.7.9.1.4.3 with the following exception:

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.9.3.4.3-1: Common Exception messages NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and CSI-SINR and CSI-IM_IMR Table H.3.6A-2 with conditions CSI-RS and PERIODIC Table H.3.6A-3 with condition PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.9.3.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.7.9.3.5	Test requirement

Table 6.7.9.3.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.3.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.9.3.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.9.3.5-3 for test configuration 3.

Table 6.7.9.3.5-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65+TT | -117+TT |
|  | NR_FDD_FR1_B |  |  |  | -116.5+TT |
|  | NR_TDD_FR1_C |  |  |  | -116+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115+TT |
|  | NR_FDD_FR1_F |  |  |  | -114.5+TT |
|  | NR_FDD_FR1_G |  |  |  | -114+TT |
|  | NR_FDD_FR1_H |  |  |  | -113.5+TT |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65+TT | -117+TT |
|  | NR_FDD_FR1_B |  |  |  | -116.5+TT |
|  | NR_TDD_FR1_C |  |  |  | -116+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115+TT |
|  | NR_FDD_FR1_F |  |  |  | -114.5+TT |
|  | NR_FDD_FR1_G |  |  |  | -114+TT |
|  | NR_FDD_FR1_H |  |  |  | -113.5+TT |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65+TT | -114+TT |
|  | NR_FDD_FR1_B |  |  |  | -113.5+TT |
|  | NR_TDD_FR1_C |  |  |  | -114+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112+TT |
|  | NR_FDD_FR1_F |  |  |  | -111.5+TT |
|  | NR_FDD_FR1_G |  |  |  | -111+TT |
|  | NR_FDD_FR1_H |  |  |  | -110.5+TT |
|  |  | 1~3 | dB | 10+TT | -3+TT |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65+TT | -120+TT |
|  | NR_FDD_FR1_B |  |  |  | -119.5+TT |
|  | NR_TDD_FR1_C |  |  |  | -119+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118+TT |
|  | NR_FDD_FR1_F |  |  |  | -117.5+TT |
|  | NR_FDD_FR1_G |  |  |  | -117+TT |
|  | NR_FDD_FR1_H |  |  |  | -116.5+TT |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65+TT | -117+TT |
|  | NR_FDD_FR1_B |  |  |  | -116.5+TT |
|  | NR_TDD_FR1_C |  |  |  | -116+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115+TT |
|  | NR_FDD_FR1_F |  |  |  | -114.5+TT |
|  | NR_FDD_FR1_G |  |  |  | -114+TT |
|  | NR_FDD_FR1_H |  |  |  | -113.5+TT |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28+TT | -87.28+TT |
|  | NR_FDD_FR1_B |  |  |  | -86.78+TT |
|  | NR_TDD_FR1_C |  |  |  | -86.28+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28+TT |
|  | NR_FDD_FR1_F |  |  |  | -84.78+TT |
|  | NR_FDD_FR1_G |  |  |  | -84.28+TT |
|  | NR_FDD_FR1_H |  |  |  | -83.78+TT |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19+TT | -81.19+TT |
|  | NR_FDD_FR1_B |  |  |  | -80.69+TT |
|  | NR_TDD_FR1_C |  |  |  | -80.19+TT |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69+TT |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19+TT |
|  | NR_FDD_FR1_F |  |  |  | -78.69+TT |
|  | NR_FDD_FR1_G |  |  |  | -78.19+TT |
|  | NR_FDD_FR1_H |  |  |  | -77.69+TT |
|  |  | 1~3 | dB | 10+TT | -3+TT |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table 6.7.9.3.5-2: L1-SINR absolute accuracy requirements for the reported values for test configurations 1 and 2

TBD

Table 6.7.9.3.5-3: L1-SINR absolute accuracy requirements for the reported values for test configurations 3

TBD

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

## << End of changes >>