3GPP TSG RAN Meeting #93-e	                   R5-217329r1

Electronic Meeting, 8th – 19th November 2021

Agenda item:	5.3.2.17

Source:	Huawei, HiSilicon

Title: 	Discussion on statistical testing

Document for:	DISCUSSION AND ENDORSEMENT

1	Introduction

The statistical testing with early pass/fail concept was adopted by RAN5 for quite a long time through different technologies. The early pass/fail limit is documented in Annex H of TS 38.521-1 for NR RF testing. However the theory of how to derive the pass/fail limit is still missing, which makes the information not complete. This document proposes the way of adding additional sub clause to capture the theory.

In addition, when revisiting the derivation theory, some errors are found regarding the existing pass/fail limit. This document also proposes some corrections.

2	Discussion

When a test requires counting the number of successful/failing samples, such as throughput measurement, BLER measurement or successful cell selection measurement, the statistical testing concept could be adopted. Statistical testing here means the outcome of the test is a decision, which could be correct (e.g. a DUT with throughput >95% is decided as pass) or in-correct with certain confidence level. The confidence level (CL) describes the probability that the decision is correct.

For a ‘standard’ test, a fixed number of samples are run, and the decision is made by comparing the number of failing samples with a threshold. The threshold could be deviated from the specified target failing rate, e.g. 5% BLER, to reach expected confidence level. Long testing time might be needed to reach a relatively high confidence level.

The early pass/fail concept was used to reduce the test time. The decision could be made every time when a failing sample is observed. If the number of total samples are higher than early pass limit (nsp), the UE could be decided as pass; if the number of total samples are lower than early fail limit (nsf), the UE could be decided as fail. The early pass/fail limit are selected so that the overall confidence level could be reached, e.g. 95%.

There are various ways of deriving the early pass/fail limit. In UTRA, it’s derived using the poisson/Chi-square distribution. In LTE, it’s derived using the inverse binomial distribution. The LTE method is reused in NR specification TS 38.521-1 and TS 38.141-1

The detailed description of the theory is captured in TS 36.521-1 and TS 38.141-1, but still missing in TS 38.521-1. Comparing the theory description in TS 36.521-1 and TS 38.141-1, they produce the early pass/fail limit with exactly the same equations, but the way of description in TS 38.141-1 is more concise. It's more focused on the statistical concept and equations, and is easier to understand. It’s proposed to introduce the theory description into TS 38.521-1 following the similar way of TS 38.141-1.

Proposal 1:	It’s proposed to introduce the theory description into TS 38.521-1 following the similar way of TS 38.141-1.

When looking at the equations of deriving early pass/fail limit, some problems could be observed.

In clause G.7.10 of TS 36.521-1, it’s indicated that the pass/fail limit is derived by below equations with df=0.004 and clp=0.9975.

However by running simulation using different combination of {df, clp}, and comparing the result with nsp/nsf values in TS 36.521-1/38.521-1, it could be seen that:

Observation 1: 	The parameter set {df=0.004, clp=0.9975} doesn’t match existing early pass/fail limit numbers in TS 36.521-1 and TS 38.521-1.

Observation 2:	When clp=0.9945, the simulated nsp values are the closest to that in TS 36.521-1, while there is no clp value resulting the exact same values. In addition, the values in TS 36.521-1 are shifted up by one step.

Observation 3:	When df=0.0044, the simulated nsf values are the closest to that in TS 36.521-1, while there is no df value resulting the exact same values.

Table 1 Comparison between TS 36.521-1 and simulations

|  | 36.521-1 |  | df=0.004 clp=0.9975 |  | df=0.0043 clp=0.9945 |  | df=0.0044 clp=0.9945 |  | df=0.0045 clp=0.9945 |  | df=0.0001 clp=0.9945 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ne | nsp | nsf | nsp | nsf | nsp | nsf | nsp | nsf | nsp | nsf | nsp | nsf |
| 0 | 67 | N/A |  |  |  |  |  |  |  |  |  |  |
| 1 | 95 | N/A | 77 | 1 | 67 | 1 | 67 | 1 | 67 | 1 | 67 | 1 |
| 2 | 119 | 2 | 106 | 3 | 95 | 3 | 95 | 3 | 95 | 3 | 95 | 2 |
| 3 | 141 | 7 | 131 | 8 | 119 | 8 | 119 | 8 | 119 | 8 | 119 | 3 |
| 4 | 162 | 14 | 154 | 14 | 141 | 15 | 141 | 15 | 141 | 15 | 141 | 7 |
| 5 | 183 | 22 | 176 | 22 | 162 | 23 | 162 | 23 | 162 | 23 | 162 | 11 |
| 6 | 202 | 32 | 197 | 32 | 183 | 32 | 183 | 32 | 183 | 32 | 183 | 17 |
| 7 | 222 | 42 | 218 | 42 | 203 | 42 | 203 | 42 | 203 | 43 | 203 | 24 |
| 8 | 241 | 53 | 238 | 52 | 222 | 53 | 222 | 53 | 222 | 53 | 222 | 31 |
| 9 | 259 | 64 | 257 | 64 | 241 | 64 | 241 | 64 | 241 | 65 | 241 | 39 |
| 10 | 278 | 76 | 277 | 75 | 259 | 76 | 259 | 76 | 259 | 76 | 259 | 48 |
| 11 | 296 | 88 | 295 | 87 | 278 | 88 | 278 | 88 | 278 | 89 | 278 | 57 |
| 12 | 314 | 100 | 314 | 100 | 296 | 101 | 296 | 101 | 296 | 101 | 296 | 67 |
| 13 | 332 | 113 | 333 | 112 | 314 | 113 | 314 | 114 | 314 | 114 | 314 | 77 |
| 14 | 349 | 126 | 351 | 125 | 332 | 126 | 332 | 127 | 332 | 127 | 332 | 87 |
| 15 | 367 | 140 | 369 | 139 | 349 | 140 | 349 | 140 | 349 | 140 | 349 | 98 |
| 16 | 384 | 153 | 387 | 152 | 367 | 153 | 367 | 154 | 367 | 154 | 367 | 109 |
| 17 | 401 | 167 | 405 | 166 | 384 | 167 | 384 | 167 | 384 | 168 | 384 | 120 |
| 18 | 418 | 181 | 422 | 180 | 401 | 181 | 401 | 181 | 401 | 182 | 401 | 132 |
| 19 | 435 | 195 | 440 | 194 | 419 | 195 | 419 | 195 | 419 | 196 | 419 | 143 |
| 20 | 452 | 209 | 457 | 208 | 436 | 209 | 436 | 210 | 436 | 210 | 436 | 155 |
| 21 | 469 | 224 | 474 | 222 | 453 | 223 | 453 | 224 | 453 | 224 | 453 | 167 |
| 22 | 486 | 238 | 492 | 237 | 469 | 238 | 469 | 238 | 469 | 239 | 469 | 180 |
| 23 | 503 | 253 | 509 | 251 | 486 | 253 | 486 | 253 | 486 | 254 | 486 | 192 |
| 24 | 519 | 268 | 526 | 266 | 503 | 267 | 503 | 268 | 503 | 268 | 503 | 205 |
| 25 | 536 | 283 | 543 | 281 | 520 | 282 | 520 | 283 | 520 | 283 | 520 | 217 |
| 26 | 552 | 298 | 560 | 295 | 536 | 297 | 536 | 298 | 536 | 298 | 536 | 230 |
| 27 | 569 | 313 | 577 | 310 | 553 | 312 | 553 | 313 | 553 | 313 | 553 | 243 |
| 28 | 585 | 328 | 593 | 325 | 569 | 327 | 569 | 328 | 569 | 328 | 569 | 257 |
| 29 | 602 | 343 | 610 | 341 | 585 | 342 | 585 | 343 | 585 | 344 | 585 | 270 |
| 30 | 618 | 359 | 627 | 356 | 602 | 358 | 602 | 358 | 602 | 359 | 602 | 283 |
| 31 | 634 | 374 | 643 | 371 | 618 | 373 | 618 | 374 | 618 | 374 | 618 | 297 |
| 32 | 650 | 389 | 660 | 387 | 634 | 389 | 634 | 389 | 634 | 390 | 634 | 310 |
| 33 | 667 | 405 | 676 | 402 | 651 | 404 | 651 | 405 | 651 | 405 | 651 | 324 |
| 34 | 683 | 421 | 693 | 418 | 667 | 420 | 667 | 420 | 667 | 421 | 667 | 338 |
| 35 | 699 | 436 | 709 | 433 | 683 | 435 | 683 | 436 | 683 | 437 | 683 | 352 |
| 36 | 715 | 452 | 725 | 449 | 699 | 451 | 699 | 452 | 699 | 452 | 699 | 366 |
| 37 | 731 | 468 | 742 | 465 | 715 | 467 | 715 | 467 | 715 | 468 | 715 | 380 |
| NOTE: the numbers in red mark the difference between values in TS 36.521-1 and simulation results. |  |  |  |  |  |  |  |  |  |  |  |  |

In addition, as per the description in clause G.7.10 in TS 36.521-1, the values are selected by simulation with a large population of DUTs, to achieve the overall confidence level 95%.

The simulation works as follows:

-	A large population of limit DUTs with true ER = 0.05 is decided against the pass and fail limits.

-	clp    and df   are tuned such that CL (95%) of the population passes and D (5%) of the population fails.

-	A population of Bad DUTs with true ER = M*0.05 is decided against the same pass and fail limits.

-	clp    and df   are tuned such that CL (95%) of the population fails and D (5%) of the population passes.

-	This procedure and the relationship to the measurement is justified in clause G.x.9. The number of DUTs decrease during the simulation, as the decided DUTs leave the population. That number decreases with an approximately exponential characteristics. After 169 bad results all DUTs of the population are decided.

By running simulation with 10000 DUTs, the corresponding pass/fail probability could be summarized as below Table 2. The simulation was run three times for each set of parameters.

Observation 4: When nsf/nsp are calculated with variants set {df=0.0044, clp=0.9945}, the Confidence Level could reach ~95% for both good DUT and bad DUT.

Table 2	Confidence level of different nsf/nsp values

|  | 10000 DUTs with 5% BLER |  | 10000 DUTs with M*5% BLER |  |
| --- | --- | --- | --- | --- |
|  | P_false_fail | P_pass | P_fail | P_false_pass |
| nsf/nsp in TS 36.521-1 | 5.8% | 94.2% | 96.8% | 3.2% |
|  | 5.2% | 94.8% | 96.4% | 3.6% |
|  | 4.9% | 95.1% | 96.9% | 3.1% |
| df=0.004 clp=0.9975 | 5.3% | 94.7% | 97.2% | 2.8% |
|  | 5.6% | 94.5% | 97.3% | 2.7% |
|  | 5.0% | 95.0% | 97.4% | 2.6% |
| df=0.0044 clp=0.9945 | 5.5% | 94.5% | 94.8% | 5.2% |
|  | 5.6% | 94.4% | 94.8% | 5.2% |
|  | 6.0% | 94.1% | 95.3% | 4.7% |

Proposal 2:	In order to minimize the impact on existing numbers of early pass/fail limit, the parameter sets are updated to {df=0.0044, clp=0.9945}..

The theory description of statistical testing is also missing in TS 38.521-2 and TS 38.521-3. Similar change is needed in both specifications.

Proposal 3:	Theory description of statistical testing should be added to TS 38.521-2 and TS 38.521-3.

In 38.521-4, most test cases are defined in fading condition therefore early pass/fail limit is not adopted. Instead a fixed pass/fail limit for 30% and 70% throughput was derived using the same equations as previously stated. In addition, the test time was determined by the longer one of fixed pass/fail limit and the minimum testing time.

G.1.4	Pass Fail limit

Testing Throughput = 30 %, then the test limit is

Number of successes (ACK) / number of samples ≥ 59 / 233

Testing Throughput = 70 % then the test limit is

Number of fails (NACK and statDTX) / number of samples ≤ 66 / 184

…

G.1.5	Minimum Test time

Editor's Note: Simulation method to derive minimum test time for FR2 needs to be evaluated.

If a pass fail decision in clause G.1.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

While since Rel-16, more test cases are added with

- 1% BLER target under fading scenario, e.g. 5.2.2.1.7 and 5.2.2.1.14

- 0.0001% BLER target under AWGN scenario, e.g. 5.2.2.1.6

For test case with 1% BLER target under fading scenario, fixed pass/fail limit should be calculated in the same way of 30% and 70% throughput.

For test case with 0.0001% BLER target under AWGN scenario, early pass/fail limit should be considered to optimize the testing time.

Proposal 4:	In TS 38.521-4, for test case with 1% BLER target under fading scenario, fixed pass/fail limit should be calculated in the same way of 30% and 70% throughput; for test case with 0.0001% BLER target under AWGN scenario, early pass/fail limit should be considered to optimize the testing time.

In TS 38.533, early pass/fail limit was specified for 90% success ratio. However the derivation theory simply refers to TS 36.521-1 without capturing corresponding parameter set {df, clp}. In order to improve the integrity of specification, Annex G.X should be updated to add the parameter set values.

Proposal 5:	In TS 38.533, Annex G.X should be updated to add values of parameter set {df, clp} which are used for derivation of the early pass/fail limit.

Since the pass/fail limit in TS 36.521-1 and TS 36.521-3 has been used for a long time for LTE, in order to keep a stable certification environment, it’s preferred to keep current values.

Proposal 6:	It’s proposed to keep current pass/fail limits unchanged in TS 36.521-1 and TS 36.521-3.

3	Conclusions

In this contribution, the following proposals were made:

Proposal 1:	It’s proposed to introduce the theory description into TS 38.521-1 following the similar way of TS 38.141-1.

Proposal 2:	In order to minimize the impact on existing numbers of early pass/fail limit, the parameter sets are updated to {df=0.0044, clp=0.9945}.

Proposal 3:	Theory description of statistical testing should be added to TS 38.521-2 and TS 38.521-3.

Proposal 4:	In TS 38.521-4, for test case with 1% BLER target under fading scenario, fixed pass/fail limit should be calculated in the same way of 30% and 70% throughput; for test case with 0.0001% BLER target under AWGN scenario, early pass/fail limit should be considered to optimize the testing time.

Proposal 5:	In TS 38.533, Annex G.X should be updated to add values of parameter set {df, clp} which are used for derivation of the early pass/fail limit.

Proposal 6:	It’s proposed to keep current pass/fail limits unchanged in TS 36.521-1 and TS 36.521-3.

Observation 1: 	The parameter set {df=0.004, clp=0.9975} doesn’t match existing early pass/fail limit numbers in TS 36.521-1 and TS 38.521-1.

Observation 2:	When clp=0.9945, the simulated nsp values are the closest to that in TS 36.521-1, while there is no clp value resulting the exact same values. In addition, the values in TS 36.521-1 are shifted up by one step.

Observation 3:	When df=0.0044, the simulated nsf values are the closest to that in TS 36.521-1, while there is no df value resulting the exact same values.

Observation 4: When nsf/nsp are calculated with variants set {df=0.0044, clp=0.9945}, the Confidence Level could reach 95% for both good DUT and bad DUT.

4	References

[1]	T1R010141, BERBLER theory, Rohde & Schwarz, T1#19

[2]	R5-082164, Statistical testing of receiver characteristics, Rohde & Schwarz, RAN5#39bis

[3]	R4-2015094, On NR Rel-16 ultra low BLER BS demodulation requirements, Nokia, RAN4#97-e.

[4]	R4-2017504, CR for 38.141-1: URLLC testing methodology appendix, Nokia, Nokia Shanghai Bell, Intel, Ericsson, Huawei, HiSilicon, RAN4#97-e.