3GPP TSG-RAN5 Meeting #93-e	R5-217414r1

# Online, 8th Nov – 19th Nov 2021

Title:							MU and TT proposal for FR2 EVM

Source:			Anritsu

Agenda Item:			5.3.1.17

Document for:			Endorsement

# 1.	Introduction

This paper provides assumptions and MU value of FR2 EVM test case for PUSCH, PC3.

# 2.	Discussion

# 2.1	TE noise floor assumption

The testable lowest power level has been discussed in RAN5, and it can be used for TE noise floor assumption. In Minimum output power measurement, it is agreed to measure -4.6 dBm/400 MHz (= core requirement -13 + relaxation 8.4) for FR2a and +0.5 dBm/400 MHz (= core requirement -13 + relaxation 13.5) for FR2b with SNR = 6 dB [1]. Therefore, the highest sensitivity TE noise floor is -10.6 dBm/400 MHz for FR2a and -5.5 dBm/400 MHz for FR2b based on the past RAN5 discussion. In ACLR test case, it is agreed to use -7.6 dBm/400 MHz for FR2a and -5.5 dBm/400 MHz. For clarity, the following table shows the values for other channel bandwidths.

Table 1 The highest sensitivity TE noise floor [dBm] based on the past RAN5 discussion

|  | CBW = 50MHz | CBW = 100MHz | CBW = 200MHz | CBW = 400MHz |
| --- | --- | --- | --- | --- |
| FR2a | -16.6 | -13.6 | -10.6 | -7.6 |
| FR2b | -14.5 | -11.5 | -8.5 | -5.5 |

Observation 1 : Based on the past RAN5 discussion, the highest sensitivity TE noise floor is -7.6 dBm/400 MHz for FR2a and -5.5 dBm/400 MHz for FR2b.

For consistency with other test cases and past discussion, we propose to use -7.6 dBm/400 MHz for FR2a and -5.5 dBm/400 MHz for FR2b for TE noise floor assumption in EVM test case.

Proposal 1 : Use -7.6 dBm/400 MHz for FR2a and -5.5 dBm/400 MHz for FR2b in TE noise floor assumption of FR2 EVM test case.

# 2.2	UE output power assumption and available residual EVM with MPR

In FR2 EVM test case, UE output power is set by continuously sending uplink power control “up” commands. Therefore, the lowest possible power level is calculated by MinPeakEIRP – (MBR + MPR + T(MPR)) – 3 dB, where 3dB is reduction by polarization mismatch in the worst case. Table 2 shows the values of MPR + T(MPR) in each modulation type according to clause 6.2.2 in TS 38.521-2. Note that ‘Outer_Full’ is used to assume larger MPR.

Table 2 MPR value [dB] in each modulation type

| Modulation | CBW ≤ 200MHz |  |  | CBW = 400MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MPR | T(MPR) | MPR + T(MPR) | MPR | T(MPR) | MPR + T(MPR) |
| DFT-s-OFDM PI/2 BPSK | 2.0 | 1.5 | 3.5 | 3.0 | 2.0 | 5.0 |
| DFT-s-OFDM QPSK | 2.0 | 1.5 | 3.5 | 3.0 | 2.0 | 5.0 |
| DFT-s-OFDM 16 QAM | 3.5 | 3.0 | 6.5 | 4.5 | 4.0 | 8.5 |
| DFT-s-OFDM 64 QAM | 5.5 | 5.0 | 10.5 | 6.5 | 5.0 | 11.5 |
| CP-OFDM QPSK | 4.0 | 3.0 | 7.0 | 5.0 | 4.0 | 9.0 |
| CP-OFDM 16 QAM | 5.0 | 4.0 | 9.0 | 6.5 | 5.0 | 11.5 |
| CP-OFDM 64 QAM | 7.5 | 5.0 | 12.5 | 9.0 | 5.0 | 14.0 |

The UE output power in each CBW and each frequency sub-range is obtained as follows by using value of MPR + T(MPR) in Table 2, MinPeakEIRP = 22.4 dBm for FR2a and 20.6 dBm for FR2b, MBR = 0.75 dB.

Table 3 UE output power [dBm] with MPR

| Modulation | FR2a |  | FR2b |  |
| --- | --- | --- | --- | --- |
|  | CBW ≤ 200MHz | CBW = 400MHz | CBW ≤ 200MHz | CBW = 400MHz |
| DFT-s-OFDM PI/2 BPSK | 15.15 | 13.65 | 13.35 | 11.85 |
| DFT-s-OFDM QPSK | 15.15 | 13.65 | 13.35 | 11.85 |
| DFT-s-OFDM 16 QAM | 12.15 | 10.15 | 10.35 | 8.35 |
| DFT-s-OFDM 64 QAM | 8.15 | 7.15 | 6.35 | 5.35 |
| CP-OFDM QPSK | 11.65 | 9.65 | 9.85 | 7.85 |
| CP-OFDM 16 QAM | 9.65 | 7.15 | 7.85 | 5.35 |
| CP-OFDM 64 QAM | 6.15 | 4.65 | 4.35 | 2.85 |

From Table 1 and Table 3, available SNR and residual EVM produced by SNR are calculated as follows. Cells are colored by relationship between the residual EVM and the core requirement; Green indicates the residual EVM is less than the half of core requirement, and orange indicates the residual EVM is larger than the half of core requirement, and red indicates the residual EVM is larger than the core requirement.

Table 4 Available SNR [dB] and residual EVM [%] with MPR in FR2a

| Modulation | CBW = 50MHz |  | CBW = 100MHz |  | CBW = 200MHz |  | CBW = 400MHz |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR | EVM | SNR | EVM | SNR | EVM | SNR | EVM |  |
| DFT-s-OFDM PI/2 BPSK | 31.75 | 2.6 % | 28.75 | 3.7 % | 25.75 | 5.2 % | 21.25 | 8.7 % | 30.0 % |
| DFT-s-OFDM QPSK | 31.75 | 2.6 % | 28.75 | 3.7 % | 25.75 | 5.2 % | 21.25 | 8.7 % | 17.5 % |
| DFT-s-OFDM 16 QAM | 28.75 | 3.7 % | 25.75 | 5.2 % | 22.75 | 7.3 % | 17.75 | 13.0 % | 12.5 % |
| DFT-s-OFDM 64 QAM | 24.75 | 5.8 % | 21.75 | 8.2 % | 18.75 | 11.6 % | 14.75 | 18.4 % | 8.0 % |
| CP-OFDM QPSK | 28.25 | 3.9 % | 25.25 | 5.5 % | 22.25 | 7.8 % | 17.25 | 13.8 % | 17.5 % |
| CP-OFDM 16 QAM | 26.25 | 4.9 % | 23.25 | 6.9 % | 20.25 | 9.8 % | 14.75 | 18.4 % | 12.5 % |
| CP-OFDM 64 QAM | 22.75 | 7.3 % | 19.75 | 10.4 % | 16.75 | 14.6 % | 12.25 | 24.5 % | 8.0 % |

Table 5 Available SNR [dB] and residual EVM [%] with MPR in FR2b

| Modulation | CBW = 50MHz |  | CBW = 100MHz |  | CBW = 200MHz |  | CBW = 400MHz |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR | EVM | SNR | EVM | SNR | EVM | SNR | EVM |  |
| DFT-s-OFDM PI/2 BPSK | 27.85 | 4.1 % | 24.85 | 5.8 % | 21.85 | 8.1 % | 17.35 | 13.6 % | 30.0 % |
| DFT-s-OFDM QPSK | 27.85 | 4.1 % | 24.85 | 5.8 % | 21.85 | 8.1 % | 17.35 | 13.6 % | 17.5 % |
| DFT-s-OFDM 16 QAM | 24.85 | 5.8 % | 21.85 | 8.1 % | 18.85 | 11.5 % | 13.85 | 20.4 % | 12.5 % |
| DFT-s-OFDM 64 QAM | 20.85 | 9.1 % | 17.85 | 12.9 % | 14.85 | 18.2 % | 10.85 | 28.8 % | 8.0 % |
| CP-OFDM QPSK | 24.35 | 6.1 % | 21.35 | 8.6 % | 18.35 | 12.2 % | 13.35 | 21.6 % | 17.5 % |
| CP-OFDM 16 QAM | 22.35 | 7.7 % | 19.35 | 10.8 % | 16.35 | 15.3 % | 10.85 | 28.8 % | 12.5 % |
| CP-OFDM 64 QAM | 18.85 | 11.5 % | 15.85 | 16.2 % | 12.85 | 22.9 % | 8.35 | 38.3 % | 8.0 % |

As shown above, there are many test points with testability issue, if we use UE output power with MPR to assume the worst case and TE noise floor based on the past RAN5 discussion. Especially 64 QAM cannot be measured with residual EVM less than the half of core requirement at any test points. Note that the above residual EVM is based on only available SNR, and the actual residual EVM can be larger by considering TE’s uncertainty.

Observation 2 : If we use UE output power with MPR to assume the worst case and TE noise floor based on the past RAN5 discussion, there are many test points with testability issue as shown in Table 4 and Table 5.

# 2.3	UE output power assumption and available residual EVM without MPR

In order to improve the testability issue, we propose to not use MPR for UE output power in EVM test case. The value of MPR and T(MPR) becomes more than 10 dB in 64 QAM and decreases UE output power, so removing MPR will lead an improvement of available SNR and EVM. Furthermore, higher output power is generally hard for UE and leads EVM test stricter.

Proposal 2 : Do not use MPR for UE output power in EVM test case.

If MPR is removed from the test case, the lowest possible power level is calculated as 18.65 dBm for FR2a and 16.85 dBm for FR2b by MinPeakEIRP – MBR – 3 dB. From Table 1, available SNR and residual EVM produced by SNR are calculated as follows.

Table 6 Available SNR [dB] and residual EVM [%] without MPR in FR2a

| Modulation | CBW = 50MHz |  | CBW = 100MHz |  | CBW = 200MHz |  | CBW = 400MHz |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR | EVM | SNR | EVM | SNR | EVM | SNR | EVM |  |
| DFT-s-OFDM PI/2 BPSK |  |  |  |  |  |  |  |  | 30.0 % |
| DFT-s-OFDM QPSK |  |  |  |  |  |  |  |  | 17.5 % |
| DFT-s-OFDM 16 QAM |  |  |  |  |  |  |  |  | 12.5 % |
| DFT-s-OFDM 64 QAM | 35.25 | 1.8 % | 32.25 | 2.5 % | 29.25 | 3.5 % | 26.25 | 4.9 % | 8.0 % |
| CP-OFDM QPSK |  |  |  |  |  |  |  |  | 17.5 % |
| CP-OFDM 16 QAM |  |  |  |  |  |  |  |  | 12.5 % |
| CP-OFDM 64 QAM |  |  |  |  |  |  |  |  | 8.0 % |

Table 7 Available SNR [dB] and residual EVM [%] without MPR in FR2b

| Modulation | CBW = 50MHz |  | CBW = 100MHz |  | CBW = 200MHz |  | CBW = 400MHz |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SNR | EVM | SNR | EVM | SNR | EVM | SNR | EVM |  |
| DFT-s-OFDM PI/2 BPSK |  |  |  |  |  |  |  |  | 30.0 % |
| DFT-s-OFDM QPSK |  |  |  |  |  |  |  |  | 17.5 % |
| DFT-s-OFDM 16 QAM |  |  |  |  |  |  |  |  | 12.5 % |
| DFT-s-OFDM 64 QAM | 31.35 | 2.8 % | 28.35 | 3.9 % | 25.35 | 5.5 % | 22.35 | 7.7 % | 8.0 % |
| CP-OFDM QPSK |  |  |  |  |  |  |  |  | 17.5 % |
| CP-OFDM 16 QAM |  |  |  |  |  |  |  |  | 12.5 % |
| CP-OFDM 64 QAM |  |  |  |  |  |  |  |  | 8.0 % |

As shown above, the residual EVM is less than the half of the core requirement at all test points other than 64 QAM and CBW ≥ 200 MHz, if we remove MPR from the test case.

Observation 3 : If we remove MPR from EVM test case, most of testability issues can be resolved as shown in Table 6 and Table 7.

# 2.4	MU proposal

The actual residual EVM is obtained by considering TE’s uncertainty in addition to the values in Table 6 and Table 7. We propose the following values for EVM MU for FR2a and FR2b.

Table 8 EVM MU for FR2a and FR2b

| CBW = 50MHz, 100 MHz | CBW = 200MHz | CBW = 400MHz |
| --- | --- | --- |
| 3.9 % | 5.6 % | 7.9 % |

Proposal 3 : Adopt the values in Table 8 to EVM MU for FR2a and FR2b, PUSCH, PC3, if the Proposal 2 is endorsed.

The proposed EVM MU value is more than the half of the core requirement at three test points, 16QAM and CBW = 400 MHz, 64QAM and CBW = 200 MHz, 64 QAM and CBW = 400 MHz. We propose to apply the following TT for such test points to ensure that a compliant UE does not fail the test by the large EVM MU. TT value is calculated by the difference between the core requirement and measured EVM of marginal UE.

- TT = 2.3 % for 16QAM, CBW = 400 MHz

- TT = 1.8 % for 64QAM, CBW = 200 MHz

- TT = 3.2 % for 64QAM, CBW = 400 MHz

Proposal 4 : Adopt the above values to EVM TT for FR2a and FR2b, PUSCH, PC3, if the Proposal 2 is endorsed.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Proposal 1 : Use -7.6 dBm/400 MHz for FR2a and -5.5 dBm/400 MHz for FR2b in TE noise floor assumption of FR2 EVM test case.

Proposal 2 : Do not use MPR for UE output power in EVM test case.

Proposal 3 : Adopt the values in Table 8 to EVM MU for FR2a and FR2b, PUSCH, PC3, if the Proposal 2 is endorsed.

Proposal 4 : Adopt the above values to EVM TT for FR2a and FR2b, PUSCH, PC3, if the Proposal 2 is endorsed.

# 4.	References

[1] R5-204740, “On FR2 Minimum Output Power relaxation and measurement uncertainty”, Keysight, RAN5#88-e