3GPP TSG-RAN5 Meeting #93-e	R5-217488r1

# Electronic Meeting, 8th – 19th Nov. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1449 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating 6.2A.2 MPR for CA test case for intra-band UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test configuration and requirement in 6.2A.2 for Intra-band contiguous with non-contiguous RB is absent. Test case 6.2A.2 need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing testing configuration and test requirement for Intra-band contiguous CA with non-contiguous RB allocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 6.2A.2 is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: 1. Undo changes after Table 6.2A.2.0.4-1 and in 6.2A.2.1.5-1A. 2. Undo the revision of titles of Table 6.2A.2.1.5-1a and Table 6.2A.2.1.5-1b 3. Adding NOTE 3 in Table 6.2A.2.1.5-1c and Table 6.2A.2.1.5-1d |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2A.2	UE maximum output power reduction for CA

6.2A.2.0	Minimum conformance requirements

6.2A.2.0.1	FFS

6.2A.2.0.2	FFS

6.2A.2.0.3	Maximum Power Reduction for Inter-band CA

For inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.2.3 apply for each uplink component carrier.

The normative reference for this requirement is TS38.101-1[2] clause 6.2A.2.

6.2A.2.0.4	Maximum Power Reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.4-1 with contiguous RB allocation is specified in Table 6.2A.2.4-1 for UE power class 3 CA bandwidth classes B and C.

In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2A.2.0.4-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

For CA bandwidth class B and bandwidth class C with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a contiguous allocation is an inner allocation if

RBStart,Low ≤ RBStart_CA ≤ RBStart,High, and NRB_alloc ≤ ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2)), RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= LCRB1 ∙ 2^µ1 + LCRB2 ∙ 2^µ2, NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2.

If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2,

if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.0.4-1 with non-contiguous RB allocation is specified in Table 6.2A.2.0.4-2 for UE power class 3 CA bandwidth classes B and C.

Table 6.2A.2.0.4-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

For CA bandwidth classes B and C with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0 when all uplink CCs are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of CC1 and CC2 respectively.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc, RBStart_CA, BWgap and BWGB,low are as defined for the Inner region.

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither an non-contiguous Inner allocation nor an Outer 1 allocation.

The normative reference for this requirement is TS 38.101-1[2] clause 6.2A.2.4.

6.2A.2.1	UE maximum output power reduction for CA (2UL CA)

Editor’s Note:

Testing for Intra-band contiguous UL CA non-contiguous RB allocation is incomplete for Outer 1 allocation.

Testing for intra-band non-contiguous UL CA is FFS.

6.2A.2.1.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio for CA and the maximum power reduction (MPR) for Inter-band CA due to Cubic Metric (CM).

6.2A.2.1.2	Test applicability

The requirements of this test apply to all types of NR Power class 3 UE release 15 and forward that support 2 UL CA.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5A.2.4.1.1 is executed.

6.2A.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.2A.2.0.

6.2A.2.1.4	Test description

6.2A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR CA configuration specified in table 5.5A.3-1. All of these configurations shall be tested with applicable test parameters for each CA configuration of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2A.2.1.4.1-1 to table 6.2A.2.1.4.1-2b. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.2.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  |  | Modulation for all CCs (NOTE 2) | RB allocation (NOTE 1) |  |
|  |  |  |  | PCC | SCC |
| 13 | Default | N/A for Maximum Power Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full | Outer Full |
| 37 | Default |  | NOTE 5 | Inner Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 5: The modulation is DFT-s-OFDM QPSK for PCC and CP-OFDM 256 QAM for SCC. |  |  |  |  |  |

Table 6.2A.2.1.4.1-2a: Intra-band contiguous CA Test Configuration Table (contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A | DFT-s-OFDM |  | Pi/2 BPSK | Inner Full |
| 2 |  |  |  | Pi/2 BPSK | Outer Full |
| 3 |  |  |  | QPSK | Inner Full |
| 4 |  |  |  | QPSK | Outer Full |
| 5 |  |  |  | 16QAM | Inner Full |
| 6 |  |  |  | 16QAM | Outer Full |
| 7 |  |  |  | 64QAM | Inner Full |
| 8 |  |  |  | 64QAM | Outer Full |
| 9 |  |  |  | 256QAM | Inner Full |
| 10 |  |  |  | 256QAM | Outer Full |
| 11 |  | CP-OFDM |  | QPSK | Inner Full |
| 12 |  |  |  | QPSK | Outer Full |
| 13 |  |  |  | 16QAM | Inner Full |
| 14 |  |  |  | 16QAM | Outer Full |
| 15 |  |  |  | 64QAM | Inner Full |
| 16 |  |  |  | 64QAM | Outer Full |
| 17 |  |  |  | 256QAM | Inner Full |
| 18 |  |  |  | 256QAM | Outer Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 6.2A.2.1.4.1-2b: Intra-band contiguous CA Test Configuration Table (non-contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A | DFT-s-OFDM |  | Pi/2 BPSK | Inner |
| 2 |  |  |  | Pi/2 BPSK | Outer 1 |
| 3 |  |  |  | Pi/2 BPSK | Outer 2 |
| 4 |  |  |  | QPSK | Inner |
| 5 |  |  |  | QPSK | Outer 1 |
| 6 |  |  |  | QPSK | Outer 2 |
| 7 |  |  |  | 16QAM | Inner |
| 8 |  |  |  | 16QAM | Outer 1 |
| 9 |  |  |  | 16QAM | Outer 2 |
| 10 |  |  |  | 64QAM | Inner |
| 11 |  |  |  | 64QAM | Outer 1 |
| 12 |  |  |  | 64QAM | Outer 2 |
| 13 |  |  |  | 256QAM | Inner |
| 14 |  |  |  | 256QAM | Outer 1 |
| 15 |  |  |  | 256QAM | Outer 2 |
| 16 |  | CP-OFDM |  | QPSK | Inner |
| 17 |  |  |  | QPSK | Outer 1 |
| 18 |  |  |  | QPSK | Outer 2 |
| 19 |  |  |  | 16QAM | Inner |
| 20 |  |  |  | 16QAM | Outer 1 |
| 21 |  |  |  | 16QAM | Outer 2 |
| 22 |  |  |  | 64QAM | Inner |
| 23 |  |  |  | 64QAM | Outer 1 |
| 24 |  |  |  | 64QAM | Outer 2 |
| 25 |  |  |  | 256QAM | Inner |
| 26 |  |  |  | 256QAM | Outer 1 |
| 27 |  |  |  | 256QAM | Outer 2 |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1b. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2A.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.2.1.4.3.

6.2A.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level corresponding to Power Class 3.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.2A.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

Table 6.2A.2.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA Test IDs 1, 4-13, 16, 19, 23, 26, 27, 30, 37 |
|  | 17 |  | Power class 3 and Inter-band CA Test IDs 14, 15, 17, 18, 33 |
|  | 16 |  | Power class 3 and Inter-band CA Test IDs 2, 3, 22, 24, 25, 28, 29, 31, 32 |
|  | 14 |  | Power class 3 and Inter-band CA Test IDs 20, 21, 36 |
|  | 13 |  | Power class 3 and Inter-band CA Test IDs 34, 35 |

Table 6.2A.2.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n41C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 18 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |

Table 6.2A.2.1.4.3-2a: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n48B

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 19 |  | Power class 3 and Test IDs 1, 3, 5 |
|  | 18 |  | Power class 3 and Test IDs 5, 11 |
|  | 17 |  | Power class 3 and Test IDs 7, 13 |
|  | 16 |  | Power class 3 and Test IDs 2, 4, 6, 15 |
|  | 15 |  | Power class 3 and Test IDs 8, 12, 14, 16 |
|  | 12 |  | Power class 3 and Test IDs 9, 10 |
|  | 11 |  | Power class 3 and Test ID 17, 18 |

Table 6.2A.2.1.4.3-2b: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n77C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 17 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |

6.2A.2.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.2.1.5-1 to Table 6.2A.2.1.5-3.

Table 6.2A.2.1.5-1: UE Output Power for inter-band CA (2 UL CA) test requirements

| Test ID | PPowerClass (dBm) | MPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0.2 | 0.2 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 3 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 4 | 23 | 1.2 | 1.2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 6 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0.5 | 0.5 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 0.5 | 0.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 9 | 23 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 1 | 1.52 | 1.52 | 3 | 23 | 3 | 25+TT | 20-TT |
| 12 | 23 | 1 | 1 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 13 | 23 | 1 | 1 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 14 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 15 | 23 | 2 | 2 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 16 | 23 | 2 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 17 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 18 | 23 | 2.5 | 2.5 | 1.52 | 1.52 | 3 | 22.52 | 52 | 25+TT | 17.5-TT2 |
| 19 | 23 | 2.5 | 2.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 20 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 62 | 25+TT | 14-TT2 |
| 21 | 23 | 4.5 | 4.5 | 1.52 | 1.52 | 3 | 202 | 62 | 25+TT | 14-TT2 |
| 22 | 23 | 4.5 | 4.5 | 0 | 0 | 3 | 21.5 | 5 | 25+TT | 16.5-TT |
| 23 | 23 | 1.5 | 1.5 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 24 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 25 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 26 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 27 | 23 | 2 | 2 | 0 (1.52) | 0 (1.52) | 3 | 23 (22.52) | 3 (52) | 25+TT | 20-TT (17.5-TT2) |
| 28 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 29 | 23 | 3 | 3 | 1.52 | 1.52 | 3 | 21.52 | 52 | 25+TT | 16.5-TT2 |
| 30 | 23 | 3 | 3 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 31 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 32 | 23 | 3.5 | 3.5 | 1.52 | 1.52 | 3 | 212 | 52 | 25+TT | 16-TT2 |
| 33 | 23 | 3.5 | 3.5 | 0 | 0 | 3 | 22.5 | 5 2 | 25+TT | 17.5-TT |
| 34 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 35 | 23 | 6.5 | 6.5 | 1.52 | 1.52 | 3 | 182 | 52 | 25+TT | 13-TT2 |
| 36 | 23 | 6.5 | 6.5 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 6.5 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-2. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1a: UE Output Power for intra-band contiguous 2 UL CA (contiguous RB allocation) test requirements (for CA_n41C)

| Configuration ID | Power Class 3 (dBm) | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 2 | 2 | 2.5 | 25+TT | 20-TT | 25+TT | 18-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19.5-TT | 25+TT | 18-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 2 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 2 | 2.5 | 4 | 25+TT | 17.5-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 2 | 2 | 3.5 | 25+TT | 19-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 2 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 2 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |

Table 6.2A.2.1.5-1a: UE Output Power for intra-band contiguous 2 UL CA (contiguous RB allocation) test requirements (for CA_n48B, CA_n79C)

| Configuration ID | Power Class 3 (dBm) | MPR |  | TC | PCMAX_L |  | TL=2 | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 2 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 3 | 23 | 1 | 2.5 | 0 | 22 | 20.5 | 3 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 17.5-TT |
| 4 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 5 | 23 | 1.5 | 2.5 | 0 | 21.5 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18.5-TT | 25+TT | 17.5-TT |
| 6 | 23 | 3.5 | 7 | 0 | 19.5 | 16 | 3 | 3.5 | 5 | 25+TT | 16-TT | 25+TT | 11-TT |
| 7 | 23 | 3 | 5 | 0 | 20 | 18 | 3 | 2.5 | 4 | 25+TT | 17-TT | 25+TT | 14-TT |
| 8 | 23 | 4 | 7 | 0 | 19 | 16 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 11-TT |
| 9 | 23 | 5.5 | 7 | 0 | 17.5 | 16 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 11-TT |
| 10 | 23 | 6 | 7.5 | 0 | 17 | 15.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 10.5-TT |
| 11 | 23 | 2 | 3.5 | 0 | 21 | 19.5 | 3 | 2 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 12 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 13 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 14 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 15 | 23 | 3.5 | 5 | 0 | 19.5 | 18 | 3 | 3.5 | 4 | 25+TT | 16-TT | 25+TT | 14-TT |
| 16 | 23 | 4 | 8 | 0 | 19 | 15 | 3 | 3.5 | 5 | 25+TT | 15.5-TT | 25+TT | 10-TT |
| 17 | 23 | 6.5 | 7 | 0 | 16.5 | 16 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11-TT |
| 18 | 23 | 6.5 | 8 | 0 | 16.5 | 15 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 10-TT |

Table 6.2A.2.1.5-1b: UE Output Power for intra-band contiguous 2 UL CA (non-contiguous RB allocation) test requirements (for CA_n41C)

| Configuration ID | Power Class 3 (dBm) |  | MPR |  |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BW class B |  | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 |  | 2 |  | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 2 | 23 |  | 5.5 (3.5)1 |  | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 |  | 11.5 (7)2 |  | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 |  | 2 |  | 2.5 | 0 | 21 | 20.5 | 2 | 2 | 2.5 | 25+TT | 19-TT | 25+TT | 18-TT |
| 5 | 23 |  | 5.5 (3.5)1 |  | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 2 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 |  | 11.5 (7)2 |  | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 |  | 2.5 |  | 3 | 0 | 20.5 | 20 | 2 | 2.5 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 8 | 23 |  | 5.5 |  | 6 | 0 | 17.5 | 17 | 2 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 |  | 11.5 (7)2 |  | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 |  | 4.5 |  | 5 | 0 | 18.5 | 18 | 2 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 |  | 6 |  | 6 | 0 | 17 | 17 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 |  | 11.5 (7)2 |  | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 |  | 6 |  | 6.5 | 0 | 17 | 16.5 | 2 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 |  | 6.5 |  | 6.5 | 0 | 16.5 | 16.5 | 2 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 |  | 11.5 (7)2 |  | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 |  | 2.5 |  | 3.5 | 0 | 20.5 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 18-TT | 25+TT | 16-TT |
| 17 | 23 |  | 6.5 (4.5)1 |  | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 2 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 |  | 12 (7.5)2 |  | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 |  | 3 |  | 3.5 | 0 | 20 | 19.5 | 2 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 20 | 23 |  | 7 |  | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 |  | 12 (7.5)2 |  | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 |  | 5 |  | 5 | 0 | 18 | 18 | 2 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 |  | 7 |  | 7 | 0 | 16 | 16 | 2 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 |  | 12 (7.5)2 |  | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 |  | 7.5 |  | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 |  | 7.5 |  | 0 | 15.5 | 15.5 | 2 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 |  | 14 (9.5)2 |  | 0 | 11 (15.5)2 | 9 (13.5)2 | 2 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-1d: UE Output Power for intra-band contiguous 2 UL CA (non-contiguous RB allocation) test requirements (for CA_n48B, CA_n79C)

| Configuration ID | Power Class 3 (dBm) | MPR |  | TC | PCMAX_L |  | TL | T(PCMAX_L) |  | Bandwidth class B |  | Bandwidth class C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BW class B | BW class C |  | BW class B | BW class C |  | BW class B | BW class C | Upper limit (dBm) | Lower limit (dBm) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 2 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 3 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 4 | 23 | 2 | 2.5 | 0 | 21 | 20.5 | 3 | 2 | 2.5 | 25+TT | 18-TT | 25+TT | 17.5-TT |
| 5 | 23 | 5.5 (3.5)1 | 6 (4)1 | 0 | 17.5 (19.5)1 | 17 (19)1 | 3 | 5 (3.5)1 | 5 (3.5)1 | 25+TT | 12.5-TT (16-TT)1 | 25+TT | 12-TT (15.5-TT)1 |
| 6 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 7 | 23 | 2.5 | 3 | 0 | 20.5 | 20 | 3 | 2.5 | 2.5 | 25+TT | 17.5-TT | 25+TT | 17-TT |
| 8 | 23 | 5.5 | 6 | 0 | 17.5 | 17 | 3 | 5 | 5 | 25+TT | 12.5-TT | 25+TT | 12-TT |
| 9 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 10 | 23 | 4.5 | 5 | 0 | 18.5 | 18 | 3 | 4 | 4 | 25+TT | 14.5-TT | 25+TT | 14-TT |
| 11 | 23 | 6 | 6 | 0 | 17 | 17 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 12-TT |
| 12 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 13 | 23 | 6 | 6.5 | 0 | 17 | 16.5 | 3 | 5 | 5 | 25+TT | 12-TT | 25+TT | 11.5-TT |
| 14 | 23 | 6.5 | 6.5 | 0 | 16.5 | 16.5 | 3 | 5 | 5 | 25+TT | 11.5-TT | 25+TT | 11.5-TT |
| 15 | 23 | 11.5 (7)2 | 13 (8.5)2 | 0 | 11.5 (16)2 | 10 (14.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5.5-TT (11-TT)2 | 25+TT | 4-TT (9.5-TT)2 |
| 16 | 23 | 2.5 | 3.5 | 0 | 20.5 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17.5-TT | 25+TT | 16-TT |
| 17 | 23 | 6.5 (4.5)1 | 7 (5)1 | 0 | 16.5 (18.5)1 | 16 (18)1 | 3 | 5 (4)1 | 5 (4)1 | 25+TT | 11.5-TT (14.5-TT)1 | 25+TT | 11-TT (14-TT)1 |
| 18 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 19 | 23 | 3 | 3.5 | 0 | 20 | 19.5 | 3 | 2.5 | 3.5 | 25+TT | 17-TT | 25+TT | 16-TT |
| 20 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 21 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 22 | 23 | 5 | 5 | 0 | 18 | 18 | 3 | 4 | 4 | 25+TT | 14-TT | 25+TT | 14-TT |
| 23 | 23 | 7 | 7 | 0 | 16 | 16 | 3 | 5 | 5 | 25+TT | 11-TT | 25+TT | 11-TT |
| 24 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| 25 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 26 | 23 | 7.5 | 7.5 | 0 | 15.5 | 15.5 | 3 | 5 | 5 | 25+TT | 10.5-TT | 25+TT | 10.5-TT |
| 27 | 23 | 12 (7.5)2 | 14 (9.5)2 | 0 | 11 (15.5)2 | 9 (13.5)2 | 3 | 6 (5)2 | 6 (5)2 | 25+TT | 5-TT (10.5-TT)2 | 25+TT | 3-TT (8.5-TT)2 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.2.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.1.5-2: Test Tolerance for inter-band CA (Maximum Power Reduction (MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.2.1.5-3: Test Tolerance for intra-band CA (Maximum Power Reduction (MPR))

| TT for overall output power (dB) |  |  |
| --- | --- | --- |
| Aggregation BW | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |
| 100MHz < BW ≤ 200MHz | FFS | FFS |

For the UE which supports inter-band NR CA configuration, ΔTIB,c in Table 6.2A.4.0.2.3-1 applies. Unless otherwise stated, ΔTIB,c is set to zero.

<Unchanged Text Skipped>

<End of Changes>