Title:	Discussion on test point selection for NS_04 A-MPR in FR1

# Background

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

The purpose of this paper is to provide an analysis for the parameters in test configuration table and a selection of test points for test environment, test frequencies, channel bandwidths, SCS, RB allocation and modulation.

# Discussion

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] and for FR1. In general, for A-MPR the emissions minimum requirements are SEM and/or Spurious. The proposals below are basically the same as what have been proposed for NS_35 in [4]. However, some additional test frequencies are required for NS_04 A-MPR tests.

## Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

## Test Frequencies

The selection of the general test frequencies can be based on the agreed proposals for SEM [2], i.e. Low and High frequency.

PROPOSAL 2: Select Low range and High range as general test frequencies for NS_04 A-MPR in FR1.

## Additional Test Frequencies for NS_04

NS_04 can be signalled for a UE supporting band n41, where FUL_low = 2496 MHz and where FUL_high = 2690 MHz. The Maximum transmission bandwidth for NR is shown in figure 1. The additional frequencies are in addition to the test frequency Low.

Figure 1: Maximum transmission badwidth.

The following channel bandwidths (BWChannel) are supported for NR band n41:

Figure 2: Channel bandwiths for some NR bands.

The A -MPR requirement for NS_04 is shown in the figure 3 and 4  below.

Figure 3: A-MPR requirements for NS_04.

Figure 4: Parameters for the A-MPR NS_04.

The requirements for NS_04 are given in Figure 2 with certain parameters defined in Figure 3. The requirements are divided into 3 regions: IMD3 region, regrowth region and MPR (A-MPR = 0).

Figure 4: Parameters for the A-MPR.

Figure 4 shows an example of the division into regions for 10 MHz channel bandwidth, based on the equations in Figure 3. The additional frequencies separate the regions. For 10 MHz channel bandwidth, the additional frequencies are 2505 and 2511 MHz.

Table 1: Aditional test frequencies

| BWChannel (MHz) | IMD3 region (MHz) | Regrowth region (MHz) | MPR region (MHz) |
| --- | --- | --- | --- |
| 10 | FC < 2505 | 2505 ≤ FC < 2511 | 2511 ≤ FC |
| 15 | FC < 2512.5 | 2512.5 ≤ FC < 2513.5 | 2513.5 ≤ FC |
| 20 | FC < 2520 | IMD3 region overlaps Regrowth region | 2520 ≤ FC |
| 30 | FC < 2535 | IMD3 region overlaps Regrowth region | 2535 ≤ FC |
| 40 | FC < 2550 | IMD3 region overlaps Regrowth region | 2550 ≤ FC |
| 50 | FC < 2565 | IMD3 region overlaps Regrowth region | 2565 ≤ FC |
| 60 | FC < 2580 | IMD3 region overlaps Regrowth region | 2580 ≤ FC |
| 70 | FC < 2595 | IMD3 region overlaps Regrowth region | 2595 ≤ FC |
| 80 | FC < 2610 | IMD3 region overlaps Regrowth region | 2610 ≤ FC |
| 90 | FC < 2625 | IMD3 region overlaps Regrowth region | 2625 ≤ FC |
| 100 | FC < 2640 | IMD3 region overlaps Regrowth region | 2640 ≤ FC |
|  | PC3_A2, PC2_A4, PC1.5_A6 | PC3_A1. PC2_A3, PC1.5_A5 | (A-MPR=0) |

Table 1 lists the additional frequencies for different channel bandwidths and regions. For channel bandwidths greater than 15 MHz, the IMD3 region overlaps the Regrowth region. All of these additional test frequencies fit the channel raester with the exception of 2513.5 MHz for 15 MHz channel bandwidth. Then select for SCS = 15 kHz 2513.505 MHz and for, SCS = 30 kHz, SCS = 60 kHz 2513.52 MHz as additional test frequencies.

Figure 6: NS_04 A-MPR requirements for different modulations and power classes (PC).

Table 2: RB allocation and test frequencies

|  | IMD3 region (MHz) | Regrowth region (MHz) | MPR region (MHz) |
| --- | --- | --- | --- |
| Proposed RB allocation | Edge_1RB_Left | Edge_1RB_Left | Edge_1RB_Left |
| Test frequency | Low (see TS 38.805-1) | Additional test frequency (see Table 1) | Additional test frequency (see Table 1) |
| Requirement | PC3_A2, PC2_A4, PC1.5_A6 | PC3_A1. PC2_A3, PC1.5_A5 | (A-MPR=0) |

Table 2 shows the proposed test frequencies and RB allocations. Like all other A-MPR test cases, the starting point is to test Low and High frequencies with Edge_1RB allocation and Outer_Full. The same principle shall also apply to the additional frequencies.

PROPOSAL 3: Select additional frequencies (Fc) according to Table 1 and test the A-MPR requirements with both Edge_1RB and Outer_Full allocation.

## Test Channel Bandwidth

Minimum requirements for NS_04 applies for all channel bandwidths. Select channel bandwidth to test as for SEM [2]

PROPOSAL 4: Select Lowest and Highest test channel bandwidth for NS_04 A-MPR in FR1.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. lowest and highest supported subcarrier spacing.

PROPOSAL 5: Select Lowest and Highest supported subcarrier spacing for NS_04 A-MPR in FR1.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms).

PROPOSAL 6: Test all UL modulation waveforms supported by the UE for NS_04 A-MPR in FR1.

## Uplink RB allocation

For parts of the band where A-MPR=0, the selection of UL RB allocation can be based on the agreed proposals for SEM [2].

PROPOSAL 7: For NS_04 where A-MPR=0 base the uplink RB allocation on the agreed proposal for SEM.

2.7.	Number of Test Points

The number of test points for NS_04 AMPR is equal to 120 test points.

2.7.	Exceptions for A-MPR UL-MIMO test

The requirements of A-MPR for UL-MIMO is the same as the requirements of single antenna A-MPR test case 6.2.3, therefor the test point selection in proposal 1~5 and 7 can be reused without any change.

CP-OFDM is mandatory in case UL-MIMO transmission with two layer data, only CP-OFDM test points selected in proposal 6 are valid for UL-MIMO.

PROPOSAL 8: Test points selected in proposals 1~5 and 7 can be reused by A-MPR UL-MIMO testing, and only CP-OFDM test points selected in proposal 6 are reused for UL-MIMO.

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_04 A-MPR in FR1 test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select Low range and High range test frequencies for NS_04 A-MPR in FR1.

PROPOSAL 3: Select additional frequencies (Fc) according to Table 1 and test the A-MPR requirements with both Edge_1RB and Outer_Full allocation.

PROPOSAL 4: Select Lowest and Highest test channel bandwidth for NS_04 A-MPR in FR1.

PROPOSAL 5: Select Lowest and Highest supported subcarrier spacing for NS_04 A-MPR in FR1.

PROPOSAL 6: Test all UL modulation waveforms supported by the UE for NS_04 A-MPR in FR1

PROPOSAL 7: For NS_04 A-MPR base the uplink RB allocation on the agreed proposal for SEM.

PROPOSAL 8: Test points selected in proposals 1~5 and 7 can be reused by A-MPR UL-MIMO testing, and only CP-OFDM test points selected in proposal 6 are reused for UL-MIMO.

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- R5-184752, “Discussion on test point selection for NS_35 A-MPR in FR1”, (Ericsson)