3GPP TSG-WG5 Meeting #93-e	R5-217672r2

# Electronic Meeting, 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2644 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New testcase for Idle/Inactive measurements on NR cells in RRC_IDLE state with configuration through RRCRelease |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To test Idle/Inactive measurements on NR cells while UE is in RRC_IDLE state and configuration is provided through RRCRelease message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new testcase with testpoints, requirements and test description. Specific message content to be defined later. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Idle/Inactive measurements feature can’t have conformance approval. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5 RRC others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0193 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Added clauses affected information r2: Clarified ssb-MeasConfig in case of SIB4, added missing I to P-RNTI in step 26, corrected numbering error in test procedure sequence |  |  |  |  |  |  |  |  |

------------------------------------------------- Start of changes -------------------------------------------------

------------------------------------------------- Skipped chapters -------------------------------------------------

Table 8.1.5.10.1.3.3-3: RRCReconfiguration (step 14, Table 8.1.5.10.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| releasePreferenceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| releasePreferenceProhibitTimer-r16 | s30 |  |  |
| connectedReporting | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.11.3	Idle/Inactive measurements / Idle mode / RRCRelease configuration / Measurement of NR cells

8.1.5.11.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig with ssb-MeasConfig }

then { UE enters RRC_IDLE state, performs cell measurements and reports measurement results in RRC_CONNECTED state  }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig, ssb-MeasConfig from SIB11 }

then { UE enters RRC_IDLE state, performs cell measurements and reports measurement results }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig, ssb-MeasConfig parameters derived from SIB4 }

then { UE enters RRC_IDLE state, performs cell measurements and reports measurement results }

}

(4)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig with ssb-MeasConfig, T331 expires }

then { UE enters RRC_IDLE state, UE does not report measurement results in RRC_CONNECTED state after T331 expiration }

}

(5)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig without measIdleCarrierListNR }

then { UE does not report measurement results }

}

8.1.5.11.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.8.3, 5.7.8 and 5.7.10.3.

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

[TS 38.331, clause 5.7.8.1]

This procedure specifies the measurements to be performed and stored by a UE in RRC_IDLE and RRC_INACTIVE when it has an idle/inactive measurement configuration.

[TS 38.331, clause 5.7.8.1a]

The purpose of this procedure is to update the idle/inactive measurement configuration.

The UE initiates this procedure while T331 is running and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC-INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB4, or SIB11), e.g. due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB of SIB11 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB11 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in interFreqCarrierFreqList of SIB4 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB4 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

[TS 38.331, clause 5.7.8.2a]

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [9].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and the SIB1 contains idleModeMeasurementsNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports carrier aggregation or NR-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry;

5>	if the reportQuantities is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive cell measurement results for the measurement quantities indicated by reportQuantities;

5>	store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within measResultsPerCarrierListIdleNR in the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThreshold is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

5>	if beamMeasConfigIdle is included in the associated entry in measIdleCarrierListNR and if UE supports idleInactiveNR-MeasBeamReport for the FR of the carrier frequency indicated by carrierFreq within the associated entry, for each cell in the measurement results:

6>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

6>	if the reportQuantityRS-Indexes is set to rsrq:

7>	consider RSRQ as the beam sorting quantity;

6>	else:

7>	consider RSRP as the beam sorting quantity;

6>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

7>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

6>	if the includeBeamMeasurements is set to true:

7>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

3>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and it contains an entry with carrierFreq set to the value of the serving frequency:

4>	if beamMeasConfigIdle is included in that entry, and if the UE supports idleInactiveNR- MeasBeamReport for the FR of the serving cell:

5>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

5>	if the reportQuantityRS-Indexes is set to rsrq:

6>	consider RSRQ as the beam sorting quantity;

5>	else:

6>	consider RSRP as the beam sorting quantity;

5>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

6>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included in SIB2 of serving cell, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

5>	if the includeBeamMeasurements is set to true:

6>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signaling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

[TS 38.331, clause 5.7.8.3]

The UE shall:

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

8.1.5.11.3.3	Test Description

8.1.5.11.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	NR Cell 3, suitable for inter-frequency measurements

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-1.

8.1.5.11.3.3.2	Test procedure sequence

Table 8.1.5.11.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCRelease message with measIdleConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 2 | Wait for 5 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 3-10 | Steps 1 to 8 of procedure described in TS 38.508-1 table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 11 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 1 | P |
| 13 | The SS transmits RRCRelease message with measIdleConfig not including ssb-MeasConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 14 | Wait for 5 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 15-22 | Steps 1 to 8 of procedure described in TS 38.508-1 table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 23 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 24 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 2 | P |
| 25 | The SS removes measIdleCarrierListNR in SIB11. | - | - | - | - |
| 26 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 27 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 28 | The SS transmits RRCRelease message with measIdleConfig not including ssb-MeasConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 29 | Wait for 5 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 30-37 | Steps 1 to 8 of procedure described in TS 38.508-1 table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 38 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 39 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 3 | P |
| 40 | The SS transmits RRCRelease message including measIdleConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 41 | Wait for 11 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 42-49 | Steps 1 to 8 of procedure described in TS 38.508-1 table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 50 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 51 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 4 | F |
| 52 | The SS transmits RRCRelease message with measIdleConfig not including measIdleCarrierListNR to the UE. | <- | NR RRC: RRCRelease | - | - |
| 53 | Wait for 5 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 54-61 | Steps 1 to 8 of procedure described in TS 38.508-1 table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 62 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 63 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 5 | F |

8.1.5.11.3.3.3	Specific message contents

TBD

### 8.1.6	SON and MDT support for NR

#### 8.1.6.1	Intra NR MDT

##### 8.1.6.1.1	Immediate MDT

###### 8.1.6.1.1.1	Immediate MDT / Measurement reporting / Location information

8.1.6.1.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and measurement with event A2 configured with includeLocationInfo included in the reportConfig }

ensure that {

when { Entry condition for event A2 is met and detailed location information that has not been reported is available }

then { UE sends MeasurementReport message with locationInfo included }

}

------------------------------------------------- Skipped chapters -------------------------------------------------

------------------------------------------------- End of changes -------------------------------------------------