3GPP TSG-RAN5 Meeting #93-e	R5-217677r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1474 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of CBW 70MHz into 6.5C of 38.521-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The CBW 70M is currently missing in TC 6.5C. Test requirement for 70MHz CBW need to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated CBW 70M into TC 6.5C.1, Updated CBW 70M into TC 6.5C.2, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CBW 70MHz will be still missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5C.1 6.5C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1:Undo the revision of note in Table 6.5C.2.2.5-1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5C.1	Occupied bandwidth for SUL

6.5C.1.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE supporting SUL are less than their specific limits when UE is configured using SUL transmission.

6.5C.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5.1.3

6.5C.1.4	Test description

Same test description as specified in clause 6.5.1.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.1.4.1-1 use Table 6.5C.1.4-1

Table 6.5C.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low Range, Mid Range, High Range for SUL carrier Mid Range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | All for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 15kHz for both SUL carrier and Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration | UL Configuration |  | SUL Configuration |  |
| Test ID | N/A | N/A |  | Modulation | RB allocation |
| 1 |  |  |  | CP-OFDM QPSK | Outer_full |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

-	Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Tables 4.6.1-28 with condition SUL AND (RF OR RRM), 4.6.3-14 with condition SUL_SUL for SUL carrier, with condition SUL, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL.

6.5C.1.5	Test requirement

The measured Occupied Bandwidth on SUL carrier shall not exceed values in Table 6.5C.1.5-1.

Table 6.5C.1.5-1: Occupied channel bandwidth

|  | NR channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Occupied channel bandwidth (MHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |

6.5C.2	Out of band emission for SUL

6.5C.2.1	General

Void

6.5C.2.2	Spectrum emission mask for SUL

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies greater than (ΔfOOB) the spurious requirements in subclause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5C.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth.

6.5C.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.2.2.3	Minimum conformance requirements

Same minimum conformance requirements as in the clause 6.5.2.2.3.

6.5C.2.2.4	Test description

Same test description as PC 3 with contiguous allocation specified in clause 6.5.2.2 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	Instead of table 6.5.2.2.4.1-1 use Table 6.5C.2.2.4.1-1

Table 6.5C.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  | Low range, High range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  |  |  |  |  |  |  | 15kHz for both SUL carrier and Non-SUL carrier |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID |  | Freq |  | Downlink Configuration |  | UL Configuration |  | SUL Configuration |  |  |  |  |
|  |  |  |  | N/A |  | N/A |  | Modulation |  |  | RB allocation |  |
| 1 |  | Low |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |  |
| 2 |  | High |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Right |  |
| 3 |  | Default |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer_Full |  |
| 4 |  | Low |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edger_1RB_Left |  |
| 5 |  | High |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edge_1RB_Right |  |
| 6 |  | Default |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Outer_Full |  |
| 7 |  | Low |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Left |  |
| 8 |  | High |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Right |  |
| 9 |  | Default |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Outer_Full |  |
| 10 |  | Low |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Edge_1RB_Left |  |
| 11 |  | High |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Edge_1RB_Right |  |
| 12 |  | Default |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Outer_Full |  |
| 13 |  | Low |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Left |  |
| 14 |  | High |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Right |  |
| 15 |  | Default |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Outer_Full |  |
| 16 |  | Low |  |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Left |  |
| 17 |  | High |  |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Right |  |
| 18 |  | Default |  |  |  |  |  | CP-OFDM QPSK |  |  | Outer_Full |  |
| 19 |  | Low |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Left |  |
| 20 |  | High |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Right |  |
| 21 |  | Default |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Outer_Full |  |
| 22 |  | Low |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Left |  |
| 23 |  | High |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Right |  |
| 24 |  | Default |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Outer_Full |  |
| 25 |  | Low |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Edge_1RB_Left |  |
| 26 |  | High |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Edge_1RB_Right |  |
| 27 |  | Default |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Outer_Full |  |
| 284 |  | Low |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  |  | Edge_1RB_Left |  |  |
| 294 |  | High |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  |  | Edge_1RB_Right |  |  |
| 304 |  | Default |  |  |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  |  | Outer Full |  |  |
| Note 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. Note 2: The specific configuration of each RF allocation is defined in Table 6.1-1. Note 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. Note 4: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. |  |  |  |  |  |  |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

-	Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND (RF OR RRM), Tables 4.6.3-14 with condition SUL_NUL for SUL carrier, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL. Message contents in Table 6.5.2.2.4.3-2 in clause 6.5.2.2 only apply to Test ID 28-30 in Table 6.5C.2.2.4.1-1.

6.5C.2.2.5	Test requirement

The measured sum of the UE mean power in the channel bandwidth on the SUL carrier, derived in step 3, shall fulfil requirements in Tables 6.2C.4.5-1 as appropriate, and the power of any UE emission shall fulfil requirements in Table 6.5C.2.2.5-1.

Table 6.5C.2.2.5-1: NR General spectrum emission mask

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  |  |  |  | -24 | -24 | -24 | -24 | -24 | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 | -25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  | -25 |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  | -25 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 |  |  |  | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  |  |  |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  |  |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  |  |  |  | -25 |  |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. Note 4: TT = 1.5 dB for f ≤ 3GHz, TT = 1.8 dB for 3GHz < f ≤ 4.2GHz, TT = 1.8 dB for 4.2GHz < f ≤ 6.0GHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<Start of Changes>