3GPP TSG-RAN5 Meeting #93-e                                                    R5-217703r1

Electronic Meeting, 8th – 19th November 2021

Title: 		        Beam correspondence Measurement Uncertainties derivation

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.1.17

Document for:	Discussion and Endorsement

# 1.	Introduction

Measurement uncertainties for FR2 beam correspondence tolerance is still undefined ([1][2]).

This document makes a proposal on how to derive such measurement uncertainty.

# 2.	Core requirements overview

Analysing minimum conformance requirements and test procedure for beam correspondence tolerance, it can be seen that beam correspondence tolerance is defined as:

6.6.1.3.3.2 Beam correspondence tolerance for PC3 The beam correspondence tolerance requirement ∆EIRPBC for power class 3 UEs is defined based on a percentile of the distribution of ∆EIRPBC, defined as ∆EIRPBC = EIRP2 - EIRP1 over the link angles spanning a subset of the spherical coverage grid points, such that - EIRP1 is the total EIRP in dBm calculated based on the beam the UE chooses autonomously (corresponding beam) to transmit in the direction of the incoming DL signal, which is based on beam correspondence without relying on UL beam sweeping. - EIRP2 is the best total EIRP (beam yielding highest EIRP in a given direction) in dBm which is based on beam correspondence with relying on UL beam sweeping. - The link angles are the ones corresponding to the top Nth percentile of the EIRP2 measurement over the whole sphere, where the value of N is according to the test point of EIRP spherical coverage requirement for power class 3, i.e. N = 50. For power class 3 UEs, the requirement is fulfilled if the UE’s corresponding UL beams satisfy the maximum limit in Table 6.6.1.3.3.2-1. Table 6.6.1.3.3.2-1: UE beam correspondence tolerance for power class 3 Operating band; Max ∆EIRPBC at 85 %-tile ∆EIRPBC CDF (dB). n257; 3.0. n258; 3.0. n260; 3.2. n261; 3.0. NOTE: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1.

As shown, beam correspondence tolerance requirement is defined as a given power percentile around the sphere of a differential EIRP measurement.

The EIRP difference for a compliant UE will be no higher than 3.2 dB for at least 85% of the measurement grid points around the sphere. Hence, it seems to be reasonable to assume that test system configuration to make each of these measurements will be similar.

# 3.	Measurement uncertainties contributions

Similarly as in [3], for the beam correspondence tolerance measurement, it is thought that all the MU factors associated to calibration stage in EIRP absolute power measurement MU budget table are cancelled out as the 2 signals experience the same absolute level distortion.

In the same way, the following MU factors in the DUT measurement stage also cancel out in the FR2 beam correspondence tolerance measurement MU budget:

- Measure distance uncertainty

- Quality of the Quiet Zone

- Mismatch

- Standing wave between the DUT and measurement antenna

- Phase curvature

- Random uncertainty

- Influence of XPD

- Insertion Loss Variation

- RF leakage (from measurement antenna to the receiver/transmitter)

- Multiple measurement antenna uncertainty

- Positioning misalignment

Hence the MU factors remaining will be:

- Positioning misalignment

- Uncertainty of the RF Power measurement equipment

- Amplifier uncertainties

- Influence of noise

Proposal 1: Adopt N/A for the MU elements other than uncertainty of the RF Power measurement equipment, amplifier uncertainties and influence of noise in FR2 Beam Correspondence tolerance measurement MU.

Proposal 2: Adopt 0.5 dB for the uncertainty value of amplifier uncertainty (as agreed in R5-215319 [4] Proposal 4) in FR2 Beam Correspondence tolerance measurement MU.

Proposal 3: In the absence of a CDF reference cure and to enable progress on this priority 1 test case, adopt 1.0dB (systematic error) as influence of noise(SNR) for FR2 Beam Correspondence tolerance measurement MU as proposed in [3] for FR2 relative power measurement MU.

Proposal 4: Adopt +/- 0.4dB (as expanded uncertainty) for Uncertainty of the RF Power measurement equipment as proposed in [3].

Proposal 5: Define total Beam Correspondence tolerance MU as +/- 1.63dB.

# 4.	Conclusion

This document describes how to calculate FR2 beam correspondence tolerance measurement MU and makes the following proposals:

Proposal 1: Adopt N/A for the MU elements other than uncertainty of the RF Power measurement equipment, amplifier uncertainties and influence of noise in FR2 Beam Correspondence tolerance measurement MU.

Proposal 2: Adopt 0.5 dB for the uncertainty value of amplifier uncertainty (as agreed in R5-215319 [4] Proposal 4) in FR2 Beam Correspondence tolerance measurement MU.

Proposal 3: In the absence of a CDF reference cure and to enable progress on this priority 1 test case, adopt 1.0dB (systematic error) as influence of noise(SNR) for FR2 Beam Correspondence tolerance measurement MU as proposed in [3] for FR2 relative power measurement MU.

Proposal 4: Adopt +/- 0.4dB (as expanded uncertainty) for Uncertainty of the RF Power measurement equipment as proposed in [3].

Proposal 5: Define total Beam Correspondence tolerance MU as +/- 1.63dB.

# 5.	References

[1] 38.521-2 v16.9.0 “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”

[2] 38.903 v16.9.0 “Derivation of test tolerances and measurement uncertainty for User equipment (UE) conformance test cases”

[3] R5-210845, “On FR2 relative power measurement uncertainty”, Anritsu, RAN5#90-e

[4] R5-215319, “On MU of FR2 power control test cases”, Anritsu, RAN5#92-e