3GPP TSG-RAN WG5 Meeting #94-e                                                              R5-220059r2

# Electronic Meeting, 21st February– 4th March 2022

Agenda Item:	5.3.31.4

Source: 	CAICT

Title: 	Discussion on Alignment of test points for MPR and ACLR in TS 38.521-1

Document for:	Discussion and Endorsement

Introduction

In 38.521-1, MPR test case can be skipped if ACLR is tested. This requires ACLR to include all the test points of MPR. This discussion paper is about how to make sure ACLR always includes all the test points of MPR.

Discussion

For NR FR1, RAN5 have used the rule that the test points in the MPR test should be aligned with ACLR test cases. The test requirements of MPR test have been verified as a part of the test procedure in ACLR test cases. This is making the MPR test case redundant for certification, but it was decided to keep the MPR test in the RAN5 spec anyway to be used in R&D testing.

One underlying reason why the UE is allowed to have power backoff (MPR) in demanding modulations is that the waveform makes it difficult for the UE PA to reach max power without introducing non-linearities that will cause spectral regrowth. By allowing MPR, it is possible to fulfil output power requirements while simultaneously fulfilling RF emisssions. This means that if a demanding waveform is tested in MPR test but NOT in emissions test then the UE will not need to care about non-linearities in this case which could result in damage to system performance, since UE may use too little MPR (less than it would need to pass RF emission requirements). Only if the UE will also be testing RF emissions in this scenario the UE will be forced to fully optimize the RF performance.

Test applicability of MPR has a NOTE indicating that if ACLR is executed MPR can be skipped, for example:

- 6.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1.5, Power Class 2 and 3 UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed.

The NOTE is based on the assumption that test procedure of ACLR includes the testing of MPR, which means ACLR shall include all the test points of MPR.

- 6.2A.2.1.2	Test applicability

The requirements of this test apply to all types of NR Power class 3 UE release 15 and forward that support 2 UL CA.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5A.2.4.1.1 is executed.

Observation 1: Test applicability of MPR requires ACLR to include all the test points of MPR.

But ACLR test cases do not include all the test points of MPR in the current TS 38.521-1, for example:

In the current spec of 38.521-1, test points for power class 1.5 has been introduced to 6.2.2 MPR, but is still missing in 6.5.2.4.1 ACLR.

Table 6.2.2.4.1-2b: Test Configuration Table for power class 1.5 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Default |  | DFT-s-OFDM_64_QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 17 | Default |  | DFT-s-OFDM_256_QAM | Inner Full |
| 18 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 19 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 20 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 21 | Default |  | CP-OFDM QPSK | Inner Full |
| 22 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 23 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM QPSK | Outer Full |
| 25 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 26 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 27 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 28 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 29 | Default |  | CP-OFDM 64 QAM | Inner Full |
| 30 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 33 | Default |  | CP-OFDM 256 QAM | Inner Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |

In the current spec of 38.521-1, “6.2A.2.1 UE maximum output power reduction for CA (2UL CA)” includes more test points than “6.5A.2.4.1.1 NR ACLR for CA (2UL CA)”.

Test points in “6.2A.2.1 UE maximum output power reduction for CA (2UL CA)” are as below:

Table 6.2A.2.1.4.1-1: Inter-band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  |  | Modulation for all CCs (NOTE 2) | RB allocation (NOTE 1) |  |
|  |  |  |  | PCC | SCC |
| 13 | Default | N/A for Maximum Power Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full | Outer Full |
| 37 | Default |  | NOTE 5 | Inner Full | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 5: The modulation is DFT-s-OFDM QPSK for PCC and CP-OFDM 256 QAM for SCC. |  |  |  |  |  |

Table 6.2A.2.1.4.1-2a: Intra-band contiguous CA Test Configuration Table (contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A | DFT-s-OFDM |  | Pi/2 BPSK | Inner Full |
| 2 |  |  |  | Pi/2 BPSK | Outer Full |
| 3 |  |  |  | QPSK | Inner Full |
| 4 |  |  |  | QPSK | Outer Full |
| 5 |  |  |  | 16QAM | Inner Full |
| 6 |  |  |  | 16QAM | Outer Full |
| 7 |  |  |  | 64QAM | Inner Full |
| 8 |  |  |  | 64QAM | Outer Full |
| 9 |  |  |  | 256QAM | Inner Full |
| 10 |  |  |  | 256QAM | Outer Full |
| 11 |  | CP-OFDM |  | QPSK | Inner Full |
| 12 |  |  |  | QPSK | Outer Full |
| 13 |  |  |  | 16QAM | Inner Full |
| 14 |  |  |  | 16QAM | Outer Full |
| 15 |  |  |  | 64QAM | Inner Full |
| 16 |  |  |  | 64QAM | Outer Full |
| 17 |  |  |  | 256QAM | Inner Full |
| 18 |  |  |  | 256QAM | Outer Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 6.2A.2.1.4.1-2b: Intra-band contiguous CA Test Configuration Table (non-contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A | DFT-s-OFDM |  | Pi/2 BPSK | Inner |
| 2 |  |  |  | Pi/2 BPSK | Outer 1 |
| 3 |  |  |  | Pi/2 BPSK | Outer 2 |
| 4 |  |  |  | QPSK | Inner |
| 5 |  |  |  | QPSK | Outer 1 |
| 6 |  |  |  | QPSK | Outer 2 |
| 7 |  |  |  | 16QAM | Inner |
| 8 |  |  |  | 16QAM | Outer 1 |
| 9 |  |  |  | 16QAM | Outer 2 |
| 10 |  |  |  | 64QAM | Inner |
| 11 |  |  |  | 64QAM | Outer 1 |
| 12 |  |  |  | 64QAM | Outer 2 |
| 13 |  |  |  | 256QAM | Inner |
| 14 |  |  |  | 256QAM | Outer 1 |
| 15 |  |  |  | 256QAM | Outer 2 |
| 16 |  | CP-OFDM |  | QPSK | Inner |
| 17 |  |  |  | QPSK | Outer 1 |
| 18 |  |  |  | QPSK | Outer 2 |
| 19 |  |  |  | 16QAM | Inner |
| 20 |  |  |  | 16QAM | Outer 1 |
| 21 |  |  |  | 16QAM | Outer 2 |
| 22 |  |  |  | 64QAM | Inner |
| 23 |  |  |  | 64QAM | Outer 1 |
| 24 |  |  |  | 64QAM | Outer 2 |
| 25 |  |  |  | 256QAM | Inner |
| 26 |  |  |  | 256QAM | Outer 1 |
| 27 |  |  |  | 256QAM | Outer 2 |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1b. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Test points in “6.5A.2.4.1.1 NR ACLR for CA (2UL CA)” are as below:

Table 6.5A.2.4.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest for both PCC and SCC Highest for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  | Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  |  | PCC | SCC |
| 13 | Low | N/A | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 23 | High |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 33 | Default |  | DFT-s-OFDM PI/2 BPSK | Outer_Full | Outer_Full |
| 4 | Default |  | DFT-s-OFDM PI/2 BPSK | Inner_Full | Inner_Full |
| 5 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 6 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 7 | Default |  | DFT-s-OFDM QPSK | Outer_Full | Outer_Full |
| 8 | Default |  | DFT-s-OFDM QPSK | Inner_Full | Inner_Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner_Full | Inner_Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer_Full | Outer_Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer_Full | Outer_Full |
| 16 | Default |  | DFT-s-OFDM 256 QAM | Outer_Full | Outer_Full |
| 17 | Default |  | CP-OFDM QPSK | Inner_Full | Inner_Full |
| 18 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 19 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 20 | Default |  | CP-OFDM QPSK | Outer_Full | Outer_Full |
| 21 | Default |  | CP-OFDM 16 QAM | Inner_Full | Inner_Full |
| 22 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 16 QAM | Outer_Full | Outer_Full |
| 25 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 64 QAM | Outer_Full | Outer_Full |
| 28 | Default |  | CP-OFDM 256 QAM | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |

Table 6.5A.2.4.1.1.4.1-2: Intra band contiguous CA Test Configuration Table (contiguous RB allocation)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg, Highest NRB_agg (NOTE 1) |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Lowest, Highest |  |  |
| Test Parameters for CA bandwidth class B and C |  |  |  |  |  |
| Test ID | DL configuration for PCC & SCC | UL configuration |  |  |  |
|  |  | Modulations for all CCs (NOTE 2) |  |  | RB allocation (NOTE 3) |
| 1 | N/A | DFT-s-OFDM |  | Pi/2 BPSK | Inner Full |
| 2 |  |  |  | Pi/2 BPSK | Outer Full |
| 3 |  |  |  | QPSK | Inner Full |
| 4 |  |  |  | QPSK | Outer Full |
| 5 |  |  |  | 16QAM | Inner Full |
| 6 |  |  |  | 16QAM | Outer Full |
| 7 |  |  |  | 64QAM | Inner Full |
| 8 |  |  |  | 64QAM | Outer Full |
| 9 |  |  |  | 256QAM | Inner Full |
| 10 |  |  |  | 256QAM | Outer Full |
| 11 |  | CP-OFDM |  | QPSK | Inner Full |
| 12 |  |  |  | QPSK | Outer Full |
| 13 |  |  |  | 16QAM | Inner Full |
| 14 |  |  |  | 16QAM | Outer Full |
| 15 |  |  |  | 64QAM | Inner Full |
| 16 |  |  |  | 64QAM | Outer Full |
| 17 |  |  |  | 256QAM | Inner Full |
| 18 |  |  |  | 256QAM | Outer Full |
| NOTE 1: The Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.5A.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 6.5A.2.4.1.1.4.1-3: Intra band non-contiguous CA Test Configuration Table

FFS

Message contents in “6.2A.2.1 UE maximum output power reduction for CA (2UL CA)” are as below:

- 6.2A.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4.

Table 6.2A.2.1.4.3-1: FrequencyInfoUL-SIB for inter-band CA

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA Test IDs 1, 4-13, 16, 19, 23, 26, 27, 30, 37 |
|  | 17 |  | Power class 3 and Inter-band CA Test IDs 14, 15, 17, 18, 33 |
|  | 16 |  | Power class 3 and Inter-band CA Test IDs 2, 3, 22, 24, 25, 28, 29, 31, 32 |
|  | 14 |  | Power class 3 and Inter-band CA Test IDs 20, 21, 36 |
|  | 13 |  | Power class 3 and Inter-band CA Test IDs 34, 35 |

Table 6.2A.2.1.4.3-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n41C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 18 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |

Table 6.2A.2.1.4.3-2a: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n48B

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 19 |  | Power class 3 and Test IDs 1, 3, 5 |
|  | 18 |  | Power class 3 and Test IDs 5, 11 |
|  | 17 |  | Power class 3 and Test IDs 7, 13 |
|  | 16 |  | Power class 3 and Test IDs 2, 4, 6, 15 |
|  | 15 |  | Power class 3 and Test IDs 8, 12, 14, 16 |
|  | 12 |  | Power class 3 and Test IDs 9, 10 |
|  | 11 |  | Power class 3 and Test ID 17, 18 |

Table 6.2A.2.1.4.3-2b: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_n77C

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 17 |  | Power class 3 and Test IDs 1, 3 ,5 |
|  | 16 |  | Power class 3 and Test IDs 11, 13 |
|  | 14 |  | Power class 3 and Test IDs 7, 15 |
|  | 11 |  | Power class 3 and Test IDs 2, 4, 6, 8, 9, 17 |
|  | 10 |  | Power class 3 and Test IDs 10, 12, 14, 16, 18 |

Message contents in “6.5A.2.4.1.1 NR ACLR for CA (2UL CA)” are as below:

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.2.4.1.1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Power class 3 and Inter-band CA Test IDs 3-9, 12, 15, 17, 20, 21, 24 |
|  | 17 |  | Power class 3 and Inter-band CA Test IDs 10, 11, 13, 14, 27 |
|  | 16 |  | Power class 3 and Inter-band CA Test IDs 1, 2, 16, 18, 19, 22, 23, 25, 26 |
|  | 14 |  | Power class 3 and Inter-band CA Test IDs 28 |

UL-MIMO and V2X ACLR test cases do not include all the test points of UL-MIMO and V2X MPR in the current 38.521-1 either.

Observation 2: ACLR does not include all the test points of MPR in the current 38.521-1.

It is not correct to skip MPR test case if ACLR doesn’t include all the test points of MPR.

The test points of MPR and ACLR were not aligned at the beginning, but have once been aligned by Ericsson, Keysight and CAICT together, but now they are not aligned again.

The misalignment of test point is easy to happen as delegates may forget to revise the test points of MPR/ACLR simultaneously or make mistakes when revising both of them.

Proposal: Revise ACLR to remove all the test config tables and just referring back to MPR to make the spec easier to maintain and to reduce misalignment.

Conclusion

Observation 1: Test applicability of MPR requires ACLR to include all the test points of MPR.

Observation 2: ACLR does not include all the test points of MPR in the current 38.521-1.

Proposal: Revise ACLR to remove all the test config tables and just referring back to MPR to make the spec easier to maintain and to reduce misalignment.