3GPP TSG-RAN5 Meeting #94-e	R5-220401r1

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0230 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional Rel-15 parameters for MCVideo User Profile 5.5.8.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ambient viewing parameters, PrivateEmergencyAlert IEs, and RemoteGroupSelectionURIList IEs in the MCVideo User Profile did not exist |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added IEs anyExt, allow-request-remote-initiated-ambient-viewing, and allow-request-locally-initiated-ambient-viewing. Added PrivateEmergencyAlert IEs Added RemoteGroupSelectionURIList IEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Certain MCVideo features will not be able to be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: The new entries PrivateEmergencyAlert and RemoteGroupSelectionURIList needed to be "entry[1]", "entry[2]", "entry[3]" to distinguish the different entries in the list |  |  |  |  |  |  |  |  |

<START OF CHANGES>

#### 5.5.8.7	MCVideo User Profile

The structure of a user profile document is specified in TS 24.484 [14] clause 9.3. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 13.2.

Table 5.5.8.7-1: MCVideo User Profile Defaults

| Derivation Path: TS 24.24.484, clause 9.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-user-profile |  |  |  |  |
| XUI-URI attribute | "sip:" & px_MCVideo_ID_User_A | same as the XUI value of the Document URI |  |  |
| user-profile-index attribute | "42" | value arbitrarily selected |  |  |
| Status | “true” | MCVideo user profile is enabled |  |  |
| ProfileName | "mcvideo-user-profile-" & user-profile-index & ".xml" | name of the user profile document; user-profile-index is the value of the user-profile-index attribute | TS 24.483 [13] clause 13.2.3; |  |
| Common |  |  |  |  |
| index attribute | "0" | Index for the particular MCVideo user profile |  |  |
| MCVideoUserID |  | Indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user | TS 24.483 [13] clause 13.2.7 |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCVideo_ID_User_A | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user |  |  |
| UserAlias | px_MCVideo_User_A_Alias | Alphanumeric aliases of MCVideo user | TS 24.483 [13] clause 13.2.11 |  |
| ParticipantType | px_MCVideo_User_A_ParticipantType | The functional category of the participant (e.g., first responder, second responder, dispatch, dispatch supervisor), typically defined by the MCVideo administrators. | TS 24.483 [13] clause 13.2.15 |  |
| MissionCriticalOrganization | px_MCX_DomainName_Organization_A | Indicates the organization an MCVideo user belongs to | TS 24.483 [13] clause 13.2.16 |  |
| NotifyList |  |  |  |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCVideo_ID_User_B |  |  |  |
| CatList |  |  |  |  |
| catentry | "1" |  | TS 24.483 [13] clause 13.2.38 |  |
| ReceptionPriority | "1" |  |  |  |
| OnNetwork |  |  |  |  |
| index | "1" |  |  |  |
| MCVideoGroupInfo |  |  |  |  |
| MCVideo-Group-ID | px_MCVideo_Group_A_ID |  |  |  |
| GMS-Serv-Id | tsc_MCX_GMS_Hostname |  |  |  |
| IdMS-Token-Endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| RelativePresentationPriority | "7" |  | TS 24.483 [13] clause 13.2.51 |  |
| MaxAffiliationsN2 | "10" |  | TS 24.483 [13] clause 13.2.67 |  |
| MaxTimeSingleTransmit | "600" | Value in seconds | TS 24.483 [13] clause 13.2.87 |  |
| PrivateEmergencyAlert |  |  | TS 24.483 [13] clause 13.2.87 |  |
| entry[1] | px_MCVideo_ID_User_A |  |  |  |
| entry[2] | px_MCVideo_ID_User_B |  |  |  |
| entry[3] | px_MCVideo_ID_User_C |  |  |  |
| RemoteGroupSelectionURIList |  |  | TS 24.483 [13] clause 13.2.87 |  |
| entry[1] | px_MCVideo_ID_User_A |  |  |  |
| entry[2] | px_MCVideo_ID_User_B |  |  |  |
| entry[3] | px_MCVideo_ID_User_C |  |  |  |
| OffNetwork |  |  |  |  |
| index | "1" |  |  |  |
| MCVideoGroupInfo |  |  |  |  |
| MCVideo-Group-ID | px_MCVideo_Group_A_ID |  |  |  |
| GMS-App-Serv-Id | tsc_MCX_GMS_Hostname |  |  |  |
| IdMS-Token-Endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| RelativePresentationPriority | "7" |  | TS 24.483 [13] clause 13.2.51 |  |
| User-Info-Id | '555555555555'O |  | TS 24.483 [13] clause 13.2.102 |  |
| cp:ruleset |  |  |  |  |
| cp:rule |  |  |  |  |
| cp:id attribute | "rule1" |  |  |  |
| cp:actions |  |  |  |  |
| allow-create-delete-user-alias | "true" |  |  |  |
| allow-create-group-broadcast- group | "true" |  |  |  |
| allow-create-user-broadcast-group | "true" |  |  |  |
| allow-modify-video | "true" |  |  |  |
| allow-renegotiate-codec | "true" |  |  |  |
| allow-camera-control | "true" |  |  |  |
| allow-remote-control | "true" |  |  |  |
| allow-display-remote-ue | "true" |  |  |  |
| allow-remote-camera | "true" |  |  |  |
| allow-push-video | "true" |  |  |  |
| allow-auto-send-notify | "true" |  |  |  |
| allow-request-affiliated-groups | "true" |  |  |  |
| allow-request-to-affiliate-other-users | "true" |  |  |  |
| allow-recommend-to-affiliate-other-users | "true" |  |  |  |
| allow-regroup | "true" |  |  |  |
| allow-presence-status | "true" |  |  |  |
| allow-request-presence | "true" |  |  |  |
| allow-activate-emergency-alert | "true" |  |  |  |
| allow-cancel-emergency-alert | "true" |  |  |  |
| allow-cancel-emergency-alert-any-user | "true" |  |  |  |
| allow-enable-disable-user | "true" |  |  |  |
| allow-enable-disable-UE | "true" |  |  |  |
| allow-off-network-manual-switch | "true" |  |  |  |
| allow-unlimited-video-streams | "true" |  |  |  |
| allow-auto-recv | "true" |  |  |  |
| allow-auto-recv-emergency | "true" |  |  |  |
| allow-auto-recv-imminent-peril | "true" |  |  |  |
| allow-request-override | "true" |  |  |  |
| allow-select-override | "true" |  |  |  |
| allow-override-group-call | "true" |  |  |  |
| allow-off-network | "true" |  |  |  |
| allow-private-call | "true" |  |  |  |
| allow-manual-commencement | "true" |  |  |  |
| allow-automatic-commencement | "true" |  |  |  |
| allow-failure-restriction | "true" |  |  |  |
| allow-emergency-group-call | "true" |  |  |  |
| allow-emergency-private-call | "true" |  |  |  |
| allow-cancel-group-emergency | "true" |  |  |  |
| allow-imminent-peril-call | "true" |  |  |  |
| allow-cancel-imminent-peril | "true" |  |  |  |
| allow-off-network-group-call-change-to-emergency | "true" |  |  |  |
| anyExt |  |  |  |  |
| allow-request-remote-initiated-ambient-viewing | "true" |  |  |  |
| allow-request-locally-initiated-ambient-viewing | "true" |  |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | IP address is IPv4 address |
| IPv6 | IP address is IPv6 address |

<END OF CHANGES>