3GPP TSG-RAN5 Meeting # 94-e	R5-220421r1

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0138 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FR2 CA CQI test cases applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the WP, FR2 CA CQI test cases will be introduced, and test case applicability is missing in TS38.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test case applicability in Table 4.1.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no applicability for FR2 CA CQI test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Correct meeting date information on the coversheet |  |  |  |  |  |  |  |  |

<Start of change>

4.1.4	Performance conformance test cases

Table 4.1.4-1: Applicability of performance test cases, ref. TS 38.521-4 [4]

| Clause | TC Title | Release | Applicability |  | Tested Bands Selection | Additional Information |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 5 | Demodulation performance requirements (Conducted requirements) |  |  |  |  |  |
| 5.2 | PDSCH demodulation requirements |  |  |  |  |  |
| 5.2.2.1.1_1 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| … | … |  |  |  |  |  |
| 8 | CSI reporting requirements (Radiated requirements) |  |  |  |  |  |
| 8.2.2.2.1.1 | 2Rx TDD FR2 periodic CQI reporting under AWGN performance for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 8.2.2.2.2.1 | 2Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA | Rel-15 | C061F | UEs supporting 5GS TDD FR2 | D014 | Skip TC 8.2.2.2.2.1 if TS 38.521-4 TC 8.2.2.2.2.1_1 has been executed and passed. |
| 8.2.2.2.2.1_1 | 2Rx TDD FR2 aperiodic CQI reporting under fading performance for both SA and NSA – 256QAM | Rel-16 | C126 | UEs supporting 5GS TDD FR2 and DL 256QAM | D013 | NOTE 1 |
| 8.2A.3.1.1 | 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) | Rel-15 | C006c | UEs supporting 5GS FR2 and CA (2DL CA) | E032 | Test execution not necessary if 8.2A.3.1.2 is executed. |
| 8.2A.3.1.2 | 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) | Rel-15 | C006d | UEs supporting 5GS FR2 and CA (3DL CA) | E033 | Test execution not necessary if 8.2A.3.1.3 is executed. |
| 8.2A.3.1.3 | 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) | Rel-15 | C006e | UEs supporting 5GS FR2 and CA (4DL CA) | E034 |  |
| 8.3.2.2.1 | 2Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 8.4.2.2.1 | 2Rx TDD FR2 RI reporting for both SA and NSA | FFS | FFS | FFS | FFS | NOTE 1 |
| 9 | Demodulation performance requirements for interworking |  |  |  |  |  |
| 9.4B.1.1 | Sustained downlink data rate performance for EN-DC within FR1 | Rel-15 | C020 | UEs supporting 5GS FDD FR1 or TDD FR1 (NSA) | D008 D009 D010 |  |
| 9.4B.1.2 | Sustained downlink data rate performance for EN-DC including FR2 NR carrier | FFS | FFS | FFS | FFS | NOTE 1 |
| 10 | CSI reporting requirements for interworking |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-4. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

<End of change>