3GPP TSG-RAN5 Meeting #94-e	R5-220517r2

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.901-5 | CR | 0028 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Annex.B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm communications-France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_UE_5GNR_App_Data_Perf |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reference for 38.214 and 38.508, Upper layer configuration and UL RMC are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reference added for 38.214 and 38.508 in Section , MAC, RLC, PDCP configuration and ULRMCs are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid reference will create confusion, UL RMC and upper layer configuration will be ambigious |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 2, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Added Clauses affected r2: Uploading archive without any subfolder |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.521-4: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance requirements".

[4]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[5]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification"

[6]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification"

[7]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification"

[8]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification"

[9]	3GPP TS 38.523: "5GS; User Equipment (UE) conformance specification; Part 1: Protocol"

[10]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"

[11]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification"

[12]	RFC 768

[13]	RFC 791

[14]	RFC 793

[15]	RFC 2460

[16]	RFC 8200

[xx]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[yy]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment"

# < Unchanged sections omitted >

Annex B: Specific Test Conditions and Environment

# B.1 Upper Layer configurations

## B1.1    MAC Configurations

For data throughput testing one DRB configuration shall be used and there will be no SRB data in the downlink and uplink. The Modulation order and transport block size are determined as per section 5.1.3 of TS 38.214 [xx].The parameters for DRB configurations will follow the Table 4.8.1-3 with n=1, m=0 (1 AM DRB) of TS 38.508-1 [yy].

## B.1.2    RLC Configuration

For NR data throughput testing purposes RLC in AM mode only will be used. The RLC configuration and RLC Bearer parameters shall follow Table 4.6.3-149 using condition AM and Table 4.6.3-148 using condition AM of TS 38.508-1 [yy] respectively.

## B.1.3    PDCP Configuration

For NR data throughput testing PDCP header compression will not be enabled. PDCP configuration for DRB in RLC AM mode shall follow Table 4.6.3-99 using condition AM (default is AM) of TS 38.508-1 [yy].

# B.2 UL RMC

Table B.2 (Copied from Table A.2.3.1.1-2 from 38.521-4) can be used to configure PUSCH TB to carry upper layer ACK/NAK for TCP

Table B.2-2: Reference Channels for CP-OFDM 16QAM for 30kHz SCS Parameter; Channel bandwidth; Subcarrier Spacing; Allocated resource blocks; CP-OFDM Symbols per slot (Note 1); Modulation; MCS Index (Note 2); Target Coding Rate; Payload size for slots 8, 9, 18 and 19; Transport block CRC; LDPC Base Graph; Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3); Total number of bits per slot for slots 8, 9, 18 and 19; Total modulated symbols per slot for slots 8, 9, 18 and 19. Unit; MHz; KHz; Bits; Bits; Bits. 5-50; 30; 1; 11; 16QAM; 10; 1/3; 176; 16; 2; 1; 528; 132. 5; 30; 6; 11; 16QAM; 10; 1/3; 1064; 16; 2; 1; 3168; 792. 5; 30; 11; 11; 16QAM; 10; 1/3; 1928; 16; 2; 1; 5808; 1452. 10; 30; 12; 11; 16QAM; 10; 1/3; 2088; 16; 2; 1; 6336; 1584. 10; 30; 24; 11; 16QAM; 10; 1/3; 4224; 24; 1; 1; 12672; 3168. 15; 30; 19; 11; 16QAM; 10; 1/3; 3368; 16; 2; 1; 10032; 2508. 15; 30; 38; 11; 16QAM; 10; 1/3; 6656; 24; 1; 1; 20064; 5016. 20; 30; 26; 11; 16QAM; 10; 1/3; 4480; 24; 1; 1; 13728; 3432. 20; 30; 51; 11; 16QAM; 10; 1/3; 8968; 24; 1; 2; 26928; 6732. 25; 30; 33; 11; 16QAM; 10; 1/3; 5760; 24; 1; 1; 17424; 4356. 25; 30; 65; 11; 16QAM; 10; 1/3; 11272; 24; 1; 2; 34320; 8580. 30; 30; 39; 11; 16QAM; 10; 1/3; 6784; 24; 1; 1; 20592; 5148. 30; 30; 78; 11; 16QAM; 10; 1/3; 13576; 24; 1; 2; 41184; 10296. 40; 30; 53; 11; 16QAM; 10; 1/3; 9224; 24; 1; 2; 27984; 6996. 40; 30; 106; 11; 16QAM; 10; 1/3; 18432; 24; 1; 3; 55968; 13992. 50; 30; 67; 11; 16QAM; 10; 1/3; 11784; 24; 1; 2; 35376; 8844. 50; 30; 133; 11; 16QAM; 10; 1/3; 23040; 24; 1; 3; 70224; 17556. 60; 30; 81; 11; 16QAM; 10; 1/3; 14088; 24; 1; 2; 42768; 10692. 60; 30; 162; 11; 16QAM; 10; 1/3; 28168; 24; 1; 4; 85536; 21384. 80; 30; 109; 11; 16QAM; 10; 1/3; 18960; 24; 1; 3; 57552; 14388. 80; 30; 217; 11; 16QAM; 10; 1/3; 37896; 24; 1; 5; 114576; 28644. 90; 30; 123; 11; 16QAM; 10; 1/3; 21504; 24; 1; 3; 64944; 16236. 90; 30; 245; 11; 16QAM; 10; 1/3; 43032; 24; 1; 6; 129360; 32340. 100; 30; 137; 11; 16QAM; 10; 1/3; 24072; 24; 1; 3; 72336; 18084. 100; 30; 273; 11; 16QAM; 10; 1/3; 48168; 24; 1; 6; 144144; 36036. Note 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM’ed] with PUSCH data. Note 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [12]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).

# < Unchanged sections omitted >

# < Unchanged sections omitted >

# < Unchanged sections omitted >