3GPP TSG-RAN WG5 Meeting #94-e	R5-220523r1

Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0157 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the assistance data elements for NR positioning support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The SRS periodicity is not suitable to both FR1 and FR2. - In TC 9.3.1.2, the number of simulated cells for UE-Based methods are missing now. - IE nr-dl-tdoa-LocationInformation-r16 and nr-dl-AoD-LocationInformation-r16 are missing in the LPP ProvideLocationInformation messages now. The value of locationInformationType is not specified in the default message content in clause 8.4.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed the SRS periodicity to 160 slots in FR1 and 1280 slots in FR2. - Added simulated cells for sub-test 20 and sub-test 21 in TC 9.3.1.2. - Added IE nr-dl-tdoa-LocationInformation-r16 and nr-dl-AoD-LocationInformation-r16 in the message content in TC 9.3.3.1B, TC 9.3.4.1 and TC 9.3.4.2. - Specified the value of locationInformationType in the default message content in clause 8.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The assistance data elements used for protocol conformance test cases for NR positioning are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2, 8.4.1.6, 8.4.3, 9.3.1.2, 9.3.3.1B, 9.3.4.1, 9.3.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Added the definetion of missing IE prs-OnlyTP-r16 - Editorial errors |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

Table 8.3.2-4: SRS-PosResource-r16

| Derivation Path: TS 38.331, clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-PosResource-r16::=SEQUENCE { |  |  |  |
| srs-PosResourceId-r16 | 0 |  |  |
| transmissionComb-r16 CHOICE { |  |  |  |
| n4-r16 SEQUENCE { |  |  |  |
| combOffset-n4-r16 | 0 |  |  |
| cyclicShift-n4-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceMapping-r16 SEQUENCE { |  |  |  |
| startPosition-r16 | 0 |  |  |
| nrofSymbols-r16 | n4 |  |  |
| } |  |  |  |
| freqDomainShift-r16 | 0 |  |  |
| freqHopping-r16 SEQUENCE { |  |  |  |
| c-SRS-r16 | 0 |  |  |
| } |  |  |  |
| groupOrSequenceHopping-r16 | neither |  |  |
| resourceType-r16 CHOICE { |  |  |  |
| periodic-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-p-r16 CHOICE { |  |  |  |
| sl160 | 20 |  | FR1 |
| sl1280 | 160 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sequenceId-r16 | 0 |  |  |
| spatialRelationInfoPos-r16 CHOICE { |  |  |  |
| servingRS-r16 SEQUENCE { |  |  |  |
| servingCellId | 0 |  |  |
| referenceSignal-r16 CHOICE { |  |  |  |
| ssb-IndexServing-r16 | 1 | Set according to Table 4.4.2-2 in TS 38.508-1. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# < Unchanged sections omitted >

Table 8.4.1.6-1: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 1 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | 1 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 1 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 1 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 1 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 1 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| prs-OnlyTP-r16 | Not present |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | In case of sub-test 20 UE-based DL-AoD or sub-test 21 DL-TDOA method supported by the UE as defined in clause 8.2.10 and clause 8.2.11. |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | 2 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 2 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 2 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 2 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| prs-OnlyTP-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[2] SEQUENCE { |  | entry 2 | In case of sub-test 21 UE-based DL-TDOA method supported by the UE as defined in clause 8.2.11 |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 3 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 3 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | 3 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined in TS 38.508-1 [30] Table 4.4.2-3 for NR Cell 3 |  |  |
| mnc-r15 | As defined in TS 38.508-1 [30] Table 4.4.2-3 for NR Cell 3 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 3 Identifier defined in TS 38.508-1 [30] Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 3 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| prs-OnlyTP-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-SSB-Config-r16 SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF NR-SSB-Config-r16 { | 1 entry |  | In case of sub-test 21 UE-based DL-TDOA method supported by the UE as defined in clause 8.2.11 |
| NR-SSB-Config-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-PhysCellID-r16 | 3 |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 3 frequency | As defined in 38.508 -1 [30] subclause 6.2.3. |  |
| ss-PBCH-BlockPower-r16 | 0 |  |  |
| halfFrameIndex-r16 | 0 |  |  |
| ssb-periodicity-r16 | ms20 |  |  |
| ssb-PositionsInBurst-r16 | Not present |  |  |
| ssb-SubcarrierSpacing-r16 | Subcarrier spacing of SSB for NR Cell 3 |  |  |
| sfn-SSB-Offset-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

NR-SelectedDL-PRS-IndexList

Table 8.4.1.6-2: NR-SelectedDL-PRS-IndexList

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-SelectedDL-PRS-IndexList-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-SelectedDL-PRS-PerFreq-r16 { | 2 entries |  |  |
| NR-SelectedDL-PRS-PerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-SelectedDL-PRS-FrequencyLayerIndex-r16 | 0 |  |  |
| nr-SelectedDL-PRS-IndexListPerFreq-r16 | Not present |  |  |
| } |  |  |  |
| NR-SelectedDL-PRS-PerFreq-r16[2] SEQUENCE { |  | entry 2 | Sub-test 21 UE-Based only as defined in clause 8.2.11 |
| nr-SelectedDL-PRS-FrequencyLayerIndex-r16 | 1 |  |  |
| nr-SelectedDL-PRS-IndexListPerFreq-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

# < Unchanged sections omitted >

-	LPP REQUEST LOCATION INFORMATION

Table 8.4-3: RequestLocationInformation

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | Dependent on test case |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | 32 |  |  |
| responseTimeEarlyFix-r12 | Not present | Rel-12 onwards |  |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | As defined in Table 5.4-4 |  | Sub-tests 7 and 15 |
| otdoa-RequestLocationInformation | As defined in Table 5.4-5 |  | Sub-test 5 and 7 |
| ecid-RequestLocationInformation | As defined in Table 5.4-6 |  | Sub-test 6 |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | As defined in Table 5.4-10 | Rel-13 onwards | Sub-test 14, 18 |
| tbs-RequestLocationInformation-r13 | As defined in Table 5.4-7 | Rel-13 onwards | Sub-tests 12, 16 |
| wlan-RequestLocationInformation-r13 | As defined in Table 5.4-8 | Rel-13 onwards | Sub-test 11, 17 |
| bt-RequestLocationInformation-r13 | As defined in Table 5.4-9 | Rel-13 onwards | Sub-test 13 |
| nr-ECID-RequestLocationInformation-r16 | As defined in Table 8.4-4 | Rel-16 onwards | Sub-test 22 |
| nr-Multi-RTT-RequestLocationInformation-r16 | As defined in Table 8.4-5 | Rel-16 onwards | Sub-test 19 |
| nr-DL-AoD-RequestLocationInformation-r16 | As defined in Table 8.4-6 | Rel-16 onwards | Sub-test 20 |
| nr-DL-TDOA-RequestLocationInformation-r16 | As defined in Table 8.4-7 | Rel-16 onwards | Sub-test 21 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# < Unchanged sections omitted >

9.3.1.2	LPP Abort

Editor's note: Test configuration D is incomplete:

-	The corresponding attach procedure for NG-RAN E-UTRA has not yet been defined.

-	The message contents need to be revised for Test Configuration D.

9.3.1.2.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {
  when {	UE receives a LPP Abort message carrying the transaction ID of an on-going procedure }

then { UE aborts the on-going procedure }

}

9.3.1.2.2	Conformance requirements

As defined in clause 7.3.5.1.2.

9.3.1.2.3	Test description

9.3.1.2.3.1	Pre-test conditions

System Simulator:

-	For Test Configuration B (Table 9.3.1.2.3.2-1):

-	Sub-tests 11, 12, 13, 15, 16, 17, 19: NR Cell 1.

-	Sub-test 5: NR Cell 1 and independent LTE Cell 1, as specified in 8.2.2.

-	Sub-test 20: NR Cell 1 and NR Cell 2 (only applicable to UE-Based DL-TDOA).

-	Sub-test 21: NR Cell 1, NR Cell 2 and NR Cell 3 (only applicable to UE-Based DL-AoD).

-	For Test Configuration D (Table 9.3.1.2.3.2-1):

-	Sub-tests 11, 12, 13, 15, 16, 17: LTE Cell 1.

-	Sub-test 5: LTE Cell 1, as specified in 8.2.2.

# < Unchanged sections omitted >

Table 9.3.3.1B.3.3-7: LPP Provide Location Information (step 3, Table 7.3.3.1B.3.2-1)

| Derivation Path: Table 7.3.3.1B.3.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.3.1B.3.3-7 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 SEQUENCE { |  | Present if UE supports UE-assisted Multi-RTT. | Rel-16 onwards |  |
| nr-Multi-RTT-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-Multi-RTT-Error-r16 |  | May be present |  |  |
| } |  |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { |  | Present if UE supports UE-assisted DL-AoD. | Rel-16 onwards |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-dl-AoD-LocationInformation-r16 |  | Not present |  |  |
| nr-DL-AoD-Error-r16 |  | May be present |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  | Present if UE supports UE-assisted DL-TDOA. | Rel-16 onwards |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  | Not present |  |  |
| nr-DL-TDOA-Error-r16 |  | May be present |  |  |
| } |  |  |  |  |
| nr-ECID-ProvideLocationInformation-r16 SEQUENCE { |  | Present if UE supports UE-assisted NR E-CID. | Rel-16 onwards |  |
| nr-ECID-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-ECID-Error-r16 |  | May be present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.4.1.3.3-6: LPP Provide Location Information (step 4, Table 7.3.4.1.3.2-1)

| Derivation Path: Table 7.3.4.1.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.1.3.3-8 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 20 | Rel-16 onwards |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-dl-AoD-LocationInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-DL-AoD-Error-r16 |  | Not present |  |  |
| } |  |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  | Present for sub-test 21 | Rel-16 onwards |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  | Present. Any value acceptable |  |  |
| nr-DL-TDOA-Error-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.4.2.3.3-5: LPP Request Location Information (step 3, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 8.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 8.4-3 with the following exceptions: |  |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |  |
| locationInformationType |  | locationMeasurementsRequired |  |  |
| } |  |  |  |  |
| ecid-RequestLocationInformation SEQUENCE { |  |  |  |  |
| requestedMeasurements |  | Test Configuration D: bits 0, 1, 2 = 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

# < Unchanged sections omitted >

Table 9.3.4.2.3.3-7: LPP Provide Location Information (steps 4 a1, 4b1 and 4b3, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 7.3.4.2.3.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 7.3.4.2.3.3-9 with the following exceptions: |  |  |  |
| LPP-Message ::= SEQUENCE { |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| nr-ECID-ProvideLocationInformation-r16 SEQUENCE { | Present for sub-test 22 | Rel-16 onwards |  |
| nr-ECID-SignalMeasurementInformation-r16 | Present. Any value acceptable |  |  |
| nr-ECID-Error-r16 | May be present |  |  |
| } |  |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 SEQUENCE { | Present for sub-test 19 | Rel-16 onwards |  |
| nr-Multi-RTT-SignalMeasurementInformation-r16 | Present. Any value acceptable |  |  |
| nr-Multi-RTT-Error-r16 | May be present |  |  |
| } |  |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 SEQUENCE { | Present for sub-test 20 | Rel-16 onwards |  |
| nr-DL-AoD-SignalMeasurementInformation-r16 | Present. Any value acceptable |  |  |
| nr-dl-AoD-LocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-Error-r16 | May be present |  |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { | Present for sub-test 21 | Rel-16 onwards |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 | Present. Any value acceptable |  |  |
| nr-dl-tdoa-LocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-Error-r16 | May be present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |