3GPP TSG-RAN5 Meeting #94-e	R5-220674r1

# Online, , 21st Feb 2022 – 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5398 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing editors note in CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Removing editor’s notes due to adding applicability in 36.521-2 and Annex F in 36.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove editor’s notes in test cases 9.6.1.1_A.5, 9.6.1.1_A.6, 9.6.1.2_A.5 and 9.6.1.2_A.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.1.1_A.5, 9.6.1.1_A.6, 9.6.1.2_A.5, 9.6.1.2_A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correct the CR coversheet title. |  |  |  |  |  |  |  |  |

<< Beginning of changes >>

9.6.1.1_A.5	FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (6DL CA)

9.6.1.1_A.5.1	Test purpose

To verify that the UE reports different CQI indices for Pcell and Scells if each cell experiences a different SNR.

<< Skip unchanged sections >>

9.6.1.1_A.6	FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (7DL CA)

9.6.1.1_A.6.1	Test purpose

To verify that the UE reports different CQI indices for Pcell and Scells if each cell experiences a different SNR.

<< Skip unchanged sections >>

9.6.1.2_A.5	TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (6DL CA)

9.6.1.2_A.5.1	Test purpose

To verify that the UE reports different CQI indices for Pcell and Scells if each cell experiences a different SNR.

# << Skip unchanged sections >>

9.6.1.2_A.6	TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (7DL CA)

9.6.1.2_A.6.1	Test purpose

To verify that the UE reports different CQI indices for Pcell and Scells if each cell experiences a different SNR.

9.6.1.2_A.6.2	Test applicability

This test case applies to all types of E-UTRA TDD release 14 and forward UE of category 8 and 11 or higher that support 7DL with Intra-band contiguous and Inter-band CA or 7DL with Intra-band non-contiguous and Inter-band CA or 7DL with Inter-band CA.

Note: This test also applies to UE supporting 4Rx antenna ports

9.6.1.2_A.6.3	Minimum conformance requirements

The following requirements for 7DL CA apply to UE Category 8 and ≥11. For CA with 7 DL CC, for the parameters specified in Table 9.6.1.2_A.6.3-1 and Table 9.6.1.2_A.6.3-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell 1 and SCell2, SCell 1 and SCell 3, SCell 1 and SCell 4, SCell 1 and SCell 5, SCell 1 and SCell 6 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ [2]

wideband CQISCell1 – wideband CQISCell2 ≥ [2]

wideband CQISCell1 – wideband CQISCell3 ≥ [2]

wideband CQISCell1 – wideband CQISCell4 ≥ [2]

wideband CQISCell1 – wideband CQISCell5 ≥ [2]

wideband CQISCell1 – wideband CQISCell6 ≥ [2]

for more than 90% of the time.

Table 9.6.1.2_A.6.3-1: Parameters for PUCCH 1-0 static test on multiple cells (TDD, 7 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3, 4, 5, 6 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 (shift of 10 ms relative to Pcell), 53 for Scell3 (shift of 15ms relative to Pcell), 58 for Scell4 (shift of 20ms relative to Pcell), 63 for Scell5 (shift of 25ms relative to Pcell), 68 for Scell6 (shift of 30ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1. Note 2: Void |  |  |  |  |  |

Table 9.6.1.2_A.6.3-2: PUCCH 1-0 static test (TDD, 7 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 7x20 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The normative reference for this requirement is TS 36.101 [2] clause 9.6.1.2.

9.6.1.2_A.6.4	Test description

9.6.1.2_A.6.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Maximum Wgap for Intra-band non-contiguous CA, otherwise Mid Range, as defined in TS 36.508 [7] clause 4.3.1.

Channel Bandwidths to be tested: Select one according to Table 9.6.1.2_A.6.4.1-1.

CA Capability to be tested: Select one according to Table 9.6.1.2_A.6.4.1-1.

Table 9.6.1.2_A.6.4.1-1: Test point selection for PUCCH 1-0 static test

| CA Applicability | Bandwidth combination |
| --- | --- |
|  | 6x20 |
| Intra-band contiguous (CA7_C) | Test 1 |
| Inter-band (CA7_A2) | Test 1 |
| Inter-band (CA7_A3) | Test 1 |
| Inter-band (CA7_A4) | Test 1 |
| Inter-band (CA7_A5) | Test 1 |
| Intra-band non-contiguous (CA7_N2) | Test 1 |
| Note 1: Only one test point is tested. Select the largest aggregated CA bandwidth combination supported by the UE among the UE supported CA capabilities. |  |

1.	Connect the SS and AWGN noise source to the UE antenna connector (s) as shown in TS 36.508 [7] Annex A, Figure A.59 without faders for UE supporting only 2Rx RF bands on all CC. Annex A, Figure A.86 for UE supporting 4Rx RF band on any of the CC.

2.	The parameter settings for the cell are set up according to Table 9.6.1.2_A.6.3-1.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 9.6.1.2_A.6.4.3.

9.6.1.2_A.6.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4. Message contents for RadioResourceConfigDedicatedSCell-r10-DEFAULT are defined Table 9.6.1.2_A.6.4.3-4, PhysicalConfigDedicatedSCell-r10-DEFAULT is defined in Table 9.6.1.2_A.6.4.3-5, CQI- ReportConfigSCellr10-DEFAULT is defined in Table 9.6.1.2_A.6.4.3-6, CQI-ReportPeriodic-r10-DEFAULT is defined in Table 9.6.1.2_A.6.4.3-7.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at 	least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	Set the parameters of bandwidth, reference channel, propagation condition, antenna configuration and the SNR according to Table 9.6.1.2_A.6.5-1 as appropriate.

5.	For a UE with 4Rx antenna capability, SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC on both PCC and SCCs, according to Table 9.6.1.2_A.6.5-1. The SS sends downlink MAC padding bits on the DL RMC. The SS sends uplink scheduling information via PDCCH DCI format 0 with CQI request bit set to 0 to schedule UL RMC when receiving CQI report. The UE will send ACK/NACK and periodic CQI report using PUSCH.

6.	The SS shall start gathering CQI reports, and will continue gathering CQI reports until 2000 wideband CQI reports have been gathered for each P-Cell and S-Cells. For each CSI report calculate the respective difference CQIP-S1 = wideband CQIPcell – wideband CQIScell1, the respective difference CQIS1-S2 = wideband CQIScell1 – wideband CQIScell2, the respective difference CQIS1-S3 = wideband CQIScell1 – wideband CQIScell3 and the respective difference CQIS1-S4 = wideband CQIScell1 – wideband CQIScell4. the respective difference CQIS1-S5 = wideband CQIScell1 – wideband CQIScell5 and the respective difference CQIS1-S6 = wideband CQIScell1 – wideband CQIScell6

7.	If more than 1800 values of CQIP-S1 are ≥ 2 and more than 1800 values of CQI S1-S2 are ≥ 2 and more than 1800 values of CQI S1-S3 are ≥ 2 and more than 1800 values of CQI S1-S4 are ≥ 2 and more than 1800 values of CQI S1-S5 are ≥ 2 and more than 1800 values of CQI S1-S6 are ≥ 2 and if all the conditions are met, then pass the UE. Otherwise fail the UE.

9.6.1.2_A.6.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the following exceptions:

Table 9.6.1.2_A.6.4.3-1: RadioResourceConfigDedicated-SRB2-DRB(n,m)

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB(n, m) ::= SEQUENCE { |  |  |  |  |
| physicalConfigDedicated |  | PhysicalConfigDedicated- DEFAULT |  | RBC |
| } |  |  |  |  |

Table 9.6.1.2_A.6.4.3-2: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig | CQI-ReportConfig- DEFAULT |  | RBC |
| } |  |  |  |

Table 9.6.1.2_A.6.4.3-3: CQI-ReportConfig-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { | setup |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex | 0 |  |  |
| cqi-pmi-ConfigIndex | 38 | (see Table 7.2.2-1A in TS 36.213) |  |
| cqi-FormatIndicatorPeriodic CHOICE { | widebandCQI |  |  |
| widebandCQI | NULL |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL | (see Table 7.2.2-1B in TS 36.213) |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.6.1.2_A.6.4.3-4: RadioResourceConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| physicalConfigDedicatedSCell-r10 | PhysicalConfigDedicatedSCell-r10-DEFAULT |  |  |
| } |  |  |  |

Table 9.6.1.2_A.6.4.3-5: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  |  |
| cqi-ReportConfigSCell-r10 | CQI-ReportConfigSCell-r10-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.6.1.2_A.6.4.3-6: CQI-ReportConfigSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfigSCell-r10 ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic-r10 | Not present |  |  |
| nomPDSCH-RS-EPRE-Offset-r10 | 0 |  |  |
| cqi-ReportPeriodicSCell-r10 | CQI-ReportPeriodic-r10-DEFAULT |  | CQI_PERIODIC |
| pmi-RI-Report-r10 | Not present |  |  |
| } |  |  |  |

Table 9.6.1.2_A.6.4.3-7: CQI-ReportPeriodic-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportPeriodic-r10 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cqi-PUCCH-ResourceIndex-r10 | 0 |  |  |
| cqi-PUCCH-ResourceIndexP1-r10 | Not present |  |  |
| cqi-pmi-ConfigIndex | 43 |  | Scell1 |
|  | 48 |  | Scell2 |
|  | 53 |  | Scell3 |
|  | 58 |  | Scell4 |
|  | 63 |  | Scell5 |
|  | 68 |  | Scell6 |
| cqi-FormatIndicatorPeriodic-r10 CHOICE { |  |  |  |
| widebandCQI-r10 SEQUENCE { |  |  |  |
| csi-ReportMode-r10 | Not present |  |  |
| } |  |  |  |
| ri-ConfigIndex | NULL |  |  |
| simultaneousAckNackAndCQI | FALSE |  |  |
| cqi-Mask-r9 | Not present |  |  |
| csi-ConfigIndex-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

9.6.1.2_A.6.5	Test requirement

Table 9.6.1.2_A.6.5-1: Parameters for PUCCH 1-0 static test on multiple cells (TDD, 7 DL CA)

| Parameter |  | Unit | Pcell | Scell1 | Scell2, 3, 4, 5, 6 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 |  |  |
| Special subframe configuration |  |  | 4 |  |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
| for CC on 2Rx supported RF band |  | dB[mW/15kHz] | -86 | -92 | -98 |
| for CC on 4Rx supported RF band |  | dB[mW/15kHz] | -89 | -95 | -101 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 (shift of 10 ms relative to Pcell), 53 for Scell3 (shift of 15ms relative to Pcell), 58 for Scell4 (shift of 20ms relative to Pcell), 63 for Scell5 (shift of 25ms relative to Pcell), 68 for Scell6 (shift of 30ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH.Note 2: The power level requirement for CC on 4Rx supported RF band is as specified 9.1.1.4.2. Note 3: For a UE without 4Rx antenna capability, no PDSCH for user data is scheduled for the UE with one-sided dynamic OCNG pattern OP.1 TDD as described in Annex A.5.2.1 Note 4: For a UE with 4Rx antenna capability, use reference measurement channel RC.1 TDD according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 for both PCell and SCells |  |  |  |  |  |

The pass fail decision is as specified in the test procedure in clause 9.6.1.2_A.6.4.2.

There are no parameters in the test setup or measurement process whose variation impacts the results so there are no applicable test tolerances for this test.

# << End of changes >>