3GPP TSG-RAN5 Meeting #94-e	R5-220677r1

# Online, , 21st Feb 2022 – 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2611 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case in Annex F.1.2 and Annex F.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Annex F are missing for test cases 9.1.71, 9.1.72, 9.2.58 and 9.2.59 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 9.1.71, 9.1.72, 9.2.58 and 9.2.59 need to be added to Annex F.1.2 and Annex F.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F.1.2 and Annex F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 : correct the Annex F.1.2 for test cases 9.1.72 |  |  |  |  |  |  |  |  |

<< Beginning of changes >>

F.1.2	Measurement of RRM requirements

Table F.1.2-1: Maximum Test System Uncertainty for RRM Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- |
| 4.2.1 E-UTRA FDD - FDD cell re-selection intra frequency | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |  |
| 4.2.2 E-UTRA TDD - TDD cell re-selection intra frequency | Same as 4.2.1 |  |  |  |
| << Unchanged rows omitted >> |  |  |  |  |
| 9.1.68 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.37 |  | Same as 9.1.37 |  |
| 9.1.69 4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.44 |  | Same as 9.1.44 |  |
| 9.1.70 5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.48 |  | Same as 9.1.48 |  |
| 9.1.71 6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN Noc5 is the AWGN on frequency 5 Ês8 / Noc5 is the ratio of cell 8 signal / AWGN Ês9 / Noc5 is the ratio of cell 9 signal / AWGN Noc6 is the AWGN on frequency 6 Ês10 / Noc6 is the ratio of cell 10 signal / AWGN Ês11 / Noc6 is the ratio of cell 11 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |  |
| 9.1.72 7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc7 ±1.3 dB over UE Measurement bandwidth Noc7 ±1.0 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6, Ês12 / Noc7, Ês13 / Noc7 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6, Ês12 / Noc7, Ês13 / Noc7 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN Noc5 is the AWGN on frequency 5 Ês8 / Noc5 is the ratio of cell 8 signal / AWGN Ês9 / Noc5 is the ratio of cell 9 signal / AWGN Noc6 is the AWGN on frequency 6 Ês10 / Noc6 is the ratio of cell 10 signal / AWGN Ês11 / Noc6 is the ratio of cell 11 signal / AWGN Noc7 is the AWGN on frequency 7 Ês12 / Noc7 is the ratio of cell 12 signal / AWGN Ês13 / Noc7 is the ratio of cell 13 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |  |
| 9.2.1.1 FDD Intra Frequency Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-2 7 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 9.2.1.1_1 FDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  |
| 9.2.1.1_2 FDD Intra Frequency Absolute RSRQ Accuracy for UE category 1bis | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  |
| 9.2.1.1_3 FDD Intra Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  |
| 9.2.2.1 TDD Intra Frequency Absolute RSRQ Accuracy | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  |
| 9.2.2.1_1 TDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  |
| 9.2.2.1_2 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 1bis | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  |
| 9.2.3.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |  |
| 9.2.3.1_2 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 |  | Same as 9.2.3.1 |  |
| 9.2.3.1_3 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier | Same as 9.2.3.1 |  | Same as 9.2.3.1 |  |
| 9.2.3.1_4 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Non-Overlapping Carrier | Same as 9.2.3.1 |  | Same as 9.2.3.1 |  |
| 9.2.3.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy | Same as 9.2.3.1 |  |  |  |
| 9.2.3.2_2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 |  | Same as 9.2.3.1 |  |
| 9.2.4.1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy | Same as 9.2.3.1 |  |  |  |
| 9.2.4.1_1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy (Rel-12 and forward) | Same as 9.2.3.1 |  |  |  |
| 9.2.4.1_2 TDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 |  | Same as 9.2.3.1 |  |
| 9.2.4.2 TDD - TDD Inter Frequency Relative RSRQ Accuracy | Same as 9.2.3.1 |  |  |  |
| 9.2.4.2_1 TDD - TDD Inter Frequency Relative RSRQ Accuracy (Rel-12 and forward) | Same as 9.2.3.2 |  |  |  |
| 9.2.4.2_2 TDD - TDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 |  | Same as 9.2.3.1 |  |
| 9.2.4A.1 FDD - TDD Inter Frequency Absolute RSRQ Accuracy | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |  |
| 9.2.4A.1_2 FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.4A.1 |  | Same as 9.2.4A.1 |  |
| 9.2.4A.2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ | Same as 9.2.4A.1 |  |  |  |
| 9.2.4A.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis | Same as 9.2.4A.1 |  | Same as 9.2.4A.1 |  |
| 9.2.5.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |  |
| 9.2.5.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |  |  |
| 9.2.6.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |  |  |
| 9.2.6.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |  |  |
| 9.2.7.1 FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 9.2.8.1 TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.2.7.1 |  | Same as 9.2.7.1 |  |
| 9.2.9.1 FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 9.2.10.1 TDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 9.2.9.1 |  |  |  |
| 9.2.11.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #47-52 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |  |
| 9.2.11.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |  |  |
| 9.2.12.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |  |  |
| 9.2.12.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |  |  |
| 9.2.15.1 FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |  |
| 9.2.16.1 TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 9.2.15.1 |  | Same as 9.2.15.1 |  |
| 9.2.17.1 FDD Intra Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #10-15 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #10-15 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 9.2.18.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #10-15 |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.18.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for 5MHz Bandwidth | Same as 9.2.18.1 |  |  |
| 9.2.19.1 FDD-FDD Inter Frequency absolute WB-RSRQ accuracy | FFS |  | FFS |
| 9.2.20.1 TDD-TDD Inter Frequency absolute WB-RSRQ accuracy | FFS |  | FFS |
| 9.2.21.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #22-27 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.21.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |  |
| 9.2.22.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |  |
| 9.2.22.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |  |
| 9.2.23.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.23.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 |  | Same as 9.2.23.1 |
| 9.2.24.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 |  | Same as 9.2.23.1 |
| 9.2.24.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 |  | Same as 9.2.23.1 |
| 9.2.25.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.25.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Same as 9.2.25.1 |  |  |
| 9.2.26.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Same as 9.2.25.1 |  |  |
| 9.2.26.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Same as 9.2.25.1 |  |  |
| 9.2.27.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.27.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.28 FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.29 TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Same as 9.2.28 |  | Same as 9.2.28 |
| 9.2.30 FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.31 TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Same as 9.2.30 |  | Same as 9.2.30 |
| 9.2.32 FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.33 TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Same as 9.2.32 |  | Same as 9.2.32 |
| 9.2.38 3DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.39 3DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.38 |  | Same as 9.2.38 |
| 9.2.40 3DL FDD RSRQ for E-UTRA Carrier Aggregation | Same as 9.2.38 |  | Same as 9.2.38 |
| 9.2.41 3DL TDD RSRQ for E-UTRA Carrier Aggregation | Same as 9.2.38 |  | Same as 9.2.38 |
| 9.2.42.1 FD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.43.1 HD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Same as 9.2.42.1 |  | Same as 9.2.42.1 |
| 9.2.44.1 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Same as 9.2.42.1 |  | Same as 9.2.42.1 |
| 9.2.45 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.46 4DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.45 |  | Same as 9.2.45 |
| 9.2.47 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc4 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.48 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.47 |  | Same as 9.2.47 |
| 9.2.51 FDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB averaged over UE Measurement bandwidth Noc2 ±1.5 dB averaged over BWConfig Noc2 ±1.8 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB averaged over UE Measurement bandwidth Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) |  | Note: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.52 TDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Same as 9.2.51 |  | Same as 9.2.51 |
| 9.2.55 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.38 |  | Same as 9.2.38 |
| 9.2.56 4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.45 |  | Same as 9.2.45 |
| 9.2.57 5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.47 |  | Same as 9.2.47 |
| 9.2.58 6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5ㄝÊs6 / Noc6 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.59 7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Noc7 ±1.0 dB averaged over BWConfig Noc7 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6, Ês7 / Noc7 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6, Ês7 / Noc7 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) |  | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Noc7 is the AWGN on frequency 7 Ês7 / Noc7 is the ratio of cell 7 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.3.1 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB |  | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 9.3.2 E-UTRAN TDD - UTRA FDD CPICH RSCP absolute accuracy | Same as 9.3.1 |  |  |
| 9.3.3 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth | Same as 9.3.1 |  |  |
| 9.4.1 E-UTRAN FDD – UTRA FDD CPICH Ec/No absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB |  | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 9.4.2 E-UTRAN TDD – UTRA FDD CPICH Ec/No absolute accuracy | Same as 9.4.1 |  | Same as 9.4.1 |
| 9.4.3 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth | Same as 9.4.1 |  | Same as 9.4.1 |
| 9.5.1 E-UTRAN FDD – UTRA TDD P-CCPCH RSCP absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB PCCPCH EC/Ior ±0.1 dB DwPCH_EC/Ior ±0.1 dB |  | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec / Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 9.5.2 E-UTRAN TDD – UTRA TDD P-CCPCH RSCP absolute accuracy | Same as 9.5.1 |  |  |
| 9.6.1 GSM RSSI accuracy for E-UTRAN FDD | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell BCCH1 Signal level ±0.7 dB GSM cell BCCH 2 to 6 Signal level ±2.0 dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN GSM cells BCCH 1 to 6 have only the wanted signal, without AWGN |
| 9.6.2 GSM RSSI accuracy for E-UTRAN TDD | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell BCCH1 Signal level ±0.7 dB GSM cell BCCH 2 to 6 Signal level ±2.0 dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN GSM cells BCCH 1 to 6 have only the wanted signal, without AWGN |
| 9.9.1.1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.1.1_1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.9.1.1 |  | Same as 9.9.1.1 |
| 9.9.1.2 FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.2.1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.2.1_1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.9.2.1 |  | Same as 9.9.2.1 |
| 9.9.2.2 TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.11.1 FS3 average RSSI accuracy case (PCell using FDD) | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for measurement PRBs Noc2 ±1.5 dB averaged over BWConfig Noc2 ±1.8 dB for measurement PRBs Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for measurement PRBs |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.11.2 FS3 average RSSI accuracy case (PCell using TDD) | Same as 9.11.1 |  | Same as 9.11.1 |
| 9.12.1 FS3 channel occupancy test (PCell using FDD) | Same as 9.11.1 |  | Same as 9.11.1 |
| 9.12.2 FS3 channel occupancy test (PCell using TDD) | Same as 9.11.1 |  | Same as 9.11.1 |
| 10.2 E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication | ± 0.6dB |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 10.4 E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication | ± 0.6dB |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 11.1 V2V UE Transmission Timing Accuracy Test | ±3Ts Uplink signal transmit timing relative to GNSS synchronization source Absolute GNSS signal level, ±1 dB |  | Note: TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] Same value as the A-GNSS Time Assistance tests in section 5 of TS 37.571-1 |
| 11.2 Interruptions due to V2V sidelink communication | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB Absolute GNSS signal level, ±1 dB |  | Note: Ês / Noc is the ratio of cell 1 signal / AWGN Same value as the A-GNSS Time Assistance tests in section 5 of TS 37.571-1 |
| 12.1.1 V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference | synchronization source Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to GNSS |  | Note: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 12.1.2 V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink |  | Note: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 12.2.1 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês / Noc is the ratio of cell 1 signal / AWGN |
| 12.2.2 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference | Noc ±1.5 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês / Noc is the ratio of SyncRef UE signal / AWGN |
| 12.3.1 V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority | Noc2 ±1.0 dB averaged over BWConfig SCH Ês1 / Noc ±0.3 dB averaged over BWConfig SCH Ês2 / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Ês2 / Noc is the ratio of SyncRef UE 2 signal / AWGN |
| 12.3.2 V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority | Noc2 ±1.0 dB averaged over BWConfig SCH Ês1 / Noc ±0.3 dB averaged over BWConfig SCH Ês2 / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Ês2 / Noc is the ratio of SyncRef UE 2 signal / AWGN |
| 12.4 Congestion Control Measurement Test for V2X UE | Noc ±1.5 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês / Noc is the ratio of Active UE signal / AWGN |
| 12.5 Interruptions due to V2X Sidelink Communication | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| 12.6.1 V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| 12.6.2 V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig |  | Note: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| In addition, the following Test System uncertainties and related constraints apply. Any additional constraints are defined in the specific tests. |  |  |  |
| AWGN Bandwidth |  |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty |  |  | Test-specific |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  |  | ±2 dB |
| AWGN peak to average ratio |  |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty |  |  | Test-specific |
| Fading profile power uncertainty |  |  | ±0.5 dB |
| Fading profile delay uncertainty, relative to frame timing |  |  | ±5 ns (excludes absolute errors related to baseband timing) |

<< Skip unchanged sections >>

F.3.2	Measurement of RRM requirements

Because the relationships between the Test system uncertainties and the Test Tolerances are often complex, it is not always possible to give a simple derivation of the Test Requirement in this document. The analysis is recorded in 3GPP TR 36 903 [20].

Table F.3.2-1: Derivation of Test Requirements (RRM tests)

| Test |  |  |  |  |  |  |  |  |  | Minimum Requirement in TS 36.133 |  |  | Test Tolerance (TT) |  |  | Test Requirement in TS 36.521-3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.1 E-UTRA FDD – FDD cell re-selection intra frequency |  |  |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: +13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13.00dB |  |
| << Unchanged rows omitted >> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.43.1 TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 |  |  |  |  |  |  |  |  |  | Same as 9.1.41.1 |  |  | Same as 9.1.41.1 |  |  | Same as 9.1.41.1 |  |  |  |
| 9.1.43.2 TDD Intra Frequency Relative RSRP Accuracy for UE category 0 |  |  |  |  |  |  |  |  |  | Same as 9.1.41.2 |  |  | Same as 9.1.41.2 |  |  | Same as 9.1.41.2 |  |  |  |
| 9.1.61 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell |  |  |  |  |  |  |  |  |  | Same as 9.1.60 |  |  | Same as 9.1.60 |  |  | Same as 9.1.60 |  |  |  |
| 9.1.68 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  |  |  |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |
| 9.1.69 4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  |  |  |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |
| 9.1.70 5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  |  |  |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |
| 9.1.71 6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Ês8 / Noc5: +3dB Ês9 / Noc5: -1dB Ês10 / Noc6: +3dB Ês11 / Noc6: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 – Cell 1) and (Cell 4 – Cell 1), (Cell 6 – Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10): ±3dB |  |  |  | 0dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Ês8 / Noc5: +3dB Ês9 / Noc5: -0.2dB Ês10 / Noc6: +3dB Ês11 / Noc6: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2 and Cell 4, Cell 6, Cell 8, Cell 10 absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Relative: (Cell 2 – Cell 1) and (Cell 4 – Cell 1),(Cell 6 –Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
| 9.1.72 7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Ês8 / Noc5: +3dB Ês9 / Noc5: -1dB Ês10 / Noc6: +3dB Ês11 / Noc6: -1dB Ês12 / Noc7: +3dB Ês13 / Noc7: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 – Cell 1) and (Cell 4 – Cell 1), (Cell 6 – Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1), (Cell 12 – Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10), (Cell 13 – Cell 12): ±3dB |  |  |  | 0dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Ês8 / Noc5: +3dB Ês9 / Noc5: -0.2dB Ês10 / Noc6: +3dB Ês11 / Noc6: -0.2dB Ês12 / Noc6: +3dB Ês13 / Noc6: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2 and Cell 4, Cell 6, Cell 8, Cell 10, Cell 12 absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Relative: (Cell 2 – Cell 1) and (Cell 4 – Cell 1),(Cell 6 –Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1), (Cell 12 – Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10), (Cell 13 – Cell 12): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
| 9.2.1.1 FDD Intra Frequency Absolute RSRQ Accuracy |  |  |  |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  | Test 1: Noc: -85.51Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_14 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |
| 9.2.1.1_2 FDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  |  |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |
| 9.2.1.1_3 FDD Intra Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements |  |  |  |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |
| 9.2.2.1 TDD Intra Frequency Absolute RSRQ Accuracy |  |  |  |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -116dBm/15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  | Test 1: Noc: -85.51Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRQ_14 Test 3: Noc: -116dBm/15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 2 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |
| 9.2.2.1_2 TDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  |  |  |  |  |  |  | Same as 9.2.1.1_2 |  |  |  | Same as 9.2.1.1_2 |  |  | Same as 9.2.1.1_2 |  |  |
| 9.2.3.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy |  |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm/15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -119.5dBm to-116 dBm /15kHz depending on operating band Noc2: -119.2dBm -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |
| 9.2.3.1_2 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3.5dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm/15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions |  |  |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_02 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_21 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) Test 3: Noc1: -119.5dBm to-116 dBm /15kHz depending on operating band Noc2: -119.2dBm -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |
| 9.2.3.1_3 FDD – FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier |  |  |  |  |  |  |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -80 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB Reported RSRQ values: ±4 dB for normal conditions and ±5.5 dB for extreme conditions Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±5 dB for normal conditions and ±5.5 dB for extreme conditions Test 3: Noc1: -119.5 dBm to -116 dBm/15kHz depending on operating band Noc2: -119.5 dBm to -116 dBm/15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±5 dB for normal conditions and ±5.5 dB for extreme conditions |  |  |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -81,1 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB RSRQ 1 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 22 (ETC) Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 20 (ETC) Test 3: Noc1: -119.5 dBm to -116 dBm /15kHz depending on operating band Noc2: -119.2 dBm to -115.7 dBm /15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 20 (ETC) |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |
| 9.2.3.1_4 FDD – FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier |  |  |  |  |  |  |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -80 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB Reported RSRQ values: ±5 dB for normal conditions and ±6.5 dB for extreme conditions Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±6 dB for normal conditions and ±6.5 dB for extreme conditions Test 3: Noc1: -119.5 dBm to -116 dBm/15kHz depending on operating band Noc2: -119.5 dBm to -116 dBm/15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±6 dB for normal conditions and ±6.5 dB for extreme conditions |  |  |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -81,1 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 24 (ETC) Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 22 (ETC) Test 3: Noc1: -119.5 dBm to -116 dBm /15kHz depending on operating band Noc2: -119.2 dBm to -115.7 dBm /15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 22 (ETC) |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |
| 9.2.3.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy |  |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -81.1dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.2dBm to -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 |  |  |
|  |  |  |  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.2_2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±4dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB |  |  |  |  |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -81.1dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 10 to RSRQ_x + 10 (NTC) RSRQ_x - 12 to RSRQ_x + 12 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 11 to RSRQ_x + 13 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.2dBm to -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 11 to RSRQ_x + 13 |  |  |
|  |  |  |  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy |  |  |  |  |  | Same as 9.2.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1_1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy (Rel-12 and forward) |  |  |  |  |  | Same as 9.2.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1_2 TDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  |  |  |  | Same as 9.2.3.1_2 |  |  |  |  |  |  | Same as 9.2.3.1_2 |  |  | Same as 9.2.3.1_2 |  |  |
| 9.2.4.2 TDD - TDD Inter Frequency Relative RSRQ Accuracy |  |  |  |  |  | Same as 9.2.3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.2_1 TDD - TDD Inter Frequency Relative RSRQ Accuracy (Rel-12 and forward) |  |  |  |  |  | Same as 9.2.3.2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.2_2 TDD - TDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis |  |  |  |  |  | Same as 9.2.3.2_2 |  |  |  |  |  |  | Same as 9.2.3.2_2 |  |  | Same as 9.2.3.2_2 |  |  |
| 9.2.4A.1 FDD - TDD Inter Frequency Absolute RSRQ Accuracy |  |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4.0dB Ês2 / No2c: -4.0dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | Test 1: 0dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |
| 9.2.4A.1_2 FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3.5dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4.0dB Ês2 / No2c: -4.0dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions |  |  |  | Test 1: 0dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_02 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_21 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |
| 9.2.4A.2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ |  |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  |  | Test 1: -1.10dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -81.10dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 10 to RSRQ_x + 10 Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 10 to RSRQ_x + 10 |
|  |  |  |  |  |  |  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |
| 9.2.4A.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis |  |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±4dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB |  |  |  | Test 1: -1.10dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -81.10dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 10 to RSRQ_x + 10 (NTC) RSRQ_x - 12 to RSRQ_x + 12 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 12 to RSRQ_x + 12 Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 12 to RSRQ_x + 12 |
|  |  |  |  |  |  |  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |
| 9.2.5.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  |  |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |
| 9.2.5.2 FDD Relative RSRQ Accuracy E-UTRA for Carrier Aggregation |  |  |  |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |
| 9.2.6.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  |  |  |  |  |  |  | Same as 9.2.5.1 |  |  |  | Same as 9.2.5.1 |  |  | Same as 9.2.5.1 |
| 9.2.6.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  |  |  |  |  |  |  | Same as 9.2.5.2 |  |  |  | Same as 9.2.5.2 |  |  | Same as 9.2.5.2 |
| 9.2.7.1 FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  |  |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.8dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  | Test 1: Noc: -85.76Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRQ_00 to RSRQ_16 |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |
| 9.2.8.1 TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  |  |  |  |  |  |  | Subset of 9.2.7.1 |  |  |  | Same as 9.2.7.1 |  |  | Subset of 9.2.7.1 |
| 9.2.9.1 FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  |  |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.8dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  | Test 1: Noc: -85.76Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRQ_00 to RSRQ_16 |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |
| 9.2.10.1 TDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  |  |  |  |  |  |  | Subset of 9.2.9.1 |  |  |  | Same as 9.2.9.1 |  |  | Subset of 9.2.9.1 |
| 9.2.11.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  |  |  |  |  |  |  | Same as 9.2.5.1 |  |  |  | Same as 9.2.5.1 |  |  | Same as 9.2.5.1 |
| 9.2.11.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  |  |  |  |  |  |  | Same as 9.2.5.2 |  |  |  | Same as 9.2.5.2 |  |  | Same as 9.2.5.2 |
| 9.2.12.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  |  |  |  |  |  |  | Same as 9.2.5.1 |  |  |  | Same as 9.2.5.1 |  |  | Same as 9.2.5.1 |
| 9.2.12.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  |  |  |  |  |  |  | Same as 9.2.5.2 |  |  |  | Same as 9.2.5.2 |  |  | Same as 9.2.5.2 |
| 9.2.15.1 FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  |  |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: 0dB 0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB +1.0dB Via mapping |  |  | Test 1: Noc: -84.76Bm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -0.5dB RSRQ_05 to RSRQ_17 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -0.5dB RSRQ_05 to RSRQ_17 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 3 RSRQ from Ês1 / Noc, Ês2 / Noc, and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |
| 9.2.16.1 TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  |  |  |  |  |  |  | Subset of 9.2.15.1 |  |  |  | Same as 9.2.15.1 |  |  | Subset of 9.2.15.1 |  |  |  |
| 9.2.17.1 FDD Intra Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth |  |  |  |  |  |  |  |  | Test 1: Noc: -81.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -100.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  | Test 1: Noc: -82.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -100.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_14 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |  |
|  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |
| 9.2.18.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth |  |  |  |  |  |  |  |  | Test 1: Noc1: -77dBm/15kHz Noc2: -77dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: 113dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -77dBm/15kHz Noc2: -78.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -112.7dBm /15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |
| 9.2.18.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for 5MHz Bandwidth | Test 1: Noc1: -77dBm/15kHz Noc2: -77dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: 113dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  |  |  |  |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -78.1dBm/15kHz Noc2: -78.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -112.7dBm /15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 |  |  |  |  |
|  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.19.1 FDD-FDD Inter Frequency absolute WB-RSRQ accuracy | FFS |  |  |  |  |  |  |  | FFS |  |  |  | FFS |  |  |  |  |
| 9.2.20.1 TDD-TDD Inter Frequency absolute WB-RSRQ accuracy | FFS |  |  |  |  |  |  |  | FFS |  |  |  | FFS |  |  |  |  |
| 9.2.21.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.21.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |  |  |  |
|  |  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.22.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  |  |  |  | Same as 9.2.21.1 |  |  |  |  |  |  |  | Same as 9.2.21.1 | Same as 9.2.21.1 |  |  |  |  |  |
| 9.2.22.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  |  |  |  | Same as 9.2.21.2 |  |  |  |  |  |  |  | Same as 9.2.21.2 | Same as 9.2.21.2 |  |  |  |  |  |
| 9.2.23.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -3.7dB Ês3 / Noc2: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.23.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported RSRQ values: ±4dB |  |  |  |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -3.7dB Ês3 / Noc2: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |  |  |  |
| 9.2.24.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  |  |  |  | Same as 9.2.23.1 |  |  |  |  |  |  |  | Same as 9.2.23.1 | Same as 9.2.23.1 |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.24.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  |  |  |  | Same as 9.2.23.2 |  |  |  |  |  |  |  | Same as 9.2.23.2 | Same as 9.2.23.2 |  |  |  |  |  |
| 9.2.25.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD |  |  |  |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |  |  |  |  |  | Same as 9.2.38 Cell 1 and Cell 2 | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |  |  |  |
| 9.2.25.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD |  |  |  |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |  |  |  |  |  | Same as 9.2.38 Cell 1 and Cell 2 | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |  |  |  |
| 9.2.26.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD |  |  |  |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |  |  |  |  |  | Same as 9.2.39 Cell 1 and Cell 2 | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |  |  |  |
| 9.2.26.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD |  |  |  |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |  |  |  |  |  | Same as 9.2.39 Cell 1 and Cell 2 | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |  |  |  |
| 9.2.27.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.27.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.28 FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal |  |  |  |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  |  |  |  |  | Test 1: 0dB +0.4dB +0.4dB Via mapping | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.29 TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal |  |  |  |  |  | Test 1: Noc: -116dBm to -114dBm /15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  |  |  |  |  | Test 1: 0dB +0.4dB +0.4dB Via mapping | Test 1: Noc: -116dBm to -114dBm /15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.38 3DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation |  |  |  |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) and (Cell 3 – Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  |  |  |  |  | 0.3dB 0dB 0dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Absolute: Cell 1, Cell 2, and Cell 3 RSRQ_0 to RSRQ_14 Relative: (Cell 2 – Cell 1) and (Cell 3 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.30 FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal |  |  |  |  |  | Test 1: Noc1: -117.5dBm to -114dBm/15kHz depending on operating band Noc2: -117.5dBm to -114dBm/15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / No2c: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  |  |  |  |  | Test 1: 0dB 0.3dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -117.5dBm to-114 dBm /15kHz depending on operating band Noc2: -117.2dBm -113.7dBm /15kHz depending on operating band Ês1 / Noc1: -5.2dB Ês2 / No2c: -5.2dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 10 to RSRQ_x + 10 |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the absolute RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.31 TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal |  |  |  |  |  | Test 1: Noc1: -117.5dBm to -115.5dBm/15kHz depending on operating band Noc2: -117.5dBm to -115.5dBm/15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / No2c: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  |  |  |  |  | Test 1: 0dB 0.3dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -117.5dBm to-115.5 dBm /15kHz depending on operating band Noc2: -117.2dBm -115.2dBm /15kHz depending on operating band Ês1 / Noc1: -5.2dB Ês2 / No2c: -5.2dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 10 to RSRQ_x + 10 |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the absolute RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.39 3DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  |  |  |  | Same as 9.2.38 |  |  |  |  |  |  |  | Same as 9.2.38 | Same as 9.2.38 |  |  |  |  |  |
| 9.2.40 3DL FDD RSRQ for E-UTRA Carrier Aggregation |  |  |  |  |  | Same as 9.2.38 |  |  |  |  |  |  |  | Same as 9.2.38 | Same as 9.2.38 |  |  |  |  |  |
| 9.2.41 3DL TDD RSRQ for E-UTRA Carrier Aggregation |  |  |  |  |  | Same as 9.2.38 |  |  |  |  |  |  |  | Same as 9.2.38 | Same as 9.2.38 |  |  |  |  |  |
| 9.2.42.1 FD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  |  |  |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.43.1 HD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  |  |  |  | Same as 9.2.42.1 |  |  |  |  |  |  |  | Same as 9.2.42.1 | Same as 9.2.42.1 |  |  |  |  |  |
| 9.2.44.1 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  |  |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  |  |  |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 2 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.45 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  |  |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) and (Cell 4 – Cell 1) : ±4dB for normal conditions ±4dB for extreme conditions |  |  |  |  |  |  |  | 0.3dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês3 / Noc3: -5.20dB Absolute: Cell 1, Cell 2, Cell 3 and Cell 4 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) and (Cell 4 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.2.38, and no separate analysis is needed |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.46 4DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  |  |  |  | Same as 9.2.45 |  |  |  |  |  |  |  | Same as 9.2.45 | Same as 9.2.45 |  |  |  |  |  |
| 9.2.47 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation |  |  |  |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1) : ±4dB for normal conditions ±4dB for extreme conditions |  |  |  |  |  |  |  | 0.3dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.48 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  |  |  |  | Same as 9.2.47 |  |  |  |  |  |  |  | Same as 9.2.47 | Same as 9.2.47 |  |  |  |  |  |
| 9.2.51 FDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 |  |  |  |  |  | Noc1: -119dBm to -116dBm/15kHz depending on operating band Noc2 and Noc3: Noc1 +3.5dB Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  |  |  |  |  | 0dB 0dB 0dB 0dB 0.3dB Via mapping | Noc1: -119dBm to -116dBm/15kHz depending on operating band Noc2 and Noc3: Noc1 +3.5dB Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -3.7dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 12 to RSRQ_x + 9 |  |  |  |  |  |
| 9.2.52 TDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 |  |  |  |  |  | Same as 9.2.51 |  |  |  |  |  |  |  | Same as 9.2.51 | Same as 9.2.51 |  |  |  |  |  |
| 9.2.55 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  | Same as 9.2.38 |  |  |  |  |  |  |  | Same as 9.2.38 | Same as 9.2.38 |  |  |  |  |  |
| 9.2.56 4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  | Same as 9.2.45 |  |  |  |  |  |  |  | Same as 9.2.45 | Same as 9.2.45 |  |  |  |  |  |
| 9.2.57 5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  | Same as 9.2.47 |  |  |  |  |  |  |  | Same as 9.2.47 | Same as 9.2.47 |  |  |  |  |  |
| 9.2.58 6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Ês6 / Noc6: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  |  |  |  |  | 0.3dB 0dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Ês6 / Noc6: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.59 7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  |  |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Noc7: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Ês6 / Noc6: -6dB Ês7 / Noc7: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1), (Cell 7 – Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  |  |  |  |  | 0.3dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Noc7: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Ês6 / Noc6: -5.20dB Ês7 / Noc7: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1), (Cell 7 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.3.1 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy |  |  |  |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -60.00dBm/3.84MHz Ior / Ioc: +9.54dB CPICH_Ec/Ior: -10.00dB Reported CPICH_RSCP values: ±8dB Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -94.46dBm or -92.46dBm or -90.96dBm or -91.46dBm or -93.46dBm /3.84MHz depending on operating band Ior / Ioc: -9.54dB CPICH_Ec/Ior: -10.00dB Reported CPICH_RSCP values: ±6dB |  |  |  |  |  |  |  | Test 1: 0dB 0dB -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0.7dB 0.35dB 0dB Via mapping | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -60.75dBm/3.84MHz Ior / Ioc: +9.54dB CPICH_Ec/Ior: -10.00dB CPICH_RSCP_46 to CPICH_RSCP_63 Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -93.76dBm or -91.76dBm or -90.26dBm or -90.76dBm or -92.76dBm /3.84MHz depending on operating band Ior / Ioc: -9.19dB CPICH_Ec/Ior: -10.00dB CPICH_RSCP_-04 to CPICH_RSCP_9 CPICH_RSCP_-02 to CPICH_RSCP_11 CPICH_RSCP_-01 to CPICH_RSCP_12 CPICH_RSCP_-03 to CPICH_RSCP_10 depending on operating band |  |  |  |  |  |
|  |  |  |  |  |  | The derivation of the CPICH_RSCP values takes into account the uncertainty in Cell 2 CPICH_RSCP from Ioc, Ior / Ioc and CPICH_Ec/Ior, the allowed UE reporting accuracy, and the UE mapping function. The CPICH_RSCP values given above are for normal conditions. In all cases the CPICH_RSCP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.3.2 E-UTRAN TDD - UTRA FDD CPICH RSCP absolute accuracy |  |  |  |  |  | Same as 9.3.1 |  |  |  |  |  |  |  | Same as 9.3.1 | Same as 9.3.1 |  |  |  |  |  |
| << Unchanged rows omitted >> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>