3GPP TSG-RAN5 Meeting #94-e	R5-220785r1

# Electronic Meeting, 21st Feb – 4th Mar 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1543 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.2E.2.2 MPR for concurrent opration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | V2X MPR for con-current operation is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 6.2E.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The V2X Tx test case is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updating configuration table to improve clarity. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2E.2.1D.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.1D.5-1.

Table 6.2E.2.1D.5-1: UE MPR test requirement for contiguous PSCCH/PSSCH (Bands n38, n47)

| Test ID | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 + TT |
| 2 | 23 | 2.5 | 0 | 20.5 | 6 | 3 | 25.0 + TT | 14.5 + TT |
| 3 | 23 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 + TT |
| 4 | 23 | 2.5 | 0 | 20.5 | 6 | 3 | 25.0 + TT | 14.5 + TT |
| 5 | 23 | 4.5 | 0 | 18.5 | 5 | 3 | 25.0 + TT | 13.5 + TT |
| 6 | 23 | 7.0 | 0 | 16 | 5 | 3 | 25.0 + TT | 11.0 + TT |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.2E.2.1D.5-2. |  |  |  |  |  |  |  |  |

Table 6.2E.2.1D.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | FFS | FFS |

6.2E.2.2	UE maximum output power reduction for V2X / concurrent operation

6.2E.2.2.1	Test purpose

To verify that the reduction of UE transmitted power due to higher order modulations and transmit bandwidth configurations is within the allowed range. An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2E.2.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

6.2E.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2E.2.0.

6.2E.2.2.4	Test description

6.2E.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2E.2.2.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annexes A.7.5. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2E.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for NR Uu and V2X carrier High range for NR Uu and V2X carrier |  |  |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for NR Uu and V2X carrier Highest for NR Uu and V2X carrier |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | NR Uu Configuration |  |  |  | NR V2X Configuration to Transmit |  |  |
|  | Freq | Downlink Configuration | Uplink Configuration |  | Freq | Modulation | PSCCH and PSSCH RB allocation (Note 2) |
|  |  |  | Modulation | RB allocation (NOTE 1) |  |  |  |
| 1 | Default | N/A | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM QPSK | Inner_Full |
| 2 | Default |  | CP-OFDM 256 QAM | Outer Full | Default | CP-OFDM 256QAM | Outer_Full |
| 3 | Default |  | DFT-s-OFDM QPSK | Inner Full | Default | CP-OFDM 256QAM | Outer_Full |
| 4 | Default |  | CP-OFDM 256 QAM | Outer Full | Default | CP-OFDM QPSK | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.3 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The V2X Reference Measurement Channel and NR UL Reference Measurement Channel are set according to Table 6.2E.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameter Connectivity NR, Connected without release on, Test Mode On and Sidelink On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2E.2.2.4.3.

6.2E.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2E.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	SS sends sidelink scheduling information for each SL-HARQ process via PDCCH DCI format 3_0 for C_RNTI to schedule the Sidelink RMC according to Table 6.2E.2.2.4.1-1. UE is configured to transmit at Pcmax on the sidelink carrier. Since the UE has no payload and no loopback data to send the UE sends MAC padding bits on the Sidelink RMC.

4.	Measure the sum of the mean power of the UE at NR Uu carrier and V2X sidelink carrier in the channel bandwidth according to the test configuration from Table 6.2E.2.2.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.2E.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

Table 6.2E.2.2.4.3-1: SL-ResourcePool

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-TimeResourcePSCCH-r16 |  | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 |  | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-SubchannelSize-r16 |  | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 |  | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 |  | As defined in Table 6.1E-2 |  |  |
| sl-PowerControl-r16 SEQUENCE { |  |  |  |  |
| sl-MaxTransPower-r16 |  | 24 |  |  |
| sl-Alpha-PSSCH-PSCCH-r16 |  | Not present |  |  |
| dl-Alpha-PSSCH-PSCCH-r16 |  | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r16 |  | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r16 |  | Not present |  |  |
| dl-Alpha-PSFCH-r16 |  | Not present |  |  |
| dl-P0-PSFCH-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.2E.2.2.5	Test requirement

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2E.2.2.5-1.

Table 6.2E.2.2.5-1: UE MPR test requirement for inter-band con-current NR V2X operation

|  | PPowerClass,NR (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,c, NR (dBm) | PCMAX_H,c, NR (dB) | PPowerClass,V2X (dBm) | PCMAX_H,f,c,V2X (dBm) | PCMAX_L (dBm) | PCMAX_H (dBm) | T(PCMAX_L) (dB) | T(PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 2 | 23 | 6.5 | 0 | 16.5 | 23 | 23 | 23 | 16.5 | 26 | 5 | 2 | 28+TT | 11.5-TT |
| 3 | 23 | 0 | 0 | 23 | 23 | 23 | 23 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 4 | 23 | 6.5 | 0 | 16.5 | 23 | 23 | 23 | 16.5 | 26 | 5 | 2 | 28+TT | 11.5-TT |

Table 6.2E.2.2.5-2: Test Tolerance (inter-band con-current NR V2X operation)

|  | f ≤ 3.0GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB |

6.2F	Transmitter power for shared spectrum channel access

6.2F.1	UE maximum output power for shared spectrum channel access

Editor’s Note: The following aspects are not yet determined:

- No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4. Testing with 1.5dB MPR has been covered in [6.2F.2].

- MU and TT for >6GHz (band n96).

- RMC in Annex A.

- Test coverage for UL-MIMO

- Message exceptions

- Test state and generic procedure are TBD in 38.508-1

6.2F.1.1	Test purpose

Same test purpose as in 6.2.1.1

6.2F.1.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR standalone shared spectrum channel access.

<End of Changes >