3GPP TSG-RAN5 Meeting #94-e	R5-220883r3

# Online, 21st Feb – 4th Mar 2022

Title:							MU and TT proposal for FR2 EVM

Source:			Anritsu

Agenda Item:			5.3.1.17

Document for:			Endorsement

# 1.	Introduction

This paper provides assumptions and MU value of FR2 EVM test case for PUSCH, PC3.

# 2.	Discussion

In the last RAN5 meeting [1], it was agreed that the noise floor assumption in ACLR test case is used in EVM test case by considering single polarization measurement.

Proposal 1 : Use -10.6 dBm/400 MHz for FR2a and -8.5 dBm/400 MHz for FR2b in TE noise floor assumption of FR2 EVM test case. Note that it assumes measuring single polarization.

According to the above noise floor assumption, residual EVM values are calculated as below. The calculation process is shown in the section 2.2 of [1]. Cells are colored by relationship between the residual EVM and the core requirement; Green indicates the residual EVM is less than the half of core requirement, and orange indicates the residual EVM is larger than the half of core requirement, and red indicates the residual EVM is larger than the core requirement.

Table 1 Residual EVM in FR2a based on available SNR

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 1.22 % | 1.73 % | 2.44 % | 3.45 % | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 1.83 % | 2.58 % | 3.65 % | 6.14 % | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 1.22 % | 1.73 % | 2.44 % | 3.45 % | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 1.83 % | 2.58 % | 3.65 % | 6.14 % | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 2.17 % | 3.07 % | 4.34 % | 9.18 % | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 2.58 % | 3.65 % | 5.16 % | 9.18 % | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 3.44 % | 4.86 % | 6.88 % | 12.96 % | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 4.09 % | 5.78 % | 8.17 % | 12.96 % | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 2.58 % | 3.65 % | 5.16 % | 9.72 % | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 2.73 % | 3.86 % | 5.46 % | 9.72 % | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 3.44 % | 4.86 % | 6.88 % | 12.96 % | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 3.44 % | 4.86 % | 6.88 % | 12.96 % | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 5.14 % | 7.27 % | 10.28 % | 17.28 % | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 5.14 % | 7.27 % | 10.28 % | 17.28 % | 8.0% |

Table 2 Residual EVM in FR2b based on available SNR

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 1.91 % | 2.71 % | 3.82 % | 5.41 % | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.86 % | 4.05 % | 5.72 % | 9.61 % | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 1.91 % | 2.71 % | 3.82 % | 5.41 % | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.86 % | 4.05 % | 5.72 % | 9.61 % | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.40 % | 4.81 % | 6.80 % | 14.38 % | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 4.04 % | 5.71 % | 8.08 % | 14.38 % | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 5.39 % | 7.62 % | 10.77 % | 20.31 % | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 6.40 % | 9.05 % | 12.80 % | 20.31 % | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 4.04 % | 5.71 % | 8.08 % | 15.23 % | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 4.28 % | 6.05 % | 8.56 % | 15.23 % | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 5.39 % | 7.62 % | 10.77 % | 20.31 % | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 5.39 % | 7.62 % | 10.77 % | 20.31 % | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 8.06 % | 11.39 % | 16.11 % | 27.08 % | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 8.06 % | 11.39 % | 16.11 % | 27.08 % | 8.0% |

Note that the above residual EVM is based on only available SNR, and the actual residual EVM is obtained by considering TE’s uncertainty in addition to the values in Table 1 and Table 2. We propose the following values for EVM MU for FR2a and FR2b.

Table 3 EVM MU in FR2a

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 9.00% | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 9.00% | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.31% | 4.50% | 6.36% | 11.22% | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.60% | 4.73% | 6.68% | 11.22% | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.26% | 5.95% | 8.41% | 15.85% | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 5.00% | 7.07% | 9.99% | 15.85% | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.60% | 4.73% | 6.68% | 11.89% | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.71% | 4.99% | 7.06% | 11.89% | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.26% | 5.95% | 8.41% | 15.85% | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.26% | 5.95% | 8.41% | 15.85% | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.29% | 8.89% | 12.58% | 21.13% | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.29% | 8.89% | 12.58% | 21.13% | 8.0% |

Table 4 EVM MU in FR2b

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 4.54% | 6.26% | 8.91% | 18.06% | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 5.08% | 7.18% | 10.15% | 18.06% | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 6.77% | 9.57% | 13.53% | 25.51% | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.04% | 11.37% | 16.08% | 25.51% | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 5.08% | 7.18% | 10.15% | 19.13% | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 5.38% | 7.60% | 10.75% | 19.13% | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 6.77% | 9.57% | 13.53% | 25.51% | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 6.77% | 9.57% | 13.53% | 25.51% | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 10.12% | 14.31% | 20.24% | 34.02% | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 10.12% | 14.31% | 20.24% | 34.02% | 8.0% |

According to the Proposal 2 in [3], the above EVM MU values in Table 3 and Table 4 are calculated by the RSS of the residual EVM due to available SNR (Table 1 and Table 2) and the permitted TE EVM (Table 3a and Table 4a).

Table 3a Permitted TE EVM in FR2a

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.50% | 3.44% | 4.86% | 6.87% | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.50% | 3.26% | 4.61% | 6.58% | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.50% | 3.44% | 4.86% | 6.87% | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.50% | 3.26% | 4.61% | 6.58% | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 2.50% | 3.29% | 4.65% | 6.45% | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 2.50% | 3.01% | 4.25% | 6.45% | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.50% | 3.43% | 4.84% | 9.12% | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 2.88% | 4.07% | 5.75% | 9.12% | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 2.50% | 3.01% | 4.25% | 6.85% | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 2.50% | 3.16% | 4.48% | 6.85% | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 2.50% | 3.43% | 4.84% | 9.12% | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 2.50% | 3.43% | 4.84% | 9.12% | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 3.63% | 5.12% | 7.25% | 12.16% | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 3.63% | 5.12% | 7.25% | 12.16% | 8.0% |

Table 4a Permitted TE EVM in FR2b

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.00% | 4.00% | 5.75% | 8.00% | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.00% | 4.00% | 5.75% | 8.00% | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.00% | 4.00% | 5.75% | 8.00% | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.00% | 4.00% | 5.75% | 8.00% | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.00% | 4.00% | 5.75% | 10.93% | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.08% | 4.35% | 6.14% | 10.93% | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.10% | 5.79% | 8.19% | 15.44% | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 4.87% | 6.88% | 9.73% | 15.44% | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.08% | 4.35% | 6.14% | 11.58% | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.26% | 4.60% | 6.50% | 11.58% | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.10% | 5.79% | 8.19% | 15.44% | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.10% | 5.79% | 8.19% | 15.44% | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.12% | 8.66% | 12.25% | 20.59% | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.12% | 8.66% | 12.25% | 20.59% | 8.0% |

Proposal 1 : Adopt the values in Table 3 and Table 4 to EVM MU for FR2a and FR2b, PUSCH, PC3.

There is a concern that a compliant UE could fail the test by large MU. Therefore, we propose to apply the following TT for test points shown in orange cells, and test points shown in red cells should be removed. TT value is calculated by the difference between the core requirement and measured EVM of marginal UE

Table 5 TT in FR2a

| Modulation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- |
|  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| DFT-s-OFDM PI/2 BPSK | 0.0% | 0.0% | 0.0% | 0.0% | 30.0% |
| DFT-s-OFDM QPSK | 0.0% | 0.0% | 0.0% | 2.7% | 17.5% |
| DFT-s-OFDM 16 QAM | 0.0% | 0.0% | 2.2% | 5.0% | 12.5% |
| DFT-s-OFDM 64 QAM | 1.9% | 3.2% | N/A | N/A | 8.0% |
| CP-OFDM QPSK | 0.0% | 0.0% | 0.0% | 4.2% | 17.5% |
| CP-OFDM 16 QAM | 0.0% | 1.8% | 3.2% | N/A | 12.5% |
| CP-OFDM 64 QAM | 2.7% | N/A | N/A | N/A | 8.0% |

Table 6 TT in FR2b

| Modulation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- |
|  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| DFT-s-OFDM PI/2 BPSK | 0.0% | 0.0% | 0.0% | 0.0% | 30.0% |
| DFT-s-OFDM QPSK | 0.0% | 0.0% | 0.0% | 4.4% | 17.5% |
| DFT-s-OFDM 16 QAM | 0.0% | 2.4% | 4.3% | N/A | 12.5% |
| DFT-s-OFDM 64 QAM | N/A | N/A | N/A | N/A | 8.0% |
| CP-OFDM QPSK | 0.0% | 0.0% | 3.6% | N/A | 17.5% |
| CP-OFDM 16 QAM | 2.2% | 3.9% | N/A | N/A | 12.5% |
| CP-OFDM 64 QAM | N/A | N/A | N/A | N/A | 8.0% |

Proposal 2 : Remove test points in red cells in Table 3 and Table 4 from EVM MU for FR2a and FR2b, PUSCH, PC3.

Proposal 3 : Adopt the values in Table 5 and Table 6 to EVM MU for FR2a and FR2b, PUSCH, PC3.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Proposal 1 : Adopt the values in Table 3 and Table 4 to EVM MU for FR2a and FR2b, PUSCH, PC3.

Proposal 2 : Remove test points in red cells in Table 3 and Table 4 from EVM MU for FR2a and FR2b, PUSCH, PC3.

Proposal 3 : Adopt the values in Table 5 and Table 6 to EVM MU for FR2a and FR2b, PUSCH, PC3.

# 4.	References

[1] R5-218212, “MU and TT proposal for FR2 EVM”, Anritsu, RAN5#93-e

[2] R5-215476, “On the MU of Tx Modulation Quality test cases”, R&S, RAN5#92-e

[3] R5-220353, “Discussion on MU and TT proposal for FR2 EVM”, Keysight, RAN5#94-e

# Annex

The following table shows the original values of total EVM MU and TE EVM from Anritsu. The EVM values in section 2 is calculated from the maximum value of [2], [3], and the following tables.

Table 3 EVM MU in FR2a

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.72% | 3.85% | 5.44% | 7.69% | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.94% | 4.16% | 5.88% | 9.00% | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.72% | 3.85% | 5.44% | 7.69% | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.94% | 4.16% | 5.88% | 9.00% | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.18% | 4.50% | 6.36% | 11.22% | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.34% | 4.73% | 6.68% | 11.22% | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.21% | 5.95% | 8.41% | 15.85% | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 5.00% | 7.07% | 9.99% | 15.85% | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.34% | 4.73% | 6.68% | 11.89% | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.53% | 4.99% | 7.06% | 11.89% | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.21% | 5.95% | 8.41% | 15.85% | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.21% | 5.95% | 8.41% | 15.85% | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.29% | 8.89% | 12.58% | 21.13% | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.29% | 8.89% | 12.58% | 21.13% | 8.0% |

Table 4 EVM MU in FR2b

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.76% | 3.89% | 5.50% | 7.78% | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.59% | 5.08% | 7.18% | 12.07% | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.76% | 3.89% | 5.50% | 7.78% | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.59% | 5.08% | 7.18% | 12.07% | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 4.27% | 6.04% | 8.54% | 18.06% | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 5.08% | 7.18% | 10.15% | 18.06% | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 6.77% | 9.57% | 13.53% | 25.51% | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.04% | 11.37% | 16.08% | 25.51% | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 5.08% | 7.18% | 10.15% | 19.13% | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 5.38% | 7.60% | 10.75% | 19.13% | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 6.77% | 9.57% | 13.53% | 25.51% | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 6.77% | 9.57% | 13.53% | 25.51% | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 10.12% | 14.31% | 20.24% | 34.02% | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 10.12% | 14.31% | 20.24% | 34.02% | 8.0% |

Table 3a Permitted TE EVM in FR2a

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.43% | 3.44% | 4.86% | 6.87% | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.30% | 3.26% | 4.61% | 6.58% | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.43% | 3.44% | 4.86% | 6.87% | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.30% | 3.26% | 4.61% | 6.58% | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 2.32% | 3.29% | 4.65% | 6.45% | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 2.12% | 3.01% | 4.24% | 6.45% | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.43% | 3.43% | 4.84% | 9.12% | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 2.88% | 4.07% | 5.75% | 9.12% | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 2.12% | 3.01% | 4.24% | 6.85% | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 2.24% | 3.16% | 4.48% | 6.85% | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 2.43% | 3.43% | 4.84% | 9.12% | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 2.43% | 3.43% | 4.84% | 9.12% | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 3.63% | 5.12% | 7.25% | 12.16% | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 3.63% | 5.12% | 7.25% | 12.16% | 8.0% |

Table 4a Permitted TE EVM in FR2b

| Test ID | Modulation | RB allocation | CBW |  |  |  | Requirement for EVM [%] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50MHz | 100MHz | 200MHz | 400MHz |  |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 1.99% | 2.79% | 3.96% | 5.59% | 30.0% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 2.17% | 3.07% | 4.34% | 7.30% | 30.0% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 1.99% | 2.79% | 3.96% | 5.59% | 17.5% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 2.17% | 3.07% | 4.34% | 7.30% | 17.5% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 2.58% | 3.65% | 5.17% | 10.93% | 12.5% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.08% | 4.35% | 6.14% | 10.93% | 12.5% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.10% | 5.79% | 8.19% | 15.44% | 8.0% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 4.87% | 6.88% | 9.73% | 15.44% | 8.0% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.08% | 4.35% | 6.14% | 11.58% | 17.5% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.26% | 4.60% | 6.50% | 11.58% | 17.5% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.10% | 5.79% | 8.19% | 15.44% | 12.5% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.10% | 5.79% | 8.19% | 15.44% | 12.5% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.12% | 8.66% | 12.25% | 20.59% | 8.0% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.12% | 8.66% | 12.25% | 20.59% | 8.0% |