3GPP TSG-RAN5 Meeting #94-e	R5-220975r1

# Electronic Meeting, 21st Feb. – 04th Mar. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1307 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of transmit power configuration for EN-DC reference sensitivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The transmit power is limited to min(+20 dBm, PCMAX_L,f,c) for the REFSENS MSD due to dual uplink. However, the default message contents in REFSENS test case don't implement the power limitation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the transmit power limitation into REFSENS MSD due to dual uplink to align with TS 38.101-3. - Adding message exception of power configuration for test points with 2UL intermodulation exception. - Correcting some editorial mistakes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The REFSENS test case would not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.0.3.5, 7.3B.2.3, 7.3B.2.3_1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependent |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: Correct NOTE 1 in Table 7.3B.2.0.3.5.1-1a to NOTE 4 to resolve overlap with R5-221337. |  |  |  |  |  |  |  |  |

<Start of Changes>

7.3B.2.0.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex M  the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [5] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [5] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [5], with parameters specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1A_n5A | 1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
|  | n5 | 836.5 | 5 | 25 | 876.5 | N/A | N/A |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
| DC_1A_SUL_n77A- |  |  |  |  |  | 32.54 |  |
| n84A | n77, n78 | 4090 | 10 | 50 | 4090 | N/A | N/A |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
| DC_1A-SUL_n77A-n84A |  |  |  |  |  | 10.74 |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A | n77, n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_1A_n78A, | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
| DC_1A_SUL_n78A- |  |  |  |  |  | 10.74 |  |
| n84A | n78 | 3710 | 10 | 25 | 3710 | N/A |  |
| DC_2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_2A_n66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
| DC_2A_n78A |  |  |  |  |  | 28.74 |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
| DC_2A_n78A |  |  |  |  |  | 10.74 |  |
|  | n78 | 3690 | 10 | 50 | 3700 | N/A | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
| DC_3_n5 | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_3C_n7A | n7 | 2535 | 10 | 50 | 2655 | 10.25 | IMD4 |
|  | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
| DC_3A_n20A | n20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
| DC_3C_n41A | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | IMD4 |
| DC_3A_SUL_n41A-n80A, | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
| DC_3C_SUL_n41A-n80A | n41 | 2657.5 | 10 | 52 | 2657.5 | N/A | N/A |
| DC_3A_n77A | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
| DC_3A_SUL_n77A-n80A DC_3A_n78A |  |  |  |  |  | 28.74 |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
| DC_3A_SUL_n77A-n80A, DC_3A_n78A |  |  |  |  |  | 10.74 |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n77A | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_7A_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | 4th IMD |
| DC_7A-7A_n66A DC_7C_n66A | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_8A_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
|  | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
| DC_8A_n3A | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_SUL_n41A- | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
| n81A | n41 | 2685 | 10 | 52 | 2685 | N/A | N/A |
| DC_8A_n77A DC_8A_n78A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
| DC_8A-n79C, DC_8A-SUL_n78A-n81A | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | H4 |
| DC_12_n78 | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_20A_n3A | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_8A_n79A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
| DC_8A-SUL_n79A-n81A | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_18A_n77A DC_18A_n78A | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_19A_n78A | 19 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 21 | IMD3 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | IMD3 |
| DC_20A_n77A DC_20A_n78A, | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
| DC_20A-SUL_n78A-n82A | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_20A_n77A | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD54 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD3 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
|  | 28 | 730 | 10 | 50 | 775 | 15.3 | IMD 2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
| DC_28_n50 | 28 | 740 | 10 | 50 | 785 | 6 | IMD 4 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 0.5 | IMD 5 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD 4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_26A_n77A, | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
| DC_26A_n78A | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28A_n77A, | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_28A_n78A, DC_28A-SUL_n78A-n83A | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_48A_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_66A_n2A | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
| DC_66A_n25A | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_66A_n78A | 66 | 1730 | 5 | 25 | 2130 | 5.0 | IMD5 |
|  | n78 | 3660 | 5 | 50 | 3660 | N/A | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L_E-UTRA,c) or min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2.B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| NOTE 4: Both of the transmitters shall be set min(+23 dBm, PCMAX_L_E-UTRA,c) or min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2.B.4.1.3. |  |  |  |  |  |  |  |

7.3B.2.0.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.0.3.5.2-0: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 |  | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: Both of the transmitters shall be set min(+20 dBm, PCMAX_L_E-UTRA,c) or min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2.B.4.1.3. |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |  |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |  |
| DC_1A-3A_n28A DC_1A-3C_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |  |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |  |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |  |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |  |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |  |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |  |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |  |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |  |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |  |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |  |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |  |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |  |
| DC_1A-7A_n78A DC_1A-7C_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |  |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |  |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |  |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |  |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |  |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |  |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |  |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-8A_n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |  |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |  |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | IMD3 |  |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | 8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 |  |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | N/A |  |
| DC_1A-18A_n77A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |  |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |  |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |  |
| DC_1A-18A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |  |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |  |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |  |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |  |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |  |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |  |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |  |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |  |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |  |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |  |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |  |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |  |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |  |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |  |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |  |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |  |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |  |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |  |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD2 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |  |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |  |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |  |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.8 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 | IMD5 |  |
|  | n77 | 3330 | 10 | 50 | 3330 | N/A | N/A |  |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |  |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |  |
| DC_1A-28A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |  |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |  |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |  |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | IMD5 |  |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |  |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |  |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |  |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |  |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |  |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |  |
| DC_1A_n40A-n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |  |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 ¦3*fB1-fn40¦ |  |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |  |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 ¦3*fB1 -fn78¦ |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |  |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |  |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 |  |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A |  |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | IMD4 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |  |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A |  |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | IMD5 |  |
| DC_1A-41A_n78A | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |  |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |  |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
| DC_1A-41A_n78A | 1 | 1955 | 5 | 25 | 2145 | 8.7 | IMD4 |  |
|  | 41 | 2507.5 | 10 | 50 | 2507.5 | N/A | N/A |  |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |  |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |  |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A |  |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | IMD2 |  |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |  |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |  |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |  |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |  |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |  |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |  |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |  |
| DC_1A-SUL_n77A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |  |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |  |
| DC_1A-SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |  |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |  |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |  |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |  |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |  |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |  |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |  |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A | N/A | N/A | IMD4 |  |
|  | 12 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |  |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |  |
| DC_3A-5A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | 5 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |  |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |  |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |  |
| DC_3A-8A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | 9.7 | IMD4 |  |
| DC_3A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |  |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |  |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | N/A |  |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | N/A |  |
|  | 8 | 910 | 5 | 25 | 955 | 15.3 | IMD3 |  |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |  |
|  | 3 | 1755 | 5 | 25 | 1850 | 8.8 | IMD4 |  |
| DC_3A_n7A-n78A DC_3C_n7A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 ¦2*fn7-fB3¦ |  |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |  |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |  |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |  |
| DC_3A-18A_n77A DC_3A-18A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | 18 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A-19A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |  |
|  | 19 | 840 | 5 | 25 | 885 | [18.5] | IMD3 |  |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |  |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |  |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |  |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |  |
|  | 3 | 1733 | 5 | 25 | 1828 | 9.4 | IMD4 |  |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |  |
| DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |  |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |  |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |  |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |  |
|  | 3 | N/A | N/A | N/A | N/A | N/A | IMD2 |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |  |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |  |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |  |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |  |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |  |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |  |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |  |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |  |
| DC_3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | IMD3 |  |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |  |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |  |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |  |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |  |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |  |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |  |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |  |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |  |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |  |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |  |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |  |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 |  |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |  |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |  |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 |  |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 |  |
| DC_3A-41A_n77A DC_3A-41C_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |  |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 |  |
|  | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |  |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |  |
| DC_3A-41A_n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |  |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |  |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |  |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |  |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |  |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |  |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |  |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |  |
| DC_3A-SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |  |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |  |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |  |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |  |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |  |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |  |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |  |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |  |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |  |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |  |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |  |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |  |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | IMD2 |  |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | N/A |  |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |  |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | IMD5 |  |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | N/A |  |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |  |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 |  |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |  |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |  |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |  |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 |  |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |  |
| DC_7A-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |  |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |  |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |  |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |  |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |  |
|  | 20 | 855 | 5 | 25 | 814 | N/A | N/A |  |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |  |
| DC_7A-20A_n28A | 20 | 842 | 5 | 25 | 801 | N/A | N/A |  |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |  |
|  | 7 | 2520 | 10 | 50 | 2640 | 5.9 | IMD5 |  |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 |  |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |  |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |  |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |  |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |  |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |  |
| DC_7A-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |  |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |  |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |  |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |  |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |  |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |  |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |  |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |  |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |  |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |  |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |  |
|  | 28 | 720 | 5 | 25 | 780 | 8.3 | IMD2 |  |
|  | n78 | 3350 | 10 | 50 | 3421 | N/A | N/A |  |
|  | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |  |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 | IMD5 |  |
|  | n78 | 3460 | 10 | 50 | 3421 | N/A | N/A |  |
|  | 7 | 2570 | 5 | 25 | 2650 | 30.5 | IMD2 |  |
|  | 28 | 740 | 5 | 25 | 768 | N/A | N/A |  |
|  | n78 | 3390 | 10 | 50 | 3421 | N/A | N/A |  |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |  |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |  |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |  |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |  |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |  |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |  |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |  |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |  |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |  |
|  | 28 | 723 | 5 | 25 | 778 | 4.4 | IMD5 |  |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | N/A |  |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | N/A |  |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |  |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |  |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |  |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |  |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |  |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |  |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |  |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |  |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |  |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |  |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |  |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |  |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |  |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |  |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |  |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |  |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |  |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |  |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |  |
|  | n78 | 3630 | 10 | 50 | 3630 | 16.0 | IMD3 |  |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |  |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | IMD3 |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |  |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |  |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |  |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |  |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |  |
| DC_20A_SUL_n78A-n80A | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |  |
|  | n80 | 1735 | 5 | 25 |  | N/A | N/A |  |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |  |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |  |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |  |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |  |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_28A-42A_79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |  |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |  |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |  |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |  |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |  |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |  |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |  |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |  |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |  |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |  |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |  |
| DC_20A_n28A-n78A DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |  |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |  |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |  |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |  |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |  |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |  |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |  |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |  |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |  |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |  |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |  |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |  |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: Both of the transmitters shall be set min(+20 dBm, PCMAX_L_E-UTRA,c) or min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2.B.4.1.3. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Mid range for E-UTRA and Mid range for NR |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY |  | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| NOTE 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. NOTE 2: LTE anchor agnostic configuration NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 4: The test configuration table only applies to EN-DC only capable devices and devices not supporting SA mode in the tested band. NOTE 5: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B. |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  | Range |  |  |  | NRB |  |  |  |  |  |  | Band |  |  |  |  | Range |  |  | NRB |  |  |  |
|  | UL MOD |  |  | DL MOD |  |  |  | CH BW |  |  |  | DLalloc / UL alloc |  |  | UL MOD |  |  |  |  | DL MOD |  |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | Mid |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz | All RBs / 0 |  |  |
| 24,10 | 1 |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 36 | 1 |  |  | Low |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | High |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz | All RBs / 0 |  |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3710 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | Mid |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 26 | 3 |  |  | High |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | Low |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 34 | 3 |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 |  |  | High |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 26 | 3 |  |  | High |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | Low |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |
| 34, 10 | 3 |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | 2657.5 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_3A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3495 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 3 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3525 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 20 MHz | All RBs / 0 |  |  |
| 35 | 3 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3685 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 3a9 | 3 |  |  | Low |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | High |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 |  |  |  | All RBs/0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 20 | All RBs / REFSENS_ENDC_2 |  |  |
| 44 | 3 |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3575 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 54 | 3 |  |  | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3435 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  | DL 2655 |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | DL 1825 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  | 10 MHz |  |  |  | N/A |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  | DL 932.5 |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | DL 2155 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  | DL 945 |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | DL 1850 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 8 |  |  | DL 942.5 |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | DL 1842.5 |  |  |  |  |  |  |
|  | QPSK |  |  | N/A |  |  |  | 5 MHz |  |  |  | N/A |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_8A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3590 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  | QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 2,8 | 8 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3520 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  | QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 8 |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  | 3635 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | 4457.7 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 11 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | 4512.7 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | QPSK |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | Low |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| 2,9 | 19 |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  | 4445.02 |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1775 / DL 1870 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| 24 | 20 |  |  | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1735 / DL 1830 |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | Mid |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  | QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 23 | 20 |  |  | Mid |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3388 MHz |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  | 100 MHz | All RBs / 0 |  |  |
| 34 | 20 |  |  | UL 850 / DL 809 MHz |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3359 MHz |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | 2524.5 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  | QPSK |  |  | 50 MHz |  |  |  | All RBs / 0 |
| 2,8 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | 2572 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  | QPSK |  |  | 50 MHz |  |  |  | All RBs / 0 |
| 34 | 26 |  |  | UL 839 |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | 2562 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3366 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  | QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| 2,8 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3446 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 15 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  | QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| 34 | 26 |  |  | UL 836.5 |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3391 |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4432.5 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 15 MHz |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 60 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 22,9 | 26 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | 4470.5 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 15 MHz |  |  |  | All RBs / 0 |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 60 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | 3540 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |  |  |
| 28 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | Low |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |  |  |
| 34 |  | 28 |  | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | 3582.5 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 10 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| 45 |  | 28 |  | Low |  |  |  |  |  |  |  |  |  | n77 |  |  |  | 3815 |  |  |  |  |  |  |  |  |  |
|  |  | N/A |  | QPSK |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | 3525 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 2,9 | 40 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | Mid |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | 3750 |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_2 |
| 26 | 41 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |
|  | N/A |  |  | QPSK |  |  | 20 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / REFSENS_ENDC_3 |
| 36 | 41 |  |  | High |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |
|  | QPSK |  |  | QPSK |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  | 100 MHz |  |  |  | All RBs / 0 |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 48 |  | 3557.6MHZ |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 1778.8 MHZ |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 23 |  |  | 48 |  | 3581.0 MHz |  |  |  |  |  |  |  | n66 |  |  |  |  |  | 1777.5 MHz |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  | QPSK |  |  |  | 20 MHz |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  | 40 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |
| 34 |  |  | 48 |  | 3630 MHz |  |  |  |  |  |  |  | n66 |  |  |  |  |  | UL 1715 / DL 2115 MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  | 20 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 |  |  | 66 |  | UL1775 MHz/ DL2175 MHz |  |  |  |  |  |  |  | n2 |  |  |  |  |  | UL 1855 / DL 1935MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 24, 10 |  |  | 66 |  | UL 1750 MHz/ DL 2150 MHz |  |  |  |  |  |  |  | n2 |  |  |  |  |  | UL 1883.3 / DL 1963.3MHz |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  | QPSK |  |  |  | 5 MHz |  | All RBs / REFSENS_ENDC_4 |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84.. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-0:

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-0 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-0 Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in Tables 7.3B.2.3.5-1 to 7.3B.2.3.5-4. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with NOTE 4 in Table 7.3B.2.3.4.2.1-6, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable for two, three, four and five bands operation.

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n772 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n782 | 15 |  | -71.9+TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
| 2 | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.9 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | --95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| 3 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  |  |  | -74.1 +TT15 | -74.1 +TT15 | -74.0 +TT15 |  |  | -73.9 +TT15 | -74.0 +TT15 |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  |  |  | -74.4 +TT15 | -74.2 +TT15 | -74.2 +TT15 |  |  | -74.0 +TT15 | -74.1 +TT15 | --74.1 +TT15 | -74.0 +TT15 | -74.0 +TT15 | -74.0 +TT15 |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  |  |  |  | -74.8 +TT15 | -74.5 +TT15 | -74.4 +TT15 |  |  | -74.2 +TT15 | -74.2 +TT15 | -74.2 +TT15 | -74.1 +TT15 | -74.1 +TT15 | -74.1 +TT15 |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -96.9 +TT15 | -95.4 +TT15 | -94.6 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.2 +TT15 | -95.5 +TT15 | -94.8 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.6 +TT15 | -95.8 +TT15 | -95.0 +TT15 |  |  |  |  |  |  |  |  |
| 5 | n78 | 15 |  | -84.2 +TT | -84.4 +TT |  |  |  | -84.2 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -84.3 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.3 +TT | -84.5 +TT | -84.4 +TT | -84.3 +TT | -84.0 +TT | -84.2 +TT |
|  |  | 60 |  | -84.5 +TT | -84.6 +TT | -84.5 +TT |  |  | -84.5 +TT | -84.6 +TT | -84.5 +TT | -84.4 +TT | -84.1 +TT | -84.3 +TT |
| 8 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -84.0 +TT | -83.9 +TT |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -84.3 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 8 | n41 | 15 |  | -81.8 +TT | -81.7 +TT | -81.7 +TT |  |  | -81.6 +TT | -81.5 +TT |  |  |  |  |
|  |  | 30 |  | -82.1 +TT | -81.8 +TT | -81.9 +TT |  |  | -81.7 +TT | -81.6 +TT | -81.4 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
|  |  | 60 |  | -82.5 +TT | -82.1 +TT | -81.1 +TT |  |  | -81.9 +TT | -81.7 +TT | -81.6 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
| 8 | n794,5 | 15 |  |  |  |  |  |  | -82.8 +TT | -82.4 +TT |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  | -82.9 +TT | -82.5 +TT | -82.3 +TT | -81.7 +TT |  | -81.2 +TT |
|  |  | 60 |  |  |  |  |  |  | -83.1 +TT | -82.6 +TT | -82.4 +TT | -81.8 +TT |  | -81.3 +TT |
| 12 | n66 | 15 | -89.5 +TT | -88.8 +TT | -88.3 +TT | -87.8 +TT |  |  | -87.7 +TT |  |  |  |  |  |
|  |  | 30 |  | -89.1 +TT | -88.4 +TT | -88.0 +TT |  |  | -87.8 +TT |  |  |  |  |  |
|  |  | 60 |  | -89.5 +TT | -88.7 +TT | -88.2 +TT |  |  | -87.9 +TT |  |  |  |  |  |
| 12 | n78 | 15 |  | -85.4 +TT | -85.1 +TT | -84.9 +TT |  |  | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  | -85.7 +TT | -85.2 +TT | -85.1 +TT |  |  | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  | -86.1 +TT | -85.5 +TT | -85.3 +TT |  |  | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| 18， 19 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 28 | n51 | 15 | -72.2 +TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n774,5 n784,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 20 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n41 | 15 |  | -84.5 +TT | -84.6 +TT | -84.4 +TT |  |  | -83.6 +TT | -83.3 +TT | 3.9 +TT | 3.1 +TT | 2.7 +TT |  |
|  |  | 30 |  | -84.8 +TT | -84.7 +TT | -84.6 +TT |  |  | -83.7 +TT | -83.4 +TT | -83.0 +TT | -82.5 +TT | -82.4 +TT |  |
|  |  | 60 |  | -85.2 +TT | -85.0 +TT | -84.8 +TT |  |  | -83.9 +TT | -83.5 +TT | -83.2 +TT | -82.5 +TT | -82.4 +TT |  |
| 26 | n776,7 n786,7 | 15 |  | -85.0 +TT | -84.9 +TT | -84.7 +TT |  |  | -83.6 +TT | -84.6 +TT |  |  |  |  |
|  |  | 30 |  | -85.3 +TT | -85.0+TT | -84.9 +TT |  |  | -83.7 +TT | -84.7 +TT | -84.7 +TT | -84.6 +TT | -84.6 +TT | -84.6 +TT |
|  |  | 60 |  | -85.7 +TT | --85.3 +TT | --85.1 +TT |  |  | -83.9 +TT | -84.8 +TT | -84.8 +TT | -84.7 +TT | -84.7 +TT | -84.7 +TT |
| 26 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -85.6 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -85.7 +TT | -84.5 +TT |
| n28 | 18,9,10 | N/A | -89.1 | -88.7 | -88.3 | -88.0 |  |  |  |  |  |  |  |  |
| n71 | 211 | N/A | -92.7 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 212 | N/A | -95.6 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 7 | N/A | -82.7 | -82.6 | -82.4 | -82.3 |  |  |  |  |  |  |  |  |
| 66 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2+TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 | N/A | -70.7 | -70.6 | -70.8 | -70.8 |  |  |  |  |  |  |  |  |
| n66 | 48,3 | N/A | -96.1 | -93.6 | -92.3 | -91.6 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n77, DC_66_n77, DC_3_n77 and DC_3_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 15: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n41 | 264 | N/A | -72.5 | -69.5 | -69.5 | N/A |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 282 | N/A | -69.8 | -69.8 | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -90.6 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.310 | -92.010 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-3 with uplink configuration specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.5-3: Reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n1 | 3 | N/A | -93.3 | -91 | -89.5 | -88.5 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | -94 +TT | -91.6 +TT | -90.1 +TT | -89.1 +TT | -88.1 +TT | -87.4 +TT | -86.2 +TT |  |  |  |  |  |  |
| 1 | n40 | 15 | -93.4 +TT | -90.2 +TT | -88.4 +TT | -87.2 +TT | -86.1 +TT |  | -85.3 +TT | -84.0 +TT |  |  |  |  |  |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
|  |  |  |  | -97.1 +TT4 | -95.1 +TT4 | -94 +TT4 |  | -92.0 +TT4 | -90.7 +TT4 | -89.7 +TT4 | -88.9 +TT4 |  | -87.6 +TT4 | -87.1 +TT4 | -86.7 +TT4 |
| n40 | 1 | N/A | -91.0 | -88.0 | -86.2 | -85.0 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | N/A | 3 | n41 | 30 | 3 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | N/A | -95.2 | -92.2 | -90.4 | -89.2 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| n41 | 66 | N/A | -95.3 | -92.3 | -90.5 | -89.3 |  |  |  |  |  |  |  |  |  |
| 41 | n773 | 30 |  | -87.3 +TT | -85.3 +TT | -84.1 +TT |  |  | -82.9 +TT | -82.9 +TT | -82.9 +TT |  | -82.1 +TT | -81.7 +TT | -81.3 +TT |
| 3 | n51 | 15 | -93.6 +TT |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 15 | -91.2 +TT | -88.0 +TT | -86.2 +TT | -85.0 +TT |  |  |  | -81.8 +TT |  |  |  |  |  |
| n78 | 71 | N/A | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 30 |  | -87.8 +TT | -85.8 +TT | -84.6 +TT |  |  | -83.4 +TT | -83.4 +TT | -83.4 +TT |  | -82.6 +TT | -82.2 +TT | -81.8 +TT |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-3a: Reference sensitivity exceptions due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87.0 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
|  |  |  |  | -97.1 +TT3 | -95.1 +TT3 | -94 +TT3 |  | -92.0 +TT3 | -90.7 +TT3 | -89.7 +TT3 | -88.9 +TT3 |  | -87.6 +TT3 | -87.1 +TT3 | -86.7 +TT3 |
| n41 | 3 | N/A | -94.0 | -91.0 | -89.2 | -88.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | -96.73 | -93.73 | -91.93 | -90.73 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-4 with uplink configuration specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n5A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS |  | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n78A | 2 | N/A | -71.3 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.3 | - | - | - | - | N/A |  |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| CA_3A-n20A | 3 | N/A | -92.3 | - | - | - | - | IMD4 | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | -88.0+TT | - | - | - | - | IMD4 | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  |  |  | -90.87 | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  |  |  | -88.37 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | N/A | -91.5 | - | - | - | - | IMD5 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20A_n77A | 20 | N/A | -85.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 20 | N/A | -89.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_28_n50 | 28 | N/A | - | -79.5 | - | - | - | IMD 2 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -88.8 | - | - | - | IMD 4 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -94.3 | - | - | - | IMD 5 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-5: Reference sensitivity exceptions due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm)) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order) | Duplex mode |
| DC_1A_n78A | 1 | N/A | REFSENS-17.8 | - | - | - | - | IMD4 |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n41A | 3 | N/A | -77.9 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.67 | - | - | - | - |  |  |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -64.4 | - | - | - | - | IMD2 | FDD |
|  |  |  | -67.17 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 | N/A | -77.8 | - | - | - | - | IMD4 | FDD |
|  |  |  | -80.57 | - | - | - | - |  |  |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |

#### 7.3B.2.3_1	Reference sensitivity for EN-DC within FR1 (>2 CCs)

7.3B.2.3_1.1	Reference sensitivity for EN-DC within FR1 (3 CCs)

Editor's note:	The test requirements section of this test case contains some EN-DC configurations that have no test points defined meaning these cannot be tested.

Editor's note:	For EN-DC configurations with >1NR CC, test points in Table 7.3B.2.3_1.1.4.1-0 only need to be tested if the configuration is a max NR CC of the UE, where detailed execution rules are FFS.

7.3B.2.3_1.1.1	Test purpose

To verify the ability of UE that support EN-DC configurations to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when no CA exceptions are allowed and single carrier requirements apply whenever possible. A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B.2.3_1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 3CC EN-DC.

7.3B.2.3_1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.3_1.1.4	Test description

7.3B.2.3_1.1.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.1.4.1-0 and 7.3B.2.3_1.1.4.1-1.

Table 7.3B.2.3_1.1.4.1-0: Test Configuration Table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | DC_XA_nYA-nZA Mid range for MCG and SCG DC_XA_nYC: Mid range for XA Low range, High Range for nXC DC_(n)XCA: Low range, High Range DC_XA_nY(2A): TBD |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range |  |  | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc/UL alloc | UL MOD | DL MOD |  |  | UL/DL Ch BW | DLalloc/ ULalloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc/UL alloc |
| Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | X | default |  |  |  |  | nX | default |  |  |
|  | QPSK | QPSK | Highest | All RBs / REFSENS_LTE | N/A | QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA-XA_nXA/ DC_XA_(n)XAA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| 2 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | Highest | All RBs / REFSENS_NR | N/A | CP-OFDM QPSK | Highest | All RBs / 0 |
| Note 1: LTE anchor agnostic configuration Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: LTE anchor agnostic configuration with LTE CA where LTE CA fallback to 1CC is sufficient to test Note 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 5: For UE supporting multiple EN-DC configurations with the same NR CA combination, only one EN-DC configuration is tested. Note 6: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EN-DC configurations containing the following band combinations: 20-n78: Mid in band 20 and Mid in band 78. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test DC Combination setting (NRB_agg) as specified in clause [TBD] for the DC Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB" and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CG -NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  | Range | NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Band |  |  | Range |  |  |  |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  |  |  |  | Band |  |  |  |  | Range |  |  |  |  |  |  |  |  |  | NRB |  |  |  |  |  |  |  |
|  | UL MOD |  |  |  | DL MOD | CH BW |  |  |  |  |  | DLalloc/UL alloc |  |  |  |  |  |  |  |  | UL MOD |  |  | DL MOD |  |  |  |  |  |  |  | UL/DL Ch BW |  |  |  |  |  |  |  |  | DLalloc/ ULalloc |  |  |  | UL MOD |  |  |  |  | DL MOD |  |  |  |  |  |  |  |  |  | UL/DL Ch BW |  |  | DLalloc/UL alloc |  |  |  |  |
| Default Test Settings for a DC_XA_nYC, DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_1A-3A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  |  |  | UL 1975 / DL 2165 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  | DL 1818.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |
| 2 | 3 |  |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | DL 2139 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s- OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |
| Test Settings for DC_1A-7A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  |  |  | UL 1935 / DL 2125 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  | DL 2653 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 718 / DL 773 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  |  |  |  |
| Test Settings for DC_1A-3A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3757.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | DL 2125 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3725 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-5A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2122 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3780 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1975 / DL 2165 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  | DL 885 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3405 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1977.5 / DL 2167.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2627.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3305 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3580 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2120 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 835 / DL 794 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3790 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3330 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-28A_n3A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2139 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A_n28A-n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  |  | 3416 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  |  | UL 733 / DL 788 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | N/A |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |
| 2 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  |  | DL 790 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  |  | 3320 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | N/A |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |
| Test Settings for DC_1A-28A_n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | UL 740 / DL 795 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | DL 2150 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1970 / DL 2160 |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 794 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3352 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-7A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  |  |  | UL 1712.5 / DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  | DL 2682 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |
| 2 | 7 |  |  |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  | DL 1832.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |
| Test Settings for DC_3A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3475 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-8A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3640 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1828 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 852 / DL 811 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3510 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | UL 740 / DL 760 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1750/ DL 1845 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3764 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-40A_n1A Configuration – Note 3, 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1735 / DL 1830 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |  |  |  |  |  |  | DL 2380 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1A |  |  |  |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_5A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | UL 844 / DL 889 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2645 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3489 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 5 |  | DL 879 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2550 / DL 2670 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3429 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 5 |  | DL 875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2525 / DL 2645 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n1A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | UL 1940 / DL 2130 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 806 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1737 / DL 1832 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 855 / DL 896 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 20 |  | UL 842/ DL 801 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  | DL 2640 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  | N/A |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2560 / DL 2680 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3370 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2560 / DL 2680 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3435 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 7 |  | DL 2675 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3520 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-28A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 796 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | 1842 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | 1810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2567.5 / DL 2687.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2567.5 / DL 2687.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3460 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 7 |  | DL 2650 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 740 / DL 795 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3390 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2565/ DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2565/ DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3365 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_2A-14A_n66A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | DL 1954 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 |  |  |  |  |  |  |  | UL 793 / DL 763 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1770 / DL 2170 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_14A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 |  | DL 763 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 66 |  |  |  |  |  |  |  | UL 1762 / DL 2162 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1874 / DL 1954 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_20A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | UL 857 / DL 816 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3314 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 20 |  | UL 837 / DL 796 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | UL 744 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 5MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_20A_n28A-n75A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n75 |  |  |  |  | DL 1476MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / N/A |  |  |  |
| 2Note 8 | 20 |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n75 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / N/A |  |  |  |
| Note 1: Void Note 2: Void Note 3: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. Note 4: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 6: Test only applicable for to UEs not supporting UE capability singleUL-Transmissionn for the correspondent uplink configuration. Note 7: NR CA fallback to 1CC is sufficient to test, unless both NR cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 8: Test ID with UL harmonic exception avoided. Note 9: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.1.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2.

7.3B.2.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with Note 3 in Table 7.3B.2.3_1.1.4.1-1, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3_1.1.5	Test requirement

Reference sensitivity test requirements for EN-DC configurations affected by 3 band 2UL intermodulation interference, are specified in Table 7.3B.2.3_1.1.5-1 and Table 7.3B.2.3_1.1.5-2 with uplink configuration specified in Table 7.3B.2.3_1.1.4.1-1.

Reference sensitivity test requirements for test points in Table 7.3B.2.3_1.1.4.1-1, are specified in  Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.3_1.1.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | Test ID |  | EUTRA/ NR band |  | SCS (kHz) |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 20 MHz (dBm) |  | 40 MHz (dBm) |  | Duplex mode |  | IMD order |  | Single UL allowed |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A |  | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  |  | 3 |  | N/A |  | -92.3 |  | - |  | - |  | - |  |  |  | IMD5 |  |  |  |
|  |  | 2 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  |  | 1 |  | N/A |  | -88.3 |  | - |  | - |  | - |  |  |  | IMD4 |  |  |  |
| DC_1A-7A_n28A | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  | 7 |  | N/A |  | - |  | -64.3 |  | - |  | - |  |  |  | IMD2 |  |  |  |
| DC_1A-3A_n77A | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | -64.8 |  | - |  | - |  | - |  |  |  | IMD2 |  |  |  |
|  |  |  | n77 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | -87.8 |  | - |  | - |  | - |  |  |  | IMD4 |  |  |  |
|  |  |  | n77 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 1 |  | N/A |  | -68.3 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  | n77 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 |  | 1 |  | N/A |  | -91.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | -65.1 |  | - |  | - |  | - |  |  |  | IMD2 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 1 |  | N/A |  | -96.5 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-5A_n78A | 1 |  | 1 |  | N/A |  | -91.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 5 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 5 |  | N/A |  | -89.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 1 |  | N/A |  | -81.2 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 5 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 4 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 5 |  | N/A |  | -94.2 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-7A_n78A | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 7 |  | N/A |  | -88.2 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 1 |  | N/A |  | -90.6 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 7 |  | N/A |  | - |  | REFSENS |  | - |  |  |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-20A_n78A | 1 |  | 1 |  | N/A |  | -79.0 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-20A_n78A | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | -93.3 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-28A_n3A | 1 |  | 28 |  | N/A |  | REFSENS |  |  |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  |  |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 1 |  | N/A |  | -88.3 |  |  |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
| DC_1A_n28A-n78A |  | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | -80.1 + TT |  | - |  | - |  | TDD |  | IMD3 |  |  |  |
|  |  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | -95.2 + TT |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
| DC_1A-28A_n78A | 1 |  | 1 |  | N/A |  | -83.6 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |  |
|  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |  |
|  |  |  | 28 |  | N/A |  | -93.6 |  | - |  | - |  |  |  | FDD |  | IMD5 |  |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |  |
| DC_2A-14A_n66A | 1 |  | 2 |  | N/A |  | -90.1 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 14 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n66 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_3A-5A_n78A | 1 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 5 |  | N/A |  | -89.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 3 |  | N/A |  | -70.3 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 5 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | -88.3 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  |  |  |  |  | [TBD] |  |  |  |  |  | 10.75 |  |  |  |  |  |  |  |
| DC_3A-7A_n28A |  | 1 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  |  | 7 |  | N/A |  | - |  | -77.4 |  | - |  | - |  |  |  | IMD3 |  |  |  |
|  |  | 2 |  | 7 |  | N/A |  | - |  | REFSENS |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  |  | 3 |  | N/A |  | -70.3 |  | - |  | - |  | - |  |  |  | IMD2 |  |  |  |
| DC_3C-7C_n78A | 1 |  | 3 |  | N/A |  | -78.7 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 3 |  | N/A |  | -87.7 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-7A_n78A DC_3C-7A_n78A | 1 |  | 3 |  | N/A |  | -78.7 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-7A_n78A DC_3C-7A_n78A | 2 |  | 3 |  | N/A |  | -87.7 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-8A_n78A | 1 |  | 3 |  | N/A |  | -79.8 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 8 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-20A_n28A |  | 1 |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | 3 |  | N/A |  | -86.9 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
| DC_3A-20A_n78A DC_3C-20A_n78A | 1 |  | 3 |  | N/A |  | -79.0 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A_n28A-n78A | 1 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |  |
|  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | -91.3+TT |  | - |  | - |  | TDD |  | IMD5 |  |  |  |  |
| DC_3A-28A_n78A | 1 |  | 3 |  | N/A |  | -79 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-40A_n1A | 1 |  | n1 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 40 |  | N/A |  | -91.3 |  | - |  | - |  | - |  | TDD |  | IMD5 |  |  |  |
| DC_5A-7A_n78A | 1 |  | 5 |  | N/A |  | -89.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 5 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 7 |  | N/A |  | -67.2 |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 5 |  | N/A |  | -67.1 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 4 |  | 5 |  | N/A |  | -94.0 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A-20A_n1A | 1 |  | 7 |  |  |  |  |  | REFSENS |  |  |  |  |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  |  |  | -88.8 |  |  |  |  |  |  |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n1 |  |  |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
| DC_7A-20A_n3A | 1 |  | 7 |  | N/A |  | - |  | REFSENS |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | - |  | -82.8 |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | - |  | -68.3 |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n3 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_7A-20A_n28A |  | 1 |  | 7 |  | N/A |  | - |  | -91.4 |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_7A-20A_n78A | 1 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | -65.8 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A-20A_n78A | 2 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | -93.3 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A-20A_n78A | 3 |  | 7 |  | N/A |  | -66.5 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A-28A_n3A | 1 |  | 7 |  | N/A |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
|  |  |  | 28 |  | N/A |  | -77.8 |  |  |  |  |  |  |  | FDD |  | IMD2 |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | -79.3 |  |  |  |  |  |  |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
| DC_7A-28A_n78A | 1 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 28 |  | N/A |  | -89.5 |  | - |  | - |  | - |  |  |  | IMD2 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 28 |  | N/A |  | -94.8 |  | - |  | - |  | - |  |  |  | IMD5 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 7 |  | N/A |  | -66.8 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A_n28A-n78A |  | 1 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | -66.1+TT |  | - |  | - |  | TDD |  | IMD2 |  |  |  |
|  |  | 2 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | -69.7+TT |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_20A_n28A-n78A |  | 1 |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | -87.1+TT |  | - |  | - |  | TDD |  | IMD4 |  |  |  |
|  |  | 2 |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | -89.1+TT |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
| DC_14A-66A_n2A | 1 |  | 14 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 66 |  | N/A |  | -91.2 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n2 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| NOTE 1: Both of the transmitters shall be set min (+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 6: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the Band 46. The reference sensitivity should only be verified when this is not the case (the requirements for Band 46 specified in the CA_7A-46A in clause 7.3.1 of TS 36.101 [5] apply). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78C | 1 | N/A | -92.0+TT | - | - | - | FDD | IMD4 |  |
|  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | n78 | 15 | - | REFSENS | - | - |  | N/A |  |
| NOTE 1: Both of the transmitters shall be set min (+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |  |  |

Test tolerance is the same as given in Table 7.3B.2.3.5-2.

<End of Changes>