3GPP TSG-RAN WG5 Meeting #94-e	R5-221036r1

# Online, , 21st February 2022 - 4th March 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0331 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new IMS over 5GS TC 11.8 eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Fulfilling the work plan by introducing new test case 11.8 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 11.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.8 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0305 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-221036r1: 1. Updated numbering. |  |  |  |  |  |  |  |  |

<Start of modified section>

11.8	eCall only mode / Automatic initiation / Emergency Registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS

11.8.1	Test Purpose (TP)

(1)

with { UE is in state 5GMM-DEREGISTERED.eCALL-INACTIVE }

ensure that {

when { UE is being made to initiate an automatic eCall over IMS }

then { UE performs IMS and emergency registration and sends correctly composed INVITE with MSD }

}

(2)

with { UE having sent an  INVITE }

ensure that {

when { UE receives 600 Busy Everywhere }

then { UE performs domain selection to a cell supporting CS domain (UTRAN/GERAN) based on capability supported and initiates ecall over CS domain }

}

11.8.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-16 requirements.

[TS 24.229 clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

[TS 24.229 clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2015 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

3)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B]; and

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

11.8.3	Test description

11.8.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell 1 connected to 5GC, default parameters.

-	if px_NR_RATComb_Tested = NR_UTRA, UTRA Cell 5 as defined in TS 36.508 [52]

-	if px_NR_RATComb_Tested = NR_GERAN, GERAN cell 24 as defined in TS 36.508 [52]

-	NR Cell 1 power level is set as “serving cell”

-     Cell 24/Cell 5 power level is set as “suitable cell” as defined in TS 36.508 [52].

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 38.508-1 [21] Table 6.4.1-24.

Preamble:

-	UE is in state 5GMM-DEREGISTERED.eCALL-INACTIVE.

11.8.3.2	Test procedure sequence

Table 11.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start an automatic eCall | - | - | - | - |
| 2-19 | Steps 2 to 19 of generic procedure specified in Table 4.5.2.2-2 of 38.508-1 [21] takes place. | - | - | - | - |
| 20-22 | Steps 8-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 23 and 24, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 23 | Step 1 of Annex A.23 happens Check: Does the UE include MSD in the message body? | --> | INVITE | 1 | P |
| 24 | TE sends 600 Busy Everywhere response | <-- | 600 Busy Everywhere | - | - |
|  | EXCEPTION: Steps 25a1-25b5 takes place, depending on the value of px_NR_RATComb_Tested | - | - | - | - |
| 25a1 | IF px_NR_RATComb_Tested = NR_UTRA UE performs domain selection to a cell supporting CS domain (UTRAN) and performs eCall establishment in CS domain together with MM registration | - | - | 2 | P |
| 25a2 | SS configures NR Cell 1 as a “non-suitable cell” | - | - | - | - |
| 25a3 | eCall is maintained for 5 seconds | - | - | - | - |
| - | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP re-registration defined in Annex A.12 can take place | - | - | - | - |
| - | EXCEPTION: Depending on the UE implementation, the generic test procedure for mobile initiated IMS SIP de-registration defined in Annex A.11 can take place | - | - | - | - |
| 25a4 | eCall is cleared by SS | - | - | - | - |
| 25b1 | IF px_NR_RATComb_Tested = NR_GERAN UE performs domain selection to a cell supporting CS domain (GERAN cell) and performs eCall establishment in CS domain | - | - | 2 | P |
| 25b2 | SS configures NR Cell 1 as a “non-suitable cell” | - | - | - | - |
| 25b3 | eCall is maintained for 5 seconds | - | - | - | - |
| 25b2 | eCall is cleared by SS | - | - | - | - |
| 25b5 | UE performs MM/GMM registration | - | - | - | - |

11.8.3.3	Specific message contents

Table 11.8.3.3-1: INVITE (step 23, table 11.8.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.8.3.3-2: 600 Busy Everywhere (step 24, table 11.8.3.2-1)

Derivation path: TS 34.229-1 [2], Annex A.2.22 with Condition A1

Table 11.8.3.3-4: EMERGENCY SETUP (step 25a1 or 25b1, table 11.8.3.2-1)

| Information Element | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Service Category IE |  | bit 7 = 1 and all other bits are set to 0. |  |  |

Table 11.8.3.3-3: ROUTING AREA UPDATE ACCEPT (step 25b5, table 11.8.3.2-1)

| Derivation path: TS 36.508, Table 4.7B.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Cond | Value/remark | Rel | Reference |  |
| PDP Context Status |  | 0 NSAPI(0) - NSAPI(15) is set to 0, which means that the SM state of all PDP contexts is PDP-INACTIVE |  |  |  |

<End of modifed section>