3GPP TSG-RAN5 Meeting #94-e	R5-221169r1

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0502 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to HST Demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test applicability for multiple HST test cases is incorrectly specified as Rel16 onwards. Based on TS 38.307, all HST Demodulation test cases are release independent. Hence applicable for UE Rel15 and onwards Typo in the test requirement SNR for FDD HST-SFN test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -- updated Test Applicability section for multiple HST test cases for release applicability -- updated the test requirement SNR for FDD HST-SFN test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation of the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.x.9, 5.2.2.x.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependent on discussion paper R5-221168 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: removing editor’s note for Annex F MU/TT since Annex F contents are introduced via other CRs R5-220636, R5-220637 Added an editor’s note to state that the minimum test time values are in []. Annex G.1.5 updated for HST single tap, HST-SFN and HST-DPS scenarios |  |  |  |  |  |  |  |  |

###### 5.2.2.1.9_1	2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA

###### Editor's note:	The minimum test time value is in []

5.2.2.1.9_1.1	Test purpose

To verify the PDSCH performance under 2 receive antennas conditions in the HST-SFN scenario defined in B.3.2 when highSpeedDemodFlag-r16 IE [20] is configured and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.1.9_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

<many sections skipped>

5.2.2.1.9_1.4	Test requirement

Tables 5.2.2.1.9_1.4-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.9_1.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.9_1.4-1: Test Requirements for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.3 FDD | 10 / 15 | 16QAM, 0.48 | HST-SFN | 2x2 | 70 | 13.6 |

<many sections skipped>

###### 5.2.2.1.10_1	2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.1.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.1.10_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15.

<many sections skipped>

###### 5.2.2.2.9_1	2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	The minimum test time value is in []

5.2.2.2.9_1.1	Test purpose

To verify the PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 when highSpeedDemodFlag-r16 IE [20] is configured and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.2.9_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC that supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

<many sections skipped>

###### 5.2.2.2.10_1	2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's note:	The minimum test time value is in []

5.2.2.2.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.2.10_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

<many sections skipped>

###### 5.2.3.1.9_1	4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	The minimum test time value is in []

5.2.3.1.9_1.1	Test purpose

To verify the PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 when highSpeedDemodFlag-r16 IE [20] is configured and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.3.1.9_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

<many sections skipped>

###### 5.2.3.1.10_1	4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	The minimum test time value is in []

5.2.3.1.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.3.1.10_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15.

<many sections skipped>

###### 5.2.3.2.9_1	4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA

Editor's note:	The minimum test time value is in [].

5.2.3.2.9_1.1	Test purpose

To verify the PDSCH performance under 4 receive antenna conditions in the HST-SFN scenario defined in B.3.2 when highSpeedDemodFlag-r16 IE [20] is configured and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.3.2.9_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC supporting enhanced demodulation processing for HST-SFN joint transmission scheme.

<many sections skipped>

# G.1	Statistical testing of Performance Requirements with throughput

<many sections skipped>

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern | Reference Channel | Propagation condition | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- | --- |
| NA | R.PDSCH.1-8.1 FDD | HST-750 | 750 Hz | 6000 (Note 1) | 1.0526 | 6400 |
| NA | R.PDSCH.1-8.2 FDD | HST-972 | 972 Hz | 6000 (Note 1) | 1.0526 | 6400 |
| NA | R.PDSCH.1-8.1 FDD | TDLC300-600 | 600 Hz | 8000 (Note 1) | 1.0526 | 9000 |
| NA | R.PDSCH.1-1.1 FDD | TDLB100-400 | 400 Hz | 10000 (Note 1) | 1.0526 | 11000 |
| NA | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD, | TDLC300-100 | 100 Hz | 20000 (Note 1) | 1.0526 | 22000 |
| NA | R.PDSCH.1-1.3 FDD R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-2.5 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-3.2 FDD, R.PDSCH.1-3.3 FDD, R.PDSCH.1-3.4 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-12.1 FDD, R.PDSCH.2-1.1 FDD, | TDLA30-10 | 10 Hz | 75000 (Note 1) | 1.0526 | 79000 |
| NA | R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD | TDLA30-10 | 10 Hz | 75000 (Note 1) | 1.25 | 94000 |
| NA | R.PDSCH.1-8.3 FDD | HST-SFN | 870 Hz | 30000 (Note 1) | 1.0526 | 32000 |
| NA | R.PDSCH.1-8.4 FDD, R.PDSCH.1-8.4 FDD | HST-DPS | 870 Hz | 30000 (Note 1) | 1.0526 | 32000 |
| FR1.15-1 | R.PDSCH.1-1.1 TDD R.PDSCH.1-1.2 TDD | TDLA30-10 | 10Hz | 75000 (Note 1) | 2.8571 | 215000 |
| FR1.30-1A | R.PDSCH.2-1.1 TDD | TDLB100-400 | 400 Hz | 10000 (Note 1) | 1.2903 | 13000 |
| FR1.30-1 | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD | TDLC300-100 | 100 Hz | 20000 (Note 1) | 1.2903 | 26000 |
| FR1.30-1 | R.PDSCH.2-2.2 TDD R.PDSCH.2-2.3 TDD R.PDSCH.2-2.4 TDD R.PDSCH.2-2.5 TDD R.PDSCH.2-3.1 TDD, R.PDSCH.2-3.2 TDD, R.PDSCH.2-3.3 TDD, R.PDSCH.2-3.4 TDD, R.PDSCH.2-4.1 TDD, | TDLA30-10 | 10 Hz | 75000 (Note 1) | 1.2903 | 97000 |
| FR1.30-1 | R.PDSCH.2-1.3 TDD | TDLA30-10 | 10 Hz | 75000 (Note 1) | 1.4815 | 112000 |
| FR1.30-2 | R.PDSCH.2-5.1 TDD | TDLA30-10 | 10 Hz | 75000 (Note 1) | 1.2903 | 97000 |
| FR1.30-2 | R.PDSCH.2-17.1 TDD | TDLA30-10 | 10 Hz | 75000 (Note 1) | 5 | 375000 |
| FR1.30-3 | R.PDSCH.2-6.1 TDD | TDLA30-10 | 10 Hz | 75000 (Note 1) | 1.4815 | 112000 |
| FR1.30-4 | R.PDSCH.2-9.1 TDD | TDLA30-10 | 10 Hz | 75000 (Note 1) | 1.2903 | 97000 |
| FR1.30-5 | R.PDSCH.2-11.1 TDD | TDLB100-400 | 400Hz | 10000 (Note 1) | 1.2903 | 13000 |
| FR1.30-6 | R.PDSCH.2-12.1 TDD | TDLB100-400 | 400Hz | 10000 (Note 1) | 1.2903 | 13000 |
| FR1.30-1 | R.PDSCH.2-10.1 TDD | HST-1000 | 1000 Hz | 15000 (Note 1) | 1.4815 | 23000 |
| FR1.30-1 | R.PDSCH.2-10.1 TDD | HST-1667 | 1667 Hz | 15000 (Note 1) | 1.4815 | 23000 |
| FR1.30-1 | R.PDSCH.2-10.4 TDD | HST-SFN | 1667 Hz | 30000 (Note 1) | 1.4815 | 45000 |
| FR1.30-1 | R.PDSCH.2-10.5 TDD | HST-DPS | 1667 Hz | 30000 (Note 1) | 1.4815 | 45000 |
| FR2.60-1 | R.PDSCH.4-1.1 TDD | TDLA30-75 | 75 Hz | 20000 (Note 2) | 1.33 | 27000 |
| FR2.120-1A | R.PDSCH.5-1.1 TDD | TDLC60-300 | 300 Hz | 10000 (Note 2) | 1.25 | 13000 |
| FR2.120-1 | R.PDSCH.5-2.1 TDD R.PDSCH.5-2.2 TDD R.PDSCH.5-2.3 TDD R.PDSCH.5-3.1 TDD | TDLA30-300 | 300 Hz | 10000 (Note 2) | 1.25 | 13000 |
| FR2.120-1 | R.PDSCH.5-1.2 TDD | TDLA30-75 | 75 Hz | 20000 (Note 2) | 1.25 | 25000 |
| FR2.120-2 | R.PDSCH.5-4.1 TDD R.PDSCH.5-5.1 TDD R.PDSCH.5-5.2 TDD R.PDSCH.5-6.1 TDD | TDLA30-75 | 75 Hz | 20000 (Note 2) | 1.33 | 27000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |  |