3GPP TSG-WG5 Meeting #94-e	R5-221199r1

# Electronic Meeting, 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2835 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New testcase for Idle/Inactive measurements on E-UTRA cells in RRC_INACTIVE state with configuration through SIB11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To test Idle/Inactive measurements while UE is in RRC_INACTIVE state and measured E-UTRA cells are informed through SIB 11. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new testcase testing Idle/Inactive measurements of E-UTRA cells while UE is in RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Idle/Inactive measurements functionality is not tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6 MR-DC RRC others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0216 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Added clauses affected. |  |  |  |  |  |  |  |  |

------------------------------------------------- Start of changes --------------------------------------------------

------------------------------------------------- Skipped chapters -------------------------------------------------

Table 8.2.6.3.2.3.3-6: RRCResumeComplete (step 6a7, Table 8.2.6.2.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleMeasAvailable-r16 |  | Not Present |  |  |
| measResultIdleEUTRA-r16 |  | Not Present |  |  |
| measResultIdleNR-r16 |  | Not Present |  |  |
| scg-Response-r16 CHOICE{ |  |  |  |  |
| nr-SCG-Response |  | RRCReconfigurationComplete defined as per TS 38.508-1[4] Table 4.6.1-14 with condition NR-DC |  |  |
| eutra-SCG-Response |  | Not Present |  |  |
| } |  |  |  |  |
| ue-MeasurementsAvailable-r16 |  | Not Present |  |  |
| mobilityHistoryAvail-r16 |  | Not Present |  |  |
| mobilityState-r16 |  | Not Present |  |  |
| needForGapsInfoNR-r16 |  | Not Present |  |  |
| nonCriticalExtension |  | Not Present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 8.2.6.3	Idle/Inactive measurements

##### 8.2.6.3.3	Idle/Inactive measurements / Inactive mode / NE-DC / SIB11 configuration

8.2.6.3.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig and suspendConfig, SIB11 including measIdleCarrierListEUTRA }

then { UE enters RRC_INACTIVE state, performs measurements on E-UTRA cells and reports measurement results in RRC_CONNECTED state }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig, SIB11 including measIdleCarrierListEUTRA, T331 expires }

then { UE enters RRC_INACTIVE state, UE does not report measurement results in RRC_CONNECTED state after T331 expiration }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCRelease message including measIdleConfig and suspendConfig, SIB11 does not include measIdleCarrierListEUTRA }

then { UE enters RRC_INACTIVE state, UE does not report measurement results in RRC_CONNECTED state due to missing configuration }

}

8.2.6.3.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.8.3.

[TS 38.331, clause 5.3.8.3]

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListEUTRA:

4>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

[TS 38.331, clause 5.7.8.3]

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

[TS 38.331, clause 5.7.8.1a]

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCRelease message:

2>	if the UE supports idleInactiveEUTRA-MeasReport:

3>	if the SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListEUTRA:

4>	store or replace the measIdleCarrierListEUTRA of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListEUTRA in VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

[TS 38.331, clause 5.7.8.2a]

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListEUTRA and the SIB1 contains idleModeMeasurementsEUTRA:

3>	for each entry in measIdleCarrierListEUTRA within VarMeasIdleConfig:

4>	if UE supports NE-DC between the serving carrier and the carrier frequency indicated by carrierFreqEUTRA within the corresponding entry:

5>	perform measurements in the carrier frequency and bandwidth indicated by carrierFreqEUTRA and allowedMeasBandwidth within the corresponding entry;

5>	if the reportQuantitiesEUTRA is set to rsrq:

6>	consider RSRQ as the sorting quantity;

5>	else:

6>	consider RSRP as the sorting quantity;

5>	if the measCellListEUTRA is included:

6>	consider cells identified by each entry within the measCellListEUTRA to be applicable for idle/inactive mode measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive measurement results for the measurement quantities indicated by reportQuantitiesEUTRA;

5>	store the derived measurement results as indicated by reportQuantitiesEUTRA within the measReportIdleEUTRA in VarMeasIdleReport in decreasing order of the sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThresholdEUTRA is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThresholdEUTRA;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signaling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

8.2.6.3.3.3	Test Description

8.2.6.3.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	LTE Cell 1, suitable for inter-frequency measurements

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-1.

8.2.6.3.3.3.2	Test procedure sequence

Table 8.2.6.3.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCRelease message including measIdleConfig and suspendConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 2 | Wait for 66 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 3-10 | Steps 1 to 8 of procedure described in TS 38.508-1 table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 11 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 1 | P |
| 13 | The SS transmits RRCRelease message including measIdleConfig and suspendConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 14 | Wait for 130 s for UE to enter RRC_INACTIVE mode and T331 to expire. | - | - | - | - |
| 15-22 | Steps 1 to 8 of procedure described in TS 38.508-1 table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 23 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 24 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 2 | F |
| 25 | The SS removes measIdleCarrierListEUTRA in SIB11. | - | - | - | - |
| 26 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 27 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 28 | The SS transmits RRCRelease message including measIdleConfig and suspendConfig to the UE. | <- | NR RRC: RRCRelease | - | - |
| 29 | Wait for 66 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 30-37 | Steps 1 to 8 of procedure described in TS 38.508-1 table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 38 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <- | NR RRC: UEInformationRequest | - | - |
| 39 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | -> | NR RRC: UEInformationResponse | 3 | F |

8.2.6.3.3.3.3	Specific message contents

Table 8.2.6.3.3.3.3-1: RRCRelease (steps 1, 13 and 28, Table 8.2.6.3.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig | SuspendConfig |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r16 SetupRelease { |  |  |  |
| setup | MeasIdleConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.3.3.3-2: SuspendConfig (Table 8.2.6.3.3.3.3-1)

| Derivation Path: TS 38.331 [XYZ] Chapter 6.2.2 Message definitions - RRCRelease |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SuspendConfig ::= SEQUENCE { |  |  |  |
| fullI-RNTI | I-RNTI-Value | 38.508-1[4] Table 4.6.3-64 |  |
| shortI-RNTI | ShortI-RNTI-Value | 38.508-1[4] Table 4.6.3-170 |  |
| ran-PagingCycle | rf32 |  |  |
| ran-NotificationAreaInfo CHOICE { |  |  |  |
| cellList SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell { | 2 entries |  |  |
| PLMN-RAN-AreaCell[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity | PLMN1 |  |  |
| ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity { | 1 entry |  |  |
| CellIdentity[1] | CellIdentity of NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| PLMN-RAN-AreaCell[2] SEQUENCE { |  | entry 2 |  |
| plmn-Identity | PLMN2 |  |  |
| ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity { | 1 entry |  |  |
| CellIdentity[1] | CellIdentity of LTE Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| t380 | Not Present |  |  |
| nextHopChainingCount | NextHopChainingCount | 38.508-1[4] Table 4.6.3-83 |  |
| } |  |  |  |

Table 8.2.6.3.3.3.3-3: MeasIdleConfigDedicated (Table 8.2.6.3.3.3.3-1)

| Derivation Path: TS 38.331 [XYZ] Chapter 6.3.2 Radio resource control information elements - MeasIdleConfig |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not Present |  |  |
| measIdleCarrierListEUTRA-r16 | Not Present |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not Present |  |  |
| } |  |  |  |

Table 8.2.6.3.3.3.3-4: SIB11 (Table 8.2.6.3.3.3.3-1, steps 1 to 24)

| Derivation Path: TS 38.331 [XYZ] Chapter 6.3.1 System information blocks – SIB11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB11-r16 ::= SEQUENCE { |  |  |  |
| measIdleConfigSIB-r16 SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not Present |  |  |
| measIdleCarrierListEUTRA-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16 { | 1 entry |  |  |
| MeasIdleCarrierEUTRA-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreqEUTRA-r16 | ARFCN value corresponding to LTE cell 1 |  |  |
| allowedMeasBandwidth-r16 | mbw50 |  |  |
| measCellListEUTRA-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF EUTRA-PhysCellIdRange { | 1 entry |  |  |
| EUTRA-PhysCellIdRange[1] SEQUENCE { |  | entry 1 |  |
| start | EUTRA-PhysCellId corresponding to LTE cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuatitiesEUTRA-r16 | both |  |  |
| qualityThresholdEUTRA-r16 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.3.3.3-5: SIB11 (Table 8.2.6.3.3.3.3-1, steps 25 to 39)

| Derivation Path: TS 38.331 [XYZ] Chapter 6.3.1 System information blocks – SIB11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB11-r16 ::= SEQUENCE { |  |  |  |
| measIdleConfigSIB-r16 SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not Present |  |  |
| measIdleCarrierListEUTRA-r16 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.3.3.3-6: UEInformationRequest (Table 8.2.6.3.3.3.3-1)

| Derivation Path: TS 38.331 [XYZ] Chapter 6.2.2 Message definitions – UEInformationRequest |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 8.2.7

#### 8.2.7.1

#### 8.2.7.2	RRC Resume

##### 8.2.7.2.1	RRC Resume / NR-DC

8.2.7.2.1.1	Test Purpose (TP)

------------------------------------------------- Skipped chapters -------------------------------------------------

------------------------------------------------- End of changes -------------------------------------------------