3GPP TSG-RAN5 Meeting #94-e	R5-221220r1

# Electronic Meeting,  21th February 2022 - 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5082 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to test case 7.1.4.42 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is text missing in the Conformance requirements clause. A reference to core requirement specification is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added missing text to the TS 36.213, clause 8.1.1, core requirement. Changed the reference to TS 36.312 to TS 36.213 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.4.42 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated reference in Table 7.1.4.42.3.3-1: RadioResourceConfigCommon-DEFAULT to preamble. Added contents to the ce-PUSCH-FlexibleStartPRB-AllocConfig-r15 IE in Table 7.1.4.42.3.3-2. |  |  |  |  |  |  |  |  |

#### 7.1.4.42	Enhanced Coverage / UL Flexible starting PRB

7.1.4.42.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE has pending data for transmission and receives an uplink grant with RIV calculated by values of  and }

then { UE transmits MAC PDU on PUSCH on the granted resources using DCI format 6-0A and the received resource allocation information }

}

7.1.4.42.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.213, clause 8.1.1.

[TS 36.213, clause 8.1.1]

The resource allocation information for uplink resource allocation type 0 indicates to a scheduled UE a set of contiguously allocated virtual resource block indices denoted by . A resource allocation field in the scheduling grant consists of a resource indication value (RIV) corresponding to a starting resource block () and a length in terms of contiguously allocated resource blocks ( 1).

For a BL/CE UE,

-	uplink resource allocation type 0 is only applicable for UE configured with CEModeA, and

-	, if the UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig; otherwise  and,

-	if the UE is configured with higher layer parameters ce-PUSCH-FlexibleStartPRB-AllocConfig, shall not exceed  with ={, where  is the number of edge RB(s) not belonging to any narrowband in one side of system bandwidth , and  is the number of narrowbands. PUSCH resource allocations shall not contain PRB(s) not belonging to any narrowband unless it is the centre PRB in the uplink system bandwidth, and,

-	if the UE is not configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig,  is always set to 6 in this subclause regardless of the system bandwidth.

For PDCCH/SPDCCH DCI format 7-0A/7-0B and , VRB allocations for a UE vary from 4 VRB(s) up to  VRBs with an increment step of 4 VRBs. A type 0 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block  using  and a length in terms of virtually contiguously allocated resource blocks , where  is defined if configured by higher layer parameter resourceAllocationOffset; otherwise set to 0. The resource indication value is defined by:

if  then

else

where , and , and where,

1 and shall not exceed .

Otherwise, the resource indication value is defined by

if  then

if a BL/CE UE in TDD is configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig, then

else

else

7.1.4.42.3	Test description

7.1.4.42.3.1	Pre-test conditions

System Simulator:

- Cell 1

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 1.

UE:

None.

Preamble:

-	The UE is in state Loopback Activated (state 4-CE) according to [18].

7.1.4.42.3.2	Test procedure sequence

Table 7.1.4.42.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 2 | The SS transmits a MAC PDU. | <-- | MAC PDU | - | - |
| 3 | The SS allocates an uplink grant indicating DCI Format 6-0A with a RIV calculated by values of and set according to clause 8.1.1 in TS 36.213, with the UE configured with higher layer parameter ce-PUSCH-FlexibleStartPRB-AllocConfig. | <-- | (UL Grant) | - | - |
| 4 | CHECK: Does UE return the same MAC PDU as received in step 1? | --> | MAC PDU | 1 | P |

7.1.4.42.3.3	Specific message contents

Table 7.1.4.42.3.3-1: RadioResourceConfigCommon-DEFAULT (preamble)

Derivation Path : 36.508 table 4.6.3-13 with condition FullConfig and CEmodeA

Table 7.1.4.42.3.3-2: PhysicalConfigDedicated-DEFAULT (preamble)

| Derivation Path: 36.508 Table 4.8.2.1.6-1 with CEmodeA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| pusch-ConfigDedicated-v1530 CHOICE { |  |  |  |
| ce-PUSCH-FlexibleStartPRB-AllocConfig-r15 CHOICE { |  |  |  |
| setup SEQUENCE{ |  |  |  |
| offsetCE-ModeB-r15 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |