3GPP TSG-RAN5 Meeting #94-e	R5-221222r2

# Electronic Meeting,  21th February 2022 - 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2280 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NE-DC support for UECapabilityEnquiry and UECapabilityInformation messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no support for NE-DC in the UECapabilityEnquiry and UECapabilityInformation messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for NE-DC is ontroduce for the RAT-Type, UE-CapabilityRAT-ContainerList and UE-CapabilityRAT-RequestList IEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NE-DC will not be fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added condition NE-DC to Table 4.6.4-21. Removed condition NE-DC from ue-CapabilityRAT-Container IE in Table 4.6.4-45. Added condition NE-DC in Table 4.6.4-46 R2: Added updates to the FreqBandList, RF-Parameters, RF-ParametersMRDC, UE-CapabilityRAT-ContainerList, UE-CapabilityRAT-RequestList, UE-CapabilityRequestFilterNR, UE-MRDC-Capability and UE-NR-Capability IEs. |  |  |  |  |  |  |  |  |

#### –	FreqBandList

Table 4.6.4-21: FreqBandList

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandList::= SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation { | Number of entries depends on the conditions |  |  |
| FreqBandInformation[1] CHOICE { |  | entry 1 | EN-DC, NE-DC |
| bandInformationEUTRA SEQUENCE { |  |  |  |
| bandEUTRA | FreqBandIndicatorEUTRA |  |  |
| ca-BandwidthClassDL-EUTRA | Not checked |  |  |
| ca-BandwidthClassUL-EUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[2] CHOICE { |  | entry 2 |  |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[3] CHOICE { |  | entry 3 | EN-DC AND CA-InterBand |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR with condition CA-InterBand |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[1] CHOICE { |  | entry 1 | NR |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[2] CHOICE { |  | entry 2 | NR AND CA-InterBand |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR with condition CA-InterBand |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| FreqBandInformation[2] CHOICE { |  | entry 2 | NR-DC |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR with condition NR-DC-SecondaryBand |  |  |
| maxBandwidthRequestedDL | Not checked |  |  |
| maxBandwidthRequestedUL | Not checked |  |  |
| maxCarriersRequestedDL | Not checked |  |  |
| maxCarriersRequestedUL | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| CA-InterBand | Used in NR CA Inter-band test cases |
| NR | NG-RAN NR Radio Access |
| NR-DC | Used in NR-DC test cases |
| NE-DC | NR E-UTRA Dual Connectivity |

#### <<< TEXT SKIPPED >>>

#### –	RAT-Type

Table 4.6.4-37: RAT-Type

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RAT-Type | nr |  |  |
|  | eutra-nr |  | EN-DC, NE-DC |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| NE-DC | NR E-UTRA Dual Connectivity |

#### –	RF-Parameters

Table 4.6.4-38: RF-Parameters

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RF-Parameters ::= SEQUENCE { |  |  |  |
| supportedBandListNR SEQUENCE (SIZE (1..maxBands)) OF BandNR { | At least 1 entry |  |  |
| BandNR[1] SEQUENCE { |  | entry 1 |  |
| bandNR | FreqBandIndicatorNR with condition NR |  |  |
| modifiedMPR-Behaviour | Not checked |  |  |
| mimo-ParametersPerBand | Not checked |  |  |
| extendedCP | Not checked |  |  |
| multipleTCI | Not checked |  |  |
| bwp-WithoutRestriction | Not checked |  |  |
| bwp-SameNumerology | Not checked |  |  |
| bwp-DiffNumerology | Not checked |  |  |
| crossCarrierScheduling-SameSCS | Not checked |  |  |
| pdsch-256QAM-FR2 | Not checked |  |  |
| pusch-256QAM | Not checked |  |  |
| ue-PowerClass | Not checked |  |  |
| rateMatchingLTE-CRS | Not checked |  |  |
| channelBWs-DL CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| channelBWs-UL CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| BandNR[2] SEQUENCE { |  | entry 2 | NR-DC-SecondaryBand NR_CA-InterBand |  |
| bandNR | FreqBandIndicatorNR with condition NR-DC-SecondaryBand |  | NR-DC-SecondaryBand |  |
|  | FreqBandIndicatorNR with condition NR_CA-InterBand |  | NR_CA-InterBand |  |
| modifiedMPR-Behaviour | Not checked |  |  |  |
| mimo-ParametersPerBand | Not checked |  |  |  |
| extendedCP | Not checked |  |  |  |
| multipleTCI | Not checked |  |  |  |
| bwp-WithoutRestriction | Not checked |  |  |  |
| bwp-SameNumerology | Not checked |  |  |  |
| bwp-DiffNumerology | Not checked |  |  |  |
| crossCarrierScheduling-SameSCS | Not checked |  |  |  |
| pdsch-256QAM-FR2 | Not checked |  |  |  |
| pusch-256QAM | Not checked |  |  |  |
| ue-PowerClass | Not checked |  |  |  |
| rateMatchingLTE-CRS | Not checked |  |  |  |
| channelBWs-DL CHOICE { |  |  |  |  |
| fr1 SEQUENCE { |  |  |  |  |
| scs-15kHz | Not checked |  |  |  |
| scs-30kHz | Not checked |  |  |  |
| scs-60kHz | Not checked |  |  |  |
| } |  |  |  |  |
| fr2 SEQUENCE { |  |  |  |  |
| scs-60kHz | Not checked |  |  |  |
| scs-120kHz | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| channelBWs-UL CHOICE { |  |  |  |  |
| fr1 SEQUENCE { |  |  |  |  |
| scs-15kHz | Not checked |  |  |  |
| scs-30kHz | Not checked |  |  |  |
| scs-60kHz | Not checked |  |  |  |
| } |  |  |  |  |
| fr2 SEQUENCE { |  |  |  |  |
| scs-60kHz | Not checked |  |  |  |
| scs-120kHz | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |
| maxUplinkDutyCycle-PC2-FR1 | Not checked |  |  |
| pucch-SpatialRelInfoMAC-CE | Not checked |  |  |
| powerBoosting-pi2BPSK | Not checked |  |  |
| maxUplinkDutyCycle-FR2 | Not checked |  |  |
| channelBWs-DL-v1590 | Not checked |  |  |
| channelBWs-UL-v1590 | Not checked |  |  |
| asymmetricBandwidthCombinationSet | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-r16 | Not checked |  |  |
| cancelOverlappingPUSCH-r16 | Not checked |  |  |
| multipleRateMatchingEUTRA-CRS-r16 | Not checked |  |  |
| overlapRateMatchingEUTRA-CRS-r16 | Not checked |  |  |
| pdsch-MappingTypeB-Alt-r16 | Not checked |  |  |
| oneSlotPeriodicTRS-r16 | Not checked |  |  |
| olpc-SRS-Pos-r16 | Not checked |  |  |
| spatialRelationsSRS-Pos-r16 | Not checked |  |  |
| simulSRS-MIMO-TransWithinBand-r16 | Not checked |  |  |
| channelBW-DL-IAB-r16 | Not checked |  |  |
| channelBW-UL-IAB-r16 | Not checked |  |  |
| rasterShift7dot5-IAB-r16 | Not checked |  |  |
| ue-PowerClass-v1610 | Not checked |  |  |
| condHandover-r16 | Not checked |  |  |
| condHandoverFailure-r16 | Not checked |  |  |
| condHandoverTwoTriggerEvents-r16 | Not checked |  |  |
| condPSCellChange-r16 | Not checked |  |  |
| condPSCellChangeTwoTriggerEvents-r16 | Not checked |  |  |
| mpr-PowerBoost-FR2-r16 | Not checked |  |  |
| activeConfiguredGrant-r16 | Not checked |  |  |
| maxNumberConfigsPerBWP-r16 | Not checked |  |  |
| maxNumberConfigsAllCC-r16 | Not checked |  |  |
| jointReleaseConfiguredGrantType2-r16 | Not checked |  |  |
| sps-r16 | Not checked |  |  |
| maxNumberConfigsPerBWP-r16 | Not checked |  |  |
| maxNumberConfigsAllCC-r16 | Not checked |  |  |
| jointReleaseSPS-r16 | Not checked |  |  |
| simulSRS-TransWithinBand-r16 | Not checked |  |  |
| trs-AdditionalBandwidth-r16 | Not checked |  |  |
| handoverIntraF-IAB-r16 | Not checked |  |  |
| simulTX-SRS-AntSwitchingIntraBandUL-CA-r16 | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-v1630 | Not checked |  |  |
| handoverUTRA-FDD-r16 | Not checked |  |  |
| enhancedUL-TransientPeriod-r16 | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-v1640 | Not checked |  |  |
| supportedBandCombinationList | Not checked |  |  |
| appliedFreqBandListFilter | Not present |  |  |
|  | FreqBandList |  | FILTER_REQUESTED |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FILTER_REQUESTED | This condition shall be set to true when UE is requested to filter the information via ‘capabilityRequestFilter’ IE in the NR5GC UECapabilityEnquiry message or via ‘requestedFreqBandsNR-MRDC’ IE in the EN-DC UECapabilityEnquiry message |
| NR-DC-SecondaryBand | Used in NR-DC test cases |
| NR_CA-InterBand | NG-RAN NR Radio Access using NR CA Inter-band test cases |

#### –	RF-ParametersMRDC

Table 4.6.4-39: RF-ParametersMRDC

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RF-ParametersMRDC ::= SEQUENCE { |  |  |  |
| supportedBandCombinationList | BandCombinationList |  |  |
| appliedFreqBandListFilter | FreqBandList with condition EN-DC |  | EN-DC |
|  | FreqBandList with condition NE-DC |  | NE-DC |
|  | FreqBandList with condition EN-DC and CA-InterBand |  | ENDC_CA-InterBand |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| EN-DC_CA-InterBand | E-UTRA-NR Dual Connectivity used in NR CA Inter-band test cases |
| NE-DC | NR E-UTRA Dual Connectivity |

#### <<< TEXT SKIPPED >>>

#### –	UE-CapabilityRAT-ContainerList

Table 4.6.4-45: UE-CapabilityRAT-ContainerList

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRAT-ContainerList ::= SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Container { | n entries |  |  |
| UE-CapabilityRAT-Container[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | RAT-Type |  |  |
| ue-CapabilityRAT-Container | UE-NR-Capability with condition NR |  |  |
|  | UE-NR-Capability with condition NR-DC |  | NR-DC |
|  | UE-NR-Capability with condition NR_CA-InterBand |  | NR_CA-InterBand |
| } |  |  |  |
| UE-CapabilityRAT-Container[2] SEQUENCE { |  | entry 2 | EN-DC, NE-DC, EN-DC_CA-InterBand |
| rat-Type | RAT-Type with condition NE-DC |  |  |
| ue-CapabilityRAT-Container | UE-MRDC-Capability with condition EN-DC |  | EN-DC |
|  | UE-MRDC-Capability with condition NE-DC |  | NE-DC |
|  | UE-MRDC-Capability with condition EN-DC_CA-InterBand |  | EN-DC_CA-InterBand |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| EN-DC_CA-InterBand | E-UTRA-NR Dual Connectivity using NR CA Inter-band test cases |
| NE-DC | NR E-UTRA Dual Connectivity |
| NR_CA-InterBand | NG-RAN NR Radio Access using NR CA Inter-band test cases |
| NR-DC | Used in NR-DC test cases |

#### –	UE-CapabilityRAT-RequestList

Table 4.6.4-46: UE-CapabilityRAT-RequestList

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRAT-RequestList ::= SEQUENCE (SIZE (0.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request { | n entries |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 | NR, NR-DC, NR_CA-InterBand |
| rat-Type | RAT-Type |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR with condition NR | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
|  | UE-CapabilityRequestFilterNR with condition NR-DC | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) | NR-DC |
|  | UE-CapabilityRequestFilterNR with condition NR_CA-InterBand | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) | NR_CA-InterBand |
| } |  |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 | NE-DC, EN-DC, EN-DC_CA-InterBand |
| rat-Type | RAT-Type with condition NR |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR with condition NR | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
| } |  |  |  |
| UE-CapabilityRAT-Request[2] SEQUENCE { |  | entry 2 | NE-DC |
| rat-Type | RAT-Type with condition NE-DC |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR with condition NE-DC | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
| } |  |  |  |
| UE-CapabilityRAT-Request[2] SEQUENCE { |  | entry 2 | EN-DC, EN-DC_CA-InterBand |
| rat-Type | RAT-Type with condition EN-DC |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR with condition EN-DC | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
|  | UE-CapabilityRequestFilterNR with condition EN-DC_CA-InterBand |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| EN-DC_CA-InterBand | E-UTRA-NR Dual Connectivity using NR CA Inter-band test cases |
| NR | NG-RAN NR Radio Access |
| NR-DC | Used in NR-DC test cases |
| NR_CA-InterBand | NG-RAN NR Radio Access using NR CA Inter-band test cases |
| NE-DC | NR E-UTRA Dual Connectivity |

#### <<< TEXT SKIPPED >>>

#### –	UE-CapabilityRequestFilterNR

Table 4.6.4-47: UE-CapabilityRequestFilterNR

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRequestFilterNR ::= SEQUENCE { |  |  |  |
| frequencyBandListFilter | FreqBandList with condition NR |  | NR |
|  | FreqBandList with condition EN-DC |  | EN-DC |
|  | FreqBandList with condition EN-DC AND CA-InterBand |  | EN-DC_CA-InterBand |
|  | FreqBandList with condition NR AND CA-InterBand |  | NR_CA-InterBand |
|  | FreqBandList with condition NR and NR-DC |  | NR-DC |
|  | FreqBandList with condition NR and NE-DC |  | NE-DC |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| srs-SwitchingTimeRequest | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| srs-SwitchingTimeRequest | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| EN-DC_CA-InterBand | E-UTRA-NR Dual Connectivity using NR CA Inter-band test cases |
| NR_CA-InterBand | NG-RAN NR Radio Access using NR CA Inter-band test cases |
| NR | NG-RAN NR Radio Access |
| NR-DC | Used in NR-DC test cases |
| NE-DC | NR E-UTRA Dual Connectivity |

#### –	UE-MRDC-Capability

Table 4.6.4-48: UE-MRDC-Capability

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-MRDC-Capability ::= SEQUENCE { |  |  |  |
| measAndMobParametersMRDC | Not checked |  |  |
| phy-ParametersMRDC-v1530 | Not checked |  |  |
| rf-ParametersMRDC | RF-ParametersMRDC with condition EN-DC |  |  |
|  | RF-ParametersMRDC with condition NE-DC |  | NE-DC |
|  | RF-ParametersMRDC with condition EN-DC_CA-InterBand |  | EN-DC_CA-InterBand |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA | Not checked |  |  |
| } |  |  |  |
| fdd-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Not checked |  |  |
| sftd-MeasNR-Cell | Not checked |  |  |
| } |  |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Not checked |  |  |
| sftd-MeasNR-Cell | Not checked |  |  |
| } |  |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Not checked |  |  |
| splitDRB-withUL-Both-MCG-SCG | Not checked |  |  |
| srb3 | Not checked |  |  |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-MRDC-Capabilities |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombinations | Not checked |  |  |
| pdcp-ParametersMRDC-v1530 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| UE-MRDC-Capability-v1560 SEQUENCE { |  |  |  |
| receivedFilters | Not checked |  |  |
| measAndMobParametersMRDC-v1560 SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff-v1560 SEQUENCE { |  |  |  |
| sftd-MeasPSCell-NEDC | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fdd-Add-UE-MRDC-Capabilities-v1560 SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff-v1560 SEQUENCE { |  |  |  |
| sftd-MeasPSCell-NEDC | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-MRDC-Capabilities-v1560 SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff-v1560 SEQUENCE { |  |  |  |
| sftd-MeasPSCell-NEDC | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| EN-DC_CA-InterBand | E-UTRA-NR Dual Connectivity used in NR CA Inter-band test cases |
| NE-DC | NR E-UTRA Dual Connectivity |

#### –	UE-NR-Capability

Table 4.6.4-49: UE-NR-Capability

| Derivation Path: TS 38.331 [6], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |
| accessStratumRelease | AccessStratumRelease |  |  |
| pdcp-Parameters | Not checked |  |  |
| rlc-Parameters | Not checked |  |  |
| mac-Parameters | Not checked |  |  |
| phy-Parameters | Not checked |  |  |
| rf-Parameters | RF-Parameters |  |  |
|  | RF-Parameters with condition NR-DC |  | NR-DC |
|  | RF-Parameters with condition NR_CA-InterBand |  | NR_CA-InterBand |
| measAndMobParameters | Not checked |  |  |
| fdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Not checked |  |  |
| logicalChannelSR-DelayTimer | Not checked |  |  |
| longDRX-Cycle | Not checked |  |  |
| shortDRX-Cycle | Not checked |  |  |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Not checked |  |  |
| eventA-MeasAndReport | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Not checked |  |  |
| logicalChannelSR-DelayTimer | Not checked |  |  |
| longDRX-Cycle | Not checked |  |  |
| shortDRX-Cycle | Not checked |  |  |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Not checked |  |  |
| eventA-MeasAndReport | Not checked |  |  |
| } |  |  |  |
| fr1-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { | Not checked |  |  |
| dynamicSFI | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSets | Not checked |  |  |
| featureSetCombinations | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| fdd-Add-UE-NR-Capabilities-1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA Not Checked |  |  |  |
| } |  |  |  |
| } |  |  |  |
| dummy | Not checked |  |  |
| interRAT-Parameters | Not checked |  |  |
| inactiveState | Not checked |  |  |
| delayBudgetReporting | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdap-Parameters | Not checked |  |  |
| overheatingInd | Not checked |  |  |
| ims-Parameters | Not checked |  |  |
| fr1-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| freqHoppingPUCCH-F0-2 | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH- MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |
| reducedCP-Latency | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nrdc-Parameters | Not checked |  |  |
| receivedFilters | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NR-DC | Used in NR-DC test cases |
| NR_CA-InterBand | NG-RAN NR Radio Access using NR CA Inter-band test cases |