3GPP TSG-RAN5 Meeting #94-e	R5-221271r1

# Electronic Meeting, 21st February – 4th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5401 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of maximum output power for CA test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2022-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some allocations in Table 6.2.2A.2.4.1-1 are incorrect. Allocation for 3 MHz CHBW has to be limited to 4 RBs in order to avoid MPR for QPSK. Due to test purpose MPR has to be zero. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The allocations have been corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MOP for CA cannot be tested for 5MHz/3MHz CHBWs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2A.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: corrected typo in position of allocation |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 6.2.2A.2	UE Maximum Output Power for CA (inter-band DL CA and UL CA)

##### 6.2.2A.2.1	Test purpose

Same as in clause 6.2.2A.1.1, but for the inter-band carrier aggregation case.

##### 6.2.2A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 11 and forward that support inter-band DL CA and UL CA.

##### 6.2.2A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2.2A.0.

##### 6.2.2A.2.4	Test description

6.2.2A.2.4.1	Initial conditions

Same as in clause 6.2.2A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.2.2A.1.4.1-1 use Table 6.2.2A.2.4.1-1.

-	Instead of clause 6.2.2A.1.4.3 use clause 6.2.2A.2.4.3.

Table 6.2.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC (Note 3) |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@0 | S_8@0 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@1 | S_8@17 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@1 | S_12@38 | - | - |
| 15 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 |  |  |
| 15 | 25 |  | QPSK | 2 | P_1@14 | S_1@24 |  |  |
| 15 | 25 |  | QPSK | 12 | P_4@0 | S_8@0 |  |  |
| 15 | 25 |  | QPSK | 12 | P_4@11 | S_8@17 |  |  |
| 25 | 6 |  | QPSK | 2 | P_1@0 | S_1@0 |  |  |
| 25 | 6 |  | QPSK | 2 | P_1@24 | S_1@5 |  |  |
| 25 | 6 |  | QPSK | 13 | P_8@0 | S_5@0 |  |  |
| 25 | 6 |  | QPSK | 13 | P_8@17 | S_5@1 |  |  |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@0 | S_4@0 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@17 | S_4@11 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@17 | S_12@38 | - | - |
| 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 25 |  | QPSK | 2 | P_1@49 | S_1@24 | - | - |
| 50 | 25 |  | QPSK | 8 | P_12@0 | S_8@0 | - | - |
| 50 | 25 |  | QPSK | 8 | P_12@38 | S_8@17 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@38 | S_12@38 | - | - |
| 25 | 75 |  | QPSK | 2 | P_1@0 | P_1@0 |  |  |
| 25 | 75 |  | QPSK | 2 | P_1@24 | P_1@74 |  |  |
| 25 | 75 |  | QPSK | 24 | P_8@0 | P_16@0 |  |  |
| 25 | 75 |  | QPSK | 24 | P_8@17 | P_16@59 |  |  |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@38 | S_18@82 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@59 | S_16@59 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@82 | S_12@38 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@82 | S_18@82 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 3: For Low range use only test points with RBstart=0. For High range use only test points with RBstart=RBend-LCRB+1. |  |  |  |  |  |  |  |  |

< End of changes >