3GPP TSG-RAN5 Meeting #94-e                                        R5-221279r1

Electronic Meeting, 8th – 19th November 2021

Title: 	Discussion on HPUE_PC1_5_n77_n78-UEConTest Work plan review to include dependencies with NR_RF_TxD-UEConTest work plan

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.29.4

Document for:	Discussion and Endorsement

# 1.	Introduction

RAN5 has been working in Work item for UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 in RAN5#93e and its status reports indicates that the completion level reached is 50% with a target completion date in Sep’22 ([1], [2]).

Having a look to core requirements, it can be seen that such power class is achieved via dual Tx:

It implies that UL MIMO and Tx diversity core requirements should be tested as part of the on HPUE_PC1_5_n77_n78-UEConTest, depending on UE capabilities.

RAN4 and RAN5 are currently working on work items for UE RF requirements for Transparent Tx Diversity (TxD) for NR ([3], [4]).

This document makes an analysis on the dependencies between the work plan for these 2 features and makes a proposal for reviewing UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 work plan.

# 2.	Discussion

In [5], some analysis is performed regarding the details to be defined in the work plan for Tx diversity feature and proposes to define new test cases with new clause suffix G in test specifications aligned with core requirements for Tx Diversity:

  - UE maximum output power

  - MPR

  - Additional MPR

  - Configured transmitted power

  - Minimum output power

  - Transmit OFF power

  - Transmit ON/OFF time mask

  - Power Control

  - Frequency error

  - Transmit modulation quality

  - Occupied bandwidth

  - Out of band emission

  - Spurious emission

  - Transmit intermodulation

The definition of the new Tx Diversity test cases, if proposals in [5] are endorsed, require that some of the changes performed in the test specification for HPUE_PC1_5_n77_n78-UEConTest should be revisited to update the proper test cases.  These changes should be included in HPUE_PC1_5_n77_n78-UEConTest work plan.

There should be no conflict on Release applicability as power class 1.5 functionality has been defined in 38.307 as release independent from Rel-15, same as current understanding for Tx Diversity:

Proposal 1: Review HPUE_PC1_5_n77_n78-UEConTest work plan to align to Tx Diversity work plan and review Work item completion in Status Report accordingly.

# 3. Conclusion

This document makes an analysis on the dependencies between the UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 and Tx Diversity work plans and makes the following proposal:

Proposal 1: Review HPUE_PC1_5_n77_n78-UEConTest work plan to align to Tx Diversity work plan and review Work item completion in Status Report accordingly.

# 4.	References

[1] RP-212581, “New WID on UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78”, Verizon, RAN#93e.

[2] RP-212809, “Status report for UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78”, Verizon, RAN#94e.

[3] RP-213229, “New WID on UE Conformance – UE RF requirements for Transparent diversity (TxD) for NR”, Huawei, HiSilicon, RAN5#93e.

[4] RP-211940, “WID on UE RF requirements for Transparent Tx Diversity (TxD) for NR”, Qualcomm, RAN#93e.

[5] R5-221281, “Discussion on Transparent Tx diversity work plan”, Keysight Technologies UK Ltd, RAN5#94e.