3GPP TSG-RAN5 Meeting #94-e                      	R5-221294r1

# Electronic Meeting, February 21 - March 4 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0153 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FR1 DL Interruptions test cases applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM test cases 6.5.7.1 and 6.5.7.2 are missing in the applicability table. |  |  |  |  |  |  |  |  |
| R4- |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Condition C001d has been added and following RRM test cases has been added to the applicability table. : 6.5.7.1, 6.5.7.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: Corrected the condition to match changes in the r1 of R5-221293. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

## 4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The tested bands selection criteria given in Table 4.0-2 apply. The tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2 and Table 4.0-3 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-1: Applicability of conformance test cases conditions

C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C001a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7/3 THEN R ELSE N/A

C001b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C001c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.1-2/2e OR A.4.3.1-2/12) THEN R ELSE N/A

C001d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/QQ) THEN R ELSE N/A

C002 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C003 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C003b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) AND (A.4.3.2-1/58 OR A.4.3.2-1/59 OR A.4.3.2-1/60) THEN R ELSE N/A

C003a IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C004 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-2/1 THEN R ELSE N/A

C005 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.1-2/4 AND A.4.3.2A.1-1/1 AND A.4.1-3/1 THEN R ELSE N/A

C006 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 THEN R ELSE N/A

C006a IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C006b IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/31a THEN R ELSE N/A

C006c IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C006d IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-1/2 THEN R ELSE N/A

C006e IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-1/3 THEN R ELSE N/A

C006f IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-1/4 THEN R ELSE N/A

C006g IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-1/5 THEN R ELSE N/A

C006h IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-1/6 THEN R ELSE N/A

C006i IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-1/7 THEN R ELSE N/A

C007 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/22 THEN R ELSE N/A

C008 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND NOT(A.4.3.2-1/22) THEN R ELSE N/A

C009 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C009a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C009z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C010 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C010a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C010z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C011 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C011a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C011b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2A/1 THEN R ELSE N/A

C011c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-1A/1 THEN R ELSE N/A

C011z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2A/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C012 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C012a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1A/1 THEN R ELSE N/A

C012b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A

C012c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/3 THEN R ELSE N/A

C012d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/4 THEN R ELSE N/A

C012e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A

C012f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/31a THEN R ELSE N/A

C012g IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1A/3 THEN R ELSE N/A

C012h IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1A/4 THEN R ELSE N/A

C012z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C013 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (A.4.1-4/3 OR A.4.1-4/4) THEN R ELSE N/A

C014 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/4) THEN R ELSE N/A

C015 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/6 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015c IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/66 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015x IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015y IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/33 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C016 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016b IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/6 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016c IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/66 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016x IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016y IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/20 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C017 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C017b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/6 THEN R ELSE N/A

C017c IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/66 THEN R ELSE N/A

C017x IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.9-1/1 THEN R ELSE N/A

C017y IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/33 THEN R ELSE N/A

C018 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A

C019 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C019b IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.2-1/6 THEN R ELSE N/A

C019c IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.2-1/66 THEN R ELSE N/A

C019x IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.9-1/1 THEN R ELSE N/A

C020 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) THEN R ELSE N/A

C021 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C021a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C021b IF A.4.1-1/1 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C022 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C022a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C023 IF A.4.1-4/5 AND A.4.1-3/2 THEN R ELSE N/A

C023a IF A.4.1-4/5 AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C024 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C025 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 THEN R ELSE N/A

C025a IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C026 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND 4.3.6-1/11 THEN R ELSE N/A

C027 Void

C028 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.6-1/11 THEN R ELSE N/A

C029 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A

C030 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/9 THEN R ELSE N/A

C030a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/9 THEN R ELSE N/A

C031 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C032 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C033 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/2 THEN R ELSE N/A

C034 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/6 THEN R ELSE N/A

C035 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/6 THEN R ELSE N/A

C036 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/3 THEN R ELSE N/A

C037 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/41 THEN R ELSE N/A

C037a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C038 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 THEN R ELSE N/A

C038a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C039 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.6-1/41 THEN R ELSE N/A

C040 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 THEN R ELSE N/A

C041 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 THEN R ELSE N/A

C041a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C041b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C042 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 THEN R ELSE N/A

C042a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C042b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C043 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND (4.3.6-1/43 OR 4.3.6-1/44) THEN R ELSE N/A

C043a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C044 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND 4.3.6-1/42 THEN R ELSE N/A

C045 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) THEN R ELSE N/A

C046 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/3 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 ) THEN R ELSE N/A

C047 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/4 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) THEN R ELSE N/A

C048 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/2 AND A.4.1-4/1 THEN R ELSE N/A

C049 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/3 AND A.4.1-4/1 THEN R ELSE N/A

C050 IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/36 THEN R ELSE N/A

C051 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.3.2A.1-2/1 AND A.4.3.2-1/37 THEN R ELSE N/A

C052 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.11-1/1 THEN R ELSE N/A

C053 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/1 THEN R ELSE N/A

C054 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/2 THEN R ELSE N/A

C055 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/3 THEN R ELSE N/A

C056 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/4 THEN R ELSE N/A

C057 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/5 THEN R ELSE N/A

C058 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/6 THEN R ELSE N/A

C059 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/7 THEN R ELSE N/A

C060 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C061 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) THEN R ELSE N/A

C062 IF A.4.1-1/2 AND A.4.1.2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 THEN R ELSE N/A

C063 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A

C064 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/3 THEN R ELSE N/A

C064a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/4 THEN R ELSE N/A

C064b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/5 THEN R ELSE N/A

C065 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) AND A.4.1-3/2 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) THEN R ELSE N/A

C065a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) AND A.4.1-3/2 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C065b IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) AND A.4.1-3/2 AND (A.4.3.2-1/42 OR A.4.3.2-1/43) THEN R ELSE N/A

C066 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) THEN R ELSE N/A

C066a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C066b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/42 OR A.4.3.2-1/43) THEN R ELSE N/A

C067 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.1-3/2 THEN R ELSE N/A

C068 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND [10] A.4.6-1/1 AND A.4.1-3/2 THEN R ELSE N/A

C069 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/6 THEN R ELSE N/A

C070 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C071 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C072 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C073 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C074 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C075 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C076 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C077 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C078 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) AND A.4.1-4A/1 AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C079 IF A.4.1-1/3 AND A.4.1-2/7 THEN R ELSE N/A

C080 IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND A.4.1-1/2 AND A.4.1-2/8 THEN R ELSE N/A

C080a IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND A.4.1-1/2 AND A.4.1-2/8 AND A.4.3.5-1/1 THEN R ELSE N/A

C081 IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/5) AND (A.4.3.6-1/46 OR A.4.3.6-1/47) THEN R ELSE N/A

C081a IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/5) AND [10]A.4.4-1a/5 AND (A.4.3.6-1/46 OR A.4.3.6-1/47) THEN R ELSE N/A

C082 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/63 AND A.4.3.2-1/65 THEN R ELSE N/A

C082a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/63 AND A.4.3.2-1/65 AND A.4.3.5-1/1 THEN R ELSE N/A

C083 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/41 AND A.4.3.2-1/64 AND A.4.3.2-1/65 THEN R ELSE N/A

C083a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/41 AND A.4.3.2-1/64 AND A.4.3.2-1/65 AND A.4.3.5-1/1 THEN R ELSE N/A

C084 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/63 AND A.4.3.2-1/65 THEN R ELSE N/A

C084a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/63 AND A.4.3.2-1/65 AND A.4.3.5-1/1THEN R ELSE N/A

C085 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/41 AND A.4.3.2-1/64 AND A.4.3.2-1/65 THEN R ELSE N/A

C085a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/41 AND A.4.3.2-1/64 AND A.4.3.2-1/65 AND A.4.3.5-1/1 THEN R ELSE N/A

C086 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 THEN R ELSE N/A

C086a IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 AND A.4.3.5-1/1 THEN R ELSE N/A

C087 IF A.4.1-1/2 AND A.4.1.2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/3 THEN R ELSE N/A

C088 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/2 AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C089 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C090 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/3 AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C091 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C092 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C093 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/45 THEN R ELSE N/A

C094 IF ((A.4.1-1/1 AND A.4.1-1/1) OR (A.4.1-1/1 AND A.4.1-1/2) OR (A.4.1-1/2 AND A.4.1-1/1) OR (A.4.1-1/2 AND A.4.1-1/2)) AND A.4.1-3/1 AND A.4.3.6-1/45 THEN R ELSE N/A

C095 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.6-1/45 THEN R ELSE N/A

C096 IF ((A.4.1-1/1 OR A.4.1-1/2) AND [10] A.4.1-1/3) THEN R ELSE N/A

C097 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.11-1/1 OR A.4.3.11-1/4) THEN R ELSE N/A

C098 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 AND (A.4.3.11-1/1 OR A.4.3.11-1/3) THEN R ELSE N/A THEN R ELSE N/A

C099 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.11-1/2 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C100 IF A.4.1-1/3 AND A.4.1-2/7 AND A.4.3.10-1/3 THEN R ELSE N/A

C101 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/9 THEN R ELSE N/A

C102 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/16 THEN R ELSE N/A

C103 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.8-1/15 AND A.4.3.8-1/18 THEN R ELSE N/A

C104 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.8-1/17 AND A.4.3.8-1/18 THEN R ELSE N/A

C105 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/11 THEN R ELSE N/A

C106 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.8-1/11 THEN R ELSE N/A

C107 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/15 THEN R ELSE N/A

C108 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/17 THEN R ELSE N/A

C109 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/15 AND A.4.3.8-1/18 THEN R ELSE N/A

C110 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/17 AND A.4.3.8-1/18 THEN R ELSE N/A

C111 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/31 AND A.4.3.2-1/57 THEN R ELSE N/A

C112 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/31 AND A.4.3.2-1/57 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C113 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C113a IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C113b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C113c IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C114 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/54 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C114a IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/54 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C114b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/54 AND A.4.3.2-1/56 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C114c IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/54 AND A.4.3.2-1/56 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C115 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/55 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C115a IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/55 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C115b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/55 AND A.4.3.2-1/56 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C115c IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/55 AND A.4.3.2-1/56 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C116 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/61 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C117 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/61 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C118 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/61 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C119 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/61 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C120 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/12 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C121 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/62 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C122 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/12 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C123 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/62 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C124 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/12 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C125 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/12 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C126 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) and A.4.3.2-1/3 THEN R ELSE N/A

C126a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2A/1 AND A.4.3.2-1/37 THEN R ELSE N/A

C127 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.11-1/2 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C128 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/67 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C129 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/67 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C130 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/67 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C131 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/67 AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25).

Table 4.0-2: Tested Bands Selection Criteria

| Code | Tested Bands Selection Criteria | Comment |
| --- | --- | --- |
| D001 | (A.4.3.1-1 OR A.4.3.1-2) AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FR1 Bands without SUL/SDL bands |
| D002 | A.4.3.1-4 | All supported FR1 PC2 Bands |
| D003 | A.4.3.1-5 | All supported FR1 SUL Bands |
| D004 | {1,2,3,5,7,8,12,14,20,25,28,30,34,38,39,40,41,50,51,65,66,70,71,74,75,76} | UE supported bands among n1,n2,n3,n5,n7,n8,n12,n14,n20,n25,n28,n30,n34,n38,n39,n40,n41,n50,n51,n65,n66,n70,n71,n74,n75,n76 |
| D005 | A.4.3.1-3 | All supported FR2 Bands |
| D006 | A.4.3.1-1 OR A.4.3.1-2 | All supported FR1 Bands |
| D007 | A.4.3.1-1 OR A.4.3.1-2 OR A.4.3.1-3 | All supported NR Bands |
| D008 | ANY((A.4.3.1-1) AND 10MHz) | Any FDD FR1 band within the set supporting 10 MHz UE Channel BW |
| D009 | ANY((A.4.3.1-2) AND 20MHz) | Any TDD FR1 band within the set supporting 20 MHz UE Channel BW |
| D010 | ANY((A.4.3.1-2) AND 40MHz) | Any TDD FR1 band within the set supporting 40 MHz UE Channel BW |
| D011 | A.4.3.9-4a OR A.4.3.9-4b | All supported 4 Rx antenna ports Bands |
| D012 | A.4.3.1-1 AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FDD bands without SUL/SDL bands |
| D013 | ANY((A.4.3.1-3) AND 50MHz) | Any TDD FR2 band within the set supporting 50 MHz UE Channel BW |
| D014 | ANY((A.4.3.1-3) AND 100MHz) | Any TDD FR2 band within the set supporting 100 MHz UE Channel BW |
| D015 | ANY((A.4.3.1-3) AND 200MHz) | Any TDD FR2 band within the set supporting 200 MHz UE Channel BW |
| D016 | A.4.3.1-9 | All supported FR1 sidelink bands |
| D017 | {40, 41, 77, 78, 79} | UE supported TDD bands among n40, n41, n77, n78, n79 |
| D018 | A.4.3.1-2/2e OR A.4.3.1-2/12 | All supported FR1 Bands for operation with shared spectrum channel access |
| NOTE 1: Band Selection is based on set theory. For each feature, item number shall correspond to the Band number. The result is the set of bands for which the test shall be conducted. The following operators are used: AND: Set intersection ( ). {1,2} AND {2,3} = {2} OR: Set union ( ). {1,2} OR {2,3} = {1,2,3} NOT: Set complement (\), full set being all bands. NOT{1} = {2 ...256} Also note that this is set without repetitions so {1} AND {1} = {1} The following basic sets are used: {1,2}: Explicitly given band set 10MHz: All bands supporting 10 MHz |  |  |

Table 4.0-3: Tested CA/DC Configuration Selection Criteria

| Code | Tested CA/DC Configuration Selection Criteria | Comment |
| --- | --- | --- |
| E001 | DL_2CC(A.4.3.2A.2.1-3) AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.2.1-2) | All supported intra-band contiguous CA Configurations with 2 carriers in DL but no CA in UL |
| E002 | DL_2CC(A.4.3.2A.4.1-3) AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.4.1-2) | All supported inter-band CA Configurations with 2 carriers in DL but no CA in UL |
| E003 | UL_2CC(A.4.3.2B.2.1-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band contiguous EN-DC configurations in FR1 (2UL CCs) |
| E003a | DL_2CC(A.4.3.2B.2.1-2) AND A.4.3.2B.2.0-1/1 | All supported Intra-band contiguous EN-DC configurations in FR1 (2DL CCs) |
| E004 | UL_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2UL CCs) |
| E004a | DL_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1/1 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2DL CCs) |
| E005 | UL_2CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-2/1 | All supported Inter-band EN-DC configurations within FR1 (2UL CCs) |
| E005a | DL_2CC(A.4.3.2B.2.3.1-2) AND A.4.3.2B.2.0-1/1 | All supported Inter-band EN-DC configurations within FR1 (2DL CCs) |
| E005b | UL_NR_1CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-2A/1 | All supported Inter-band EN-DC configurations within FR1 with 1 NR UL CC and one or more LTE UL CC(s) |
| E005c | DL_NR_1CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/1 | All supported Inter-band EN-DC configurations within FR1 with 1 NR DL CC and one or more LTE DL CC(s) |
| E005d | A.4.3.2B.2.3.1-3 | All supported PC2 Inter-band EN-DC configurations within FR1 |
| E006 | DL_3CC(A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/2 | All supported EN-DC configurations within FR1 (3DL CCs) |
| E007 | DL_4CC(A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/3 | All supported EN-DC configurations within FR1 (4DL CCs) |
| E008 | DL_5CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/4 | All supported EN-DC configurations within FR1 (5DL CCs) |
| E009 | DL_6CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/5 | All supported EN-DC configurations within FR1 (6DL CCs) |
| E010 | UL_NR_1CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-2 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/1 | All supported Inter-band EN-DC configurations including FR2 (1NR UL CC) |
| E010a | DL_NR_1CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-2 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/1 | All supported Inter-band EN-DC configurations including FR2 (1NR DL CC) |
| E011 | UL_NR_2CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/2 AND A.4.3.2B.2.0-2A/2 | All supported Inter-band EN-DC configurations including FR2 (2UL NR CCs) |
| E011a | DL_NR_2CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/2 AND A.4.3.2B.2.0-1A/2 | All supported Inter-band EN-DC configurations including FR2 (2DL NR CCs) |
| E012 | UL_NR_3CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/3 AND NR_A.4.3.2B.2.0-2A/3 | All supported Inter-band EN-DC configurations including FR2 (3UL NR CCs) |
| E012a | DL_NR_3CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/3 AND A.4.3.2B.2.0-1A/3 | All supported Inter-band EN-DC configurations including FR2 (3DL NR CCs) |
| E013 | UL_NR_4CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/4 AND A.4.3.2B.2.0-2A/4 | All supported Inter-band EN-DC configurations including FR2 (4UL NR CCs) |
| E013a | DL_NR_4CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/4 AND A.4.3.2B.2.0-1A/4 | All supported Inter-band EN-DC configurations including FR2 (4DL NR CCs) |
| E014 | DL_NR_5CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/5 AND A.4.3.2B.2.0-1A/5 | All supported Inter-band EN-DC configurations including FR2 (5DL NR CCs) |
| E015 | UL_2CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-2/1 | All supported FR1 2UL CA configurations |
| E016 | DL_2CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3) AND A.4.3.2A.1-1/1 | All supported FR1 2DL CA configurations |
| E017 | DL_3CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-1/2 | All supported FR1 3DL CA configurations |
| E018 | DL_4CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-1/3 | All supported FR1 4DL CA configurations |
| E019 | ULTxSwitching(A.4.3.2A.4.1-3) | All supported FR1 2UL CA configurations with ULTxSwitching capability |
| E020 | UL_2CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/1 | All supported FR2 2UL CA configurations |
| E021 | UL_3CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/2 | All supported FR2 3UL CA configurations |
| E022 | UL_4CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/3 | All supported FR2 4UL CA configurations |
| E023 | UL_5CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/4 | All supported FR2 5UL CA configurations |
| E024 | UL_6CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/5 | All supported FR2 6UL CA configurations |
| E025 | UL_7CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/6 | All supported FR2 7UL CA configurations |
| E026 | UL_8CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/7 | All supported FR2 8UL CA configurations |
| E027 | DL_NR_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/2 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2DL NR CCs) |
| E028 | DL_NR_3CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/3 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (3DL NR CCs) |
| E028a | DL_NR_4CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/4 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (4DL NR CCs) |
| E028b | DL_NR_5CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/5 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (5DL NR CCs) |
| E029 | DL_NR_2CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/2 | All supported Inter-band EN-DC configurations within FR1 (2DL NR CCs) |
| E030 | DL_NR_3CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/3 | All supported Inter-band EN-DC configurations within FR1 (3DL NR CCs) |
| E030a | DL_NR_4CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/4 | All supported Inter-band EN-DC configurations within FR1 (4DL NR CCs) |
| E030b | DL_NR_5CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/5 | All supported Inter-band EN-DC configurations within FR1 (5DL NR CCs) |
| E031 | A.4.3.2C.3-2 | All supported FR1 intra-band contiguous 2DL CA with SUL in uplink Configurations |
| E031b | ULTxSwitching(A.4.3.2B.2.3.1-2) | All supported FR1 2UL inter-band EN-DC configurations with ULTxSwitching capability |
| E032 | DL_2CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-1/1 | All supported FR2 2DL CA configurations |
| E033 | DL_3CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-1/2 | All supported FR2 3DL CA configurations |
| E034 | DL_4CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-1/3 | All supported FR2 4DL CA configurations |
| E035 | DL_5CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-1/4 | All supported FR2 5DL CA configurations |
| E036 | DL_6CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-1/5 | All supported FR2 6DL CA configurations |
| E037 | DL_7CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-1/6 | All supported FR2 7DL CA configurations |
| E038 | DL_8CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-1/7 | All supported FR2 8DL CA configurations |
| NOTE 1: UL(table_index) includes all supported CA Configurations where at least one UL CA configuration was declared in column "Supported CA Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8]. NOTE 2: UL_nCC(table_index) includes all supported CA or DC Configurations where at least one n-carrier UL CA or DC configuration was declared in column "Supported CA Bandwidth Class(es) in UL" or “Supported EN-DC Bandwidth Class(es) in UL” in Table table_index in TS 38.508-2 [8]. NOTE 3: UL_NR_nCC(table_index) includes all supported DC Configurations where at least one DC configuration with n-carrier NR UL CA configuration was declared in column "Supported EN-DC Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8]. NOTE 4: DL_nCC(table_index) includes all supported n-carrier CA/DC Configurations in Table table_index in TS 38.508-2 [8]. NOTE 5: DL_NR_nCC(table_index) includes all supported DC Configurations with n-carrier NR DL CA configuration in Table table_index in TS 38.508-2 [8]. NOTE 6: ULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported ULTxSwitching Band Pair" in Table table_index in TS 38.508-2 [8]. |  |  |

///Unchanged clauses skipped

## 4.2	RRM conformance test cases

Table 4.2-1: Applicability of RRM EN-DC FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 4.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 4.3.2 | RRC connection mobility control |  |  |  |  |  |
| 4.3.2.2 | Random access |  |  |  |  |  |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | Rel-15 | C030 | UEs supporting EN-DC FR1 and CSI-RS based PRACH |  | 2Rx 4Rx |
| 4.3.2.2.3 | EN-DC FR1 2-step contention based random access | Rel-16 | C021 | UEs supporting EN-DC FR1 | NOTE 1 | 2Rx 4Rx |
| 4.3.2.2.4 | EN-DC FR1 2-step non-contention based random access | Rel-16 | C021 | UEs supporting EN-DC FR1 | NOTE 1 | 2Rx 4Rx |
| 4.4 | Timing |  |  |  |  |  |
| 4.4.1 | UE Transmit Timing |  |  |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.4.2 | UE timer accuracy |  |  |  |  |  |
| 4.4.3 | Timing Advance |  |  |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5 | Signalling characteristics |  |  |  |  |  |
| 4.5.1 | Radio link monitoring |  |  |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM |  | 2Rx 4Rx |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM |  | 2Rx 4Rx |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle |  | 2Rx 4Rx |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle |  | 2Rx 4Rx |
| 4.5.2 | Interruption |  |  |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx |
| 4.5.3 | SCell activation and deactivation delay |  |  |  |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |
| 4.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C032 | UEs supporting EN-DC FR1 and SUL |  | 2Rx 4Rx |
| 4.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C082 | UEs supporting EN-DC FR1 and link recovery |  | 2Rx 4Rx |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C082a | UEs supporting EN-DC FR1 and long DRX cycle and link recovery |  | 2Rx 4Rx |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C083 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and link recovery |  | 2Rx 4Rx |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C083a | UEs supporting EN-DC FR1 and long DRX cycle and CSI-RS-based RLM and link recovery |  | 2Rx 4Rx |
| 4.5.6 | Active BWP switch delay |  |  |  |  |  |
| 4.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C065 | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 4.5.6.1.2 | EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | Rel-15 | C065a | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  | 2Rx 4Rx |
| 4.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 4.5.6.2.1 | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C065b | UEs supporting EN-DC FR1 and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 4.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 4.5.7.1 | EN-DC FR1 addition and release delay of known PSCell | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.5.8 | UL switching |  |  |  |  |  |
| 4.5.8.1 | EN-DC FR1 interruptions at switching between two uplink carriers | Rel-16 | C126a | UEs supporting EN-DC and dynamic UL Tx switching in case of inter-band EN-DC |  | 2Rx 4Rx |
| 4.6 | Measurement procedures |  |  |  |  |  |
| 4.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 4.6.1.1 | EN-DC FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.1.2 | EN-DC FR1 event-triggered reporting without gap in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.6.1.3 | EN-DC FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C042 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  | 2Rx 4Rx |
| 4.6.1.4 | EN-DC FR1 event-triggered reporting with gap in DRX | Rel-15 | C042a | UEs supporting EN-DC FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle |  | 2Rx 4Rx |
| 4.6.1.5 | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Rel-15 | C021b | UEs supporting EN-DC FDD FR1 |  | 2Rx 4Rx |
| 4.6.1.6 | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Rel-15 | C042b | UEs supporting EN-DC FDD FR1 and CSI-RS based RLM and BWP operation without bandwidth restriction |  | 2Rx 4Rx |
| 4.6.1.7 | EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C097 | UEs supporting EN-DC FR1 and long DRX cycle and measurement enhancements in HST |  | 2Rx 4Rx |
| 4.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 4.6.2.1 | EN-DC FR1-FR1 event-triggered reporting in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.2.2 | EN-DC FR1-FR1 event-triggered reporting in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.6.2.5 | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.2.6 | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.6.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 4.6.4.1 | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.4.2 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.6.4.3 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.6.4.4 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |
| 4.7 | Measurement performance requirements |  |  |  |  |  |
| 4.7.1 | SS-RSRP |  |  |  |  |  |
| 4.7.1.1 | Intra-frequency measurements |  |  |  |  |  |
| 4.7.1.1.1 | EN-DC FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.1.1.2 | EN-DC FR1 SS-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.1.2 | Inter-frequency measurements |  |  |  |  |  |
| 4.7.1.2.1 | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.1.2.2 | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.2 | SS-RSRQ |  |  |  |  |  |
| 4.7.2.1 | EN-DC FR1 SS-RSRQ measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.2.2.1 | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.2.2.2 | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.3 | SS-SINR |  |  |  |  |  |
| 4.7.3.1 | EN-DC FR1 SS-SINR measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 4.7.3.2.1 | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 4.7.3.2.2 | EN-DC FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 4.7.4 | L1-RSRP |  |  |  |  |  |
| 4.7.4.1.1 | EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.4.1.2 | EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.4.2.1 | EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.4.2.2 | EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |
| 4.7.5 | SFTD |  |  |  |  |  |
| 4.7.5.1 | EN-DC FR1 SFTD measurement accuracy | Rel-15 | C043 | UEs supporting EN-DC FR1 and SFTD measurements between E-UTRA PCell and NR PSCell |  | 2Rx 4Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |

Table 4.2-1a: Void

Table 4.2-2: Applicability of RRM EN-DC FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 5.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 5.3.2 | RRC connection mobility control |  |  |  |  |  |
| 5.3.2.2 | Random access |  |  |  |  |  |
| 5.3.2.2.1 | EN-DC FR2 contention based random access | Rel-16 | C022 | UEs supporting EN-DC FR2 |  | 2Rx 4Rx |
| 5.3.2.2.2 | EN-DC FR2 non-contention based random access | Rel-16 | C030a | UEs supporting EN-DC FR2 and CSI-RS based PRACH |  | 2Rx 4Rx |
| 5.3.2.2.3 | EN-DC FR2 2-step contention based random access | Rel-16 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx 4Rx |
| 5.3.2.2.4 | EN-DC FR2 2-step non-contention based random access | Rel-16 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx 4Rx |
| 5.4 | Timing |  |  |  |  |  |
| 5.4.1 | UE transmit timing |  |  |  |  |  |
| 5.4.1.1 | EN-DC FR2 UE transmit timing accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.4.2 | UE timer accuracy |  |  |  |  |  |
| 5.4.3 | Timing advance |  |  |  |  |  |
| 5.4.3.1 | EN-DC FR2 timing advance adjustment accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.5 | Signalling characteristics |  |  |  |  |  |
| 5.5.1 | Radio link monitoring |  |  |  |  |  |
| 5.5.1.1 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.2 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.3 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.4 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.5 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.6 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.7 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.8 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.1.9 | EN-DC FR2 radio link monitoring UE scheduling restrictions | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2 | Interruption |  |  |  |  |  |
| 5.5.2.1 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.2 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.3 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.4 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.5 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.2.6 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.3 | SCell activation and deactivation delay |  |  |  |  |  |
| 5.5.3.1 | EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 5.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 5.5.5.1 | EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.5.2 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.5.3 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.5.4 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.5.5 | EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.5.6 | Active BWP switch delay |  |  |  |  |  |
| 5.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 5.5.6.1.1 | EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.6.1.2 | EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 5.5.6.2.1 | EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 5.5.7.1 | EN-DC FR2 addition and release delay of known PSCell | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.5.8 | Active TCI state switch delay |  |  |  |  |  |
| 5.5.8.1 | EN-DC FR2 MAC-CE based active TCI state switch | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.5.8.2 | EN-DC FR2 RRC based active TCI state switch | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.6 | Measurement procedures |  |  |  |  |  |
| 5.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 5.6.1.1 | EN-DC FR2 event-triggered reporting without gap in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.6.1.2 | EN-DC FR2 event-triggered reporting without gap in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.1.3 | EN-DC FR2 event-triggered reporting with gap in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.6.1.4 | EN-DC FR2 event-triggered reporting with gap in DRX | Rel-15 | C043a | UEs supporting EN-DC FR2, long DRX cycle, CSI-RS based RLM and BWP operation without BW restriction |  | 2Rx |
| 5.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 5.6.2.1 | EN-DC FR2-FR2 event-triggered reporting in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.2.2 | EN-DC FR2-FR2 event-triggered reporting in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.3 | EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.2.4 | EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | Rel-15 | C023 | UEs supporting EN-DC FR1 and FR2 |  | 2Rx |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | Rel-15 | C023a | UEs supporting EN-DC FR1 and FR2 and long DRX cycle |  | 2Rx |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C023 | UEs supporting EN-DC FR1 and FR2 |  | 2Rx |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C023a | UEs supporting EN-DC FR1 and FR2 and long DRX cycle |  | 2Rx |
| 5.6.3 | L1-RSRP for beam reporting |  |  |  |  |  |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |
| 5.7 | Measurement performance requirements |  |  |  |  |  |
| 5.7.1 | SS-RSRP |  |  |  |  |  |
| 5.7.1.1 | EN-DC FR2 SS-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.1.2 | EN-DC FR2-FR2 SS-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.1.3 | EN-DC FR1-FR2 SS-RSRP measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 5.7.2 | SS-RSRQ |  |  |  |  |  |
| 5.7.2.1 | EN-DC FR2 SS-RSRQ measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |
| 5.7.2.2 | EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |
| 5.7.3 | SS-SINR |  |  |  |  |  |
| 5.7.3.1 | EN-DC FR2 SS-SINR measurement accuracy | Rel-15 | C069 | UEs supporting EN-DC FR2 and SS-SINR-meas |  | 2Rx |
| 5.7.3.2 | EN-DC FR2-FR2 SS-SINR measurement accuracy | Rel-15 | C069 | UEs supporting EN-DC FR2 and SS-SINR-meas | NOTE 1 | 2Rx |
| 5.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

Table 4.2-2a: Void

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 6.1 | RRC_IDLE state mobility |  |  |  |  |  |
| 6.1.1 | NR cell re-selection |  |  |  |  |  |
| 6.1.1.1 | NR SA FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST | NOTE 1 | 2Rx 4Rx |
| 6.1.2 | NR – E-UTRA cell re-selection |  |  |  |  |  |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.1.2.3 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C094 | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.2.4 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C094 | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  | 2Rx 4Rx |
| 6.1.2.5 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C098 | UEs supporting 5GS NR SA FR1 and E-UTRA and measurement enhancements in HST |  | 2Rx 4Rx |
| 6.2 | RRC_INACTIVE state mobility |  |  |  |  |  |
| 6.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 6.3.1 | Handover |  |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.3 | NR SA FR1-FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.3.1.6 | NR SA FR1 – UTRAN FDD handover with known target cell | Rel-16 | C096 | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  | 2Rx 4Rx |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | Rel-16 | C101 | UEs supporting 5GS NR SA FR1 and intra-frequency DAPS handover |  | 2Rx 4Rx |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | Rel-16 | C102 | UEs supporting 5GS NR SA FR1 and intra-frequency async DAPS handover |  | 2Rx 4Rx |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Rel-16 | C107 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover |  | 2Rx 4Rx |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Rel-16 | C108 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover |  | 2Rx 4Rx |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | Rel-16 | C107 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx |
|  |  |  | C109 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell | For test configuration 3, 6, 7, 8 | 2Rx 4Rx |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | Rel-16 | C108 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx |
|  |  |  | C110 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell | For test configuration 3, 6, 7, 8 | 2Rx 4Rx |
| 6.3.2 | RRC connection mobility control |  |  |  |  |  |
| 6.3.2.1 | RRC re-establishment |  |  |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.1.2 | NR SA FR1 – FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.2 | Random access |  |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | Rel-15 | C029 | UEs supporting 5GS NR SA FR1 and CSI-RS based PRACH |  | 2Rx 4Rx |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | Rel-16 | C001 | UEs supporting 5GS NR SA FR1 | NOTE 1 | 2Rx 4Rx |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | Rel-16 | C001 | UEs supporting 5GS NR SA FR1 | NOTE 1 | 2Rx 4Rx |
| 6.3.2.3 | RRC connection release with redirection |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.3.2.3.2 | NR SA FR1 – E-UTRA RRC connection release with redirection | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 6.3.3 | Conditional handover |  |  |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | Rel-16 | C105 | UEs supporting 5GS NR SA FR1 and Conditional handover |  | 2Rx 4Rx |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | Rel-16 | C105 | UEs supporting 5GS NR SA FR1 and Conditional handover |  | 2Rx 4Rx |
| 6.4 | Timing |  |  |  |  |  |
| 6.4.1 | UE transmit timing |  |  |  |  |  |
| 6.4.1.1 | NR SA FR1 transmit timing accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.4.2 | UE timer accuracy |  |  |  |  |  |
| 6.4.3 | Timing advance |  |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.5 | Signalling characteristics |  |  |  |  |  |
| 6.5.1 | Radio Link Monitoring |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM |  | 2Rx 4Rx |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM |  | 2Rx 4Rx |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle |  | 2Rx 4Rx |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle |  | 2Rx 4Rx |
| 6.5.2 | Interruption |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3 | Scell activation and deactivation delay |  |  |  |  |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |
| 6.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C002 | UEs supporting 5GS NR SA FR1 and SUL |  | 2Rx 4Rx |
| 6.5.5 | Link recovery procedures |  |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C084 | UEs supporting 5GS NR SA FR1 and link recovery |  | 2Rx 4Rx |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C084a | UEs supporting 5GS NR SA FR1 and long DRX cycle and link recovery |  | 2Rx 4Rx |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C085 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and link recovery |  | 2Rx 4Rx |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C085a | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and link recovery |  | 2Rx 4Rx |
| 6.5.6 | Active BWP switch delay |  |  |  |  |  |
| 6.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | Rel-15 | C066a | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  | 2Rx 4Rx |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | Rel-15 | C066 | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 6.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 6.5.7 | DL interruptions at switching between two uplink carriers |  |  |  |  |  |
| 6.5.7.1 | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | Rel-16 | C001d | UEs supporting 5GS NR SA FR1 and dynamic UL Tx switching | NOTE 1 | 2Rx |
| 6.5.7.2 | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | Rel-16 | C001d | UEs supporting 5GS NR SA FR1 and dynamic UL Tx switching | NOTE 1 | 2Rx |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | Rel-15 | C066b | UEs supporting 5GS NR SA FR1 and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |
| 6.6 | Measurement procedures |  |  |  |  |  |
| 6.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 6.6.1.1 | NR SA FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.1.2 | NR SA FR1 event-triggered reporting without gap in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.1.3 | NR SA FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C041 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  | 2Rx 4Rx |
| 6.6.1.4 | NR SA FR1 event-triggered reporting with gap in DRX | Rel-15 | C041a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle |  | 2Rx 4Rx |
| 6.6.1.5 | NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading | Rel-15 | C024 | UEs supporting 5GS NR FDD SA FR1 |  | 2Rx 4Rx |
| 6.6.1.6 | NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading | Rel-15 | C041b | UEs supporting 5GS NR FDD SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  | 2Rx 4Rx |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  | 2Rx 4Rx |
| 6.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.3 | Inter-RAT measurements |  |  |  |  |  |
| 6.6.3.1 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRAN |  | 2Rx 4Rx |
| 6.6.3.2 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Rel-15 | C025a | UEs supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle |  | 2Rx 4Rx |
| 6.6.3.3 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Rel.16 | C052 | UEs supporting 5GS NR SA FR1 and E-UTRAN configured with highSpeedMeasFlag-r16 |  | 2Rx 4Rx |
| 6.6.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | 2Rx 4Rx |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST | NOTE 1 | 2Rx 4Rx |
| 6.6.5 | FFS |  |  |  |  |  |
| 6.6.5.1 | NR SA FR1 – UTRAN FDD event triggered reporting in non-DRX | Rel-16 | C096 | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  | 2Rx 4Rx |
| 6.7 | Measurement performance requirements |  |  |  |  |  |
| 6.7.1 | SS-RSRP |  |  |  |  |  |
| 6.7.1.1 | Intra-frequency measurements |  |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.1.2 | Inter-frequency measurements |  |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.2 | SS-RSRQ |  |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.3 | SS-SINR |  |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | 2Rx 4Rx |
| 6.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB-based L1-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.4.1.2 | NR SA FR1 SSB-based L1-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.4.2.1 | NR SA FR1 CSI-RS-based L1-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.4.2.2 | NR SA FR1 CSI-RS-based L1-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.5 | E-UTRAN RSRP |  |  |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.6 | E-UTRAN RSRQ |  |  |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| 6.7.7 | E-UTRAN RS-SINR |  |  |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |

Table 4.2-3a: Void

Table 4.2-4: Applicability of RRM NR SA FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 7.1 | RRC_IDLE state mobility |  |  |  |  |  |
| 7.1.1 | NR cell re-selection |  |  |  |  |  |
| 7.1.1.1 | NR SA FR2 cell re-selection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | 2Rx |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | 2Rx |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | 2Rx |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  | 2Rx |
| 7.2 | RRC_INACTIVE state mobility |  |  |  |  |  |
| 7.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 7.3.1 | Handover |  |  |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | Rel-16 | C103 | UEs supporting 5GS NR SA FR1 and 5GS NR SA FR2 and inter-frequency DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell | NOTE 1 | 2Rx |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | Rel-16 | C104 | UEs supporting 5GS NR SA FR1 and 5GS NR SA FR2 and inter-frequency async DAPS handover and supporting different SCSs in source PCell and inter-frequency target PCell | NOTE 1 | 2Rx |
| 7.3.2 | RRC connection mobility control |  |  |  |  |  |
| 7.3.2.1 | RRC re-establishment |  |  |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.3.2.1.2 | NR SA FR2 - FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.3.2.2 | Random access |  |  |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.3.2.3 | RRC connection release with redirection |  |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2 RRC connection release with redirection | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.3.3 | Conditional Handover |  |  |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover | NOTE 1 | 2Rx |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover | NOTE 1 | 2Rx |
| 7.4 | Timing |  |  |  |  |  |
| 7.4.1 | UE transmit timing |  |  |  |  |  |
| 7.4.2 | UE timer accuracy |  |  |  |  |  |
| 7.4.3 | Timing advance |  |  |  |  |  |
| 7.5 | Signalling characteristics |  |  |  |  |  |
| 7.5.1 | Radio Link Monitoring |  |  |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.2 | Interruption |  |  |  |  |  |
| 7.5.3 | Scell activation and deactivation delay |  |  |  |  |  |
| 7.5.3.1 | NR SA FR2-FR2 intra-band SCell activation and deactivation delay | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.3.2 | NR SA FR1-FR2 inter-band SCell activation and deactivation delay | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |
| 7.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.5.5 | NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.6 | Active BWP switch delay |  |  |  |  |  |
| 7.5.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 7.5.6.1.1 | NR SA FR2 2DL CA DCI-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.6.1.2 | NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.6.1.3 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |
| 7.5.6.2.1 | NR SA FR2 RRC-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.7 | PSCell addition and release delay |  |  |  |  |  |
| 7.5.7.1 | NR SA FR2 addition and release delay of known PSCell | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.5.7.2 | NR SA FR2 addition and release delay of unknown PSCell | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.6 | Measurement procedures |  |  |  |  |  |
| 7.6.1 | Intra-frequency measurements |  |  |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 | 2Rx |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle | NOTE 1 | 2Rx |
| 7.6.2 | Inter-frequency measurements |  |  |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.3 | L1-RSRP for beam reporting |  |  |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | 2Rx |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | 2Rx |
| 7.7 | Measurement performance requirements |  |  |  |  |  |
| 7.7.1 | SS-RSRP |  |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.1.3 | Inter-frequency measurements between FR1 and FR2 |  |  |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.2 | SS-RSRQ |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.3 | SS-SINR |  |  |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | FFS | FFS | FFS | NOTE 1 | 2Rx |
| 7.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

Table 4.2-4a: Void

Table 4.2-5: Applicability of E-UTRA – NR Inter-RAT conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 8.2 | RRC_IDLE state mobility |  |  |  |  |  |
| 8.2.1 | Inter-RAT cell re-selection |  |  |  |  |  |
| 8.2.1.1 | E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 8.2.1.2 | E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | Rel.16 | C052 | UEs supporting 5GS NR SA FR1 and E-UTRAN configured with highSpeedMeasFlag-r16 |  | 2Rx 4Rx |
| 8.3 | RRC_CONNECTED state mobility |  |  |  |  |  |
| 8.3.1 | Inter-RAT cell handover |  |  |  |  |  |
| 8.3.1.1 | E-UTRA – NR FR1 handover with known target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |
| 8.4 | Measurement procedures |  |  |  |  |  |
| 8.4.1 | SFTD measurement delay |  |  |  |  |  |
| 8.4.1.1 | E-UTRA – NR FR1 SFTD measurement delay in non-DRX | Rel-15 | C081 | UEs supporting EN-DC and E-UTRA and SFTD measurements between E-UTRA PCell and NR neighbour cell |  | 2Rx 4Rx |
| 8.4.1.2 | E-UTRA – NR FR1 SFTD measurement delay in DRX | Rel-15 | C081a | UEs supporting EN-DC and E-UTRA and long DRX cycle and SFTD measurements between E-UTRA PCell and NR neighbour cell |  | 2Rx 4Rx |
| 8.4.2 | Inter-RAT measurements |  |  |  |  |  |
| 8.4.2.1 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.4.2.2 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Rel-15 | C086a | UEs supporting E-UTRA and NR FR1 measurement and long DRX cycle |  | 2Rx 4Rx |
| 8.4.2.3 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.4.2.4 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Rel-15 | C086a | UEs supporting E-UTRA and NR FR1 measurement and long DRX cycle |  | 2Rx 4Rx |
| 8.4.2.5 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement | NOTE 1 | 2Rx |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | Rel-15 | C080a | UEs supporting E-UTRA and NR FR2 measurement and long DRX cycle | NOTE 1 | 2Rx |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement | NOTE 1 | 2Rx |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | Rel-15 | C080a | UEs supporting E-UTRA and NR FR2 measurement and long DRX cycle | NOTE 1 | 2Rx |
| 8.4.2.9 | E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | Rel.16 | C052 | UEs supporting 5GS NR SA FR1 and E-UTRAN configured with highSpeedMeasFlag-r16 |  | 2Rx 4Rx |
| 8.5 | Measurement performance requirements |  |  |  |  |  |
| 8.5.1 | SFTD measurement accuracy |  |  |  |  |  |
| 8.5.1.1 | E-UTRA – NR FR1 SFTD measurement accuracy | Rel-15 | C081 | UEs supporting EN-DC and E-UTRA and SFTD measurements between E-UTRA PCell and NR neighbour cell | NOTE 1 | 2Rx 4Rx |
| 8.5.2 | Inter-RAT |  |  |  |  |  |
| 8.5.2.1 | SS-RSRP |  |  |  |  |  |
| 8.5.2.1.1.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.1.2 | E-UTRA SS-RSRP measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| 8.5.2.2 | SS-RSRQ |  |  |  |  |  |
| 8.5.2.2.1 | E-UTRA SS-RSRQ measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.2.2 | E-UTRA SS-RSRQ measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| 8.5.2.3 | SS-SINR |  |  |  |  |  |
| 8.5.2.3.1 | E-UTRA SS-SINR measurement accuracy of a NR FR1 neighbour cell | Rel-15 | C086 | UEs supporting E-UTRA and NR FR1 measurement |  | 2Rx 4Rx |
| 8.5.2.3.2 | E-UTRA SS-SINR measurement accuracy of a NR FR2 neighbour cell | Rel-15 | C080 | UEs supporting E-UTRA and NR FR2 measurement |  | 2Rx |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. |  |  |  |  |  |  |

Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN5#76 | R5-173911 | - | - | - | Draft skeleton | 0.0.1 |
| 2018-01 | RAN5#1-5G-NR Adhoc | R5-180107 | - | - | - | Updated after RAN5#1-5G-NR Adhoc: - Foreword, scope, references, definitions, symbols and abbreviations, recommended test case applicability updated - clause 4.1.1, 4.1.2, 4.1.3 and 4.1.4 added - change history added | 0.1.0 |
| 2018-03 | RAN5 #78 | R5-181687 | - | - | - | TP for Clause 4.1.1 Range 1 standalone conformance test cases | 0.2.0 |
| 2018-03 | RAN5 #78 | R5-181688 | - | - | - | TP for Clause 4.1.2 Range 2 standalone conformance test cases | 0.2.0 |
| 2018-03 | RAN5 #78 | R5-181689 | - | - | - | TP for Clause 4.1.3 NR interworking between NR range1 and NR range2 and between NR and LTE conformance test cases | 0.2.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-182013 | - | - | - | TP for Clause 3 Definitions, symbols and abbreviations | 0.3.0 |
| 2018-04 | RAN5#2-5G-NR Adhoc | R5-182047 | - | - | - | TP for Clause 4 Recommended test case applicability | 0.3.0 |
| 2018-08 | RAN5#80 | R5-185209 | - | - | - | TP for Clause 4.1.1 of TS 38.522 | 1.0.1 |
| 2018-08 | RAN5#80 | R5-185210 | - | - | - | TP for Clause 4.1.2 of TS 38.522 | 1.0.1 |
| 2018-08 | RAN5#80 | R5-185211 | - | - | - | TP for Clause 4.1.3 of TS 38.522 | 1.0.1 |
| 2018-09 | RAN#81 | - | - | - | - | raised to v15.0.0 with editorial changes only | 15.0.0 |
| 2018-12 | RAN#82 | R5-186501 | 0013 | - | F | Applicability rules implementation in 38.522 | 15.1.0 |
| 2018-12 | RAN#82 | R5-188223 | 0015 | - | F | Applicability for RRM NR tests | 15.1.0 |
| 2018-12 | RAN#82 | R5-187566 | 0016 | - | F | Update note in section 4.1 to include CBW and SCS in RF test applicability | 15.1.0 |
| 2018-12 | RAN#82 | R5-187849 | 0014 | 1 | F | Adding applicability for new 38.521-1 CA TCs | 15.1.0 |
| 2018-12 | RAN#82 | R5-187881 | 0008 | 1 | F | Update Clause 1 Scope of TS 38.522 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187884 | 0011 | 1 | F | TP for Clause 4.1.2 of TS 38.522 | 15.1.0 |
| 2018-12 | RAN#82 | R5-187922 | 0017 | - | F | Removing FR2 test case 7.4 from TS 38.522 due to testability issue | 15.1.0 |
| 2019-01 | RAN#82 | R5-187882 | 0009 | 1 | F | Update Clause 3 of TS 38.522 | 15.1.1 |
| 2019-01 | RAN#82 | R5-187883 | 0010 | 1 | F | TP for Clause 4.1.1 of TS 38.522 | 15.1.1 |
| 2019-01 | RAN#82 | R5-187885 | 0012 | 1 | F | TP for Clause 4.1.3 of TS 38.522 | 15.1.1 |
| 2019-03 | RAN#83 | R5-191722 | 0021 | - | F | addition of applicability for BFD and measurement | 15.2.0 |
| 2019-03 | RAN#83 | R5-192507 | 0020 | 1 | F | TP for TS 38.522 | 15.2.0 |
| 2019-03 | RAN#83 | R5-192508 | 0022 | 1 | F | Addition of RRM Test Cases Applicability | 15.2.0 |
| 2019-06 | RAN#84 | R5-195444 | 0027 | 1 | F | TP for TS 38.522 | 15.3.0 |
| 2019-06 | RAN#84 | - | - | - | - | Administrative release upgrade to match the release of 3GPP TS 38.508-1 and TS 38.521-1 which were upgraded at RAN#84 to Rel-16 due to Rel-16 relevant CR(s) | 16.0.0 |
| 2019-06 | RAN#84 | - | - | - | - | Addition of missing Table part of R5-195444 and part of a note. | 16.0.1 |
| 2019-06 | RAN#84 | - | - | - | - | Formatted big tables to landscape | 16.0.2 |
| 2019-09 | RAN#85 | R5-197650 | 0030 | 1 | - | TP for TS 38.522 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197650 | 0030 | 1 | - | Added missing changes of R5-197650 | 16.1.1 |
| 2019-12 | RAN#86 | R5-199089 | 0032 | 2 | - | TP for TS 38.522 | 16.2.0 |
| 2020-03 | RAN#87 | R5-201036 | 0033 | 1 | F | TP and format updated for TS 38.522 | 16.3.0 |
| 2020-06 | RAN#88 | R5-202958 | 0040 | 1 | F | R16 TDD ENDC PC2 TP for TS 38.522 | 16.4.0 |
| 2020-06 | RAN#88 | R5-203114 | 0037 | 2 | F | TP updated to applicability table | 16.4.0 |
| 2020-09 | RAN#89 | R5-204098 | 0046 | - | F | Correct applicability EN-DC event-triggered inter-frequency tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204099 | 0047 | - | F | Correct applicability NR SA event-triggered inter-frequency tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204939 | 0043 | 1 | F | TP for TS 38.522 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204940 | 0044 | 1 | F | Correction of 38.522 | 16.5.0 |
| 2020-12 | RAN#90 | R5-206905 | 0051 | 1 | F | Update to applicability spec for 5G test cases | 16.6.0 |
| 2021-03 | RAN#91 | R5-210506 | 0055 | - | F | Correction of applicability definitions for long DRX cycle related test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210792 | 0058 | - | F | Adding the test applicability of RF test cases for eMIMO | 16.7.0 |
| 2021-03 | RAN#91 | R5-211158 | 0060 | - | F | Addition of applicability new test case 6.3.2.1.3 in TS 38.521-4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211159 | 0061 | - | F | Addition of applicability new test case 6.3.3.1.3 in TS 38.521-4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211610 | 0067 | - | F | Applicability of Error Vector Magnitude for V2X for non-concurrent operation | 16.7.0 |
| 2021-03 | RAN#91 | R5-211720 | 0054 | 1 | F | Correction of applicability definitions for PUSCH HalfPi BPSK related test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211853 | 0059 | 1 | F | Update to applicability spec for 5G test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211913 | 0057 | 1 | F | Adding test applicability for switching test case | 16.7.0 |
| 2021-03 | RAN#91 | R5-211917 | 0066 | 1 | F | Addition of new RRM test cases to the applicability table in 4.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211918 | 0068 | 1 | F | Applicability for RRM NR HST test case 6.1.1.7 and 6.6.1.7 | 16.7.0 |
| 2021-03 | RAN#91 | - | - | - | - | Administrative release upgrade to match the release of TS 38.508-1, TS 38.508-2 and TS 38.521-1 which were upgraded at RAN#91 to Rel-17 due to Rel-17 relevant CRs | 17.0.0 |
| 2021-06 | RAN#92 | R5-212078 | 0069 | - | F | Addition of applicability for new test case 6.3.2.1.4 and 6.3.3.1.4 in TS 38.521-4 | 17.1.0 |
| 2021-06 | RAN#92 | R5-212932 | 0075 | - | F | Addition of test applicability for V2X RF test cases | 17.1.0 |
| 2021-06 | RAN#92 | R5-212939 | 0076 | - | F | Adding test applicability for eMIMO demod test cases | 17.1.0 |
| 2021-06 | RAN#92 | R5-212948 | 0077 | - | F | Adding test applicability for URLLC demod test cases | 17.1.0 |
| 2021-06 | RAN#92 | R5-213095 | 0078 | - | F | Adding test applicability for new test cases introduced in R17 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214006 | 0074 | 1 | F | Addition of new V2X test cases to the applicability table in 4.1.1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214089 | 0073 | 1 | F | Update to applicability spec for 5G test cases | 17.1.0 |
| 2021-06 | RAN#92 | R5-214096 | 0071 | 1 | F | Update of Applicability for Inter-band EN-DC Including FR2 | 17.1.0 |
| 2021-09 | RAN#93 | R5-214480 | 0081 | - | F | 38.522 Jumbo CR for R16 CADC configurations | 17.2.0 |
| 2021-09 | RAN#93 | R5-214534 | 0082 | - | F | Addition of applicability for FR2 DL 256QAM demodulation test case | 17.2.0 |
| 2021-09 | RAN#93 | R5-214571 | 0083 | - | F | Adding test applicability for UE power saving test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214609 | 0084 | - | F | Correction of condition C30 C37 C37a C41 C41a and introduction of condition C37b and C37c | 17.2.0 |
| 2021-09 | RAN#93 | R5-214834 | 0085 | - | F | Applicability for 5G-SRVCC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215033 | 0092 | - | F | Addition of applicability for NR HST TCs | 17.2.0 |
| 2021-09 | RAN#93 | R5-215045 | 0093 | - | F | Addition of R16 FDD-TDD PC2 inter-band EN-DC baseline implementation capabilities into 38.522 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215079 | 0095 | - | F | Addition of test applicability for V2X test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215245 | 0099 | - | F | Addition of test applicability for RRM test case 6.6.4.5 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215399 | 0102 | - | F | Add 2-Step PRACH test cases to Applicability spec | 17.2.0 |
| 2021-09 | RAN#93 | R5-215411 | 0103 | - | F | Correction of RRM HST test cases applicability | 17.2.0 |
| 2021-09 | RAN#93 | R5-215931 | 0090 | 1 | F | Addition of applicability for Mob_Enh TCs | 17.2.0 |
| 2021-09 | RAN#93 | R5-215935 | 0096 | 1 | F | Adding test applicability for eMIMO test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215960 | 0098 | 1 | F | Addition of applicability of URLLC demod test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215981 | 0086 | 1 | F | FR2 standalone RF conformance test case applicability | 17.2.0 |
| 2021-09 | RAN#93 | R5-216077 | 0097 | 1 | F | Test applicability for FR2 256QAM CQI reporting | 17.2.0 |
| 2021-09 | RAN#93 | R5-216097 | 0101 | 1 | F | Update to applicability spec for 5G test cases | 17.2.0 |
| 2021-12 | RAN#94 | R5-216539 | 0105 | - | F | Addition of applicability for HST test case 5.2.3.1.9_1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-216540 | 0106 | - | F | Addition of applicability for HST test case 5.2.3.1.10_1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-216784 | 0109 | - | F | Correction to Test Bands Selection Criteria for performance test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-216852 | 0110 | - | F | Correction to applicability of RLM TCs | 17.3.0 |
| 2021-12 | RAN#94 | R5-216870 | 0111 | - | F | Correction to applicability of Mob_enh RRM TCs | 17.3.0 |
| 2021-12 | RAN#94 | R5-216911 | 0112 | - | F | Correction to applicability of HST TCs | 17.3.0 |
| 2021-12 | RAN#94 | R5-217219 | 0114 | - | F | Correct of condition for RRM Test Cases with BWP switch | 17.3.0 |
| 2021-12 | RAN#94 | R5-217319 | 0116 | - | F | Addition of applicability for new type II PMI repoering test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217349 | 0118 | - | F | Update of 3.1 for definitions of CA and DC configurations | 17.3.0 |
| 2021-12 | RAN#94 | R5-217381 | 0120 | - | F | Addition of test applicability for URLLC test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217529 | 0122 | - | F | Jumbo CR for updating applicability of NR perf enh WI test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217568 | 0123 | - | F | Correction of RRM HST test cases applicability | 17.3.0 |
| 2021-12 | RAN#94 | R5-217569 | 0124 | - | F | Correction of RRM test cases applicability - Note 1 removal | 17.3.0 |
| 2021-12 | RAN#94 | R5-217597 | 0125 | - | F | Update applicability for Tx modulation quality test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217729 | 0129 | - | F | 38.522 applicability updates for Rel.16 FR2 RF enhancements | 17.3.0 |
| 2021-12 | RAN#94 | R5-218249 | 0115 | 1 | F | Update of MPR applicability for intra-band contiguous EN-DC | 17.3.0 |
| 2021-12 | RAN#94 | R5-218370 | 0107 | 1 | F | Addition of content for FR2 standalone RF conformance test case applicability | 17.3.0 |
| 2021-12 | RAN#94 | R5-218371 | 0128 | 1 | F | NR U test case applicability | 17.3.0 |
| 2021-12 | RAN#94 | R5-218390 | 0108 | 1 | F | Addition of Power Class 1.5 into applicability of RF SA FR1 conformance test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-218437 | 0113 | 1 | F | 6.2B.2.2 MPR IBNC EN-DC applicability correction if 6.5B.2.2.3 ACLR IBNC EN-DC is executed | 17.3.0 |
| 2021-12 | RAN#94 | R5-218438 | 0127 | 1 | F | Update to applicability spec for 5G test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-218460 | 0117 | 1 | F | Adding test applicability for switching time mask for inter-band EN-DC | 17.3.0 |
| 2021-12 | RAN#94 | R5-218463 | 0119 | 1 | F | Addition of test applicability e-MIMO test cases | 17.3.0 |

///End of changes