3GPP TSG-RAN5 Meeting #95-e	R5-222111r1

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2878 | rev | - | Current version: | 16.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - At step 16 SS allocates UL grant using DCI format 0_1 for UE to loopback the MAC PDU sent by SS at step 2. As SS stopped responding to scheduling requests at step 3, after multiple SR transmissions UE falls back to RACH procedure. During this process, UE releases SRS resources as well. As a result, a conformant UE may not be able to accept the UL grant allocated via DCI format 0_1 which contains SRS resources. - At Step 0AA, The SS transmits an updated system information which includes prach_ConfigurationIndex = (94) configuration for TDD .Prach is expected to be received in the subframe = 4/9 ( every odd SFN ) ;with the configured default TDD DL UL Pattern, If Prach is received in Subframe =9 and slot = 1 , no issue is observed ; But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) - similar topic is already discussed in R5s220280 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the pre test condition to use Short_DCI in NR Serving cell configuration. - It is proposed that in Table 7.1.1.1.2.3.3-7 IE ra-ResponseWindow should be set to sl40 for condition FR1 TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st Revision – Added prose changes associated with TTCN CR R5s220673 from R&S. Also added R&S as co-source |  |  |  |  |  |  |  |  |

##### 7.1.1.1.2	Random access procedure / Successful / C-RNTI Based / Preamble selected by MAC itself

7.1.1.1.2.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected NR SpCell TimeAlignmentTimer expired, and has UL Data to send }

ensure that {

when { the UL MAC PDU Size is less than messageSizeGroupA }

then { UE transmits a random access preamble using a preamble in group A of random access preambles }

}

(2)

with { UE in RRC_Connected state after transmission of a PRACH preamble on NR SpCell }

ensure that {

when { SS does not answer with a matching Random Access Response within ra-ResponseWindowSize }

then { UE retransmits a PRACH preamble from same group }

}

(3)

with { UE in RRC_Connected state after transmission of a PRACH preamble on NR SpCell }

ensure that {

when { UE receives while ra-ResponseWindowSizeTimer is running MAC PDU containing multiple RARs but none of the subheaders contains a RAPID corresponding to the UE }

then { UE retransmits a PRACH preamble from same group }

}

(4)

with { UE in RRC_Connected state after transmission of a PRACH preamble on NR SpCell }

ensure that {

when { SS sends a Random Access Response including a Backoff Indicator and the Random Access Preamble identifier is different from the value received from the UE }

then { UE triggers RA preamble after a random time between 0 and the indicated Backoff parameter from same group }

}

(5)

with { UE in RRC_Connected state after transmission of a PRACH preamble on NR SpCell }

ensure that {

when { UE receives while ra-ResponseWindowSizeTimer is running MAC PDU containing multiple RARs and one of the subheaders contains a RAPID corresponding to the UE and containing Backoff Indicator }

then { UE stores Backoff Indicator UE transmits RACH procedure MSG3 }

}

(6)

with { UE in RRC_Connected state after transmission of Msg3 on NR SpCell without dedicated preamble }

ensure that {

when { The SS does not schedule any PDCCH transmission addressed to UE C-RNTI before Contention resolution timer expiry }

then { UE transmits a random access preamble using a preamble in the same group of random access preambles as used for the first transmission of Msg3 }

}

(7)

with { UE in RRC_Connected state after transmission of Msg3 on NR SpCell without dedicated preamble }

ensure that {

when { UE receive PDCCH transmission addressed to its C-RNTI before Contention resolution timer expiry }

then { UE considers RACH procedure as complete }

}

(8)

with { UE in RRC CONNECTED state and Random Access Preambles group B is configured }

ensure that {

when { UE has data available for transmission and the MAC PDU Size carrying this data is greater than ra-Msg3SizeGroupA and TimeAlignmentTimer expires }

then {UE transmits a random access preamble using a preamble in group B of random access preambles}

}

(9)

with { UE in RRC_Connected state and having initiated a random access procedure in NR SpCell }

ensure that {

when { The SS transmits a Timing Advance Command in a Random Access Response message }

then {the UE applies the received Timing Advance value in the next transmitted MAC PDU }

}

7.1.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clauses 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.2, 6.1.3.2, 6.1.5 and 6.2.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.1.2]

The MAC entity shall:

…

1>	else (i.e. for the contention-based Random Access preamble selection):

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B is configured:

4>	if the potential Msg3 size (UL data available for transmission plus MAC header and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) –preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA:5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	if the association between Random Access Preambles and SSBs is configured:

3>	select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group.

2>	else:

3>	select a Random Access Preamble randomly with equal probability from the Random Access Preambles within the selected Random Access Preambles group.

2>	set the PREAMBLE_INDEX to the selected ra-PreambleIndex.

…

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if ra-AssociationPeriodIndex and si-RequestPeriod are configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to subclause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

1>	else if an SSB is selected above:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to subclause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB).

1>	else if a CSI-RS is selected above:

2>	if there is no contention-free Random Access Resource associated with the selected CSI-RS:

3>	determine the next available PRACH occasion from the PRACH occasions, permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, corresponding to the SSB in candidateBeamRSList which is quasi-collocated with the selected CSI-RS as specified in TS 38.214 [7] (the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the SSB which is quasi-collected with the selected CSI-RS).

2>	else:

3>	determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers, corresponding to the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS).

1>	perform the Random Access Preamble transmission procedure (see subclause 5.1.3).

[TS 38.321, clause 5.1.3]

The MAC entity shall, for each Random Access Preamble:

1>	if PREAMBLE_TRANSMISSION_COUNTER is greater than one; and

1>	if the notification of suspending power ramping counter has not been received from lower layers; and

1>	if SSB selected is not changed (i.e. same as the previous Random Access Preamble transmission):

2>	increment PREAMBLE_POWER_RAMPING_COUNTER by 1.

1>	select the value of DELTA_PREAMBLE according to subclause 7.3;

1>	set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP;

1>	except for contention-free Random Access Preamble for beam failure recovery request, compute the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted;

1>	instruct the physical layer to transmit the Random Access Preamble using the selected PRACH, corresponding RA-RNTI (if available), PREAMBLE_INDEX and PREAMBLE_RECEIVED_TARGET_POWER.

The RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI= 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the specified PRACH (0 ≤ s_id < 14), t_id is the index of the first slot of the specified PRACH in a system frame (0 ≤ t_id < 80), f_id is the index of the specified PRACH in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Msg1 transmission (0 for NUL carrier, and 1 for SUL carrier).

[TS 38.321, clause 5.1.4]

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

…

1>	else:

2>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission is received from lower layers on the Serving Cell where the preamble was transmitted; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see subclause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for the SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see subclause 5.2);

5>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × preamblePowerRampingStep).

5>	if the Serving Cell for the Random Access procedure is SRS-only SCell:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

…

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received; or

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if the PDCCH addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted:

2> consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTxMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers.

4>	if this Random Access procedure was triggered for SI request:

5> consider the Random Access procedure unsuccessfully completed.

>	else if the Random Access Preamble is transmitted on a SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	if the criteria (as defined in subclause 5.1.2) to select contention-free Random Access Resources is met during the backoff time:

4>	perform the Random Access Resource selection procedure (see subclause 5.1.2);

3>	else:

4>	perform the Random Access Resource selection procedure (see subclause 5.1.2) after the backoff time.

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response transmission.

[TS 38.321, clause 5.1.5]

Once Msg3 is transmitted, the MAC entity shall:

1>	start the ra-ContentionResolutionTimer and restart the ra-ContentionResolutionTimer at each HARQ retransmission in the first symbol after the end of the Msg3 transmission;

1>	monitor the PDCCH while the ra-ContentionResolutionTimer is running regardless of the possible occurrence of a measurement gap;

1>	if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers:

2>	if the C-RNTI MAC CE was included in Msg3:

3>	if the Random Access procedure was initiated by the MAC sublayer itself or by the RRC sublayer and the PDCCH transmission is addressed to the C-RNTI and contains a UL grant for a new transmission; or

3>	if the Random Access procedure was initiated by a PDCCH order and the PDCCH transmission is addressed to the C-RNTI; or

3>	if the Random Access procedure was initiated by a beam failure indication from lower layer and the PDCCH transmission is addressed to the C-RNTI:

4>	consider this Contention Resolution successful;

4>	stop ra-ContentionResolutionTimer;

4>	discard the TEMPORARY_C-RNTI;

4>	consider this Random Access procedure successfully completed.

…

1>	if ra-ContentionResolutionTimer expires:

2>	discard the TEMPORARY_C-RNTI;

2>	consider the Contention Resolution not successful.

1>	if the Contention Resolution is considered not successful:

2>	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTxMax + 1:

3>	indicate a Random Access problem to upper layers.

3>	if this Random Access procedure was triggered for SI request:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3> select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	if the criteria (as defined in subclause 5.1.2) to select contention-free Random Access Resources is met during the backoff time:

4>	perform the Random Access Resource selection procedure (see subclause 5.1.2);

3>	else:

4>	perform the Random Access Resource selection procedure (see subclause 5.1.2) after the backoff time.

[TS 38.321, clause 5.2]

RRC configures the following parameters for the maintenance of UL time alignment:

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned.

The MAC entity shall:

1>	when a Timing Advance Command MAC CE is received, and if a NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG:

2>	apply the Timing Advance Command for the indicated TAG;

2>	start or restart the timeAlignmentTimer associated with the indicated TAG.

…

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with the PTAG:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

2>	else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG:

3>	flush all HARQ buffers;

3>	notify RRC to release PUCCH, if configured;

3>	notify RRC to release SRS, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble transmission when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running. Furthermore, when the timeAlignmentTimer associated with the pTAG is not running, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble transmission on the SpCell.

[TS 38.321, clause 6.1.3.2]

The C-RNTI MAC CE is identified by MAC PDU subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size and consists of a single field defined as follows (Figure 6.1.3.2-1):

-	C-RNTI: This field contains the C-RNTI of the MAC entity. The length of the field is 16 bits.

Figure 6.1.3.2-1: C-RNTI MAC CE

[TS 38.321, clause 6.1.5]

A MAC PDU consists of one or more MAC subPDUs and optionally padding. Each MAC subPDU consists one of the following:

-	a MAC subheader with Backoff Indicator only;

-	a MAC subheader with RAPID only (i.e. acknowledgment for SI request);

-	a MAC subheader with RAPID and MAC RAR.

A MAC subheader with Backoff Indicator consists of five header fields E/T/R/R/BI as described in Figure 6.1.5-1. A MAC subPDU with Backoff Indicator only is placed at the beginning of the MAC PDU, if included. 'MAC subPDU(s) with RAPID only' and 'MAC subPDU(s) with RAPID and MAC RAR' can be placed anywhere between MAC subPDU with Backoff Indicator only (if any) and padding (if any).

A MAC subheader with RAPID consists of three header fields E/T/RAPID as described in Figure 6.1.5-2.

Padding is placed at the end of the MAC PDU if present. Presence and length of padding is implicit based on TB size, size of MAC subPDU(s).

Figure 6.1.5-1: E/T/R/R/BI MAC subheader

Figure 6.1.5-2: E/T/RAPID MAC subheader

Figure 6.1.5-3: Example of MAC PDU consisting of MAC RARs

[TS 38.321, clause 6.2.3]

The MAC RAR is of fixed size as depicted in Figure 6.2.3-1, and consists of the following fields:

-	R: Reserved bit, set to "0";

-	Timing Advance Command: The Timing Advance Command field indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply in TS 38.213 [6]. The size of the Timing Advance Command field is 12 bits;

-	UL Grant: The Uplink Grant field indicates the resources to be used on the uplink in TS 38.213 [6]. The size of the UL Grant field is 27 bits;

-	Temporary C-RNTI: The Temporary C-RNTI field indicates the temporary identity that is used by the MAC entity during Random Access. The size of the Temporary C-RNTI field is 16 bits.

The MAC RAR is octet aligned.

Figure 6.2.3-1: MAC RAR

7.1.1.1.2.3	Test description

7.1.1.1.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that Short_DCI condition is applied in NR Serving cell configuration.

7.1.1.1.2.3.2	Test procedure sequence

Table 7.1.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 0AA is performed IF pc_NG_RAN_NR only. | - | - | - | - |
| 0AA | The SS transmits an updated system information as specified in Table 7.1.1.1.2.3.3-1A. | - | - | - | - |
| 0A | SS transmits an RRCReconfiguration message to configure specific parameters. (Note 1) | <-- | RRCReconfiguration | - | - |
| 0B | The UE transmits RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 1 | SS transmits Timing Advance command to SpCell. SS does not send any subsequent timing alignments. Start Timer_T1 = Time Alignment timer value on SS. | <-- | MAC PDU (Timing Advance Command MAC Control Element) | - | - |
| 2 | 40 to 50 TTI before Timer_T1 expires the SS transmits a MAC PDU containing a PDCP SDU of size 56 bits, less then ra-Msg3SizeGroupA (208 bits) on SpCell. (Note 3) | <-- | MAC PDU | - | - |
| 3 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 4 | Check: Does the UE transmit preamble on PRACH using a preamble in group A defined in CellGroupConfig in RRCReconfiguration (totalNumberOfRA-Preambles, ssb-perRACH-OccasionAndCB-PreamblesPerSSB and numberOfRA-PreamblesGroupA) on SpCell in frame number X meeting condition nSFN mod 8 =1, subframe number 2,6,9 (FDD FR1), frame number X meeting condition nSFN mod 2 =1, subframe number 4,9 (FR1 TDD) and frame number X meeting condition nSFN mod 4 =1 and slot number 8, 9, 18, 19, 28, 29, 38, 39 , 48, 49, 58, 59, 68, 69, 78, 79 (FR2 120 kHz)? | --> | PRACH Preamble | 1 | P |
| 5 | Check: Does the UE transmit a preamble on PRACH, in frame number X+8 subframe number 2,6,9 (FDD FR1), in frame number X or X+2 in subrame number 4,9 (FR1 TDD) and frame number X or X+4 and slot number 8, 9, 18, 19, 28, 29, 38, 39 , 48, 49, 58, 59, 68, 69, 78, 79 (FR2 120 kHz) using the same group A? | --> | PRACH Preamble | 2 | P |
| 6 | The SS transmits a MAC PDU addressed to UE RA-RNTI, containing multiple RARs but none of the MAC sub headers contains a matching RAPID on SpCell. | <-- | Random Access Response | - | - |
| - | EXCEPTION: In parallel with step 7, parallel behaviour defined in table 7.1.1.1.2.3.2-2 is executed. | - | - | - | - |
| 7 | Check: Does the UE re-transmit a preamble on PRACH on SpCell using the same group A? | --> | PRACH Preamble | 3 | P |
| 8 | The SS transmits a Random Access Response with the back off parameter set to value Index field '12' and with the Random Access Preamble identifier different from the value received from the UE in the Random Access Preamble. The SS sets Timer_T2 to the Back off value ‘960’ associated with the Index value ‘12’ and starts Timer_T2. | <-- | Random Access Response(BI, RAPID) | - | - |
| 9 | Check: Does UE send a Random Access Preamble on SpCell while Timer_T2 is running? | --> | Random Access Preamble | 4 | P |
| 10 | SS sends Random Access Response with an UL Grant of 56-bits, a back off parameter set to value Index field ‘13’ and the Random Access Preamble identifier value set to the same value as received from the UE in the Random Access Preamble. (Note 4) | <-- | Random Access Response(BI, RAPID) | - | - |
| 11 | Check: Does UE sends a msg3 in the grant associated to the Random Access ´Response received in step 10 on SpCell? | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | 5 | P |
| 12 | SS does not schedule any PDCCH transmission for UE C-RNTI. The SS sets Timer_T3 to the Back off value ‘1920’ associated with the Index value ‘13’ plus Contention Resolution Timer and starts Timer_T3. | - | - | - | - |
| 13 | Check: Does the UE transmit preamble on PRACH using a preamble belonging to group A for time equal to Timer_T3 on SpCell? | --> | PRACH Preamble | 6 | P |
| 14 | The SS transmits Random Access Response with an UL Grant of 56-bits and RAPID corresponding to the transmitted Preamble in step 13, including T-CRNTI. | <-- | Random Access Response | - | - |
| 15 | UE sends a msg3 using the grant associated to the Random Access ´Response received in step 14 on SpCell? | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | - | - |
| 16 | SS schedules PDCCH transmission for UE C_RNTI and allocate uplink grant. | <-- | Contention Resolution | - | - |
| - | EXCEPTION: In parallel with step 17, parallel behaviour defined in table 7.1.1.1.2.3.2-3 is executed. | - | - | - | - |
| 17 | Check: Does the UE transmit a MAC PDU with C-RNTI containing looped back PDCP SDU? | --> | MAC PDU | 7 | P |
| - | EXCEPTION: Step 17AA is performed IF pc_NG_RAN_NR only. | - | - | - | - |
| 17AA | The SS transmits an updated system information as specified in Table 7.1.1.1.2.3.3-1A. | - | - | - | - |
| 17A | The SS transmits an RRCReconfiguration message to configure specific parameters. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 17B | The UE transmits an RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 18 | SS transmits Timing Advance command to SpCell. SS does not send any subsequent timing alignments. Start Timer_T4 = Time Alignment timer value on SS. | <-- | MAC PDU (Timing Advance Command MAC Control Element) | - | - |
| 19 | 40 to 50 TTI before Timer_T4 expires the SS transmits a MAC PDU containing a PDCP SDU of size > ra-Msg3SizeGroupA (208 bits). | <-- | MAC PDU | - | - |
| 20 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 21 | Check: Does the UE transmit preamble on PRACH using a preamble in group B defined in CellGroupConfig in RRCReconfiguration (ssb-perRACH-OccasionAndCB-PreamblesPerSSB, numberOfRA-PreamblesGroupA and numberOfRA-Preambles) on SpCell? | --> | PRACH Preamble | 8 | P |
| 22 | The SS transmits Random Access Response with an UL Grant of 56-bits and RAPID corresponding to the transmitted Preamble in step 21, including T-CRNTI. | <-- | Random Access Response | - | - |
| 23 | UE sends a msg3 using the grant associated to the Random Access ´Response received in step 22 on SpCell? | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | - | - |
| 23A | SS schedules PDCCH transmission for UE C_RNTI and allocate uplink grant. | <-- | Contention Resolution | - | - |
| 24 | Check: Does the UE transmit a MAC PDU with C-RNTI containing looped back PDCP SDU? | --> | MAC PDU | 9 | P |
| Note 1: for EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: for EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: MAC PDU size of 56bits is selected to allow UE send status PDU and still stays below the limit of ra-Msg3SizeGrioupA. Note 4: UL grant of 56bits is to make UE not send any loopback data in uplink with msg3. |  |  |  |  |  |

Table 7.1.1.1.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit msg3 message on SpCell? | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | - | F |

Table 7.1.1.1.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit an PRACH preamble on SpCell? | --> | PRACH Preamble | - | F |

7.1.1.1.2.3.3	Specific message contents

Table 7.1.1.1.2.3.3-1: MAC-CellGroupConfig (preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| tag-Config SEQUENCE { |  |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG { | 1 entry |  |  |
| TAG[1] SEQUENCE { |  | entry 1 |  |
| timeAlignmentTimer | ms750 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.2.3.3-1A: SystemInformationBlockType1 (step 0AA and 17AA, Table 7.1.1.1.2.3.2-1)

| Derivation path: 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Same contents as in Table 7.1.1.1.2.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.1.2.3.3-2: RRCReconfiguration (step 0A and step 17A, Table 7.1.1.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
|  |  |  | TS 38.508-1 [4], 2. |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.2.3.3-3: CellGroupConfig (Table 7.1.1.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| newUE-Identity | RNTI-Value |  |  |
| t304 | ms2000 |  |  |
| rach-ConfigDedicated | Not Present |  |  |
| } |  |  |  |

Table 7.1.1.1.2.3.3-4: ServingCellConfigCommon (Table 7.1.1.1.2.3.3-3, Table 7.1.1.1.2.3.3-1A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.2.3.3-5: BWP-UplinkCommon (Table 7.1.1.1.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.2.3.3-6: RACH-ConfigCommon (Table 7.1.1.1.2.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 42 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| One | n32 |  |  |
| } |  |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| ra-Msg3SizeGroupA | b208 |  |  |
| messagePowerOffsetGroupB | minusinfinity |  |  |
| numberOfRA-PreamblesGroupA | 28 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| } |  |  |  |

Table 7.1.1.1.2.3.3-7: RACH-ConfigGeneric (Table 7.1.1.1.2.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 119 | As per Table 6.3.3.2-2: of TS 38.211 [24], this results in PRACH preamble transmission in a radio frame meeting nSFN mod 2=1, subframe number 2, 6, 9 and starting symbol 0 using preamble Format A2. | FR1 FDD |
| prach-ConfigurationIndex | 94 | As per Table 6.3.3.2-3: of TS 38.211 [24], this results in PRACH preamble transmission in a radio frame meeting nSFN mod 2=1, subframe number 4, 9 and starting symbol 0 using preamble Format A2. | FR1 TDD |
| prach-ConfigurationIndex | 6 | As per Table 6.3.3.2-4: of TS 38.211 [24] and clause 5.3.2 of TS 38.211 this results in PRACH preamble transmission in radio frame meeting nSFN mod 4 = 1, slot number 8, 9, 18, 19, 28, 29, 38, 39 , 48, 49, 58, 59, 68, 69, 78, 79 and starting symbol 0 using preamble format A1. | FR2 (120 kHz) |
| preambleReceivedTargetPower | dBm-104 |  |  |
| preambleTransMax | n10 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl8 |  | FR1 FDD and FR2 (120 kHz) |
|  | sl40 |  | FR1 TDD |
| } |  |  |  |

Table 7.1.1.1.2.3.3-8: Void