3GPP TSG-RAN5 Meeting #95-e	R5-222132r1

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2295 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SIB11 to common environment for early measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To remove SIB11 configuration from test cases to common environment configuration to which testcases can then refer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new system information combination NR-18 and defining SIB11 to common environment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SIB11 will remain as a testcase specific config. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.1.2, 4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-1 CR 2926, 2929, 2973, 2984, 2985, 2999 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: - NR-4 modification canceled due to too big impact to existing test cases. - NR-18 SIB4 added as NR-19 can be used for idle/inactive tests without SIB4 - SIB11 conditions updated and missing fields added - Added source to TSG |  |  |  |  |  |  |  |  |

##### ------------------------------------------------------- Start of changes -----------------------------------------------------

##### 4.4.3.1.2	Combinations of system information blocks for NR standalone and NE-DC

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, several combinations of system information blocks are defined.

Regardless of the combination of system information blocks indicated as being used by a test case, the SS shall broadcast only the NR MIB on the NR Cell(s) configured on an SDL band.

Combination NR-1 is the default combination which applies to the following test case scenarios:

-	NR FDD single cell scenario except RRM test case scenarios

-	NR TDD single cell scenario except RRM test case scenarios

-	NR FDD inter-band DC component carriers cell scenario

-	NR TDD inter-band DC component carriers cell scenario

-	NR FDD and NR TDD inter-band DC component carriers cell scenario

Combination NR-2 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario

-	NR TDD intra-frequency multi cell scenario

-	NR FDD and NR TDD dual mode multi cell roaming scenario

-	NR FDD single cell RRM test case scenario

-	NR TDD single cell RRM test case scenario

Combination NR-3 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario with neighbouring cell related information

-	NR TDD intra-frequency multi cell scenario with neighbouring cell related information

Combination NR-4 applies to the following test case scenarios:

-	NR FDD inter-frequency multi cell scenario

-	NR TDD inter-frequency multi cell scenario

-	NR FDD inter-band multi cell scenario

-	NR TDD inter-band multi cell scenario

-	NR FDD and NR TDD dual mode multi cell non-roaming scenario

-	NR FDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD inter-band carrier aggregation component carriers cell scenario

-	NR TDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario

Combination NR-5 applies to the following test case scenarios:

-	NR FDD intra-band carrier aggregation component carriers cell scenario + NR FDD intra-frequency neighbour.

-	NR FDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR TDD intra-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

Combination NR-6 applies to the following test case scenarios:

-	3GPP inter-RAT NR FDD + E-UTRA FDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA TDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA FDD multi cell scenario

Combination NR-7 applies to the following test case scenarios:

-	NR FDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

-	NR TDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

Combination NR-8 applies to the following test case scenarios:

-	NR FDD ETWS single cell scenario

-	NR TDD ETWS single cell scenario

Combination NR-9 applies to the following test case scenarios:

-	3GPP NR FDD + CMAS single cell scenario

-	3GPP NR TDD + CMAS single cell scenario

Combination NR-10 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS primary notification single cell scenario

-	3GPP NR TDD + ETWS primary notification single cell scenario

Combination NR-11 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS secondary notification single cell scenario

-	3GPP NR TDD + ETWS secondary notification single cell scenario

Combination NR-12 applies to the following test case scenarios:

-	3GPP NR FDD + SNPN only single cell scenario

-	3GPP NR TDD + SNPN only single cell scenario

Combination NR-13 applies to the following test case scenarios:

-	3GPP NR FDD + CAG cell  multi cell scenario

-	3GPP NR TDD + CAG cell  multi cell scenario

Combination NR-14 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

Combination NR-15 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD single cell scenario + positioning test case which require posSIBs.

Combination NR-16 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-17 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-18 applies to the following test case scenarios:

- 	3GPP NR FDD inter-frequency multi cell scenario + idle/inactive measurements.

-	3GPP NR TDD inter-frequency multi cell scenario + idle/inactive measurements.

Table 4.4.3.1.2-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB1 | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 | SIB14 | posSIBs |
| NR-1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-2 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-3 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-4 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |
| NR-5 | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |
| NR-6 | X | X |  |  | X |  |  |  |  |  |  |  |  |  |  |
| NR-7 | X | X |  | X | X |  |  |  |  |  |  |  |  |  |  |
| NR-8 | X |  |  |  |  | X | X |  |  |  |  |  |  |  |  |
| NR-9 | X |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| NR-10 | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| NR-11 | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| NR-12 | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| NR-13 | X | X |  |  |  |  |  |  |  | X |  |  |  |  |  |
| NR-14 | X | X |  | X |  |  |  |  |  |  |  | X |  |  |  |
| NR-15 | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-16 | X | X |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-17 | X | X |  | X |  |  |  |  |  |  |  |  |  |  | X |
| NR-18 | X |  |  | X |  |  |  |  |  |  | X |  |  |  |  |

##### 4.4.3.1.3	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables. There is no scheduling information for combination NR-1.

Table 4.4.3.1.3-1: Scheduling for combination NR-2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |

Table 4.4.3.1.3-2: Scheduling for combination NR-3

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB3 |

Table 4.4.3.1.3-3: Scheduling for combination NR-4

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |

Table 4.4.3.1.3-4: Scheduling for combination NR-5

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB3 |
| 3 | 64 | SIB4 |

Table 4.4.3.1.3-5: Scheduling for combination NR-6

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB5 |

Table 4.4.3.1.3-6: Scheduling for combination NR-7

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4, SIB5 |

Table 4.4.3.1.3-7: Scheduling for combination NR-8

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB6 |
| 2 | 32 | SIB7 |

Table 4.4.3.1.3-8: Scheduling for combination NR-9

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB8 |

Table 4.4.3.1.3-9: Scheduling for combination NR-10

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
|  | 32 | SIB6 |

Table 4.4.3.1.3-10: Scheduling for combination NR-11

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB7 |

Table 4.4.3.1.3-11: Scheduling for combination NR-12

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB10 |

Table 4.4.3.1.3-12: Scheduling for combination NR-13

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB10 |

Table 4.4.3.1.3-13: Scheduling for combination NR-14

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB12 |

Table 4.4.3.1.3-14: Scheduling for combination NR-18

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB4 |
| 2 | 64 | SIB11 |

## 4.4A	Test states

### 4.4A.1	General

The purpose of the test states is to get the UE into specific 5GC and RRC protocol states in the initial condition of test cases. Each test state is identified by a test state ID. The syntax used for test state IDs is described in sub-clause 4.4A.4. The list of defined test states and the associated UE 5GC and RRC/ N3AN protocol states are specified in sub-clause 4.4A.2.

##### ------------------------------------------------------- Skipped chapters ---------------------------------------------------

#### –	SIB10

Table 4.6.2-12: SIB10

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB10-r16 ::= SEQUENCE { |  |  |  |
| hrnn-List-r16 SEQUENCE (SIZE (1..maxNPN-r16)) OF HRNN-r16 { |  |  |  |
| HRNN-r16[1] SEQUENCE { |  | 1 entry |  |
| hrnn-r16 | “3gppTest” | charstring “3gppTest” converted to OCTETSTRING |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

#### –	SIB11

Table 4.6.2-13: SIB11

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB11-r16 ::= SEQUENCE { |  |  |  |
| measIdleConfigSIB-r16 SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not present |  |  |
| measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16 { | The same number of entries as NR cells configured for measurements | Serving cell not included | EMR_NR_SIB4, EMR_NR_SIB11 |
| MeasIdleCarrierNR-r16[n] SEQUENCE { |  | entry n |  |
| carrierFreq-r16 | ARFCN value of configured NR cell |  |  |
| ssbSubcarrierSpacing-r16 | SubcarrierSpacing | Table 4.6.3-188 |  |
| frequencyBandList | Not present |  |  |
| measCellListNR-r16 | Not present |  |  |
| reportQuantities-r16 | both |  |  |
| qualityThreshold-r16 | Not present |  |  |
| ssb-MeasConfig-r16 | Not present |  | EMR_NR_SIB4 |
| ssb-MeasConfig-r16 SEQUENCE { |  |  | EMR_NR_SIB11 |
| nrofSS-BlocksToAverage-r16 | 2 |  |  |
| absThreshSS-BlocksConsolidation-r16 SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| smtc-r16 | SSB-MTC | Table 4.6.3-185 |  |
| ssb-ToMeasure-r16 | SSB-ToMeasure | Table 4.6.3-187 |  |
| deriveSSB-IndexFromCell-r16 | false |  |  |
|  | true |  | FR1_TDD, FR2_TDD |
| ss-RSSI-Measurement-r16 | Not present |  |  |
| } |  |  |  |
| beamMeasConfigIdle-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleCarrierListEUTRA-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16 { | The same number of entries as E-UTRA cells configured for measurements | Serving cell not included | EMR_EUTRA_SIB11 |
| MeasIdleCarrierEUTRA-r16[n] SEQUENCE { |  | entry n |  |
| carrierFreqEUTRA-r16 | ARFCN value of configured E-UTRA cell |  |  |
| allowedMeasBandwidth-r16 | EUTRA-AllowedMeasBandwidth | Table 4.6.5-1. |  |
| measCellListEUTRA-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF EUTRA-PhysCellIdRange { |  |  |  |
| EUTRA-PhysCellIdRange[n] SEQUENCE { |  | entry n |  |
| start | EUTRA-PhysCellId corresponding to E-UTRA cell |  |  |
| range | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuatitiesEUTRA-r16 | both |  |  |
| qualityThresholdEUTRA-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| EMR_NR_SIB4 | For idle/inactive testcases where part of the NR configuration is derived from SIB4. |
| EMR_NR_SIB11 | For idle/inactive testcases where NR configuration is derived from SIB11. |
| EMR_EUTRA_SIB11 | For idle/inactive testcases where E-UTRA configuration is derived from SIB11. |

#### –	SIB12

Table 4.6.2-14: SIB12

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-r16 ::= SEQUENCE { |  |  |  |
| segmentNumber-r16 | index of the segment contained in segmentContainer-r16 |  |  |
| segmentType-r16 | notLastSegment |  | NOT_LAST_SEG |
|  | lastSegment |  | LAST_SEG |
| segmentContainer-r16 | OCTET STRING (CONTAINING SIB12-IEs-r16 or segment of SIB12-IEs-r16) | Note 1 |  |
| } |  |  |  |
| Note 1: Size of the SIB12-IEs-r16 or segment of SIB12-IEs-r16 contained in segmentContainer-r16 shall not exceed the maximum TBS size of the SI. For signalling test cases, the size is defined in TS 38.523-3[23] clause 7.3.3.2. |  |  |  |

| Condition | Explanation |
| --- | --- |
| NOT_LAST_SEG | Used when SIB12-IEs-r16 is segmented and the segment contained in segmentContainer-r16 is not the last segment |
| LAST_SEG | Used when SIB12-IEs-r16 is not segmented, or SIB12-IEs-r16 is segmented and the segment contained in segmentContainer-r16 is the last segment |

##### ------------------------------------------------------- End of changes -----------------------------------------------------