3GPP TSG-RAN5 Meeting #95-e	R5-222193r1

# Electronic Meeting, 9th May– 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1336 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Transmitter power test requirements for EN-DC within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - NR carrier UL power requirement is still FFS for Rel-15 Intra-Band EN-DC Power Class 2 UE. - Interband EN-DC Power Class 2 is release independent from Rel-15 for TDD Duplex-mode. NR carrier test requirements for Rel-15 Interband EN-DC Power Class 2 UE are only defined for UE indicates PC2 on NR band in 6.2B.4.1.3 (Configured Output Power). There is no NR carrier test requirements defined in 6.2B.1.3 (MOP), 6.2B.2.3 (MPR) and 6.2B.3.3 (A-MPR) for Rel-15 Interband EN-DC Power Class 2 UE. There is no NR carrier Power Class 3 test requirements defined in 6.2B.4.1.3 (Configured Output Power) for Rel-15 Interband EN-DC Power Class 2 UE. - Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs) is missing in 6.2B.1.3.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR carrier UL power requirement for Rel-15 Intra-Band EN-DC Power Class 2 UE was added. - 6.2B.1.3 (MOP), 6.2B.2.3 (MPR) and 6.2B.3.3 (A-MPR) were revised to add NR carrier test requirements for Rel-15 Inter-Band EN-DC TDD Duplex-mode Power Class 2 UE. - 6.2B.4.1.3 (Configured Output Power) was revised to add NR carrier PC3 test requirements for Rel-15 Interband EN-DC TDD Duplex-mode Power Class 2 UE. - Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs) was added in 6.2B.1.3.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - NR carrier test requirements for EN-DC within FR1 Power Class 2 UE will be incomplete. - Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs) will be missing in 6.2B.1.3.5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1.1, 6.2B.1.2, 6.2B.1.3.5, 6.2B.2.1.5, 6.2B.2.2.5, 6.2B.2.3.5, 6.2B.3.1.5.2, 6.2B.3.2.5, 6.2B.3.3.5, 6.2B.4.1.1.5, 6.2B.4.1.2.5, 6.2B.4.1.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Table 6.2B.1.3.5-2a was updated as Interband EN-DC Power Class 2 is also release independent from Rel-15 for FDD and TDD Duplex-mode. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2B.1.1	UE Maximum Output Power for Intra-Band Contiguous EN-DC

Editor's note:

-	For overlapping transmission there is no test point satisfying 0dB MPR according to RAN4 specification.

<< Unchanged sections omitted >>

6.2B.1.1.5	Test requirements

The maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the DC UE Power Class and tolerance in Table 6.2B.1.1.5-1 for power class 3 UE and Table 6.2B.1.1.5-2 for power class 2 UE. The corresponding requirements is specified in Table 6.2.2.5-1, 6.2.2_1.5-1 in TS 36.521-1 [10] or Table 6.2.1.5-1, 6.2.1.5-2 in TS 38.521-1 [8].

The maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed in Table 6.2B.1.1.5-1.

Table 6.2B.1.1.5-1: Maximum output power for EN-DC (continuous sub-blocks) for power class 3

| DC configuration | Power class2 | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA |  |  | 23 | +2+TT/-3-TT |
| DC_(n)41AA |  |  | 23 | +2+TT/-31+TT |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.1.5-3. |  |  |  |  |

Table 6.2B.1.1.5-2: Maximum output power for EN-DC (continuous sub-blocks) for Power Class 2 for Rel-15

| DC configuration | Carrier | Power class2 | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_(n)41AA | NR carrier | 26 | +2+TT/-31-TT | UE indicates PC2 on NR band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-31-TT | UE indicates PC3 on NR band | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-31-TT | UE indicates PC2 ON E-UTRA band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-31-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.1.5-3 |  |  |  |  |  |

Table 6.2B.1.1.5-2a: Maximum output power for EN-DC (continuous sub-blocks) for Power Class 2 for Rel-16 and forward

| DC configuration | Carrier | Power class2 | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_(n)41AA | NR carrier | 26 | +2+TT/-31-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-31-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-31-TT | UE indicates PC2 ON E-UTRA band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-31-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.1.5-3. |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2B.1.2	UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC

Editor's note:

-	For overlapping transmission there is no test point satisfying 0dB MPR according to RAN4 specification.

<< Unchanged sections omitted >>

6.2B.1.2.5	Test requirements

The maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the DC UE Power Class and tolerance in Table 6.2B.1.2.5-1 for power class 3 UE and Table 6.2B.1.2.5-2 for power class 2 UE. The corresponding requirements is specified in table 6.2.2.5-1, 6.2.2_1.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1, 6.2.1.5-2 in TS 38.521-1 [8].

The maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed in Table 6.2B.1.2.5-1.

Table 6.2B.1.2.5-1: Maximum output power for EN-DC (non-continuous sub-blocks) for power class 3

| DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_2A_n2A4 |  |  | 23 | +2+TT/-3-TT |
| DC_3A_n3A2 |  |  | 23 | +2+TT/-3-TT |
| DC_41A_n41A |  |  | 23 | +2+TT/-31-TT |
| DC_66A_n66A4 |  |  | 23 | +2+TT/-3-TT |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Only single switched UL is supported in Rel-15. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.2.5-3 NOTE 4: Only single switched UL is supported. |  |  |  |  |

Table 6.2B.1.2.5-2: Maximum output power for EN-DC (non-continuous sub-blocks) for power class 2 for Rel-15

| DC configuration | Carrier | Power class2 | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_41A_n41A | NR carrier | 26 | +2+TT/-31-TT | UE indicates PC2 on NR band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-21-TT | UE indicates PC3 on NR band | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-31-TT | UE indicates PC3 on E-UTRA carrier of this DC_Configutation | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-21-TT | UE indicates PC3 on E-UTRA carrier of this DC_Configutation | UE meets power class 3 requirements |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high– 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.1.5-3 |  |  |  |  |  |

Table 6.2B.1.2.5-2a: Maximum output power for EN-DC (non-continuous sub-blocks) for power class 2 for Rel-16 and forward

| DC configuration | Carrier | Power class2 | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_41A_n41A | NR carrier | 26 | +2+TT/-31-TT | UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-21-TT | UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-21-TT | UE indicates PC2 ON E-UTRA carrier of this DC_Configutation | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-21-TT | UE indicates PC3 on E-UTRA carrier of this DC_Configutation | UE meets power class 3 requirements |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.1.2.5-3 |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2B.1.3.5	Test requirements

For test ID 1~6 the maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the UE Power Class and tolerance in Table 6.2B.1.3.5-1.

For test ID 7~15 the maximum output power for the DC configuration, derived in step 3 shall be within the range prescribed by the UE Power Class and tolerance in table 6.2.2.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1 in TS 38.521-1 [8] for E-UTRA carier and NR carrier respectively for Power class 3, and in Table 6.2B.1.3.5-2a for Power class 2.

For test ID 1~6 the maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed by Power Class 3 and tolerance in Table 6.2B.1.3.5-1.

For test ID 7~15 the maximum output power for the DC configuration, derived in step 4 shall be within the range prescribed by Power Class 3 and tolerance in table 6.2.2.5-1 in TS 36.521-1 [10] or table 6.2.1.5-1 in TS 38.521-1 [8] for E-UTRA carier and NR carrier respectively.

Table 6.2B.1.3.5-1: Maximum output power for inter-band EN-DC (two bands), for overlapping UL transmission

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n7A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n40A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n51A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_1A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n5A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_2A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n66A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_2A_n71A |  |  | 23 | +2 +TT/-3-TT |
| DC_2A_n77A | 268 | +2+TT/-3-TT | 23 | +2+TT/-3-TT3 |
| DC_2A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_3A_n7A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n28A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n40A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n41A, DC_3C_n41A | 268 | +2+TT/-3-TT3 | 23 | +2+TT/-3-TT3 |
| DC_3A_n51A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n77A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n78A | 268 | +2+TT/-3-TT3 | 23 | +2 +TT/-3-TT3 |
| DC_3A_n79A DC_3A_n80A_ULSUP-TDM_n79A, DC_3A_n80A_ULSUP-FDM_n79A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_3A_n82A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_5A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_5A_n40A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_5A_n66A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_5A_n77A | 268 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_5A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_7A_n28A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_7A_n51A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_7A_n66A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_7A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n1A |  |  |  | +2 +TT/-3-TT |
| DC_8A_n3A |  |  |  | +2 +TT/-3-TT |
| DC_8A_n20A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n40A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_8A_n41A, DC_8A_n81A_ULSUP-TDM, DC_8A_n81A_ULSUP-FDM |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_8A_n78A DC_8A_n81A_ULSUP-TDM_n78A | 268 | +2+TT/-3-TT | 23 | +2 +TT/-3-TT |
| DC_8A_n79A DC_8A_n81A_ULSUP-TDM_n79A, DC_8A_n81A_ULSUP-FDM_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_11A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_11A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_11A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_12A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_13A_n77A | 268 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_14A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_14A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_18A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_19A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n7A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n8A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n28A DC_20A_n83A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n51A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A, DC_20A_n82A_ULSUP-FDM_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n28A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_21A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_25A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_26A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A n3A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n7A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A n51A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n77A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A, DC_28A_n83A_ULSUP-FDM_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_28A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_30A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_30A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_39A_n41A | 26 | +2/-31 | 23 | +2 +TT/-3-TT3 |
| DC_39A_n78A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_39A_n79A | 26 | +2/-3 | 23 | +2 +TT/-3-TT3 |
| DC_40A_n1A |  |  | 23 | +2 +TT/-3-TT |
| DC_40A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_40A_n77A |  |  | N/A | N/A |
| DC_40A_n78A |  |  | 23 | +2 +TT/-3-TT |
| DC_40A_n79A |  |  | 23 | +2 +TT/-3-TT |
| DC_41A_n77A DC_41C_n77A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_41A_n78A DC_41C_n78A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_41A_n79A DC_41C_n79A | 26 | +2/-31 | 23 | +2 +TT/-3-TT3 |
| DC_42A_n51A |  |  | 23 | +2 +TT/-3-TT |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n5A |  |  | 23 | +2 +TT/-3-TT |
| DC_48A_n66A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n2A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n5A |  |  | 23 | +2 +TT/-3-TT3 |
| DC_66A_n41A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n71A |  |  | 23 | +2 +TT/-3-TT |
| DC_66A_n77A | 268 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |
| DC_66A_n78A, DC_66A_n86A_ULSUP-TDM_n78A, DC_66A_n86A_ULSUP -FDM_n78A |  |  | 23 | +2 +TT/-3-TT |
| NOTE 1: TT applies to output power in each UL carrier with E-UTRA UL transmission not overlapping with NR UL transmission in time, and its value is the same as TT of standalone E-UTRA or NR transmission. For detailed values refer to Table 6.2B.1.3.5-2. NOTE 2: TT applies to overall output power with E-UTRA UL transmission overlapping with NR UL transmission in time, and its value is the maximum TT among all E-UTRA and NR UL carriers. For detailed values refer to Table 6.2B.1.3.5-3. NOTE 3: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. NOTE 5: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 6: Power Class 3 is the default power class unless otherwise stated. NOTE 7: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

Table 6.2B.1.3.5-2: Void

Table 6.2B.1.3.5-2a: Maximum output power for inter-band EN-DC (two bands), for non-overlapping UL transmission for power class 2

| EN-DC configuration | Carrier | Power class 2 (dBm) | Tolerance (dB) | Condition | Comment |
| --- | --- | --- | --- | --- | --- |
| DC_1A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_2A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_3A_n41A | NR carrier | 26 | +2+TT/-32-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-22-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_3A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_8A_n78A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_5A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_13A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_39A_n41A | NR carrier | 26 | +2+TT/-32-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-22-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_39A_n79A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_41A_n79A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 26 | +2+TT/-22-TT | UE indicates PC2 on E-UTRA band | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-22-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| DC_66A_n77A | NR carrier | 26 | +2+TT/-3-TT | Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16) | UE meets power class 2 requirements |
|  |  | 23 | +2+TT/-3-TT | Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR) | UE meets power class 3 requirements |
|  | E-UTRA carrier | 23 | +2+TT/-2-TT | UE indicates PC3 on E-UTRA band | UE meets power class 3 requirements |
| NOTE 1: TT applies to output power in each UL carrier with E-UTRA UL transmission not overlapping with NR UL transmission in time, and its value is the same as TT of standalone E-UTRA or NR transmission. For detailed values refer to Table 6.2B.1.3.5-4. NOTE 2: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 3: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |

Table 6.2B.1.3.5-3: Test Tolerance for UE maximum output power (Overlapping UL transmission)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.1.3.5-4: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | 1.3 dB |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

<< Unchanged sections omitted >>

6.2B.2.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the UE Power Class, the nominal maximum output power and tolerance in Table 6.2B.2.1.5-1 ~ Table 6.2B.2.1.5-6a.

The maximum output power, derived in step 4 shall be within the range prescribed by Power Class 3, the nominal maximum output power and tolerance in  Table 6.2B.2.1.5-1 ~ Table 6.2B.2.1.5-3a.

Table 6.2B.2.1.5-1: UE Power Class 3 test requirements, UE supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 3.0 | 20.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 14.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15 | 11.0 | 12.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 6.0 - TT |
|  | 30, 60 | 6.0 | 17,0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 12.0 - TT |
| 48 (NOTE 3) | 15 | 7.0 | 16.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 11.0 - TT |
|  | 30, 60 | 5.0 | 18.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 13.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-2: UE Power Class 3 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | N/A | 1.0 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 3.5 | 19.5 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 17.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 1 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 2 | 21.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 19.0 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3 | 20.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 18.0 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-3: UE Power Class 3 E-UTRA carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MCG MPRc (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 6.0 | 17.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 12.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 14.0 | 9.0 | 20.0 | 6.0 | 2.5 | 22.5 + TT | 3.0 - TT |
| 48 (NOTE 3) | 15, 30, 60 | 9.0 | 14.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 9.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-3a: UE Power Class 3 NR carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | SCG MPR’c (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 6.0 | 17.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 12.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15 | 14.0 | 9.0 | 20.0 | 6.0 | 2.5 | 22.5 + TT | 3.0 - TT |
|  | 30, 60 | 9.0 | 14.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 9.0- TT |
| 48 (NOTE 3) | 15, 30 | 9.0 | 14.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 9.0- TT |
|  | 60 | 7.0 | 16.0 | 20.0 | 5.0 | 2.5 | 22.5 + TT | 11.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-4: UE Power Class 2 test requirements, UE supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 3.0 | 23.0 | 26.0 | 3.0 | 2.0 | 28.0 + TT | 20.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15 | 11.0 | 15.0 | 26.0 | 6.0 | 2.0 | 28.0 + TT | 9.0 - TT |
|  | 30, 60 | 6.0 | 20.0 | 26.0 | 6.0 | 2.0 | 28.0 + TT | 14.0 - TT |
| 48 (NOTE 3) | 15 | 7.0 | 19.0 | 26.0 | 5.0 | 2.0 | 28.0 + TT | 14.0 - TT |
|  | 30, 60 | 5.0 | 21.0 | 26.0 | 5.0 | 2.0 | 28.0 + TT | 16.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-5: UE Power Class 2 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16))

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | N/A | 1,0 | 25.0 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 23.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-5a: UE Power Class 2 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR))

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | N/A | 1,0 | 25.0 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 23.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 0.5 | 22.5 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 1.0 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 2.0 | 21.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 19.0 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3.0 | 20.0 | 23.0 | 2.5 | 2.0 | 25.0 + TT | 17.5 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-6: UE Power Class 2 E-UTRA carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MCG MPRc (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 6.0 | 20.0 | 23.0 | 2.5 | 2.0 | 25.0 + TT | 17.5 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 14.0 | 12.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 6.0 - TT |
| 48 (NOTE 3) | 15, 30, 60 | 9..0 | 17.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 12.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-6a: UE Power Class 2 NR carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | SCG MPR’c (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 6.0 | 20.0 | 23.0 | 2.5 | 2.0 | 25.0 + TT | 17.5 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15 | 14.0 | 12.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 6.0 - TT |
|  | 30, 60 | 9.0 | 17.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 12.0 - TT |
| 48 (NOTE 3) | 15, 30 | 9.0 | 17.0 | 23.0 | 5.0 | 2.0 | 25.0 + TT | 12.0 - TT |
|  | 60 | 7.0 | 19.0 | 23.0 | 3.5 | 2.0 | 25.0 + TT | 15.5 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.1.5-7: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

<< Unchanged sections omitted >>

6.2B.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.2.2.5-1 ~ Table 6.2B.2.2.5-8a.

Table 6.2B.2.2.5-1: UE Power Class 3 test requirements, UE supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 12.0 | 11.0 | 23.0 | 6.0 | 4.0 | 27.0 + TT | 5.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15,30, 60 | 15.0 | 8.0 | 23.0 | 7.0 | 4.0 | 27.0 + TT | 1.0 - TT |
| 48 (NOTE 3) | 15 | 14.0 | 9.0 | 23.0 | 7.0 | 4.0 | 27.0 + TT | 2.0 - TT |
|  | 30, 60 | 13.0 | 10.0 | 23.0 | 7.0 | 4.0 | 27.0 + TT | 3.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-2: UE Power Class 3 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | 15, 30, 60 | 1.0 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 3.5 | 19.5 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 17.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 1 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 2 | 21.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 19.0 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3 | 20.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 18.0 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-3: UE Power Class 3 E-UTRA carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MCG MPRc (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 15.0 | 8.0 | 20.0 | 6.0 | 2.5 | 22.5 + TT | 2.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 17.0 | 6.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | -1.0 - TT |
| 48 (NOTE 3) | 15, 30, 60 | 17.0 | 6.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | -1.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-3a: UE Power Class 3 NR carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | SCG MPR’c (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 15.0 | 8.0 | 20.0 | 6.0 | 2.5 | 22.5 + TT | 2.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 17.0 | 6.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | -1.0 - TT |
| 48 (NOTE 3) | 15, 30 | 17.0 | 6.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | -1.0 - TT |
|  | 60 | 16.0 | 7.0 | 20.0 | 7.0 | 2.5 | 22.5 + TT | - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-4: Void

Table 6.2B.2.2.5-5: UE Power Class 2 test requirements, UE supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 12.0 | 14.0 | 26.0 | 6.0 | 4.0 | 30.0 + TT | 8.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15,30, 60 | 15.0 | 11.0 | 26.0 | 6.0 | 4.0 | 30.0 + TT | 5.0 - TT |
| 48 (NOTE 3) | 15 | 14.0 | 12.0 | 26.0 | 6.0 | 4.0 | 30.0 + TT | 6.0 - TT |
|  | 30, 60 | 13.0 | 13.0 | 26.0 | 6.0 | 4.0 | 30.0 + TT | 7.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-6: UE Power Class 2 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16))

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | 15, 30, 60 | 1.0 | 25.0 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 23.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3.5 | 22.5 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 20.5 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-6a: UE Power Class 2 test requirements, UE with/without supporting dynamic power sharing, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR))

| Configuration ID | Test SCS (kHz) | MPR (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX L (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (NOTE 1) | 15, 30, 60 | 1.0 | 25.0 | 26.0 | 2.0 | 2.0 | 28.0 + TT | 23.0 - TT |
| 4, 6 (NOTE 2) | 15, 30, 60 | 0.5 | 22.5 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.5 - TT |
| 11, 13 (NOTE 2) | 15, 30, 60 | 1.0 | 22.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 20.0 - TT |
| 18, 20 (NOTE 2) | 15, 30, 60 | 2.0 | 21.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 19.0 - TT |
| 31,33, 38, 40 (NOTE 2) | 15, 30, 60 | 3.0 | 20.0 | 23.0 | 2.0 | 2.0 | 25.0 + TT | 17.5 - TT |
| NOTE 1: Test configuration IDs without transmission overlap with E-UTRA allocation, MPR requirements in TS 36.101 [4] apply. NOTE 2: Test configuration IDs without transmission overlap with NR allocation, MPR requirements in TS 38.101-1 [2] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-7: UE Power Class 2 E-UTRA carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | MCG MPRc (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 15.0 | 11.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 5.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 17.0 | 9.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 3.0 - TT |
| 48 (NOTE 3) | 15, 30, 60 | 17.0 | 9.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 3.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-7a: UE Power Class 2 NR carrier test requirements, UE not supporting dynamic power sharing, E-UTRA UL transmission overlapping with NR UL transmission

| Configuration ID | Test SCS (kHz) | SCG MPR’c (dB) | PCMAX, L (dBm) | PCMAX, H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (NOTE 1) | 15, 30, 60 | 15.0 | 11.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 5.0 - TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (NOTE 2) | 15, 30, 60 | 17.0 | 9.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 3.0 – TT |
| 48 (NOTE 3) | 15, 30 | 17.0 | 9.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 3.0 – TT |
|  | 60 | 16.0 | 10.0 | 23.0 | 6.0 | 2.0 | 25.0 + TT | 4.0 - TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. |  |  |  |  |  |  |  |  |

Table 6.2B.2.2.5-8: Void

Table 6.2B.2.2.5-8a: Void

Table 6.2B.2.2.5-9: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

<< Unchanged sections omitted >>

6.2B.2.3.5	Test requirement

For PC3 UE, same test requirement as in clause 6.2.2.5 in TS 38.521-1 [8].

For PC2 UE (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)), same test requirement for PC2 UE as in clause 6.2.2.5 in TS 38.521-1 [8]. For PC2 UE (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR)), same test requirement for PC3 UE as in clause 6.2.2.5 in TS 38.521-1 [8].

<< Unchanged sections omitted >>

6.2B.3.1.5.2	Test requirement for network signalled value "NS_04"

Table 6.2B.3.1.5.2-1: UE Power Class 3 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 3 | 20 | 23 | 6 | 2 | 25+TT | 14-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 9 | 14 | 23 | 6 | 2 | 25+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 11 | 12 | 23 | 6 | 2 | 25+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 6 | 17 | 23 | 5 | 2 | 25+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 11 | 12 | 23 | 6 | 2 | 25+TT | 6-TT |
| 48 (Note 3, 4) | 15 | 7 | 16 | 23 | 5 | 2 | 25+TT | 11-TT |
| 48 (Note 3, 4) | 30, 60 | 12 | 11 | 23 | 6 | 2 | 25+TT | 5-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 7 | 16 | 23 | 5 | 2 | 25+TT | 11-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-2: UE Power Class 3 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission not overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | MPR (dB) | A-MPR (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (Note 1) | 15, 30, 60 | 0 | 3 | 20 | 23 | 4 | 2 | 25+TT | 16-TT |
| 4 (Note 2) | 15, 30, 60 | 0.5 | 0 | 22.5 | 23 | 2 | 2 | 25+TT | 20.5-TT |
| 6 (Note 2) | 15, 30, 60 | 0 | 3.5 | 19.5 | 23 | 3.5 | 2 | 25+TT | 16-TT |
| 11 (Note 2) | 15, 30, 60 | 1 | 0 | 22 | 23 | 2 | 2 | 25+TT | 20-TT |
| 13 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 (Note 2) | 15, 30, 60 | 2 | 0 | 21 | 23 | 2 | 2 | 25+TT | 19-TT |
| 20 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5-TT |
| 33 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12-TT |
| 38 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5-TT |
| 40 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12-TT |
| NOTE 1: Test configuration IDs without transmission overlap, 1RB E-UTRA allocation, A-MPR requirements in TS 36.101 [5] apply NOTE 2: Test configuration IDs without transmission overlap, 1RB NR allocation, A-MPR requirements in TS 38.101-1 [2] apply. NOTE 3: Void. NOTE 4: Void. NOTE 5: Void. NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-3: UE Power Class 3 test requirements for NS_04, not supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 6 | 17 | 23 | 5 | 2 | 25+TT | 12-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 12 | 11 | 23 | 6 | 2 | 25+TT | 5-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 14 | 9 | 23 | 7 | 2 | 25+TT | 2-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 9 | 14 | 23 | 6 | 2 | 25+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 14 | 9 | 23 | 7 | 2 | 25+TT | 2-TT |
| 48 (Note 3, 4) | 15 | 10 | 13 | 23 | 6 | 2 | 25+TT | 7-TT |
| 48 (Note 3, 4) | 30, 60 | 15 | 8 | 23 | 7 | 2 | 25+TT | 1-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 10 | 13 | 23 | 6 | 2 | 25+TT | 7-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-4: UE Power Class 2 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 3 | 23 | 26 | 3 | 2 | 28+TT | 20-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 9 | 17 | 26 | 5 | 2 | 28+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 11 | 15 | 26 | 6 | 2 | 28+TT | 9-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 6 | 20 | 26 | 6 | 2 | 28+TT | 14-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 11 | 15 | 26 | 6 | 2 | 28+TT | 9-TT |
| 48 (Note 3, 4) | 15 | 7 | 19 | 26 | 5 | 2 | 28+TT | 14-TT |
| 48 (Note 3, 4) | 30, 60 | 12 | 14 | 26 | 6 | 2 | 28+TT | 8-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 7 | 19 | 26 | 5 | 2 | 28+TT | 14-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-5: UE Power Class 2 test requirements for NS_04, supporting dynamic power sharing and E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16))

| Test ID | Test SCS (kHz) | MPR (dB) | A-MPR (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (Note 1) | 15, 30, 60 | 0 | 3 | 23 | 26 | 2 | 2 | 28+TT | 21-TT |
| 4 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 6 (Note 2) | 15, 30, 60 | 0 | 5.5 | 20.5 | 26 | 2.5 | 2 | 28+TT | 18-TT |
| 11 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 13 (Note 2) | 15, 30, 60 | 0 | 6 | 20 | 26 | 2.5 | 2 | 28+TT | 17.5-TT |
| 18 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 20 (Note 2) | 15, 30, 60 | 0 | 6 | 20 | 26 | 2.5 | 2 | 28+TT | 17.5-TT |
| 31 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 33 (Note 2) | 15, 30, 60 | 0 | 7.5 | 18.5 | 26 | 4 | 2 | 28+TT | 14.5-TT |
| 38 (Note 2) | 15, 30, 60 | 3.5 | 0 | 22.5 | 26 | 2 | 2 | 28+TT | 20.5-TT |
| 40 (Note 2) | 15, 30, 60 | 0 | 7.5 | 18.5 | 26 | 4 | 2 | 28+TT | 14.5-TT |
| NOTE 1: Test configuration IDs without transmission overlap, 1RB E-UTRA allocation, A-MPR requirements in TS 36.101 [5] apply. NOTE 2: Test configuration IDs without transmission overlap, 1RB NR allocation, A-MPR requirements in TS 38.101-1 [2] apply. NOTE 3: Void. NOTE 4: Void. NOTE 5: Void. NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-5a: UE Power Class 2 test requirements for NS_04, E-UTRA UL transmission not overlapping with NR UL transmission (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR))

| Test ID | Test SCS (kHz) | MPR (dB) | A-MPR (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 (Note 1) | 15, 30, 60 | 0 | 3 | 23 | 26 | 2 | 2 | 28+TT | 21+TT |
| 4 (Note 2) | 15, 30, 60 | 0.5 | 0 | 22.5 | 23 | 2 | 2 | 25+TT | 20.5+TT |
| 6 (Note 2) | 15, 30, 60 | 0 | 3.5 | 19.5 | 23 | 3.5 | 2 | 25+TT | 16+TT |
| 11 (Note 2) | 15, 30, 60 | 1 | 0 | 22 | 23 | 2 | 2 | 25+TT | 20+TT |
| 13 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5+TT |
| 18 (Note 2) | 15, 30, 60 | 2 | 0 | 21 | 23 | 2 | 2 | 25+TT | 19+TT |
| 20 (Note 2) | 15, 30, 60 | 0 | 4 | 19 | 23 | 3.5 | 2 | 25+TT | 15.5+TT |
| 31 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5+TT |
| 33 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12.5+TT |
| 38 (Note 2) | 15, 30, 60 | 3 | 0 | 20 | 23 | 2.5 | 2 | 25+TT | 17.5+TT |
| 40 (Note 2) | 15, 30, 60 | 0 | 5.5 | 17.5 | 23 | 5 | 2 | 25+TT | 12.5+TT |
| NOTE 1: Test configuration IDs without transmission overlap, 1RB E-UTRA allocation, A-MPR requirements in TS 36.101 [5] apply. NOTE 2: Test configuration IDs without transmission overlap, 1RB NR allocation, A-MPR requirements in TS 38.101- [2] apply. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.1.5.2-6: UE Power Class 2 test requirements for NS_04, not supporting dynamic power sharing and E-UTRA UL transmission overlapping with NR UL transmission

| Test ID | Test SCS (kHz) | A-MPRtot (dB) | PEN-DC, tot_L (dBm) | PEN-DC, tot_H (dBm) | TLOW (PCMAX_L) (dB) | THIGH (PCMAX_H) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 4) | 15, 30, 60 | 6 | 20 | 26 | 6 | 2 | 28+TT | 14-TT |
| 1, 8, 15, 22, 25, 28, 35, 42, 45 (Note 1, 5) | 15, 30, 60 | 12 | 14 | 26 | 6 | 2 | 28+TT | 8-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4, 5) | 15 | 14 | 12 | 26 | 6 | 2 | 28+TT | 6-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 4) | 30, 60 | 9 | 17 | 26 | 5 | 2 | 28+TT | 12-TT |
| 2, 5, 9, 12, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, 47 (Note 2, 5) | 30, 60 | 14 | 12 | 26 | 6 | 2 | 28+TT | 6-TT |
| 48 (Note 3, 4) | 15 | 10 | 16 | 26 | 5 | 2 | 28+TT | 11-TT |
| 48 (Note 3, 4) | 30, 60 | 15 | 11 | 26 | 6 | 2 | 28+TT | 5-TT |
| 48 (Note 3, 5) | 15, 30, 60 | 10 | 16 | 26 | 5 | 2 | 28+TT | 11-TT |
| NOTE 1: Test configuration IDs with transmission overlap with full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 2: Test configuration IDs with transmission overlap with 1RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 3: Test configuration IDs with transmission overlap with edge full RB allocation, requirements in TS 38.101-3 [4] apply. NOTE 4: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 5: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 6: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.1.5-1. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2B.3.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.3.2.5-0 to Table 6.2B.3.2.5-2a. The allowed A-MPR values specified in Table 6.2B.3.2.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.1.1.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2B.1.1.3-1 apply.

Table 6.2B.3.2.5-0: Test Tolerance for UE maximum output power (LTE, NR TX separately)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| LTE | BW ≤ 20MHz | 0.7 | 1.0 | 1.3 |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 |

Table 6.2B.3.2.5-1: UE Power Class 3 test requirements for network signalled value "NS_04"

| Test ID | Modulation | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | A-MPREN-DC (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 1 | 0 | 2 | 0 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 2 | 0 | 2 | 0 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 25+TT | 6-TT (4.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 1 | 0 | - | Note 5 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 2 | 0 | - | Note 5 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 25+TT | 6-TT (4.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 3 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 4 | 0 | 1 | Note 6 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 25+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 3 | 0 | - | Note 5 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 4 | 0 | - | Note 5 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 25+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 2 | 0 | - | Note 5 | 11 | 11 | 0 (1.5) | 9 (7.5) | 6 (6) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 | E-UTRA Note 1 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37 and 41 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 25+TT | 3-TT (0.5-TT) |
| 4 | NR | 0 | 0.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 25+TT | 20.5-TT (19-TT) |
| 6 | NR | 0 | 0 | Note 9 | 3.5 | 3.5 | 0 (1.5) | 19.5 (18) | 3.5 (4) | 2 (3.5) | 25+TT | 16-TT (14-TT) |
| 11 | NR | 0 | 1 | Note 9 | 0 | 0 | 0 (1.5) | 22 (20.5) | 2 (2.5) | 2 (3.5) | 25+TT | 20-TT (18-TT) |
| 13 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 18 | NR | 0 | 2 | Note 9 | 0 | 0 | 0 (1.5) | 21 (19.5) | 2 (3.5) | 2 (3.5) | 25+TT | 19-TT (16-TT) |
| 20 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 31 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 33 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| 38 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 40 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| NOTE 1: When FIM3,low_block,low ≥ 2490.5 MHz (Case A) NOTE 2: When FIM3,low_block,low < 2490.5 MHz (Case B) NOTE 3: When NR SCS = 15kHz. NOTE 4: When NR SCS = 30 kHz or 60 kHz. NOTE 5: NR A-MPR values for NS_04 are defined in Table 6.2.3.3.2-1. NOTE 6: E-UTRA A-MPR= 3 dB for 1 RB and fc < 2517.5 MHz, otherwise 0 dB. NOTE 7: ΔTC,c = 1.5 dB for transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, otherwise 0 dB. NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.2.5-0. NOTE 9: Apply for UE supporting dynamic power sharing. NR A-MPR values for NS_04 are defined in TS 38.101-1 [2]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.2.5-2: UE Power Class 2 test requirements for network signalled value "NS_04" (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16))

| Test ID | Modulation | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | A-MPREN-DC (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 1 | 3 | 2 | 0 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 2 | 3 | 2 | 0 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 1 | 3 | - | Note 5 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 3 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 4 | 3 | 1 | Note 6 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 2 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 3 | 3 | - | Note 5 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 4 | 3 | - | Note 5 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 9 (7.5) | 6 (6) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 | E-UTRA Note 1 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37 and 41 | E-UTRA Note 2 | 0 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 4 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 6 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 20.5 (19) | 2.5 (3.5) | 2 (3.5) | 28+TT | 18-TT (15.5-TT) |
| 11 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 13 | NR | 0 | 0 | Note 9 | 6 | 6 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 28+TT | 17.5-TT (14.5-TT) |
| 18 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 20 | NR | 0 | 0 | Note 9 | 6 | 6 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 28+TT | 17.5-TT (14.5-TT) |
| 31 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 33 | NR | 0 | 0 | Note 9 | 7.5 | 7.5 | 0 (1.5) | 18.5 (17) | 4 (5) | 2 (3.5) | 28+TT | 14.5-TT (12-TT) |
| 38 | NR | 0 | 3.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 28+TT | 20.5-TT (19-TT) |
| 40 | NR | 0 | 0 | Note 9 | 7.5 | 7.5 | 0 (1.5) | 18.5 (17) | 4 (5) | 2 (3.5) | 28+TT | 14.5-TT (12-TT) |
| NOTE 1: When FIM3,low_block,low ≥ 2490.5 MHz (Case A). NOTE 2: When FIM3,low_block,low < 2490.5 MHz (Case B). NOTE 3: When NR SCS = 15kHz. NOTE 4: When NR SCS = 30 kHz or 60 kHz. NOTE 5: NR A-MPR values for NS_04 are defined in Table 6.2.3.3.2-1. NOTE 6: E-UTRA A-MPR= 3 dB for 1 RB and fc < 2517.5 MHz, otherwise 0 dB. NOTE 7: ΔTC,c = 1.5 dB for transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, otherwise 0 dB. NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.2.5-0. NOTE 9: Apply for UE supporting dynamic power sharing. NR A-MPR values for NS_04 are defined in TS 38.101-1 [24]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.2.5-2a: UE Power Class 2 test requirements for network signalled value "NS_04" (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR))

| Test ID | Modulation | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | A-MPREN-DC (dB) | A-MPRc (dB) | ΔTC,c (dB) Note 7 | PCMAX,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 1 | 3 | 2 | 0 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | E-UTRA Note 2 | 3 | 2 | 0 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 1 | 3 | - | Note 5 | 6 | 6 | 0 (1.5) | 17 (15.5) | 5 (5) | 2 (3.5) | 28+TT | 12-TT (10.5-TT) |
| 1, 8, 15, 22, 25, 28, 35, 42 and 45 | NR, Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 12 (10.5) | 6 (6) | 2 (3.5) | 28+TT | 6-TT (4.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 3 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 1, 4 | 3 | 1 | Note 6 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | E-UTRA Note 2 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 3 | 3 | - | Note 5 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 1, 4 | 3 | - | Note 5 | 9 | 9 | 0 (1.5) | 14 (12.5) | 5 (6) | 2 (3.5) | 28+TT | 9-TT (6.5-TT) |
| 2, 5, 9, 16, 19, 23, 24, 26, 27, 29, 32, 36, 39, 43, 44, 46, and 47 | NR Note 2 | 3 | - | Note 5 | 11 | 11 | 0 (1.5) | 9 (7.5) | 6 (6) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37, 41 | E-UTRA Note 1 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 3, 7, 10, 14, 17, 21, 30, 34, 37 and 41 | E-UTRA Note 2 | 3 | 1 | Note 6 | 14 | 14 | 0 (1.5) | 9 (7.5) | 6 (7) | 2 (3.5) | 28+TT | 3-TT (0.5-TT) |
| 4 | NR | 0 | 0.5 | Note 9 | 0 | 0 | 0 (1.5) | 22.5 (21) | 2 (2) | 2 (3.5) | 25+TT | 20.5-TT (19-TT) |
| 6 | NR | 0 | 0 | Note 9 | 3.5 | 3.5 | 0 (1.5) | 19.5 (18) | 3.5 (4) | 2 (3.5) | 25+TT | 16-TT (14-TT) |
| 11 | NR | 0 | 1 | Note 9 | 0 | 0 | 0 (1.5) | 22 (20.5) | 2 (2.5) | 2 (3.5) | 25+TT | 20-TT (18-TT) |
| 13 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 18 | NR | 0 | 2 | Note 9 | 0 | 0 | 0 (1.5) | 21 (19.5) | 2 (3.5) | 2 (3.5) | 25+TT | 19-TT (16-TT) |
| 20 | NR | 0 | 0 | Note 9 | 4 | 4 | 0 (1.5) | 19 (17.5) | 3.5 (5) | 2 (3.5) | 25+TT | 15.5-TT (12.5-TT) |
| 31 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 33 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| 38 | NR | 0 | 3 | Note 9 | 0 | 0 | 0 (1.5) | 20 (18.5) | 2.5 (4) | 2 (3.5) | 25+TT | 17.5-TT (14.5-TT) |
| 40 | NR | 0 | 0 | Note 9 | 5.5 | 5.5 | 0 (1.5) | 17.5 (16) | 5 (5) | 2 (3.5) | 25+TT | 12-TT (11-TT) |
| NOTE 1: When FIM3,low_block,low ≥ 2490.5 MHz (Case A). NOTE 2: When FIM3,low_block,low < 2490.5 MHz (Case B). NOTE 3: When NR SCS = 15kHz. NOTE 4: When NR SCS = 30 kHz or 60 kHz. NOTE 5: NR A-MPR values for NS_04 are defined in Table 6.2.3.3.2-1. NOTE 6: E-UTRA A-MPR= 3 dB for 1 RB and fc < 2517.5 MHz, otherwise 0 dB. NOTE 7: ΔTC,c = 1.5 dB for transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, otherwise 0 dB. NOTE 8: TT for each frequency and channel bandwidth is specified in Table 6.2B.3.2.5-0. NOTE 9: Apply for UE supporting dynamic power sharing. NR A-MPR values for NS_04 are defined in TS 38.101-1 [2]. |  |  |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2B.3.3.5	Test requirement

For PC3 UE, same test requirement as in clause 6.2.2.5 in TS 38.521-1 [8].

For PC2 UE (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)), same test requirement for PC2 UE as in clause 6.2.3.5 in TS 38.521-1 [8]. For PC2 UE (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16) or UE reporting (PC3 by PPowerClass,NR)), same test requirement for PC3 UE as in clause 6.2.3.5 in TS 38.521-1 [8].

<< Unchanged sections omitted >>

6.2B.4.1.1.5	Test requirement

For UE supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.1.5-1.

Table 6.2B.4.1.1.5-1: PCMAX configured UE output power for UE supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | Not measured | Not measured | -10dBm ± (7+TT) |
| Test ID 1b | Not measured | Not measured | 10dBm ± (7+TT) |
| Test ID 1c | Not measured | Not measured | 15 ± (6+TT) |
| Test ID 1d | Not measured | Not measured | 18 ± (5+TT) |
| Test ID 1e | Not measured | 18 + (5+TT) / 11 – (6+TT) for PC3 UE | 21.54dBm + (3+TT) / 19.64 - (5+TT) for PC3 UE |
|  |  | 21 + (3+TT) / 14 - (6+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE repoting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) | 24 + (2+TT) / 21.8 - (5+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) |
|  |  | 21 + (3+TT) / 11 - (6+TT) for PC2 UE (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16), or UE reporting (PC3 by PPowerClass,NR)) | 24 + (2+TT) / 21.4 - (5+TT) for PC2 UE (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16), or UE reporting (PC3 by PPowerClass,NR)) |
| Test ID 1f | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] | N/A | Maximum output power with reduction as defined in Table 6.2.3-1 of TS 36.521-1 [10] |
| Test ID 2a | -10dBm ± (7+TT) | N/A- | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A- | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A- | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.1.5-3 |  |  |  |

For UE not supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.1.5-2.

Table 6.2B.4.1.1.5-2: PCMAX configured UE output power for UE not supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) | Not measured |
| Test ID 1b | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) | Not measured |
| Test ID 1c | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) | Not measured |
| Test ID 1d | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] | N/A | Not measured |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.1.5-3 |  |  |  |

Table 6.2B.4.1.1.5-3: Test Tolerance (UE configured UE output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

<< Unchanged sections omitted >>

6.2B.4.1.2.5	Test requirement

For UE supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.2.5-1.

Table 6.2B.4.1.2.5-1: PCMAX configured UE output power for UE supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | Not measured | Not measured | -10dBm ± (7+TT) |
| Test ID 1b | Not measured | Not measured | 10dBm ± (7+TT) |
| Test ID 1c | Not measured | 10 + (7+TT) / 2 – (7+TT) for PC3 UE | 13.0 + (6+TT) / 10.64 – (7+TT) for PC3 UE |
|  |  | 13 + (6+TT) / 5 - (7+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) | 15.5 + (6+TT) / 12.8 - (6+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) |
|  |  | 13 + (6+TT) / 2 - (7+TT) for PC2 UE (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16), or UE reporting (PC3 by PPowerClass,NR)) | 15.5 + (6+TT) / 12.4 - (6+TT) for PC2 UE (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16), or UE reporting (PC3 by PPowerClass,NR)) |
| Test ID 1d | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] | N/A | Maximum output power with reduction as defined in Table 6.2.3-1 of TS 36.521-1 [10] |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.1.5-3 |  |  |  |

For UE not supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.1.5-2.

Table 6.2B.4.1.2.5-2: PCMAX configured UE output power for UE not supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) | Not measured |
| Test ID 1b | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] | N/A | Not measured |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.1.5-3 |  |  |  |

Table 6.2B.4.1.2.5-3: Test Tolerance for UE configured UE output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | 1.3 dB |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.4.1.2.5-4: Test Tolerance for UE configured UE output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

<< Unchanged sections omitted >>

6.2B.4.1.3.5	Test requirement

For UE supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.3.5-1.

Table 6.2B.4.1.3.5-1: PCMAX configured UE output power for UE supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | Not measured | Not measured | -10dBm ± (7+TT) |
| Test ID 1b | Not measured | Not measured | 15dBm ± (6+TT) |
| Test ID 1c | Not measured | Not measured | 17 ± (5+TT) |
| Test ID 1d | Not measured | Not measured | 20 + (4+TT)/ - (6+TT) |
| Test ID 1e | Not measured | Not measured | 21 + (3+TT)/ - (5+TT) |
| Test ID 1f | Not measured | Not measured | 22+ (2+TT)/ - (5+TT) |
| Test ID 1g | Not measured | 23 + (2+TT) / 17 – (5+TT) for PC3 UE | 23 + (2+TT) / 21.8 - (5+TT) for PC3 UE |
|  |  | 26 + (2+TT) / 20 - (6+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) | 26 + (2+TT) / 24.8 - (3+TT) for PC2 UE (Rel-15 UE indicates PC2 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC2 or Not present by powerClassNRPart-r16)) |
|  |  | 23 + (2+TT) / 17 - (5+TT) for PC2 UE (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16), or UE reporting (PC3 by PPowerClass,NR)) | 26 + (2+TT) / 24.0 - (3+TT) for PC2 UE (Rel-15 UE indicates PC3 on NR band or Rel-16 and forward UE reporting (PC2 by PPowerClass,NR, and PC3 by powerClassNRPart-r16), or UE reporting (PC3 by PPowerClass,NR)) |
| Test ID 1h | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] | N/A | Maximum output power with reduction as defined in Table 6.2.3-1 of TS 36.521-1 [10] |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.3.5-3 and Table 6.2B.4.1.3.5-4. |  |  |  |

For UE not supporting DPS, the output power measured shall not exceed the values specified in Table 6.2B.4.1.3.5-2.

Table 6.2B.4.1.3.5-2: PCMAX configured UE output power for UE not supporting DPS

|  | E-UTRA component carrier | NR component carrier | Total power measured over E-UTRA and NR component carriers |
| --- | --- | --- | --- |
| Test ID 1a | -10 dBm ± (7+TT) | -10 dBm ± (7+TT) | Not measured |
| Test ID 1b | 10 dBm ± (6+TT) | 10 dBm ± (6+TT) | Not measured |
| Test ID 1c | 15 dBm ± (5+TT) | 15 dBm ± (5+TT) | Not measured |
| Test ID 1d | Maximum output power with reduction as defined in Table 6.2.3.5-1 of TS 36.521-1 [10] | N/A | Not measured |
| Test ID 2a | -10dBm ± (7+TT) | N/A | Not measured |
| Test ID 2b | 10dBm ± (6+TT) | N/A | Not measured |
| Test ID 2c | 15dBm ± (5+TT) | N/A | Not measured |
| Test ID 3a | N/A | -10dBm ± (7+TT) | Not measured |
| Test ID 3b | N/A | 10dBm ± (6+TT) | Not measured |
| Test ID 3c | N/A | 15dBm ± (5+TT) | Not measured |
| NOTE 1: In addition NOTE 2 in Table 6.2.2-1 in TS 36.101 [5] and/or NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] shall apply to the tolerances. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2B.4.1.3.5-3 and Table 6.2B.4.1.3.5-4. |  |  |  |

Table 6.2B.4.1.3.5-3: Test Tolerance for UE maximum output power (Separate measurements over E-UTRA and NR CCs)

| Uplink TX |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6GHz |
| --- | --- | --- | --- | --- |
| E-UTRA | BW ≤ 20MHz | 0.7 dB | 1.0 dB | 1.3 dB |
| NR | BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
|  | 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2B.4.1.3.5-4: Test Tolerance for UE maximum output power (Combined measurements of E-UTRA and NR CCs)

| TT for overall output power |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NR |  |  |  |  |  |  |  |  |
|  |  |  | BW ≤ 20MHz |  |  | 20 MHz < BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| E-UTRA | BW ≤ 20MHz | f ≤ 3.0GHz | 0.7 dB | 1.0 dB | 1.0 dB | 0.7 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band EN-DC configuration, ΔTIB,c in 6.2B.4.2 applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

<< End of changes >>