3GPP TSG-RAN5 Meeting #95-e	R5-222385r3

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2515 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G Rel-15: Test Models updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. SRB0 shall be added on the E-UTRA cell to the NE-DC test model. 2. A new PIXIT is required to provide the secondary NE-DC band combination. 3. The execution guidelines need to be updated to take into account test prose updates and the new delivered test cases. 4. In clause 7.3.5.3.4 Sequence of intra-NR inter-cell CA handover: - At step 4, if target PCell is same as source SCell: Current Source SCell is configured as Target PCell by converting it from SCell to PCell. But this step has not taken care of anything about associated PCell. - At Step 17.2, Source PCell to become normal Cell is proposed to happen on the condition only if target PCell is not same as source SCell, otherwise newly Proposed step 4.1 and Step 17.2 will do the same thing. - At Step 18, if source SCell is neither target PCell nor target SCell, it is enough only to remove source SCell from the SCell list in the source PCell. 5. The EN-DC with NR CA PDCP duplication test model needs to be documented for test cases 8.2.6.1.1.x. 6. The NR-DC with NR CA PDCP duplication test model needs to be documented for test cases 8.2.6.1.2.x. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Updated the NE-DC test model to include SRB0 on the E-UTRA cell. 2. Added the PIXIT px_NEDC_SecondaryBandCombination. 3. Updated clause 11. 4. In clause 7.3.5.3.4 Sequence of intra-NR inter-cell CA handover: - It is proposed to have below steps as part of Step 4: Step 4.1 : Source PCell to become normal cell, Step 4.2 : Source SCell to become normal cell. - Step 17.2 is proposed to happen only if target PCell is not same as source SCell. - If source SCell is neither target PCell nor target SCell, reconfiguration of source SCell to normal cell is removed. 5. Added clause 5.1.2.1.1 and 5.1.2.1.2. 6. Added clause 5.2.2.2.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1&5&6.Test model will remain incomplete. 2. The secondary band combination cannot be tested for NE-DC. 3. Execution guidelines will remain incomplete. 4. A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1, 5.2.1.5A, 5.2.2.2.5 (new clause), 7.3.5.3.4, 9.2, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: Added change 4 (merged R&S CR R5-222856, based on R5w220113 endorsed at RAN5 TTCN Workshop#57). Revision 2: Added changes 5 and 6. Revision 3: corrected arrow direction in figures 5.1.2.1.2-1 and 5.2.2.2.5-1 |  |  |  |  |  |  |  |  |

#### 5.1.2.1	PDCP

#### 5.1.2.1.1	Single NR carrier

Figure 5.1.2.1.1-1: Test model for EN-DC PDCP testing (MCG and SCG)

Figure 5.1.2.1.1-2: Test model for EN-DC PDCP testing (MCG and split DRB)

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer. On UE side Ciphering is enabled as null algorithm and ROHC is not configured.

Test Loop Mode can be active on SCG DRB as shown in Figure 5.1.2.1.1-1 or active split DRB as shown in Figure 5.1.2.1.1-2.

NOTE: Test loop Mode activation on MCG DRB with NR-PDCP is not considered in Figure 5.1.2.1.1-2.

On the SS NR, Layer 1, MAC and RLC are configured in the normal operation. The PDCP is configured in a special mode, where SS does not add any PCDP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

On the SS side the EUTRA MCG layer 1, MAC, RLC and MCG DRB’s PDCP are configured in normal operation. They shall perform their functions. In case of split DRB, MCG leg’s PDCP layer is configured as Proxy mode, TTCN shall configure EUTRA for EN-DC PDCP testing only when a Test Loop Mode is active on a split DRB.

The SS shall route DL PDCP PDUs from TTCN to PCell and/or PSCell and SS shall indicate that the UL PDCP PDU is received from PCell or PSCell.

Duplication function:

-	UL SS shall include routing information for each UL PDCP PDU.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.1.2.1.2	NR carrier aggregation

Figure 5.1.2.1.2-1: Test model for EN-DC with NR CA duplication PDCP testing (MCG and SCG)

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer. On UE side Ciphering is enabled as null algorithm and ROHC is not configured. Test Loop Mode is activated on SCG DRB as shown in Figure 5.1.2.1.2-1.

On the SS side the EUTRA MCG layer 1, MAC, RLC and MCG DRB’s PDCP are configured in normal operation. They shall perform their functions.

On the NR side, the SS configures in the PSCell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The SS configures DRB j and DRB j+1 on the PSCell, every DRB is connected to an RLC entity. The RLC entity configured on DRB j is linked to the MAC entity on the PSCell, the RLC entity configured on DRB j+1 is linked to the MAC entity on the SCell.

The SS configures SCell with Layer 1, MAC in normal operation. The MAC entity of the SCell is linked to the RLC entity on DRB j+1 of the PSCell.

The NR data routing between the RLC layer of PSCell and the lower layers of either PSCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

<End of modified section>

<Start of next modified section>

#### 5.2.1.5A	NR-E-UTRA dual connectivity

Figure 5.2.1.5A-1: Test model for NR/5GC NE-DC Layer3: MCG and SCG

Figure 5.2.1.5A-2: Test model for NR/5GC NE-DC Layer3 testing: MCG and split DRB, E-UTRA terminated

Figure 5.2.1.5A-3: Test model for NR/5GC NE-DC Layer3 testing: MCG and split DRB, NR terminated

Figure 5.2.1.5A-4: Test model for NR/5GC NE-DC Layer3 testing: Split SRB(s), DRBs removed for clarity

The NR/5GC Layer3 NE-DC test model builds on top of the NR/5GC Layer3 test model, with the differences specified hereafter:

-	SRBs 0, 1 and 2 are configured in NR.

-	Only SRB0 is configured in E-UTRA and is connected directly to the SRB port in the E-UTRA RRC/NAS emulator.

-	On the SS side E-UTRA L1, MAC and RLC are configured in normal way and shall perform all their functions.

-	Only NR PDCP is configured in both NR and E-UTRA (E-UTRA PDCP is not used).  This can be configured in split for one or more SRBs or DRBs. In this case NR PDCP is configured in one cell and a proxy PDCP will be configured in the other cell. The SS shall route data to/from either cell, via the routing information provided.

-	SDAP can be configured in both NR and E-UTRA.

<End of modified section>

<Start of next modified section>

##### 5.2.2.2.3	NR-E-UTRA dual connectivity

Figure 5.2.2.2.3-1: Test model for NE-DC PDCP testing (split DRB)

The UE is configured in Test Loop Mode, to loop back the user domain data above NR PDCP layer.

On UE side Ciphering is enabled as null algorithm and ROHC is not configured.

Test Loop Mode can be active on split DRB as shown in Figure 5.2.2.2.3-1.

On the SS NR, SRBs 0,1 and 2 are configured as per Layer3 test model. The Layer1, MAC and RLC of the MCG bearer are configured in normal operation. The PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

On the SS E-UTRA the Layer1, MAC and RLC of the SCG bearer are configured in normal operation. The PDCP layer of the SCG leg is configured as Proxy mode, TTCN shall configure E-UTRA for NE-DC PDCP testing only when a Test Loop Mode is active on a split DRB.

The SS shall route DL PDCP PDUs from TTCN to PCell and/or PSCell and SS shall indicate that the UL PDCP PDU is received from PCell or PSCell.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.2.2.2.5	NR-NR dual connectivity with NR carrier aggregation

Figure 5.2.2.2.5-1: Test model for NR/5GC NR-DC with NR CA duplication in PDCP testing

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer.

On UE side Ciphering is enabled as null algorithm and ROHC is not configured.

The SS configures in the PCell, Layer 1, MAC and RLC in the normal operation.

On the NR side, the SS configures in the PSCell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The SS configures DRB j and DRB j+1 on the PSCell, every DRB is connected to an RLC entity. The RLC entity configured on DRB j is linked to the MAC entity on the PSCell, the RLC entity configured on DRB j+1 is linked to the MAC entity on the SCell.

The SS configure SCell with Layer 1, MAC in normal operation. The MAC entity of the SCell is linked to the RLC entity on DRB j+1 of the PSCell.

The NR data routing between the RLC layer of PSCell and the lower layers of either PSCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

<End of modified section>

<Start of next modified section>

##### 7.3.5.3.4	Sequence of intra-NR inter-cell CA handover

This procedure is applicable when there are more than one CC before and after the handover.

The intra-NR inter-cell CA handover is done with activation time, i.e. the timing information for configuration of the SS and sending of the RRCReconfiguration is explicit. Time ‘T’ is set to 700 ms in advance of the handover, time T1 = T + 10 ms and time T2 = T +20 ms.

At Time T, steps 1-3:

- Source PCell: Stop periodic TA.

NOTE 1:	Unless explicitly specified UL grant configuration keeps configured as per default at the source cell.

2.	Target PCell: Configure target PCell for no RACH response transmission.

3.	Source PCell: Schedule the transmission of RRCReconfiguration message to UE requesting Handover to target PCell and SCell.

At time T1 if target PCell is the same as source SCell, else at time "Now", steps 4-6:

4.	If target PCell is same as source SCell

4.1	Source PCell: Configure from PCell to normal cell: remove source SCell from the SCell list in the source PCell.

4.2	Target PCell: Configure from SCell to PCell.

5.	Target PCell: Configuration of DRBs

6.	Transfer of the PDCP Count for AM DRBs and SRBs (if applied) from source to target PCell:

a)	Source PCell: Get PDCP COUNT.

b)	Target PCell: Set PDCP COUNT.

NOTE 2:	No further sending/receiving of DRB data before the HO is done.

NOTE 3:	For AM DRBs the PDCP count is maintained, for UM DRBs the PDCP count is reset. For SRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content.

At time T2, steps 7-12:

7.	Target PCell:	Inform the SS about the HO and about the source PCell id.

8.	Target PCell:	Configure RACH procedure either dedicated or C-RNTI based.

NOTE 4:	The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI.

9.	Target PCell:	Activate security.

10.	Target PCell:	Configure UL grant configuration ("OnSR", periodic TA is not started).

11. Target PCell:	Configure Target PCell as PCell.

NOTE 5:	The FollowOnFlag is set to true in the ASP reconfiguring PCell.

12.	Target SCell:	Configure Target SCell as SCell with new PCell association.

After time T2 (without activation time):

13.	Target PCell:	Receive RRCReconfigurationComplete.

14.	Target PCell:	Start periodic TA.

15.	Target PCell:	Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU).

16.	Target PCell:	Re-configure RACH procedure as for initial access.

17.	Source PCell:	If source PCell is not target SCell:

17.1	Reset SRBs and release DRBs.

17.2	void.

17a. Source PCell: 	If target PCell is not the same as source SCell:

17a.1 Configure from PCell to normal cell: remove source SCell from the SCell list in the source PCell.

18.	Source SCell: If source SCell is neither target PCell nor target SCell:

18.1	Release DRBs.

<End of modified section>

# <Start of next modified section>

## 9.2	E-UTRA and NR PIXIT

Table 9.2-1: E-UTRA and NR PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_ENDC_BandCombination | ENDC_BandCombination_Type | DC_1A_n28A |  | EN-DC Band Combination |
| px_ENDC_CA_BandCombination | ENDC_CA_BandCombination_Type | DC_1A_n28A_n78A |  | EN-DC CA Band Combination |
| px_ENDC_SecondaryBandCombination | ENDC_BandCombination_Type | DC_1A_n77A |  | Secondary EN-DC Band Combination |
| px_IPv4_Address_Other | charstring |  |  | Other IPv4 Address (see NOTE 4) |
| px_IPv6_Address_Other | charstring |  |  | Other IPv6 Address (see NOTE 4) |
| px_IPv4_Address4_NW | charstring |  |  | IPv4 Gateway Address in PDN4 |
| px_IPv6_Address4_NW | charstring |  |  | IPv6 Gateway Address in PDN4 |
| px_IPv4_Address4_UE | charstring |  |  | IPv4 Address connected to PDN4 |
| px_IPv6_Address4_UE | charstring |  |  | IPv6 Address connected to PDN4 |
| px_IPv4_Address5_NW | charstring |  |  | IPv4 Gateway Address in PDN5 |
| px_IPv6_Address5_NW | charstring |  |  | IPv6 Gateway Address in PDN5 |
| px_IPv4_Address5_UE | charstring |  |  | IPv4 Address connected to PDN5 |
| px_IPv6_Address5_UE | charstring |  |  | IPv6 Address connected to PDN5 |
| px_IPv4_Address6_NW | charstring |  |  | IPv4 Gateway Address in PDN6 |
| px_IPv6_Address6_NW | charstring |  |  | IPv6 Gateway Address in PDN6 |
| px_IPv4_Address6_UE | charstring |  |  | IPv4 Address connected to PDN6 |
| px_IPv6_Address6_UE | charstring |  |  | IPv6 Address connected to PDN6 |
| px_NEDC_BandCombination | NEDC_BandCombination_Type | DC_n28A_3A |  | NE-DC Band Combination |
| px_NEDC_SecondaryBandCombination | NEDC_BandCombination_Type | DC_n28A_39A |  | Secondary NE-DC Band Combination |
| px_NR_CA_BandCombination | NR_CA_BandCombination_Type | CA_n3A_n77A |  | NR CA Band Combination |
| px_NR_DC_BandCombination | NR_DC_BandCombination_Type | DC_n78A_n257A |  | NR-DC Band Combination |
| px_NR_DC_CA_BandCombination | NR_DC_CA_BandCombination_Type | DC_n78A_n257G |  | NR-DC CA Band Combination |
| px_NR_CipheringAlgorithm | CipheringAlgorithm | nea2 |  | Ciphering Algorithm (see Note 1) |
| px_NR_IntegrityProtAlgorithm | IntegrityProtAlgorithm | nia2 |  | Integrity Algorithm (see Note 1) |
| px_NR_OverlappingNotSupportedBand_MFBI | integer | 1 |  | A not supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_PrimaryBand | integer | 1 |  | NR primary band |
| px_NR_SecondaryBand | integer | 2 |  | NR secondary band. Applied to inter-band and SUL test cases. |
| px_NR_SidelinkBand | integer | 47 |  | NR sidelink band |
| px_NR_RATComb_Tested | NR_RATComb_Tested_Type | NR_UTRA | NR_UTRA, NR_GERAN | This parameter represents the network RAT capability / preference and indicates which, if any is supported, RAT combination is to be tested. |
| NOTE 1: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. NOTE 2: Void. NOTE 3: Void. NOTE 4: IP address used for specific IP-address-related test requirements. The only requirements applying to this IP address is that its value shall be different from the values of all other IP address PIXITs (see TS 36.523-3 [12] clause 9) and it shall not be used within more than one PDN (PDU session) within a test case. In the present version of this specification, it is only used within PDN1 NW. |  |  |  |  |

<End of modified section>

<Start of next modified section>

# 11	Guidelines on test execution

## 11.1	Introduction

Clause 11 provides the guidelines on test executions.

The restriction on test case execution is due to the number of frequencies available for the specific band under test specified in TS 38.508-1[5] and the number of frequencies used by the test cases specified in TS 38.523-1[8].

## 11.2	EN-DC

### 11.2.1	Single NR carrier

This clause provides the guidelines for the EN-DC test cases.

A test case using more than one radio frequency on NR, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band combination DC_1A_n51A

Band combination DC_20A_n51A

Band combination DC_28A_n51A

Band combination DC_3A_n51A

Band combination DC_41A_n41A

Band combination DC_42A_n51A

Band combination DC_7A_n51A

The list of test cases is given below:

8.2.3.6.1a, 8.2.3.7.1a, 8.2.3.8.1a, 8.2.3.10.1, 8.2.3.11.1

## 11.3	NR/5GC

### 11.3.1	NR/5GC single RAT

#### 11.3.1.1	Single NR carrier

This clause provides the guidelines for the NR/5GC test cases.

A test case using more than one radio frequency, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n51.

The list of test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.13, 6.1.2.14, 6.1.2.16, 6.1.2.18, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.3.1.1, 6.3.1.2,  6.3.1.3,  6.3.1.4, 6.3.1.5, 6.3.1.8,  6.3.1.9, 6.3.1.10, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.5.1.2, 6.5.2.4,

7.1.3.4.4

8.1.1.2.1, 8.1.1.3.1, 8.1.1.3.3, 8.1.1.3.7, 8.1.1.3.7b, 8.1.3.1.3, 8.1.3.1.6, 8.1.3.1.9, 8.1.3.1.11, 8.1.3.1.12, 8.1.3.1.14A, 8.1.3.1.15A, 8.1.3.1.20, 8.1.3.21, 8.1.4.1.2, 8.1.4.3.4, 8.1.4.3.5, 8.1.6.1.2.2, 8.1.6.1.2.10, 8.1.6.1.3.2, 8.1.6.1.3.7, 8.1.6.1.4.3, 8.1.6.1.4.7,

9.1.4.1, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.4, 9.1.5.1.8, 9.1.5.1.10, 9.1.5.1.12, 9.1.5.1.14, 9.1.9.3,

11.3.5, 11.3.6, 11.3.8, 11.3.9,

11.4.7, 11.4.8.

A test case using more than two radio frequencies, i.e. using the radio frequencies NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n14,

Band n24,

Band n30,

Band n53.

The list of test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.5, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.11, 6.1.2.20, 6.3.1.1, 6.3.1.2, 6.3.1.3, 6.3.1.5, 6.3.1.8, 6.4.1.1, 6.4.1.2, 6.4.2.2, 6.5.2.4,

8.1.1.3.3,

9.1.5.1.2, 9.1.5.1.8.

#### 11.3.1.2	NR carrier aggregation

The restriction on NR CA test case execution as listed in this clause is due to the restriction of bandwidth of an NR CA configuration accommodating the necessary number of NR radio frequencies.

These test cases with switched allocation of PCell and SCell on SCell shall avoid to be executed on an NR CA configuration with a secondary band having no UL frequency:

CA_n28A_n75A,

CA_n29A_n66A,

CA_n29A_n70A,

CA_n75A_n78A,

CA_n76A_n78A,

CA_n8A_n75A

The list of test cases is given below:

8.1.4.1.7.2

### 11.3.2	NR/5GC Inter-RAT

#### 11.3.2.1	NR/E-UTRA Inter-RAT

This clause contains the guidelines for the NR/5GC and E-UTRA inter-RAT test cases executed on the different bands. According to TS 38.508-1 [5] clause 6.2.3.3, it is assumed that the NR and E-UTRA bands under test are not overlapping.

A test case using more than one radio frequency on E-UTRA, i.e. using the radio frequencies f2, f3 or f4 specified in TS 36.508 [10], shall not be executed on:

Band 13,

Band 18,

Band 31,

Band 72,

Band 73

The list of test cases is given below:

6.4.3.1,

8.1.1.3.4,

11.1.4, 11.1.5.

A test case using more than one radio frequency on NR, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n51.

The list of test cases is given below:

6.2.1.1, 6.2.1.3,

8.1.1.3.7a.

A test case using more than two radio frequencies on NR, i.e. using the radio frequencies NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band n14,

Band n24,

Band n30,

Band n53

The list of test cases is given below:

6.2.1.1.

#### 11.3.2.2	NR/EN-DC Inter-RAT

This clause contains the guidelines for the NR/5GC and EN-DC inter-RAT test cases.

A test case using more than one radio frequency on NR, i.e. using the radio frequencies NRf2 or NRf3 or NRf4 specified in TS 38.508-1 [5], shall not be executed on:

Band combination DC_1A_n51A

Band combination DC_20A_n51A

Band combination DC_28A_n51A

Band combination DC_3A_n51A

Band combination DC_41A_n41A

Band combination DC_42A_n51A

Band combination DC_7A_n51A

The list of test cases is given below:

8.1.4.2.1.2

### 11.3.3	NR MFBI

The following NR MFBI test case shall be executed using the combinations specified in Table 11.3.3-1 for px_NR_OverlappingNotSupportedBand_MFBI and px_NR_PrimaryBand:

6.1.2.23.

Table 11.3.3-1: NR MFBI bands combinations

| px_NR_OverlappingNotSupportedBand_MFBI | px_NR_PrimaryBand (Note) |
| --- | --- |
| n2 | n25 |
| n25 | n2 |
| n38 | n41 |
| n41 | n38 |
| n77 | n78 |
| n78 | n77 |
| n257 | n258, n261 |
| n258 | n257 |
| n261 | n257 |
| Note: The UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. |  |