3GPP TSG-RAN WG5 Meeting #95-e	R5-222393r1

# Electronic Meeting, , 9th May  – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0255 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.3 - Generic test procedures for UE MCS operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Procedure for CO session modification with media control missing for MCVideo. - Specific message contents missing in generic procedures. - Transmission control ACK used in all test cases for transmission granted - in procedure 5.3B.1 missing. - Incorrect table numbers. - Incorrect message description in Step 1 of Table 5.3B.3.3-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Procedure 5.3B.11 added. - Specific message content added. - Transmission control ACK in procedure 5.3B.1 added. - Table numbers corrected. - Message description in Step 1 of Table 5.3B.3.3-1 corrected - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid, inconsistent or incomplete test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1 : Change 5 added. |  |  |  |  |  |  |  |  |

## 5.3	Generic test procedures for UE MCS operation

### 5.3.1	General

The purpose of the procedures specified in the following clauses is to facilitate test description by providing procedure sequences which can be referred from the relevant TCs specified e.g. in 3GPP TS 36.579-2 [2], 3GPP TS 36.579-3 [3], 3GPP TS 36.579-6 [84], 3GPP TS 36.579-7 [85].

The procedures specified are required to ensure that any MC service can take place or specific MC relevant pre-conditions are met before a test case can be executed.

### 5.3.2	MCX Authorization/Configuration and Key Generation

5.3.2.1	Initial conditions

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.System Simulator:

-	SS (MCX server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [6] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCX operation in the MCX configuration document).

Implementation Under Test (IUT):

-	UE (MCX client)

-	The MCX Client has been provisioned with the Initial UE Configuration Data as specified in clause 5.5.8.1 allowing for the location of the configuration management server for configuration of the MCX UE initial configuration management object (MO) and the default MCX user profile configuration management object (MO).

-	According to TS 33.180 [94] all HTTP connections are secured by TLS.
The HTTP-1 interface authentication between the HTTP client in the MC UE and the HTTP server endpoint (HTTP proxy, IdM server or KMS) shall be performed by one-way authentication of the HTTP server endpoint based on server certificate as described in TS 33.180 [94] clause 6.1.1.

-	The UE User is provided with username/password for user authentication (px_MCX_User_A_username, px_MCX_User_A_password as provided in TS 36.579-5 [5], Table 9.2-1: MCX Client Common PIXIT)

-	The test USIM set as defined in clause 5.5.10 is inserted.

The UE is attached to EPS services.

-	The UE is provisioned with the names and values of the Transport Key (TrK) and the Integrity Key (InK), since the KMS shall encrypt the key material sent to the client with the TrK and sign the response with the TrK or the InK according to TS 33.180 [94].

5.3.2.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.2.3	Procedures

Table 5.3.2.3-1: MCX user authentication

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-2 | Void | - | - | - | - |
| - | EXCEPTION: Depending on the UE capabilities, the UE (MCX client) executes the sequence described in Table 5.3.2.3-1A | - | - | - | - |
| - | EXCEPTION: The messages below up to and including step 7 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70], to the authorisation endpoint of the IdM server as specified in 3GPP TS 33.180 [94] using the configured URL of the authorisation endpoint of the IdM server as specified in the "<x>/OnNetwork/AppServerInfo/IDMSAuthEndpoint" leaf node, Table 5.5.8.1-1. | - | - | - | - |
| - | EXCEPTION: Steps 3a1-3b1 describe behaviour that depends on UE implementation of the OpenID Connect protocol; the "lower case letter" identifies a step sequence that takes place when one or the other is the case. | - | - | - | - |
| 3a1 | The UE (MCX client) sends an OpenID Connect Authentication Request using HTTP GET. | --> | HTTP GET (Authorization) | - | P |
| 3b1 | The UE (MCX client) sends an OpenID Connect Authentication Request using HTTP POST. | --> | HTTP POST (Authorization) | - | P |
| 4 | The SS sends a HTTP 200 (OK) including the HTML form requesting username and password. | <-- | HTTP 200 (OK) | - | - |
| 5 | Make the UE user provide user credentials: username and password (px_MCX_User_A_username, px_MCX_User_A_password). NOTE 2 | - | - | - | - |
| 6 | The UE (MCX client) sends an HTTP POST Request message to the SS containing user name and password. | --> | HTTP POST | - | P |
| 7 | The SS sends a HTTP 302 (Found) as the OpenID Connect Authentication Response containing an authorization code. | <-- | HTTP 302 (Found) | - | - |
| 8 | Void | - | - | - | - |
| - | EXCEPTION: The messages in steps 9 to 10 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70] to the token endpoint of the IdM server as specified in 3GPP TS 33.180 [94] using the configured URL of the token endpoint of the IdM server as specified in the "/<x>/OnNetwork/AppServerInfo/IDMSTokenEndpoint" leaf node, Table 5.5.8.1-1. | - | - | - | - |
| 9 | The UE (MCX client) sends an HTTP POST Request message to the SS (OIDC Token Request message), passing the authorization code obtained in step 7. | --> | HTTP POST | - | P |
| 10 | The SS sends a HTTP 200 (OK) providing id_token, access_token and refresh token. | <-- | HTTP 200 (OK) | - | - |
| - | EXCEPTION: The messages in steps 11 to 14 are transmitted over a secure TLS tunnel that has been established by the UE (MCX client) as specified by 3GPP TS 33.310 [70] to the HTTP Proxy as specified in 3GPP TS 33.180 [94] using the configured URL of the HTTP Proxy as specified in the "/<x>/OnNetwork/AppServerInfo/HTTPproxy" leaf node, Table 5.5.8.1-1. | - | - | -- |  |
| 11 | The UE (MCX client) sends a HTTP POST message presenting the access token obtained in step 10 to the SS over HTTP for Key Management Initialisation. NOTE: Step 11 is the start of the second stage which was started in Step 2. Steps 11 through 14 involve Key Management Authorization. The MCX Client/Key Management Client presents the access token to the Key Management Server. The end result is the user gets specific key material. | --> | HTTP POST | - | P |
| 12 | The SS replies to the UE with identity specific key information. | <-- | HTTP 200 (OK) | - | - |
| 13 | The UE (MCX client) sends a HTTP POST message presenting an access token to the SS over HTTP for Key Material Request. | --> | HTTP POST | - | P |
| 14 | The SS replies to the UE with identity specific key information. | <-- | HTTP 200 (OK) | - | - |
| 15-32 | Void | - | - | - | - |
| NOTE 1: Void. NOTE 1A: Void. NOTE 2: The UE is expected to prompt the MCX user for their username and password, or it may be stored on the UE. The provision of the username/password is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

Table 5.3.2.3-1A: MCX Initial UE Configuration Request

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE (MCX client) sends an HTTP GETrequestto retrieve the initial UE configuration from the Server | --> | HTTP GET (initial UE configuration) | - | P |
| 2 | The SS sends a HTTP 200 (OK) including the initial UE configuration document | <-- | HTTP 200 (OK) | - | - |

Table 5.3.2.3-2: MCX Service Authorization and Key Generation

| St | Procedure |  | Message Sequence |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S | Message |  |  |  |  |
| - |  | EXCEPTION: In parallel to procedure of all steps below the behaviour of table 5.3.2.3-2A, the behaviour of table 5.3.2.3-2B and the behaviour of table 5.3.2.3-2C takes place. | - | - |  | - |  | - |
| - |  | EXCEPTION: Steps 1a1-1b2 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place when one or the other is the case. NOTE: Step 1a1 is the start of the third stage which was started in Step 3 of table 5.3.2.3-1. Steps 1a1 and 1b1 involve User Service Authorization. | - | - |  | - |  | - |
| 1a1 |  | The UE (MCX client) sends a SIP REGISTER request for service authorisation. | --> | SIP REGISTER |  | - |  | P |
| 1a2 |  | The SS (MCX server) sends SIP 200 (OK). NOTE: The user is now authorized for MCX service. | <-- | SIP 200 (OK) |  | - |  | - |
| 1a3 |  | The UE (MCX client) sends a SIP PUBLISH request for update of PoC-settings (NOTE 1). | --> | SIP PUBLISH |  | - |  | P |
| 1a4 |  | The SS (MCX server) sends SIP 200 (OK). | <-- | SIP 200 (OK) |  | - |  | - |
| 1b1 |  | The UE (MCX client) sends a SIP PUBLISH request for service authorisation and update of PoC-settings (NOTE 1). | --> | SIP PUBLISH |  | -- |  | P |
| 1b2 |  | The SS (MCX server) sends SIP 200 (OK). NOTE: The user is now authorized for MCX service. | <-- | SIP 200 (OK) |  |  |  | - |
| NOTE 1: The PoC-settings document contains the user profile index of the selected user profile. In general the UE sends the SIP PUBLISH request not before it has retrieved the user profile at step 8 in Table 5.3.2.3-2A. |  |  |  |  |  |  |  |  |

Table 5.3.2.3-2A: Configuration management subscription and notification procedure

| St |  | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | U - S | Message |  |  |
| 1 | The UE (MCX client) sends a SIP SUBSCRIBE - subscription to multiple documents simultaneously - to the SS containing the access token and a resource list mime body containing a list of the following documents: MCX UE Configuration document, MCX User Profile Configuration Document, and the MCX Service configuration document. The base URI of each list entry is set to the CMS XCAP-ROOT-URI. NOTE: Step 1 is the start of the fourth stage which was started in Step 3 of table 5.3.2.3-1. Steps 1 through 10 involve Configuration Management Authorization. The end result of the fourth stage is that the MCX Client receives 3 configuration documents: UE Configuration Document, User Profile Configuration Document, and the Service Configuration Document. |  | --> | SIP SUBSCRIBE | - | P |
| 2 | The SS sends a SIP 200 (OK) message. |  | <-- | SIP 200 (OK) | - | - |
| 3 | The SS sends a SIP NOTIFY message to the UE that contains the XCAP-URI of the documents. |  | <-- | SIP NOTIFY | - | - |
| - | EXCEPTION: The order of steps 4, 5, 7 and 9 depends on UE and SS implementation and is not checked by the implementation |  | - | - | - | - |
| 4 | The UE (MCX client) sends a SIP 200 (OK) message. |  | --> | SIP 200 (OK) | - | P |
| 5 | The UE (MCX client) sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the MCX UE Configuration Document. NOTE: The MCX Client is requesting the MCX UE Configuration Document. |  | --> | HTTP GET | - | P |
| 6 | The SS sends the HTTP 200 (OK) message including the MCX UE Configuration Document. |  | <-- | HTTP 200 (OK) | - | - |
| 7 | The UE (MCX client) sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the MCX User Profile Configuration Document. NOTE: The MCX Client is requesting the MCX User Profile Configuration Document. |  | --> | HTTP GET | - | P |
| 8 | The SS sends the HTTP 200 (OK) message including the MCX User Profile Configuration Document. NOTE: The MCX User Profile Configuration Document includes information on MCX groups including for which groups the MCX Client is a member. The MCX User Profile Configuration Document includes Group A as a group for which the MCX Client is a member and is implicitly affiliated. Group A is used as the default group for all test cases in TS 36.579-2 and TS 36.579-3. |  | <-- | HTTP 200 (OK) | - | - |
| 9 | The UE (MCX client) sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the MCX Service Configuration Document. NOTE: The MCX Client is requesting the MCX Service Configuration Document. |  | --> | HTTP GET | - | P |
| 10 | The SS sends the HTTP 200 (OK) message including the MCX Service Configuration Document. |  | <-- | HTTP 200 (OK) | - | -- |

Table 5.3.2.3-2B: Group document subscription and notification procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE (MCX client) sends a SIP SUBSCRIBE to the SS, containing the access token and a resource list mime body and a list of the Groups to be obtained. The base URI of each list entry is set to the GMS XCAP-ROOT-URI, and the MCX group ID identifies a group document. NOTE: Step 1 is the start of the fifth stage which was started in Step 2 of table 5.3.2.3-1. Steps 1 through 6 involve Group Management Authorization. The end result is the MCX Client will receive group information for Group A. The MCX Client will also get the Group Master Key (GMK) for the group which will be used to derive keys for the group. There will also be a Group User Key Identifier (GUK-ID), and a Group Master Key Identifier (GMK-ID). According TS 33.180 [94], clause 7.4.1, the GMK shall be used as the MIKEY Traffic Generating Key (TGK) and the GUK-ID shall be used as the MIKEY CSB ID. These shall be used to generate the SRTP Master Key and SRTP Master Salt as specified in IETF RFC 3830 [24]. | --> | SIP SUBSCRIBE | - | P |
| 2 | The SS sends a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 3 | The SS sends a SIP NOTIFY message to the UE that contains the XCAP-URI of the Group documents. | <-- | SIP NOTIFY | - | - |
| - | EXCEPTION: The order of steps 4 and 5 depends on UE and SS implementation and is not checked by the implementation | - | - | - | - |
| 4 | The UE (MCX client) sends a SIP 200 (OK) message. | --> | SIP 200 (OK) | - | P |
| 5 | The UE (MCX client) sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the Group Configuration document. | --> | HTTP GET | - | P |
| 6 | The SS sends the HTTP 200 (OK) message including the Group Document 'MCX UE Configuration document'. NOTE 1 | <-- | HTTP 200 (OK) | - | - |
| - | EXCEPTION: Steps 7a1-7a2 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place when one or the other is the case. | - | - |  |  |
| 7a1 | IF the Resource-Lists received from the UE at step 1 contains an entry referring to an MCX-GKTP document THEN the SS sends a SIP NOTIFY message to the UE containing the group key transport payloads (GKTP) document. | <-- | SIP NOTIFY |  |  |
| 7a2 | The UE (MCX client) sends a SIP 200 (OK) message. | --> | SIP 200 (OK) |  |  |
| NOTE 1: This completes MCX service enabling on the UE. |  |  |  |  |  |

Table 5.3.2.3-2C: Group communication key retrieval procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS starts timer Timer_1 = 5 seconds. | - | - | - | - |
| - | EXCEPTION: Steps 2a5-3a1 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence that takes place when one or the other is the case. | - | - | - | - |
| 2a1 | The UE (MCX client) sends a SIP SUBSCRIBE to the SS, creating a new dialog and containing the access token and a resource list mime body containing an entry to request group key transport payloads (GKTP) document. | --> | SIP SUBSCRIBE | - | P |
| 2a2 | The SS sends a SIP 200 (OK) message | <-- | SIP 200 (OK) | - | - |
| 2a3 | The SS sends a SIP NOTIFY message to the UE containing the group key transport payloads (GKTP) document. | <-- | SIP NOTIFY | - | - |
| 2a4 | The UE (MCX client) sends a SIP 200 (OK) message. | --> | SIP 200 (OK) | - | P |
| 2a5 | The SS stops Timer_1. | - | - | - | - |
| 2b1 | Timer_1 expires | - | - |  |  |
| NOTE: This key retrieval from the GMS is necessary for the MCX UE under test to enable ciphering exchanged media in group communications. |  |  |  |  |  |

5.3.2.4	Specific message contents

Table 5.3.2.4-1: HTTP GET (Step 3a1, Table5.3.2.3-1 )

Derivation Path: Table 5.5.4.2-1, condition AUTH

Table 5.3.2.4-2: HTTP POST (Step 3b1, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.3-1, condition AUTH

Table 5.3.2.4-3: HTTP 200 (OK) (Step 4, Table 5.3.2.3-1)

| Derivation Path: Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "text/html" |  | RFC 2854 [111] |  |
| Message-body |  |  |  |  |
| HTML form | <!DOCTYPE html> <html> <body> <form action="/idms/userauth" method="post"> Username: <input type="text" name="user"><br> Password: <input type="password" name="password"><button type="submit">Login</button> </form> </body> </html> | "/idms/userauth" given by tsc_MCX_IdMS_userauth_UriPath is the URI to be used by the UE as request URI in the HTTP POST request for user authentication | HTML 4.01 Specification [105] |  |

Table 5.3.2.4-4: HTTP POST (Step 6, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.3-1, condition USERAUTH

Table 5.3.2.4-5: HTTP 302 (Found) (Step 7, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.8-1, condition AUTH.

Table 5.3.2.4-6: HTTP POST (Step 9, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.3-1, condition TOKEN

Table 5.3.2.4-7: HTTP 200 (OK) (Step 10, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.6-1, condition TOKEN

Table 5.3.2.4-8: HTTP POST (Step 11, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.33-1, condition KMSINIT.

Table 5.3.2.4-9: HTTP 200 (OK) (Step 12, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.6-1, condition KMSINIT.

Table 5.3.2.4-10: HTTP POST (Step 13, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.3-1, condition KMSKEY.

Table 5.3.2.4-11: HTTP 200 (OK) (Step 14, Table 5.3.2.3-1)

Derivation Path: Table 5.5.4.6-1, condition KMSKEY.

Table 5.3.2.4-12: SIP REGISTER (Step 1a1, Table 5.3.2.3-2)

Derivation Path: Table 5.5.2.13-1, condition CONFIG

Table 5.3.2.4-13: SIP PUBLISH (Step 1b1, Table 5.3.2.3-2)

Derivation Path: Table 5.5.2.11-1, condition CONFIG

Table 5.3.2.4-13A: SIP PUBLISH (Step 1a3, Table 5.3.2.3-2)

Derivation Path: Table 5.5.2.11-1, condition POC-SETTINGS-EVENT

Table 5.3.2.4-14: SIP SUBSCRIBE (Step 1, Table 5.3.2.3-2A)

Derivation Path: Table 5.5.2.14-1, condition CONFIG

Table 5.3.2.4-15: SIP NOTIFY (Step 3, Table 5.3.2.3-2A)

Derivation Path: Table 5.5.2.8-1, condition CONFIG

Table 5.3.2.4-16: HTTP GET (Step 5, Table 5.3.2.3-2A)

Derivation Path: Table 5.5.4.2-1, condition UECONFIG.

Table 5.3.2.4-17: HTTP GET (Step 7, Table 5.3.2.3-2A)

Derivation Path: Table 5.5.4.2-1, condition UEUSERPROF.

Table 5.3.2.4-18: HTTP GET (Step 9, Table 5.3.2.3-2A)

Derivation Path: Table 5.5.4.2-1, condition UESERVCONFIG.

Table 5.3.2.4-19: HTTP 200 (OK) (Step 6, Table 5.3.2.3-2A)

Derivation Path: Table 5.5.4.6-1, condition UECONFIG.

Table 5.3.2.4-20: HTTP 200 (OK) (Step 8, Table 5.3.2.3-2A)

Derivation Path: Table 5.5.4.6-1, condition UEUSERPROF.

Table 5.3.2.4-21: HTTP 200 (OK) (Step 10, Table 5.3.2.3-2A)

Derivation Path: Table 5.5.4.6-1, condition UESERVCONFIG.

Table 5.3.2.4-22: SIP SUBSCRIBE (Step 1, Table 5.3.2.3-2B)

Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG

Table 5.3.2.4-22A: Void

Table 5.3.2.4-22B: SIP NOTIFY (Step 3, Table 5.3.2.3-2B)

Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG

Table 5.3.2.4-23: HTTP GET (Step 5, Table 5.3.2.3-2B)

Derivation Path: Table 5.5.4.2-1, condition GROUPCONFIG

Table 5.3.2.4-24: HTTP 200 (OK) (Step 6, Table 5.3.2.3-2B)

Derivation Path: Table 5.5.4.6-1, condition GROUPCONFIG.

Table 5.3.2.4-25: Void

Table 5.3.2.4-26: SIP 200 (OK) (Steps 1a2, 1a4, 1b2, Table 5.3.2.3-2, step 2, Table 5.3.2.3-2A, step 2, Table 5.3.2.3-2B)

Derivation Path: Table 5.5.2.17.1.2-1

Table 5.3.2.4-27: SIP 200 (OK) (Step 4, Table 5.3.2.3-2A, step 4, Table 5.3.2.3-2B)

Derivation Path: Table 5.5.2.17.1.1-1

Table 5.3.2.4-28: HTTP GET (Step 1, Table 5.3.2.3-1A)

Derivation Path: Table 5.5.4.2-1, condition UEINITIALCONFIG

Table 5.3.2.4-29: HTTP 200 (OK) (Step 2, Table 5.3.2.3-1A)

Derivation Path: Table 5.5.4.6-1, condition UEINITIALCONFIG

Table 5.3.2.4-30: SIP SUBSCRIBE (Step 1, Table 5.3.2.3-2C)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/resource-lists+xml" |  |  |  |
| MIME-part-body | Resource-lists as described in Table 5.3.2.4-31 |  |  |  |

Table 5.3.2.4-31: Resource-Lists in SIP SUBSCRIBE (Table 5.3.2.4-30)

Derivation Path: Table 5.5.3.3.1-1 condition GROUPKEY

Table 5.3.2.4-32: SIP NOTIFY (Step 7a, Table 5.3.2.3-2B and Step 3, Table 5.3.2.3-2C)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Message-body |  |  |  |  |
| xcap-diff document | xcap-diff document as described in Table 5.3.2.4-33 |  |  |  |

Table 5.3.2.4-33: Xcap-Diff Document (Table 5.3.2.4-32)

Derivation Path: Table5.5.3.12-2, condition GROUPKEY

### 5.3.2A - 5.3.2B	Void

### 5.3.3	MCX pre-established session establishment CO

5.3.3.1	Initial conditions

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

System Simulator:

-	SS (MCX server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [6] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCX operation in the MCX configuration document)

IUT:

-	UE (MCX client)

-	The UE has performed the procedure for MCX Authorization/Configuration and Key Generation as specified in clause 5.3.2 and thereby the MCX client is authorised for and able to use the MCX service including making group and private calls on- and off-network, and, the MCX user is registered for receiving MCX service through the MCX Client.

5.3.3.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.3.3	Procedure

Table 5.3.3.3-1: MCX pre-established session establishment CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 1A | EXCEPTION: The E-UTRA/EPC actions which are related to the MCX call establishment as described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2-7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCX Client) send a SIP INVITE message in order to create a pre-established session? | --> | SIP INVITE | - | P |
| 8A | The SS sends SIP 100 Trying | <-- | SIP 100 Trying | - | - |
| 9 | Void | - | - | - | - |
| 10 | The SS (MCX server) responds with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 10A | Check: Does the UE (MCX Client) respond with a SIP ACK message? | --> | SIP ACK | - | P |
| 11 | Void | - | - | - | - |
| 11A | The SS waits 2 seconds to ensure that lower layer signalling (TCP) is finished. | - | - | - | - |
| 12 | The SS transmits an RRCConnectionRelease message. | <-- | RRC: RRCConnectionRelease | - | - |

5.3.3.4	Specific message contents

Table 5.3.3.4-1: SIP INVITE (step 8, Table 5.3.3.3-1)

| Derivation Path: Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  | RFC 3261 [22 RFC 3840 [33] |  |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  |  |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. |  |  |
| Accept |  |  | RFC 3261 [22] |  |
| media-range[1] | "application/sdp” |  |  |  |
| Answer-Mode | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP Message | SDP message as described in Table 5.5.3.1.1-1 with conditions PRE_ESTABLISHED_SESSION, INITIAL_SDP_OFFER |  |  |  |

Table 5.3.3.4-2: SIP 200 (OK) (step 10, Table 5.3.3.3-1)

| Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCX_SessionID_B | The URI that identifies the pre-established session |  |  |

### 5.3.3A	Void

### 5.3.4	MCX CT session establishment/modification without provisional responses other than 100 Trying

5.3.4.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.4.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.4.3	Procedure

Table 5.3.4.3-1: MCX CT session establishment/modification without provisional responses other than 100 Trying

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment as described in clause 5.4.4 'Generic Test Procedure for MCX CT communication in E-UTRA' take place. | - | - | - | - |
| 2 | The SS (MCX Server) sends a SIP INVITE requesting the establishment/modification of an MCX call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying). | - | - | - | - |
| 3a1 | The UE (MCX client) sends SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 4 | Check: Does the UE (MCX client) respond to the SIP INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 5 | The SS (MCX server) sends a SIP ACK to acknowledge the session establishment/modification | <-- | SIP ACK | - | - |

5.3.4.4	Specific message contents

All message contents are as specified in clause 5.5 with the following clarifications:

none

Table 5.3.4.4-1: Void

### 5.3.5	MCX CT group call establishment, manual commencement

5.3.5.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.5.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.5.3	Procedure

Table 5.3.5.3-1: MCX CT group call establishment, manual commencement

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment described in clause 5.4.4 'Generic Test Procedure for MCX CT communication in E-UTRA' take place. | - | - | - | - |
| 2 | The SS (MCX Server) sends an initial SIP INVITE requesting the establishment of an MCX group call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 3a1 | The UE (MCX client) sends SIP 100 (Trying). | --> | SIP 100 (Trying) | - | - |
| 4 | The SS starts timer Timer_1 = 5 seconds. | - | - | - | - |
| - | EXCEPTION: Steps 5a1 to 5c1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that may take place if the UE responds reliably or unreliably to a SIP INVITE with a SIP 183 (Session Progress) | - | - | - | - |
| 5a1 | Check: Does the UE (MCX client) send SIP 183 (Session Progress) unreliably? | --> | SIP 183 (Session Progress) | - | P |
| 5a2 | The SS stops Timer_1. | - | - | - | - |
| 5b1 | Check: Does the UE (MCX client) send SIP 183 (Session Progress) reliably? | --> | SIP 183 (Session Progress) | - | P |
| 5b2 | The SS stops Timer_1. | - | - | - | - |
| 5b3 | The SS (MCX Server) acknowledges the receipt of SIP 183 (Session Progress) | <-- | PRACK | - | - |
| 5b4 | The UE (MCX Client) responds PRACK with SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 5c1 | Check: Does Timer_1 expire? | - | - | - | P |
| 5A | Check: Does the UE (MCX client) notify the User of the incoming call request? (NOTE 1) | - | - | - | P |
| 6 | Make UE (MCX User) accept the call (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCX client) respond to the SIP INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 8 | The SS (MCX server) sends a SIP ACK to acknowledge the session establishment | <-- | SIP ACK | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3.5.4	Specific message contents

All message contents are as specified in clause 5.5 with condition GROUP-CALL where applicable and with the following clarifications:

Table 5.3.5.4-1 to 3: Void

### 5.3.6	MCX CT private call establishment, manual commencement

5.3.6.1	Initial conditions

The same initial conditions apply as specified in clause 5.3.3.1.

Within the context of this procedure, MCX refers to MCPTT or MCVideo

5.3.6.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.6.3	Procedure

Table 5.3.6.3-1: MCX CT private call establishment, manual commencement

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment described in clause 5.4.4 'Generic Test Procedure for MCX CT communication in E-UTRA' take place. | - | - | - | - |
| 2 | The SS (MCX Server) sends an initial SIP INVITE requesting the establishment of an MCX private call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 3a1 | The UE (MCX client) sends SIP 100 (Trying). | --> | SIP 100 (Trying) | - | - |
| - | EXCEPTION: Steps 4a1 to 4b3 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that takes place if the UE responds either unreliably or reliably to a SIP INVITE with a SIP 180 (Ringing) | - | - | - | - |
| 4a1 | Check: Does the UE (MCX client) send a SIP 180 (Ringing) unreliably? | --> | SIP 180 (Ringing) | - | P |
| 4b1 | Check: Does the UE (MCX client) send a SIP 180 (Ringing) reliably? | --> | SIP 180 (Ringing) | - | P |
| 4b2 | The SS (MCX Server) acknowledges the receipt of SIP 180 (Ringing) | <-- | PRACK | - | - |
| 4b3 | The UE (MCX Client) responds PRACK with SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 4A | Check: Does the UE (MCX client) notify the User of the incoming call request? (NOTE 1) | - | - | - | P |
| 5 | Make UE (MCX User) accept the call | - | - | - | - |
| 6 | Check: Does the UE (MCX client) respond to the SIP INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The SS (MCX server) sends a SIP ACK to acknowledge the session establishment | <-- | SIP ACK | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3.6.4	Specific message contents

All message contents are as specified in clause 5.5 with condition PRIVATE-CALL where applicable and in the test case calling the procedure, with the following clarifications:

Table 5.3.6.4-1 to 1A: Void

Table 5.3.6.4-2: SIP 180 (Ringing) (step 4b1, Table 5.3.6.3-1)

Derivation Path: Table 5.5.2.16.2.1-1 with condition 100rel

Table 5.3.6.4-3: Void

### 5.3.7 to 5.3.9	Void

### 5.3.10	MCX CO call release

5.3.10.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.10.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.10.3	Procedure

Table 5.3.10.3-1: MCX CO call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCX Client) send a SIP BYE request to terminate the MCX session? | --> | SIP BYE | - | P |
| 2 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 3 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. | - | - | - | - |

5.3.10.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3.11	Void

### 5.3.12	MCX CT call release

5.3.12.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.12.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.12.3	Procedure

Table 5.3.12.3-1: MCX CT call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCX Server) sends a SIP BYE request to terminate the MCX session. | <-- | SIP BYE | - | - |
| 2 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 3 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. | - | - | - | - |

5.3.12.4	Specific message contents

All message contents are as specified in clause 5.5. and in the test case calling the procedure, with the following clarifications:

none

### 5.3.13 - 21	Void

### 5.3.22	MCX NW initiated notifications regarding temporary group creation or tear down

5.3.22.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.22.2	Definition of system information messages

-

5.3.22.3	Procedure

Table 5.3.22.3-1: MCX NW initiated notifications regarding temporary group creation or tear down

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCX server) sends a SIP NOTIFY to the UE informing about change of group A's configuration document. | <-- | SIP NOTIFY | - | - |
| 2 | The UE sends a SIP 200 (OK) message to the SS. | --> | SIP 200 (OK) | - | - |
| 2A-2F | Void | - | - | - | - |
| 3 | The UE (MCX client) sends an HTTP GET Request message to the SS that contains the access token and the XCAP-URI of the Group Configuration document. | --> | HTTP GET | - | - |
| 4 | The SS (MCX server) sends the HTTP 200 (OK) message including the updated Group Document | <-- | HTTP 200 (OK) | - | - |
| 5 | The SS (MCX server) sends a SIP NOTIFY message to the UE containing the group key transport payloads (GKTP) document including the group keys. | <- | SIP NOTIFY | - | - |
| 5a1-5a2 | Void | - | - | - | - |
| 6 | The UE (MCX client) sends a SIP 200 (OK) message. | --> | SIP 200 (OK) | - | - |

5.3.22.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3.22.4-1: SIP NOTIFY (Step 1)

| Derivation Path: Table 5.5.2.8-1, condition GROUPCONFIG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |  |
| Message-body |  |  |  |  |  |
| MIME body part |  | xcap-diff |  |  |  |
| MIME-part-body | Xcap-diff as described in Table 5.3.22.4-1A |  |  |  |  |

Table 5.3.22.4-1A: Xcap-diff document in SIP NOTIFY (Table 5.3.22.4-1)

Derivation Path: Table 5.5.3.12-2, condition GROUPCONFIG

Table 5.3.22.4-2: SIP 200 (OK) (Steps 2, 6)

Derivation Path: Table 5.5.2.17.1.1-1

Table 5.3.22.4-2A..2G: Void

Table 5.3.22.4-3: HTTP GET (Step 3)

Derivation Path: Table 5.5.4.2-1, condition GROUPCONFIG

Table 5.3.22.4-4: HTTP 200 (OK) (Step 4)

| Derivation Path: Table 5.5.4.6-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| group-configuration | As described in Table 5.3.22.4-5 | Group Configuration document returned |  |  |

Table 5.3.22.4-5: Group Configuration document (Table 5.3.22.4-4)

| Derivation Path: Table 5.5.7.4-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  |  |  |  |
| mcpttgi:on-network-regrouped |  |  | TS 24.481 [31] clause 7.2.4.2 | TEMPGROUPCREATE |
| temporary-MCPTT-group-ID attribute | px_MCPTT_Group_T_ID | MCS temporary group identity | TS 24.481 [31] clause 7.2.4.2 | MCPTT |
|  | px_MCVideo_Group_T_ID |  |  | MCVIDEO |
|  | px_MCData_Group_T_ID |  |  | MCDATA |
| temporary-MCPTT-group-requestor attribute | px_MCPTT_ID_User_B | Identity of the responsible for formatting the MCS temporary group. | TS 24.481 [31] clause 7.2.4.2 | MCPTT |
|  | px_MCVideo_ID_User_B |  |  | MCVIDEO |
|  | px_MCData_ID_User_B |  |  | MCDATA |
| constituent-MCPTT-group-IDs |  |  | TS 24.481 [31] clause 7.2.4.2 |  |
| constituent-MCPTT-group-ID[1] | px_MCPTT_Group_A_ID | MCS group ID of a constituent MCS group of the temporary MCS group | TS 24.481 [31] clause 7.2.4.2 | MCPTT |
|  | px_MCVideo_Group_A_ID |  |  | MCVIDEO |
|  | px_MCData_Group_A_ID |  |  | MCDATA |
| constituent-MCPTT-group-ID[1] | px_MCPTT_Group_B_ID | MCS group ID of a constituent MCS group of the temporary MCS group | TS 24.481 [31] clause 7.2.4.2 | MCPTT |
|  | px_MCVideo_Group_B_ID |  |  | MCVIDEO |
|  | px_MCData_Group_B_ID |  |  | MCDATA |
| protect-media | "true" | Indicates whether confidentiality and integrity of media is required on the MCPTT temporary group | TS 24.481 [31] clause 7.2.4.2 |  |
| protect-floor-control-signalling | "true" | Indicates whether confidentiality and integrity of floor control signalling is required on the temporary MCPTT group | TS 24.481 [31] clause 7.2.4.2 |  |

| Condition | Explanation |
| --- | --- |
| TEMPGROUPCREATE | Procedure is used for creation of a temporary group (but not for tear down) |

Table 5.3.22.4-5A: Void

Table 5.3.22.4-6: SIP NOTIFY (Step 5)

| Derivation Path: Table 5.5.2.14-1, condition GROUPCONFIG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Message-body |  |  |  |  |
| xcap-diff document | xcap-diff document as described in Table 5.3.22.4-7 |  |  |  |

Table 5.3.22.4-7: xcap-diff document for MCX group configuration (Table5.3.22.4-6)

| Derivation Path: Table 5.5.3.12-2, condition GROUPKEY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff | encrypted according to NOTE 1 of Table 5.5.3.12-2 |  |  |  |
| element[1] |  |  |  |  |
| sel attribute | Doc-Sel & "~~" & Node-Sel | Document and node selector for Group T according to NOTEs 2a, 2b and 3 of Table 5.5.3.12-2 |  |  |
| GKTPs | group key transport payloads (GKTP) document as described in Table 5.3.22.4-8 |  |  |  |

Table 5.3.22.4-8: group key transport payloads (GKTP) document (Table 5.3.22.4-7)

| Derivation Path: TS 24.481 [11] clause 7.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| GKTPs |  |  |  |  |
| GMK-GKTPs |  |  |  |  |
| GKTP[1] | MIKEY message as used in group communication key retrieval procedure | MIKEY message containing the GMK for Group A | TS 33.180 [94] |  |
| id attribute | Same value as used in group communication key retrieval procedure |  |  |  |
| on-network-regrouped-GKTPs[1] |  |  |  | TEMPGROUPCREATE |
| temporary-MCPTT-group-ID attribute | px_MCPTT_Group_T_ID |  |  | MCPTT |
|  | px_MCVideo_Group_T_ID |  |  | MCVIDEO |
|  | px_MCData_Group_T_ID |  |  | MCDATA |
| GKTP[1] | MIKEY message as described in Table 5.3.22.4-9 | MIKEY message containing the GMK for Group T | TS 33.180 [94] |  |
| id attribute | arbitrary value | unique charstring assigned by the SS |  |  |

| Condition | Explanation |
| --- | --- |
| TEMPGROUPCREATE | Procedure is used for creation of a temporary group (but not for tear down) |

Table 5.3.22.4-9: MIKEY-SAKKE I_MESSAGE (GMK distribution by the SS) (Table 5.3.22.4-8)

| Derivation Path: Table 5.5.9.1-3 |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| General Extension Payload { |  |  |  |
| Content { |  |  |  |
| Payload { |  |  |  |
| Data { |  | See TS 33.180 [94] clause E.6 |  |
| Group IDs { |  |  |  |
| Number of Group IDs | '1' |  |  |
| Group ID | px_MCPTT_Group_T_ID | The ID for the group associated with the key. | MCPTT |
|  | px_MCVideo_Group_T_ID |  | MCVIDEO |
|  | px_MCData_Group_T_ID |  | MCDATA |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ..} |  |  |  |
| } |  |  |  |

### 5.3.23 - 25	Void

### 5.3.26	MCX CO Group Creation

5.3.26.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.26.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.26.3	Procedure

Table 5.3.26.3-1: MCX CO Group Creation procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1a2 | Void | - | - | - | - |
| 1 | Check: Does the UE (MCX Client) send a HTTP PUT to the SS to request for creation of the new group? | --> | HTTP PUT | - | P |
| 2 | The SS (MCX Server) sends a HTTP 201 (Created). | <-- | HTTP 201 (Created) | - | - |
| 3-5 | Void |  |  |  |  |

5.3.26.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

Table 5.3.26.4-1 to -5: Void

### 5.3.27	MCX CO Temporary Group Creation

5.3.27.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.27.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.27.3	Procedure

Table 5.3.27.3-1: MCX CO Temporary Group Creation procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCX Client) send a HTTP POST to the SS to request for creation of a temporary group? | --> | HTTP POST | - | P |
| 2 | The SS (MCX Server) sends a HTTP 200 (OK) containing the GMOP group-regroup-creation-response. | <-- | HTTP 200 (OK) | - | - |

5.3.27.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

Table 5.3.27.4-1 to -2: Void

### 5.3.28	MCX CO Temporary Group Tear Down

5.3.28.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.28.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.28.3	Procedure

Table 5.3.28.3-1: MCX CO Temporary Group Creation procedure

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCX Client) send a HTTP DELETE to the SS to request for tear down of a temporary group? | --> | HTTP DELETE | - | P |
| 2 | The SS (MCX Server) sends a HTTP 200 (OK). | <-- | HTTP 200 (OK) | - | - |

5.3.28.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

Table 5.3.28.4-1: Void

### 5.3.29	MCX Subscription and Notification

5.3.29.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.29.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.29.3	Procedure

Table 5.3.29.3-1: MCX Subscription and Notification

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP SUBSCRIBE message request? | --> | SIP SUBSCRIBE | - | P |
| 3 | The SS (MCX Server) responds to the SIP SUBSCRIBE message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCX Server) sends a SIP NOTIFY message | <-- | SIP NOTIFY | - | - |
| 5 | The UE (MCX Client) responds with a SIP 200 (OK) message. | --> | SIP 200 (OK) |  |  |
| 6 | SS (MCX Server) releases the E-UTRA connection. | - | - | - | - |

5.3.29.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3.30	MCX SIP MESSAGE Request - Accept CO

5.3.30.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT or MCVideo

5.3.30.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.30.3	Procedure

Table 5.3.30.3-1: MCX SIP MESSAGE Request - Accept CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment as described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCX server) sends SIP MESSAGE accepting the request. | <-- | SIP MESSAGE | - | - |
| 5 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 6 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.30.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.31	MCX SIP MESSAGE Request - Accept CT

5.3.31.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT or MCVideo

5.3.31.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.31.3	Procedure

Table 5.3.31.3-1: MCX SIP MESSAGE Request - Accept CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment as described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | The SS (MCX server) sends SIP MESSAGE | <-- | SIP MESSAGE | - | - |
| 3 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 4 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 5 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 6 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.31.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.32	MCX SIP MESSAGE CO

5.3.32.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData

5.3.32.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.32.3	Procedure

Table 5.3.32.3-1: MCX SIP MESSAGE CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment as described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP MESSAGE message? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCX Server) responds with a SIP 200 (OK) message? | <-- | SIP 200 (OK) | - | - |
| 4 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.32.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.33	MCX SIP MESSAGE CT

5.3.33.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData

5.3.33.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.33.3	Procedure

Table 5.3.33.3-1: MCX SIP MESSAGE CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment as described in clause 5.4.4 'Generic Test Procedure for MCX CT communication in E-UTRA' take place. | - | - | - | - |
| 2 | The SS (MCX server) sends SIP MESSAGE | <-- | SIP MESSAGE | - | - |
| 3 | Check: Does the UE (MCX Client) respond with a SIP 200 (OK) message? | --> | SIP 200 (OK) | - | P |
| 4 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.33.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3.34	MCX Group Affiliation Status Change

5.3.34.1	Initial conditions

As specified in the test case which calls the procedure.

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData

5.3.34.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.34.3	Procedure

Table 5.3.34.3-1: MCX Group Affiliation Status Change

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCX call establishment as described in clause 5.4.4 'Generic Test Procedure for MCX CT communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCX Client) send a SIP PUBLISH message? | --> | SIP PUBLISH | - | P |
| 3 | The SS responds to the SIP PUBLISH message with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 4 | The SS sends a SIP NOTIFY message informing about the status change progress. | <-- | SIP NOTIFY | - | - |
| 5 | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 6 | The SS sends a SIP NOTIFY informing about the affiliation status of the user. | <-- | SIP NOTIFY | - | - |
| 7 | The UE responds with a SIP 200 (OK) | --> | SIP 200 (OK) | - | - |
| 8 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. (NOTE 1) | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.34.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

## 5.3A	Generic test procedures for UE MCPTT operation

### 5.3A.1	MCPTT CO session establishment/modification without provisional responses other than 100 Trying

5.3A.1.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.1.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.1.3	Procedure

Table 5.3A.1.3-1: MCPTT CO session establishment/modification without provisional responses other than 100 Trying

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP INVITE requesting the establishment/modification of an MCPTT call? | --> | SIP INVITE | - | P |
| 3 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCPTT Client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| - | EXCEPTION: Steps 6a1 describes behaviour that depends on the test case requirements; the "lower case letter" identifies a step sequence that takes place if the UE requests implicit floor control in step 2 (i.e. the "mc_implicit_request" fmtp attribute included in the SDP offer and the SS responded with the "mc_implicit_request" fmtp attribute included and the “mc_granted” fmtp attribute not present in the SDP answer (NOTE1) | - | - | - | - |
| 6a1 | The SS (MCPTT server) sends a Floor Granted message. | <-- | Floor Granted | - | - |
| NOTE1: Possibilities in SDP-offer/answer depend on the test case requirements a. UE sends SDP offer without implicit floor request b. UE sends SDP offer with implicit floor request i. SDP answer from SS contains “mc_implicit_request” and “mc_granted” (Floor is implicitly granted) ii. SDP answer from SS contains “mc_implicit request” and but no “mc_granted” (Floor needs to be explicitly granted ar step 6a1) iii. SDP answer from SS contains no “mc_implicit_request”and no “mc_granted” (the UE needs to explicitly request the floor) |  |  |  |  |  |

5.3A.1.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3A.1.4-1: SIP INVITE (step 2, Table 5.3A.1.3-1)

Derivation Path: Table 5.5.2.5.1-1 with condition MCPTT

Table 5.3A.1.4-2: SIP 200 (OK) (step 4, Table 5.3A.1.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP and MCPTT

### 5.3A.2	MCPTT CO private call establishment, manual commencement

5.3A.2.1	Initial conditions

The same initial conditions apply as specified in clause 5.3.3.1.

5.3A.2.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.2.3	Procedure

Table 5.3A.2.3-1: MCPTT CO private call establishment, manual commencement

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP INVITE requesting the establishment of an MCPTT call? | --> | SIP INVITE | - | P |
| 3 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCPTT server) responds with a SIP 180 (Ringing) | <-- | SIP 180 (Ringing) | - | - |
| 5 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 6 | Check: Does the UE (MCPTT Client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| - | EXCEPTION: Steps 7a1 describes behaviour that depends on the test case requirements ; the "lower case letter" identifies a step sequence that takes place if the UE requests implicit floor control in step 2 (i.e. the "mc_implicit_request" fmtp attribute included in the SDP offer and the SS responded with the "mc_implicit_request" fmtp attribute included and the “mc_granted” fmtp attribute not present in the SDP answer (NOTE1) | - | - | - | - |
| 7a1 | The SS (MCPTT server) sends a Floor Granted message. | <-- | Floor Granted | - | - |
| NOTE1: Possibilities in SDP-offer/answer depend on the test case requirements a. UE sends SDP offer without implicit floor request b. UE sends SDP offer with implicit floor request i. SDP answer from SS contains “mc_implicit_request” and “mc_granted” (Floor is implicitly granted) ii. SDP answer from SS contains “mc_implicit request” and but no “mc_granted” (Floor needs to be explicitly granted ar step 7a1) iii. SDP answer from SS contains no “mc_implicit_request”and no “mc_granted” (the UE needs to explicitly request the floor) |  |  |  |  |  |

5.3A.2.4	Specific message contents

All message contents are as specified in clause 5.5 with condition PRIVATE-CALL where applicable and in the test case calling the procedure, with the following clarifications: Table 5.3A.2.4-1: SIP INVITE (step 2, Table 5.3A.2.3-1)

Derivation Path: Table 5.5.2.5.2-1 with condition MANUAL and PRIVATE-CALL and MCPTT

Table 5.3A.2.4-2: SIP 200 (OK) (step 5, Table 5.3A.2.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP and MCPTT

### 5.3A.3	MCPTT CO call establishment using a pre-established session

5.3A.3.1	Initial conditions

As specified in the test case which calls the procedure.

5.3A.3.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.3.3	Procedure

Table 5.3A.3.3-1: MCPTT CO call establishment using a pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1-1a7 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.13 'Generic Test Procedure for MCPTT radio bearer establishment for use of pre-established session' starting with step 2 take place. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP REFER message to request the establishment of an MCPTT call using a pre-established session? | --> | SIP REFER | - | P |
| 3 | The SS (MCPTT Server) responds with a SIP 200 (OK) message indicating that the MCPTT call has been established | <-- | SIP 200 (OK) | - | - |
| 4 | The SS sends a Connect message | <-- | Connect | - | - |
| 5 | Check: Does the UE (MCPTT Client) send an Acknowledgement in response to the Connect message? | --> | Acknowledge | - | P |

5.3A.3.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3A.4	MCPTT CO call release keeping the pre-established session

5.3A.4.1	Initial conditions

As specified in the test case which calls the procedure.

5.3A.4.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.4.3	Procedure

Table 5.3A.4.3-1: MCPTT CO call release keeping the pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a SIP REFER message with method “BYE” to release the MCPTT session and keep the pre-established session? | --> | SIP REFER | - | P |
| 2 | The SS (MCPTT Server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 3 | The SS waits 2 seconds before the SS releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. | - | - | - | - |

5.3A.4.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.4.4-1: SIP REFER (step 1, Table 5.3A.4.3-1)

Derivation Path: Table 5.5.2.12-1 with condition METHOD-BYE

### 5.3A.5	MCPTT CT call release keeping the pre-established session

5.3A.5.1	Initial conditions

As specified in the test case which calls the procedure.

5.3A.5.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.5.3	Procedure

Table 5.3A.5.3-1: MCPTT CT call release keeping the pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS (MCPTT Server) releases the call by sending a Disconnect message | <-- | Disconnect | - | - |
| 2 | Check: Does the UE (MCPTT Client) send an Acknowledgement to accept the release of the call? | --> | Acknowledge | - | P |
| 3 | The SS waits 2 seconds before the SS releases the RRC connection. NOTE: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. | - | - | - | - |

5.3A.5.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3A.6	MCPTT CO session modification with implicit Floor Control

5.3A.6.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.6.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.6.3	Procedure

Table 5.3A.6.3-1: MCPTT CO session modification
with implicit Floor Control

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a SIP INVITE requesting the modification of an MCPTT call? | --> | SIP re-INVITE | - | P |
| 2 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 3 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 4 | Check: Does the UE (MCPTT Client) send a SIP ACK to acknowledge the session modification? | --> | SIP ACK | - | P |
| 5 | The SS (MCPTT Server) sends a Floor Granted message with an acknowledgement required. | <-- | Floor Granted | - | - |
| 6 | Check: Does the UE (MCPTT Client) sends a Floor Ack message in response to the Floor Granted message? | --> | Floor Ack | - | P |

5.3A.6.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.6.4-1: SIP 200 (OK) (step 2, Table 5.3A.6.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP

### 5.3A.7	MCPTT CO session modification without implicit Floor Control

5.3A.7.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.7.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.7.3	Procedure

Table 5.3A.7.3-1: MCPTT CO session modification
without implicit Floor Control

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a SIP INVITE requesting the establishment/modification of an MCPTT call? | --> | SIP re-INVITE | - | P |
| 2 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 3 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 4 | Check: Does the UE (MCPTT Client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| 5 | Void | - | - | - | - |

5.3A.7.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.7.4-1: SIP 200 (OK) (step 2, Table 5.3A.7.3-1)

| Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message | RFC 4566 |  |
| MIME-part-body | SDP message as described in Table 5.3A.7.4-2 |  |  |  |

Table 5.3A.7.4-2: SDP in SIP 200 (OK) (Table 5.3A.7.4-1)

| Derivation Path: Table 5.5.3.1.2-1 SDP Message from the SS for MCPTT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media description[2] |  | Media description for media control |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format specific parameters |  |  |  |  |
| mc_implicit_request | Not present | Parameter has no value | TS 24.380 [10] cl. 12.1.2.3 |  |

### 5.3A.8	MCPTT CT Call establishment automatic commencement using a pre-established session

5.3A.8.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.8.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.8.3	Procedure

Table 5.3A.8.3-1: MCPTT CT Call establishment automatic commencement using a pre-established session

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | E-UTRA/EPC signalling according to clause 5.4.13 'Generic Test Procedure for MCPTT radio bearer establishment for use of pre-established session' takes place | - | - | - | - |
| 2 | SS initiates an on-demand pre-arranged group call with automatic commencement mode using a pre-established session by sending a Connect message | <-- | Connect | - | - |
| 3 | Check: Does the UE (MCPTT client) send an Acknowledgement to accept the incoming pre-arranged group call using a pre-established session? | --> | Acknowledge | - | P |

5.3A.8.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3A.9	UE initiated MCPTT functional alias status determination and subscription

5.3A.9.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.9.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.9.3	Procedure

Table 5.3A.9.3-1: MCPTT functional alias status determination and subscription

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCPTT User request to determine the current status of a functional alias and later notification of status changes of a functional alias. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 2a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 3 | Check: Does the UE (MCPTT Client) send a SIP SUBSCRIBE requesting the status of any existing functional aliases? | --> | SIP SUBSCRIBE | - | P |
| 4 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | The SS (MCPTT server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 6 | Check: Does the UE (MCPTT Client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 2) | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI NOTE 2: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3A.9.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3A.9.4-1: SIP SUBSCRIBE (step 3, Table 5.3A.9.3-1)

| Derivation Path: Table 5.5.2.14-1 with condition MCPTT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| value | "4294967295" | to receive the current status and later notification | TS 24.379 [9] clause 9A.2.1.3 |  |
| Message-body |  |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3A.9.4-2 |  |  |  |

Table 5.3A.9.4-2: MCPTT-Info in SIP SUBSCRIBE (Table 5.3A.9.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.3 |  |

Table 5.3A.9.4-3: SIP 200 (OK) (step 4, Table 5.3A.9.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition SUBSCRIBE-RSP

Table 5.3A.9.4-4: SIP NOTIFY (step 5, Table 5.3A.9.3-1)

| Derivation Path: Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF | TS 24.379 [9] clause 9A.2.2.2.5 |  |
| MIME-part-body | PIDF as described in Table 5.3A.9.4-5 |  |  |  |

Table 5.3A.9.4-5: PIDF in SIP NOTIFY (Table 5.3A.9.4-4)

Derivation Path: Table 5.5.3.5.2-1 (NOTE 1)

NOTE 1: PIDF document contains tuple with empty <status> element (i.e. there are no <functionalAlias> entries at all) and not containing a <p-id-fa> element

### 5.3A.10	UE initiated MCPTT functional alias status change

5.3A.10.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.10.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.10.3	Procedure

Table 5.3A.10.3-1: MCPTT functional alias status change

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCPTT User request to change the status of a functional alias to "activated". (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 2a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 3 | Check: Does the UE (MCPTT Client) send a SIP PUBLISH requesting the status change of a functional alias? | --> | SIP PUBLISH | - | P |
| 4 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | The SS (MCPTT server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 6 | Check: Does the UE (MCPTT Client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 2) | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI NOTE 2: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3A.10.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3A.10.4-1: SIP PUBLISH (step 3, Table 5.3A.10.3-1)

| Derivation Path: Table 5.5.2.11-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info | TS 24.379 [9] clause 9A.2.1.2 |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3A.10.4-2 |  |  |  |
| MIME body part |  | PIDF | TS 24.379 [9] clause 9A.2.1.2 |  |
| MIME-part-body | PIDF as described in Table 5.3A.10.4-3 |  |  |  |

Table 5.3A.10.4-2: MCPTT-Info in SIP PUBLISH (Table 5.3A.10.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.2 |  |

Table 5.3A.10.4-3: PIDF in SIP PUBLISH (Table 5.3A.10.4-1)

Derivation Path: Table 5.5.3.5.1-1 with condition FUNCTIONAL_ALIAS_STATUS_CHANGE

Table 5.3A.10.4-4: SIP 200 (OK) (step 4, Table 5.3A.10.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition PUBLISH-RSP

Table 5.3A.10.4-5: SIP NOTIFY (step 5, Table 5.3A.10.3-1)

| Derivation Path: Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF | TS 24.379 [9] clause 9A.2.2.2.5 |  |
| MIME-part-body | PIDF as described in Table 5.3A.10.4-6 |  |  |  |

Table 5.3A.10.4-6: PIDF in SIP NOTIFY (Table 5.3A.10.4-5)

Derivation Path: Table 5.5.3.5.2-1 with condition FUNCTIONAL_ALIAS_ACTIVATED, NOTIFY_FOR_PUBLISH

### 5.3A.11	MCPTT Floor Request – Floor Granted

5.3A.11.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.11.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.11.3	Procedure

Table 5.3A.11.3-1: MCPTT Floor Request – Floor Granted

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a Floor Request message? | --> | Floor Request | - | P |
| 2 | The SS (MCPTT Server) sends a Floor Granted message with an acknowledgement required. | <-- | Floor Granted | - | - |
| 3 | Check: Does the UE (MCPTT Client) send a Floor Ack message in response to the Floor Granted message? | --> | Floor Ack | - | P |
| 4 | Check: Does the UE (MCPTT Client) provide floor granted notification to the MCPTT User? (NOTE 1) | - | - | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3A.11.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3A.11.4-1: Floor Granted (Step 2, Table 5.3.16.3-1)

Derivation Path: Table 5.5.6.3-1 condition ACK

### 5.3A.12	MCPTT Floor Request – Floor Queue Position Info

5.3A.12.1	Initial conditions

As specified in the test case which calls the procedure.

5.3A.12.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.12.3	Procedure

Table 5.3A.12.3-1: MCPTT Floor Request – Floor Queue Position Info

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a Floor Request message? | --> | Floor Request | - | P |
| 2 | The SS (MCPTT Server) sends a Floor Queue Position Info message indicating that the Floor Request was queued message with no acknowledgement required. | <-- | Floor Queue Position Info | - | - |

5.3A.12.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none.

### 5.3A.13	MCPTT Queuing Position Request

5.3A.13.1	Initial conditions

As specified in the test case which calls the procedure.

5.3A.13.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.13.3	Procedure

Table 5.3A.13.3-1: MCPTT Queuing Position Request

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a Floor Queue Position Request message? | --> | Floor Queue Position Request | - | P |
| 2 | The SS (MCPTT Server) responds with a Floor Queue Position Info message with no acknowledgement required. | <-- | Floor Queue Position Info | - | - |

5.3A.13.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3A.14	MCPTT Floor Request – Floor Deny

5.3A.14.1	Initial conditions

As specified in the test case which calls the procedure.

5.3A.14.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.14.3	Procedure

Table 5.3A.14.3-1: MCPTT Floor Request – Floor Deny

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a Floor Request message? | --> | Floor Request | - | P |
| 2 | The SS (MCPTT Server) sends a Floor Deny message with no acknowledgement required | <-- | Floor Deny | - | - |
| 3 | Check: Does the UE (MCPTT Client) provide floor deny notification to the MCPTT User? (NOTE 1) | - | - | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3A.14.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3A.15	MCPTT Floor Release – Floor Idle

5.3A.15.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.15.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.15.3	Procedure

Table 5.3A.15.3-1: MCPTT Floor Release – Floor Idle

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a Floor Release message? | --> | Floor Release | - | P |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE requests an acknowledgement to the Floor Release message. | - | - | - | - |
| 2a1 | The SS (MCPTT Server) sends a Floor Ack message in response to the Floor Release message | <-- | Floor Ack | - | - |
| 3 | The SS (MCPTT Server) sends a Floor Idle message with no acknowledgement required. | <-- | Floor Idle | - | - |

5.3A.15.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3A.16	MCPTT Floor Release – Floor Taken

5.3A.16.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.16.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.16.3	Procedure

Table 5.3A.16.3-1: MCPTT Floor Release – Floor Taken

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) send a Floor Release message? | --> | Floor Release | - | P |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE requests an acknowledgement to the Floor Release message. | - | - | - | - |
| 2a1 | The SS (MCPTT Server) sends a Floor Ack message in response to the Floor Release message | <-- | Floor Ack | - | - |
| 3 | The SS (MCPTT Server) sends a Floor Taken message with no acknowledgement required. | <-- | Floor Taken | - | - |

5.3A.16.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

## 5.3B	Generic test procedures for UE MCVideo operation

### 5.3B.1	MCVideo CO session establishment/modification without provisional responses other than 100 Trying

5.3B.1.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3B.1.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.1.3	Procedure

Table 5.3B.1.3-1: MCVideo CO session establishment/modification without provisional responses other than 100 Trying

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCVideo call establishment described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCVideo Client) send a SIP INVITE requesting the establishment/modification of an MCVideo call? | --> | SIP INVITE | - | P |
| 3 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCVideo server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCVideo Client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| - | EXCEPTION: Steps 6a1-6a2 describe behaviour that depends on the test case requirements; the "lower case letter" identifies a step sequence that takes place if the UE requests implicit floor control in step 2 (i.e. the "mc_implicit_request" fmtp attribute included in the SDP offer and the SS responded with the "mc_implicit_request" fmtp attribute included and the “mc_granted” fmtp attribute not present in the SDP answer (NOTE1) | - | - | - | - |
| 6a1 | The SS (MCVideo server) sends a Floor Granted message. | <-- | Transmission Granted | - | - |
| 6a2 | Check: Does the UE (MCVideo Client) send a Transmission Control Ack message acknowledging the Transmission Granted message from the SS (MCVideo Server)? | --> | Transmission Control Ack | - | P |
| NOTE1: Possibilities in SDP-offer/answer depend on the test case requirements a. UE sends SDP offer without implicit floor request b. UE sends SDP offer with implicit floor request i. SDP answer from SS contains “mc_implicit_request” and “mc_granted” (Floor is implicitly granted) ii. SDP answer from SS contains “mc_implicit request” and but no “mc_granted” (Floor needs to be explicitly granted ar step 6a1) iii. SDP answer from SS contains no “mc_implicit_request”and no “mc_granted” (the UE needs to explicitly request the floor) |  |  |  |  |  |

5.3B.1.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3B.1.4-1: SIP INVITE (step 2, Table 5.3B.1.3-1)

Derivation Path: Table 5.5.2.5.1-1 condition MCVIDEO

Table 5.3B.1.4-2: SIP 200 (OK) (step 4, Table 5.3B.1.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 condition INVITE-RSP and MCVIDEO

Table 5.3B.1.4-3: Transmission Granted (step 6a1, Table 5.3B.1.3-1)

Derivation Path: Table 5.5.11.2.1-1, condition ACK

Table 5.3B.1.4-4: Transmission Control Ack (step 6a2, Table 5.3B.1.3-1)

| Derivation Path: Table 5.5.11.3.5-1, condition UPLINK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message Type |  |  |  |  |
| Message Type | "00010000" | Transmission Control Ack message for Transmission Granted message which requested acknowledgment. |  |  |

### 5.3B.2	MCVideo Transmission request – Transmission Granted

5.3B.2.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3B.2.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.2.3	Procedure

Table 5.3B.2.3-1: MCVideo Transmission Request – Transmission Granted

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo Client) send a Transmission Request message? | --> | Transmission Request | - | P |
| 2 | The SS (MCVideo Server) sends a Transmission Granted message with an acknowledgement required. | <-- | Transmission Granted | - | - |
| 3 | Check: Does the UE (MCVideo Client) send a Transmission Control Ack message in response to the Transmission Granted message? | --> | Transmission Control Ack | - | P |
| 4 | Check: Does the UE (MCVideo Client) provide transmission granted notification to the MCVideo User? (NOTE 1) | - | - | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.2.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.2.4-1: Transmission Granted (step 2, Table 5.3B.2.3-1)

Derivation Path: Table 5.5.11.2.1-1 condition ACK

Table 5.3B.2.4-2: Transmission Control Ack (step 3, Table 5.3B.2.3-1)

| Derivation Path: Table 5.5.11.3.5-1, condition UPLINK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message Type |  |  |  |  |
| Message Type | "00010000" | Transmission Control Ack message for Transmission Granted message which requested acknowledgment. |  |  |

### 5.3B.3	MCVideo Media Transmission Notification and Request CT

5.3B.3.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3B.3.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.3.3	Procedure

Table 5.3B.3.3-1: MCVideo Media Transmission Notification and Request CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCVideo Server) sends a Media Transmission Notification message to the UE (MCVideo Client) | <- | Media Transmission Notification | - | - |
| 2 | Check: Does the UE (MCVideo Client) provide media transmission notification to the MCVideo User? (NOTE 1) | - | - | - | P |
| 3 | Make the MCVideo User request permission to receive media. (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE (MCVideo Client) send a Receive Media Request message to the SS (MCVideo Server)? | --> | Receive Media Request | - | P |
| 5 | The SS (MCVideo Server) sends a Receive Media Response message to the UE (MCVideo Client). | <- | Receive Media Response | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.3.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3B.4	MCVideo Transmission Request - Queue Position Info

5.3B.4.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.4.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.4.3	Procedure

Table 5.3B.4.3-1: MCVideo Transmission Request – Queue Position Info

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo Client) send a Transmission Request message? | --> | Transmission Request | - | P |
| 2 | The SS (MCVidao Server) sends a Queue Position Info message indicating that the Transmission Request was queued message with no acknowledgement required. | <-- | Queue Position Info | - | - |

5.3B.4.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none.

### 5.3B.5	MCVideo Queue Position Request

5.3B.5.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.5.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.5.3	Procedure

Table 5.3B.5.3-1: MCVideo Queue Position Request

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo Client) send a Queue Position Request message? | --> | Queue Position Request | - | P |
| 2 | The SS (MCVideo Server) responds with a Queue Position Info message with no acknowledgement required. | <-- | Queue Position Info | - | - |

5.3B.5.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3B.6	MCVideo Transmission Request - Transmission Rejected

5.3B.6.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.6.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.6.3	Procedure

Table 5.3B.6.3-1: MCVideo Transmission Request – Transmission Rejected

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo Client) send a Transmission Request message? | --> | Transmission Request | - | P |
| 2 | The SS (MCVideo Server) sends a Transmission Rejected message with no acknowledgement required | <-- | Transmission Rejected | - | - |
| 3 | Check: Does the UE (MCVideo Client) provide Transmission deny notification to the MCVideo User? (NOTE 1) | - | - | - | P |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.6.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3B.7	MCVideo Transmission End Request CO

5.3B.7.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.7.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.7.3	Procedure

Table 5.3B.7.3-1: MCVideo transmission End Request CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo Client) send a Transmission End Request message indicating that it wants to terminate a MCVideo On-Demand Pre-Arranged Emergency Group Call, with implicit Transmission Control? | --> | Transmission End Request | - | P |
| 2 | The SS (MCVideo Server) responds with a Transmission End Response message verifying that the UE (MCVideo Client) is able to end an MCVideo On-Demand Pre-Arranged Emergency Group Call, with implicit Transmission Control. | <-- | Transmission End Response | - | - |
| 3 | Check: Does the UE (MCVideo Client) send a Transmit Control ACK message? | --> | Transmission Control ACK | - | P |
| 4 | The SS (MCVideo Server) sends a Transmission Idle message. Do I need this message? No difference whether ending an emergency call or normal call? | <-- | Transmission Idle |  |  |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.7.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.7.4-1: Transmission End Response (Step 2, Table 5.3B.7.3-1)

Derivation Path: Table 5.5.11.3.2-1, condition ACK

Table 5.3B.7.4-2: Transmission Control Ack (Step 3, Table 5.3B.7.3-1)

| Derivation Path: Table 5.5.11.3.5-1 condition UPLINK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message Type |  |  |  |  |
| Message Type | "00010001" | Transmission Control Ack message for Transmission End Response message which requested acknowledgment. |  |  |

### 5.3B.8	MCVideo Reception End Request CO

5.3B.8.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.8.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.8.3	Procedure

Table 5.3B.8.3-1: MCVideo Media Reception End Request CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo Client) send a Media Reception End Request to indicate it wants to stop RTP packet media? | --> | Media Reception End Request | - | P |
| 2 | The SS (MCVideo Server) sends a Receive Media Reception End Response message to the UE (MCVideo Client). | <-- | Media Reception End Response | - | - |
| 3 | The SS (MCVideo Server) sends a Transmission Idle message. | <-- | Transmission Idle | - | - |

5.3B.8.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3B.9	MCVideo Transmission End Request CT

5.3B.9.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.9.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.9.3	Procedure

Table 5.3B.9.3-1: MCVideo Transmission End Request CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCVideo Server) sends the Transmission end request. | <-- | Transmission End Request | - | - |
| 2 | Check Does the UE (MCVideo Client) inform the MCVideo User that the permission to send RTP media is being revoked? (NOTE 1) | - | - | - | P |
| 3 | Check: Does the UE (MCVideo Client) respond to the Transmission end request? | --> | Transmission End Response | - | P |
| 4 | The SS (MCVideo Server) sends a Transmission Idle message. | <-- | Transmission Idle | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.9.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3B.10	MCVideo Media Reception End Request CT

5.3B.10.1	Initial conditions

As specified in the test case which calls the procedure.

5.3B.10.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.10.3	Procedure

Table 5.3B.10.3-1: MCVideo Media Reception End Request CT

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCVideo Server) sends the Transmission end request. | <-- | Media Reception End Request | - | - |
| 2 | Check Does the UE (MCVideo Client) inform the MCVideo User that the permission to send RTP media is being revoked? (NOTE 1) | - | - | - | P |
| 3 | Check: Does the UE (MCVideo Client) respond to the Transmission end request? | --> | Media Reception End Response | - | P |
| 4 | The SS (MCVideo Server) sends a Transmission Idle message. | <-- | Transmission Idle | - | - |
| NOTE 1: This expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

5.3B.10.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

None

### 5.3B.11	MCVideo CO session modification with implicit Transmission Control

5.3B.11.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3B.11.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3B.113	Procedure

Table 5.3B.11.3-1: MCVideo CO session modification
with implicit TransmissionControl

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCVideo Client) send a SIP INVITE requesting the modification of an MCVideo call? | --> | SIP re-INVITE | - | P |
| 2 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 3 | The SS (MCVideo server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 4 | Check: Does the UE (MCVideo Client) send a SIP ACK to acknowledge the session modification? | --> | SIP ACK | - | P |
| 5 | The SS (MCVideo Server) sends a Transmission Granted message with an acknowledgement required. | <-- | Transmission Granted | - | - |
| 6 | Check: Does the UE (MCVideo Client) send a Transmission Control Ack message in response to the Transmission Granted message? | --> | Transmission Control Ack | - | P |

5.3B.11.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3B.11.4-1: SIP 200 (OK) (step 2, Table 5.3B.11.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP

Table 5.3B.11.4-2: Transmission Granted (step 5, Table 5.3B.11.3-1)

Derivation Path: Table 5.5.11.2.1-1 condition ACK

Table 5.3B.11.4-3: Transmission Control Ack (step 6, Table 5.3B.11.3-1)

| Derivation Path: Table 5.5.11.3.5-1, condition UPLINK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message Type |  |  |  |  |
| Message Type | "00010000" | Transmission Control Ack message for Transmission Granted message which requested acknowledgment. |  |  |

## 5.3C	Generic test procedures for UE MCData operation

### 5.3C.1	CO SDS or FD message transfer using signalling plane

5.3C.1.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.1.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.1.3	Procedure

Table 5.3C.1.3-1: CO SDS or FD message transfer using signalling plane

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCData Client) send a SIP MESSAGE request? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCData server) sends a SIP 202 (Accepted) response | <-- | SIP 202 (Accepted) | - | - |
| 4 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection (NOTE 1). | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3C.1.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3C.2	CO MCData Call Establishment

5.3C.2.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.2.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.2.3	Procedure

Table 5.3C.2.3-1: CO MCData Call Establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP INVITE requesting the establishment of an MCData call? | --> | SIP INVITE | - | P |
| 3 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCData server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCData client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| 6 | The UE (MCData client) connects to the TCP server at the SS side to establish an MSRP connection (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCData Client) send an empty MSRP SEND request to bind the TCP connection to the MSRP session? | --> | MSRP SEND | - | P |
| 8 | The SS (MCData Server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| NOTE 1: According to TS 24.282 [87] clauses 9.2.3.4.2, 9.2.4.4.2 and 10.2.5.4.2 the SS sets the a=setup attribute set to "passive" (see table 5.5.3.1.2-3) The UE's MCData client has the role of the active endpoint |  |  |  |  |  |

5.3C.2.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.2.4-1: MSRP SEND (Step 7, Table 5.3C.2.3-1)

Derivation Path: Table 5.5.12.1-1, condition EMPTY_SEND_REQ

### 5.3C.3	CT MCData Call Establishment

5.3C.3.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.3.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.3.3	Procedure

Table 5.3C.3.3-1: CT MCData Call Establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which described in clause 5.4.4 'Generic Test Procedure for MCX CT communication in E-UTRA' take place. | - | - | - | - |
| 2 | The SS (MCX Server) sends a SIP INVITE requesting the establishment of an MCData call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 3a1 | The UE (MCX client) sends a SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 4 | Check: Does the UE (MCX client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 5 | The SS (MCX server) sends a SIP ACK | <-- | SIP ACK | - | - |
| - | EXCEPTION: Steps 6a1 - 6b3 describe behaviour that depends on which role of an endpoint the UE (MCData Client) has chosen in its SDP answer sent at step 4 | - | - | - | - |
| 6a1 | IF the UE (MCData Client) acts as passive endpoint (NOTE 1) THEN the SS connects to the TCP server at the UE side to establish an MSRP connection | - | - | - | - |
| 6a2 | The SS sends an empty MSRP SEND request to bind the TCP connection to the MSRP session. | <-- | MSRP SEND | - | - |
| 6a3 | Check: Does the UE (MCData Client) send an MSRP 200 (OK) response? | --> | MSRP 200 (OK) | - | P |
| 6b1 | ELSE (NOTE 2) the UE (MCData client) connects to the TCP server at the SS side to establish an MSRP connection | - | - | - | - |
| 6b2 | Check: Does the UE (MCData Client) send an empty MSRP SEND request to bind the TCP connection to the MSRP session? | --> | MSRP SEND | - | P |
| 6b3 | The SS (MCData Server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| NOTE 1: The MCData Client indicates to act as passive endpoint by setting the a=setup attribute of the SDP answer at step 4 to "passive" (according to RFC 4145 [119]) NOTE 2: The MCData Client indicates to act as active endpoint by setting the a=setup attribute of the SDP answer at step 4 to "active" (according to RFC 4145 [119]) |  |  |  |  |  |

5.3C.3.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.3.4-1: MSRP SEND (Step 6a2, Table 5.3C.3.3-1)

Derivation Path: Table 5.5.12.2-1, condition EMPTY_SEND_REQ

Table 5.3C.3.4-2: MSRP SEND (Step 6b2, Table 5.3C.3.3-1)

Derivation Path: Table 5.5.12.1-1, condition EMPTY_SEND_REQ

### 5.3C.4	CO MSRP message transfer

5.3C.4.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.4.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.4.3	Procedure

Table 5.3C.4.3-1: CO MSRP message transfer

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1-2 are repeated until the UE (MCData client) indicates the end of the message by setting the continuation-flag to "$" in the End-line of the MSRP SEND request at step 1 | - | - | - | - |
| 1 | Check: Does the UE (MCData Client) send an MSRP SEND request? | --> | MSRP SEND | - | P |
| 2 | The SS (MCData Server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| 3 | In case of chunking the SS reassembles the data contained in the bodies of the MSRP SEND requests (NOTE 1) | - | - | - | - |
| NOTE 1: In case of no chunking there is only one MSRP SEND request which contains the entire data. In case of chunking there are more than one MSRP SEND requests containing the chunks of data and the content type shall be the same for all MSRP SEND requests. |  |  |  |  |  |

5.3C.4.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3C.5	CT MSRP message transfer

5.3C.5.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.5.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.5.3	Procedure

Table 5.3C.5.3-1: CT MSRP message transfer

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an MSRP SEND request containing the entire data (NOTE 1). | <-- | MSRP SEND | - | - |
| 2 | Check: Does the UE (MCData Client) send an MSRP 200 (OK) response? | --> | MSRP 200 (OK) | - | P |
| NOTE 1: No chunking is applied in DL. |  |  |  |  |  |

5.3C.5.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3C.6	CO MCData call release

5.3C.6.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.6.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.6.3	Procedure

Table 5.3C.6.3-1: CO MCData call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) send a SIP BYE request to terminate the MCData communication? | --> | SIP BYE | - | P |
| 2 | The SS (MCData Server) sends a SIP 200 (OK) response. | <-- | SIP 200 (OK) | - | - |
| - | EXCEPTION: Steps 3a1 - 3b1 describe behaviour that depends on the endpoint role the UE (MCData Client) has chosen at call establishment (NOTE 1) | - | - | - | - |
| 3a1 | IF the client is the active endpoint THEN the SS waits 3s for the client to close the MSRP TCP connection (NOTE 2) | - | - | - | - |
| 3b1 | ELSE the SS closes the MSRP TCP connection (NOTE 3) | - | - | - | - |
| 4 | The SS waits 2 seconds before it deactivates the dedicated EPS bearer (NOTE 4, 5). | - | - | - | - |
| NOTE 1: The endpoint role is negotiated in the SDP signalling at call establishment (table 5.3C.2.3-1 and 5.3C.3.3-1) NOTE 2: After the wait period the SS may stop the MSRP TCP server independent from whether or not the UE has closed the connection. NOTE 3: When the SS has the role of the active endpoint it means that the MCData client hosts the TCP server of the MSRP connection. NOTE 4: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. NOTE 5: The RRC connection is kept to allow subsequent signalling using the control plane as e.g. an SDS NOTIFICATION in case of Standalone SDS. |  |  |  |  |  |

5.3C.6.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3C.7	CT MCData call release

5.3C.7.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.7.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.7.3	Procedure

Table 5.3C.7.3-1: CT MCData call release

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS (MCData server) sends a SIP BYE request to terminate the MCData comminication. | <-- | SIP BYE | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP 200 (OK) response? | --> | SIP 200 (OK) | - | P |
| - | EXCEPTION: Steps 3a1 - 3b1 describe behaviour that depends on the endpoint role the UE (MCData Client) has chosen at call establishment (NOTE 1) | - | - | - | - |
| 3a1 | IF the client is the active endpoint THEN the SS waits 3s for the client to close the MSRP TCP connection (NOTE 2) | - | - | - | - |
| 3b1 | ELSE the SS closes the MSRP TCP connection (NOTE 3) | - | - | - | - |
| 4 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer (NOTE 4, 5). | - | - | - | - |
| NOTE 1: The endpoint role is negotiated in the SDP signalling at call establishment (table 5.3C.2.3-1 and 5.3C.3.3-1) NOTE 2: After the wait period the SS may stop the MSRP TCP server independent from whether or not the UE has closed the connection.. NOTE 3: When the SS has the role of the active endpoint it means that the MCData client hosts the TCP server of the MSRP connection. NOTE 4: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished and any not allowed behaviour captured. NOTE 5: The RRC connection is kept to allow subsequent signalling using the control plane as e.g. an SDS NOTIFICATION in case of Standalone SDS. |  |  |  |  |  |

5.3C.7.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3C.8	Discovery of the absolute URI of the media storage function (one-to-one communication)

5.3C.8.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.8.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.8.3	Procedure

Table 5.3C.8.3-1: Discovery of the absolute URI of the media storage function (one-to-one)

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called and on the UE implementation. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state and pc_MCData_MSFDiscoverySignalling, the E-UTRA/EPC actions described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| - | EXCEPTION: Steps 2a1 – 2b1 describe behaviour that depends on the UE implementation | - | - | - | - |
| 2a1 | IF pc_MCData_MSFDiscoverySignalling THEN Check: Does the UE (MCData Client) send a SIP MESSAGE request to discover the absolute URI of the media storage function? | --> | SIP MESSAGE | - | P |
| 2a2 | The SS (MCData server) sends a SIP 200 (OK) response. | <-- | SIP 200 (OK) | - | - |
| 2a3 | The SS (MCData server) sends a SIP MESSAGE request containing the absolute URI of the media storage function in the <mcdata-controller-psi> element of the mcdata-info. | <-- | SIP MESSAGE | - | - |
| 2a4 | Check: Does the UE (MCData client) send a SIP 200 (OK) response? | --> | SIP 200 (OK) | - | P |
| 2b1 | ELSE the UE determines the value of the absolute URI associated with the media storage function of the MCData content server from the <MCDataContentServerURI> element of the MCData user profile document | - | - | - | - |

5.3C.8.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.8.4-1: SIP MESSAGE from the UE (step 2a1, Table 5.3C.8.3-1)

| Derivation Path: Table 5.5.2.7.1-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 5.3C.8.4-2 |  |  |  |

Table 5.3C.8.4-2: MCDATA-Info from the UE (Table 5.3C.8.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "msf-disc-req" |  |  |  |

Table 5.3C.8.4-3: SIP MESSAGE from the SS (step 2a3, Table 5.3C.8.3-1)

| Derivation Path: Table 5.5.2.7.2-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | tsc_MCData_PublicServiceId_A | According to TS 24.282 [87] clause 10.2.1.3.3 the participating function just forwards the SIP MESSAGE received from the controlling function to the client |  |  |
| Accept-Contact |  |  |  |  |
| ac-value[2] | not present |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_A_1 | Public user ID of the calling MCData user (TS 24.282 [87] clause 10.2.1.3.4) |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 5.3C.8.4-4 |  |  |  |

Table 5.3C.8.4-4: MCDATA-Info from the SS (Table 5.3C.8.4-3)

| Derivation Path: Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "msf-disc-res" |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-controller-psi | Encrypted <mcdata-controller-psi> with mcdataURI set to tsc_MCData_MSF_URI | Encrypted according to Table 5.5.3.2.2-3A |  |  |

### 5.3C.9	Discovery of the absolute URI of the media storage function (group communication)

5.3C.9.1	Initial conditions

Same as 5.3C.8.1.

5.3C.9.2	Definition of system information messages

Same as 5.3C.8.2.

5.3C.9.3	Procedure

Same as 5.3C.8.3.

5.3C.9.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.9.4-1: SIP MESSAGE from the UE (step 2a1, Table 5.3C.8.3-1)

| Derivation Path: Table 5.5.2.7.1-1, condition MCDATA_FD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 5.3C.9.4-2 |  |  |  |

Table 5.3C.9.4-2: MCDATA-Info from the UE (Table 5.3C.9.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "msf-disc-req" |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encrypted according to Table 5.5.3.2.1-3A |  |  |

Table 5.3C.9.4-3: SIP MESSAGE from the SS (step 2a3, Table 5.3C.8.3-1)

Same as Table 5.3C.8.4-3

### 5.3C.10	FD file upload using HTTP

5.3C.10.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.10.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.10.3	Procedure

Table 5.3C.10.3-1: FD file upload using HTTP

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send an HTTP POST request to upload a file to the media storage function? | --> | HTTP POST | - | P |
| 3 | The SS (MCData server) sends an HTPP 201 Created response containing a Location header field with a URL identifying the location of the resource where the file has been stored at the media storage function. | <-- | HTTP 201 Created | - | - |
| 4 | Check: Does the UE (MCData client) send a SIP MESSAGE request containing an FD SIGNALLING PAYLOAD with Payload content type "FILEURL" and with the Payload data containing the URL of the file? | --> | SIP MESSAGE | - | P |
| 5 | The SS (MCData server) sends a SIP 202 (Accepted) response | <-- | SIP 202 (Accepted) | - | - |
| 6 | The SS waits 2 seconds before the SS releases the RRC connection (NOTE 1). | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3C.10.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none

### 5.3C.11	FD file accept and download using HTTP

5.3C.11.1	Initial conditions

As specified in the test case which calls the procedure.

5.3C.11.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3C.11.3	Procedure

Table 5.3C.11.3-1: FD file accept and download using HTTP

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP MESSAGE request containing an FD NOTIFICATION with FD disposition notification type "FILE DOWNLOAD REQUEST ACCEPTED"? | --> | SIP MESSAGE | - | P |
| 3 | The SS (MCData server) sends a SIP 202 (Accepted) response | <-- | SIP 202 (Accepted) | - | - |
| 4 | Check: Does the UE (MCData client) send an HTTP GET request to download the file? | --> | HTTP GET | - | P |
| 5 | SS (MCData server) sends an HTTP 200 OK response containing the requested file. | <-- | HTTP 200 OK | - | - |
| - | EXCEPTION: Steps 6a1 describes behaviour that depends on the test case requirements; the "lower case letter" identifies a step sequence that takes place when the SS has included a FD disposition request of "FILE DOWNLOAD COMPLETED UPDATE" in the FD SIGNALLING PAYLOAD | - | - | - | - |
| 6a1 | Check: Does the UE (MCData client) send a SIP MESSAGE request containing an FD NOTIFICATION with disposition notification type "FILE DOWNLOAD COMPLETED"? | --> | SIP MESSAGE | - | P |
| 6a2 | The SS (MCData server) sends a SIP 202 (Accepted) response | <-- | SIP 202 (Accepted) | - | - |
| 7 | The SS waits 2 seconds before the SS releases the RRC connection (NOTE 1). | - | - | - | - |
| NOTE 1: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3C.11.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

none