3GPP TSG-RAN5 Meeting #95-e	R5-222478r2

# Online,  , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0728 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update FR2 TRx MU in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MU and TT for QZ size <= 40 cm are not defined. Some of MTSU derivations in Table F.1.2-1 and F.1.3-1 are not updated. Following TCs are placed in wrong rows in the tables. - General ON/OFF time mask for UL MIMO - SRS time mask for UL MIMO - EIS spherical coverage Test cases for Transmit OFF power for CA is voided in clause 6, but stil exists in Annex F. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following MU and TT for Max Device size <= 40 cm were defined. - MOP (EIRP) in Table F.1.2-1 and F.3.2-1 - MOP (TRP) in Table F.1.2-1 and F.3.2-1 - Reference sensitivity in Table F.1.3-1 and F.3.3-1 Derivations of MTSU were updated in Table F.1.2-1 and F.1.3-1. Following TCs were moved to correct rows in the tables. - General ON/OFF time mask for UL MIMO in Table F.1.2-1 - SRS time mask for UL MIMO in Table F.1.2-1 - EIS spherical coverage in Table F.1.3-1 and F.3.3-1 Transmit OFF power for CA was removed from Table F.1.2-1 and F.3.2-1. Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QZ size <= 40 cm will not be tested. Errors for MU and TT will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2, F.1.3, F.3.2, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.903 CR R5-222479 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on the outcome of MU discussion in R5-222477. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: WIC was corrected. r2: Changes for QZ size <= 40 cm were canceled. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

F.1	Acceptable uncertainty of Test System (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. Care should be taken to ensure that each conformance test implementation including the OTA chamber aspects meets the specified measurement uncertainty for each test case by requiring the test laboratory to maintain a detailed measurement uncertainty test report showing compliance to all the measurement uncertainty requirements. The detailed measurement uncertainty report would contain the justification for each measurement uncertainty component and its value and distribution. The derivation of these values is based on the minimum conformance requirements plus relaxation, i.e., test tolerance is not to be considered. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

The downlink signal uncertainties apply at the defined quiet zone with the UE properly positioned in the quiet zone. The uplink signal uncertainties apply at the measurement equipment with the UE positioned properly in the quiet zone.

F.1.1	Measurement of test environments

Editor’s note: Various measurement accuracies for UE test environments, e.g., pressure, relative humidity, DC&AC voltage, vibration, and vibration frequency, are FFS:

The measurement accuracy of the UE test environments defined in TS 38.508-1 [5] subclause 4.1, Test environments shall be

-	Temperature			±4 degrees.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause |  | Maximum Test System Uncertainty |  | Derivation of MTSU |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) |  | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |  |
| 6.2.1.1 UE maximum output power (TRP) |  | PC3 Max TRP Max Device size ≤ 30 cm ±4.42 dB (FR2a, NTC testing) ±4.62 dB (FR2b, NTC testing) ±4.70 dB (FR2a, ETC testing) ±4.90 dB (FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) |  | PC3 Max Device size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) |  | MTSU = 1.00 x MU (from Table B.3-3 in TR 38.903) |  |
| 6.2.2 UE maximum output power reduction |  | PC3 Max Device size ≤ 30 cm ±4.92 dB (FR2a), NTC testing ±5.10 dB (FR2b), NTC testing ±5.20 dB (FR2a, ETC testing) ±5.38 dB (FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |  |
| 6.2.3 UE maximum output power with additional requirements |  | Same as 6.2.2 |  |  |  |
| 6.2.4 Configured transmitted power |  | TBD |  |  |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |  |
| 6.2A.2.2 UE maximum output power reduction for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.3 UE maximum output power reduction for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.4 UE maximum output power reduction for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.5 UE maximum output power reduction for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.6 UE maximum output power reduction for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.7 UE maximum output power reduction for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO |  | Same as 6.2.2 |  |  |  |
| 6.3.1 Minimum output power |  | PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm FFS (FR2a) FFS dB (FR2b) PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing ±6.41 dB (FR2a & FR2b, ETC testing)) |  | MTSU = 1.00 x MU (from Table B.7-1 in TR 38.903) |  |
| 6.3.2 Transmit OFF power |  | PC3: Max Device size ≤ 30 cm ±5.49 dB (FR2a) |  | MTSU = 1.00 x MU (from Table B.8-1 in TR 38.903) |  |
| 6.3.3.2 General ON/OFF time mask |  | ON power: Same as 6.2.1.1 (EIRP) for the respective power class OFF power: Same as 6.3.1 for the respective power class |  |  |  |
| 6.3.3.4 PRACH time mask |  | PC3: PRACH power: TBD OFF power: Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing) ±6.41 dB (FR2a & FR2b, ETC testing) |  |  |  |
| 6.3.3.6 SRS time mask |  | TBD |  |  |  |
| 6.3.4.2 Absolute power tolerance |  | PC3 Max Device size ≤ 30 cm ±8.05 dB (FR2a & FR2b, NTC testing) ±8.42 dB (FR2a & FR2b, ETC testing) |  | MTSU = SQRT (UL Meas Uncer2 + DL Meas Uncer2) UL Meas Uncer: Same as 6.3.1 DL Meas Uncer: Same as 7.3.2 |  |
| 6.3.4.3 Relative power tolerance |  | PC3 Max Device size ≤ 30 cm [±1.7 dB] (FR2a) [±1.7 dB] (FR2b) |  | MTSU = 1.00 x MU (from Table B.9a.2.2-2 in TR 38.903) |  |
| 6.3.4.4 Aggregate power tolerance |  | PC3 Max Device size ≤ 30 cm ±1.4 dB (FR2a) ±1.4 dB (FR2b) |  | MTSU = 1.00 x MU (from Table B.9a.3.2-2 in TR 38.903) |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.3.1.1 General ON/OFF time mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.2 General ON/OFF time mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.3 General ON/OFF time mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.4 General ON/OFF time mask for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.5 General ON/OFF time mask for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.6 General ON/OFF time mask for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.7 General ON/OFF time mask for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.1 Relative power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.2 Relative power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.3 Relative power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.4 Relative power tolerance for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.5 Relative power tolerance for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.6 Relative power tolerance for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.7 Relative power tolerance for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.1 Aggregate power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.2 Aggregate power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.3 Aggregate power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.4 Aggregate power tolerance for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.5 Aggregate power tolerance for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.6 Aggregate power tolerance for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.7 Aggregate power tolerance for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO |  | PC3: OFF Power Max Device size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |  |
| 6.3D.3.4 SRS time mask for UL MIMO |  | PC3: OFF Power Max Device size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |  |
| 6.4.1 Frequency error |  | ± 0.01 ppm (NTC & ETC testing) |  | MTSU = 1.00 x MU (from B.10.1 and B.10.2 in TR 38.903) |  |
| 6.4.2.1 Error vector magnitude |  | PUSCH, PC3, FR2a: As defined in Table F.1.2-2. PUSCH, PC3, FR2b: As defined in Table F.1.2-3. Otherwise: TBD |  |  |  |
| 6.4.2.2 Carrier leakage |  | PC3 Max Device size ≤ 30 cm ±5.44 dB (FR2a) ±5.57 dB (FR2b) uplink absolute power measurement uncertainty: 6.15 dB (FR2a & FR2b, NTC testing) uplink relative power measurement uncertainty: 1.4 dB (FR2a & FR2b, NTC testing) |  | MTSU = 1.00 x MU (from Table B.11-1 in TR 38.903) |  |
| 6.4.2.3 In-band emissions |  | TBD |  |  |  |
| 6.4.2.4 EVM equalizer spectrum flatness |  | TBD |  |  |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation |  | TBD |  |  |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.4 Frequency error for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.5 Frequency error for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.6 Frequency error for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.7 Frequency error for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.5.1 Occupied bandwidth |  | Max Device size ≤ 30cm PC3: FR2a: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.2 [%CBW] (BW 200MHz) ±1.2 [%CBW] (BW 400MHz) FR2b: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.3 [%CBW] (BW 200MHz) ±1.3 [%CBW] (BW 400MHz) FR2c: TBD |  |  |  |
| 6.5.2.1 Spectrum Emission Mask |  | Max Device size ≤ 30 cm ±4.94 dB (FR2a) ±5.32 dB (FR2b) |  | MTSU = 1.00 x MU (from Table B.16-1 in TR 38.903) |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio |  | Max Device size ≤ 30cm FR2a, NTC & ETC testing: ±5.63 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) FR2b, NTC & ETC testing: ±6.09 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) |  | MTSU = 1.00 x MU (from Table B.17-1B in TR 38.903) |  |
| 6.5.3.1 Transmitter Spurious emissions |  | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz) ±5.11 dB (12.75GHz < f ≤ 23.45GHz) ±5.41 dB (23.45GHz < f < 40.8GHz) ±7.42 dB (40.8GHz ≤ f ≤ 66GHz) ±7.72 dB (66GHz ≤ f ≤ 80GHz) |  | MTSU = 1.00 x MU (from Table B.18-1 in TR 38.903) |  |
| 6.5.3.2 Spurious emission band UE co-existence |  | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz Protected band n260, n261, n257: ±6.00 dB Protected frequency 23.6 GHz ≤ f ≤ 24.0 GHz:±6.00 dB Protected frequency 57 GHz ≤ f ≤ 66GHz: ±8.01 dB Protected frequency 36 GHz ≤ f ≤ 37GHz: ±6.00 dB |  | MTSU = 1.00 x MU (from Table B.18-1a in TR 38.903) |  |
| 6.5.3.3 Additional Spurious emission |  | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz ±5.14 dB (6GHz ≤ f ≤ 12.75GHz), NS_202 ±5.70 dB (12.75GHz < f ≤ 23.45GHz), NS_202 ±6.00 dB (23.45GHz < f < 40.8GHz), NS_202, NS_203 ±8.01 dB (40.8GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band), NS_202 |  | MTSU = 1.00 x MU (from Table B.18-1b in TR 38.903) |  |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Max Device size ≤ 30cm PC3: FR2a: TBD FR2b: TBD FR2c: TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5A.1.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | MTSU = 1.00 x MU (from Table B.17-1B in TR 38.309) |  |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.4 Transmitter Spurious emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.5 Transmitter Spurious emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.6 Transmitter Spurious emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.7 Transmitter Spurious emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.2.1 Spurious emission band UE co-existence for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.2.2 Spurious emission band UE co-existence for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.2.3 Spurious emission band UE co-existence for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.2.4 Spurious emission band UE co-existence for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.3.2.5 Spurious emission band UE co-existence for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.3.2.6 Spurious emission band UE co-existence for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.3.2.7 Spurious emission band UE co-existence for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.1 Additional spurious emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.2 Additional spurious emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.3 Additional spurious emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.4 Additional spurious emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.5 Additional spurious emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.6 Additional spurious emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.3.3.7 Additional spurious emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.5D.2.1 Spectrum Emission Mask for UL MIMO |  | Same as 6.5.2.1 |  |  |  |
| 6.5D.2.2 Adjacent channel leakage ratio for UL MIMO |  | Same as 6.5.2.3 |  |  |  |
| 6.6.1 Beam correspondence – EIRP |  | PC3 Max Device size ≤ 30 cm 2.67 dB (FR2a, NTC testing) 3.80 dB (FR2b, NTC testing) |  | MTSU = 1.00 x MU (from Table B.18a.2-2 in TR 38.309) |  |
| 6.6.2 Enhanced Beam correspondence - EIRP |  | TBD |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |  |  |  |

Table F.1.2-2: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2a (23.45GHz <= f <= 32.125GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 2.78% | 3.85% | 5.44% | 7.69% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 3.10% | 4.16% | 5.88% | 8.99% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 3.31% | 4.50% | 6.36% | 11.21% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 3.60% | 4.73% | 6.68% | 11.21% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 5.01% | 7.08% | 9.99% | 15.84% |
| 9 | CP-OFDM QPSK | Inner_Full | 3.60% | 4.73% | 6.68% | 11.89% |
| 10 | CP-OFDM QPSK | Outer_Full | 3.71% | 4.99% | 7.07% | 11.89% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 4.26% | 5.96% | 8.41% | 15.84% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 6.31% | 8.91% | 12.59% | 21.13% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 6.31% | 8.91% | 12.59% | 21.13% |

Table F.1.2-3: EVM Measurement Uncertainty (MU) for PUSCH, PC3, FR2b (32.125GHz < f <= 40.8GHz)

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 3.56% | 4.83% | 6.91% | 9.65% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 4.15% | 5.69% | 8.11% | 12.50% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 4.54% | 6.26% | 8.91% | 18.06% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 5.09% | 7.19% | 10.15% | 18.06% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 8.06% | 11.38% | 16.09% | 25.50% |
| 9 | CP-OFDM QPSK | Inner_Full | 5.09% | 7.19% | 10.15% | 19.13% |
| 10 | CP-OFDM QPSK | Outer_Full | 5.39% | 7.61% | 10.75% | 19.13% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 6.78% | 9.58% | 13.54% | 25.50% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 10.14% | 14.33% | 20.25% | 34.01% |
| 14 | CP-OFDM 64 QAM | Outer_Full | 10.14% | 14.33% | 20.25% | 34.01% |

F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty (MTSU) for receiver tests

| Sub clause |  | Maximum Test System Uncertainty |  | Derivation of MTSU |  |
| --- | --- | --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level |  | PC3 Max Device size ≤ 30 cm ±5.19 dB (FR2a, FR2b, NTC testing) ±5.45 dB (FR2a, FR2b, ETC testing) |  | MTSU = 1.00 x MU (from Table B.19-1 in TR 38.903) |  |
| 7.3.4 EIS spherical coverage |  | PC3 ±4.90 dB (Max Device size ≤ 30 cm, FR2a, FR2b) |  | MTSU = 1.00 x MU (from Table B.19-2 in TR 38.903) |  |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.4 Reference sensitivity power level for CA (5DL CA) |  | TBD |  |  |  |
| 7.3A.2.5 Reference sensitivity power level for CA (6DL CA) |  | TBD |  |  |  |
| 7.3A.2.6 Reference sensitivity power level for CA (7DL CA) |  | TBD |  |  |  |
| 7.3A.2.7 Reference sensitivity power level for CA (8DL CA) |  | TBD |  |  |  |
| 7.3A.3.1 EIS spherical coverage for CA (2DL CA) |  | TBD |  |  |  |
| 7.3A.3.2 EIS spherical coverage for CA (3DL CA) |  | TBD |  |  |  |
| 7.3A.3.3 EIS spherical coverage for CA (4DL CA) |  | TBD |  |  |  |
| 7.3A.3.4 EIS spherical coverage for CA (5DL CA) |  | TBD |  |  |  |
| 7.3A.3.5 EIS spherical coverage for CA (6DL CA) |  | TBD |  |  |  |
| 7.3A.3.6 EIS spherical coverage for CA (7DL CA) |  | TBD |  |  |  |
| 7.3A.3.7 EIS spherical coverage for CA (8DL CA) |  | TBD |  |  |  |
| 7.4 Maximum input level |  | TBD |  |  |  |
| 7.4A.1 Maximum input level for CA (2DL CA) |  | TBD |  |  |  |
| 7.4A.2 Maximum input level for CA (3DL CA) |  | TBD |  |  |  |
| 7.4A.3 Maximum input level for CA (4DL CA) |  | TBD |  |  |  |
| 7.4A.4 Maximum input level for CA (5DL CA) |  | TBD |  |  |  |
| 7.4A.5 Maximum input level for CA (6DL CA) |  | TBD |  |  |  |
| 7.4A.6 Maximum input level for CA (7DL CA) |  | TBD |  |  |  |
| 7.4A.7 Maximum input level for CA ((DL CA) |  | TBD |  |  |  |
| 7.5 Adjacent channel selectivity |  | PC3 ±7.84 dB (Max Device size ≤ 30 cm, FR2a, FR2b) |  | MTSU = 1.00 x MU (from Table B.21-1 in TR 38.903) |  |
| 7.5A.1 Adjacent channel selectivity for CA (2UL CA) |  | TBD |  |  |  |
| 7.5A.2 Adjacent channel selectivity for CA (3UL CA) |  | TBD |  |  |  |
| 7.5A.3 Adjacent channel selectivity for CA (4UL CA) |  | TBD |  |  |  |
| 7.5A.4 Adjacent channel selectivity for CA (5UL CA) |  | TBD |  |  |  |
| 7.5A.5 Adjacent channel selectivity for CA (6UL CA) |  | TBD |  |  |  |
| 7.5A.6 Adjacent channel selectivity for CA (7UL CA) |  | TBD |  |  |  |
| 7.5A.7 Adjacent channel selectivity for CA (8UL CA) |  | TBD |  |  |  |
| 7.6.2 In-band blocking |  | Same as 7.5 |  |  |  |
| 7.6A.2.1 In-band blocking for CA (2UL CA) |  | TBD |  |  |  |
| 7.6A.2.2 In-band blocking for CA (3UL CA) |  | TBD |  |  |  |
| 7.6A.2.3 In-band blocking for CA (4UL CA) |  | TBD |  |  |  |
| 7.6A.2.4 In-band blocking for CA (5UL CA) |  | TBD |  |  |  |
| 7.6A.2.5 In-band blocking for CA (6UL CA) |  | TBD |  |  |  |
| 7.6A.2.6 In-band blocking for CA (7UL CA) |  | TBD |  |  |  |
| 7.6A.2.7 In-band blocking for CA (8UL CA) |  | TBD |  |  |  |
| 7.9 Spurious emissions |  | Max Device size ≤ 30 cm Maximum in-band BW ≤ 400MHz For Band n257, n258, n260, n261: ±5.50dB (6GHz ≤ f ≤ 12.75GHz) ±5.46dB (12.75GHz < f ≤ 23.45GHz) ±6.11dB (23.45GHz < f < 40.8GHz) ±7.65dB (40.8GHz ≤ f ≤ 66GHz) ±7.95 dB (66GHz ≤ f ≤ 80GHz) |  | MTSU = 1.00 x MU (from Table B.25-1 in TR 38.903) |  |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.1.2-1. |  |  |  |  |  |

F.2	Interpretation of measurement results (normative)

The actual measurement uncertainty of the Test System for the measurement of each parameter shall be included in the test report.

The recorded value for the Test System uncertainty shall be, for each measurement, equal to or lower than the appropriate figure in clause F.1 of the present document.

If the Test System using one of the permitted test methods defined in TR38.903 [20] for a test is known to have a measurement uncertainty greater than that specified in clause F.1, it is still permitted to use this apparatus provided that an adjustment is made value as follows:

Any additional uncertainty in the Test System over and above that specified in clause F.1 shall be used to tighten the Test Requirement, making the test harder to pass. For some tests, for example receiver tests, this may require modification of stimulus signals. This procedure will ensure that a Test System not compliant with clause F.1does not increase the chance of passing a device under test where that device would otherwise have failed the test if a Test System compliant with clause F.1 had been used.

F.3	Test Tolerance and Derivation of Test Requirements (informative)

TBD

F.3.1	Measurement of test environments

TBD

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) |  | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.87 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) Max EIRP 0 dB |  | Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) |  |
| 6.2.1.1 UE maximum output power (TRP) |  | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.65 dB (FR2a, NTC) 2.77 dB (FR2b, NTC) 2.82 dB (FR2a, ETC) 2.94 dB (FR2b, ETC) |  | Max TRP TT = 0.60 x MTSUIFF |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) |  | PC1 TBD PC2 TBD PC3 IFF (Max Device size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD |  | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) |  |
| 6.2.2 UE maximum output power reduction |  | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.11 dB (FR2a, NTC) 3.11 dB (FR2b, NTC) 3.30 dB (FR2a, ETC) 3.30 dB (FR2b, ETC) |  | Minimum peak EIRP TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) |  |
| 6.2.3 UE maximum output power with additional requirements |  | Same as 6.2.2 |  |  |  |
| 6.2.4 Configured transmitted power |  | TBD |  |  |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |  |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |  |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |  |  |
| 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) |  | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |  |  |
| 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) |  | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |  |  |
| 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) |  | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |  |  |
| 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) |  | Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.2 UE maximum output power reduction for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.3 UE maximum output power reduction for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.2A.2.4 UE maximum output power reduction for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.5 UE maximum output power reduction for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.6 UE maximum output power reduction for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2A.2.7 UE maximum output power reduction for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO |  | Same as 6.2.2 |  |  |  |
| 6.3.1 Minimum output power |  | PC3 Minimum EIRP IFF (Max Device size ≤ 30 cm) NTC 4.21 dB (FR2a 50 MHz) 2.52 dB (FR2a 100 MHz) 0.66 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.17 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) ETC 4.37 dB (FR2a 50 MHz) 2.68 dB (FR2a 100 MHz) 0.82 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.33 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) |  | Minimum EIRP TT = max(R, ΔSNRmr + 0.65 x (MTSUIFF – 1.0)) -R R: Relaxation needed to limit influence of TE noise to 1 dB (specified in clause 6.3.1.5) ΔSNRmr: Systematic offset due to noise when measuring at minimum requirement level (-13 dBm) FR2a 50 MHz: ΔSNRmr = 0.86 dB FR2a 100 MHz: ΔSNRmr = 1.57 dB FR2a 200 MHz: ΔSNRmr = 2.71 dB FR2a 400 MHz: ΔSNRmr = 4.35 dB FR2b 50 MHz: ΔSNRmr = 2.32 dB FR2b 100 MHz: ΔSNRmr = 3.82 dB FR2b 200 MHz: ΔSNRmr = 5.82 dB FR2b 400 MHz: ΔSNRmr = 8.21 dB |  |
| 6.3.2 Transmit OFF power |  | 0 dB |  |  |  |
| 6.3.3.2 General ON/OFF time mask |  | PC3: ON Power Same as 6.2.1.1 (EIRP) OFF Power 0 dB |  | ON Power: Same as 6.2.1.1 (EIRP) OFF Power: Same as 6.3.1 |  |
| 6.3.3.4 PRACH time mask |  | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD |  | ON Power TBD |  |
| 6.3.4.2 Absolute power tolerance |  | PC3 Max Device size ≤ 30 cm ±8.05 dB (FR2a & FR2b, NTC testing) ±8.42 dB (FR2a & FR2b, ETC testing) |  | TT = MTSU |  |
| 6.3.4.3 Relative power tolerance |  | PC3 IFF (Max Device size ≤ 30 cm) [0.46 dB] (FR2a) [0.46 dB] (FR2b) |  | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |  |
| 6.3.4.4 Aggregate power tolerance |  | PC3 IFF (Max Device size ≤ 30 cm) 0.26 dB (FR2a) 0.26 dB (FR2b) |  | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |  |
| 6.3A.1.1 Minimum output power for CA (2UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) |  | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |  |  |
| 6.3A.3.1.1 General ON/OFF time mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.2 General ON/OFF time mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.3 General ON/OFF time mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.3.1.4 General ON/OFF time mask for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.5 General ON/OFF time mask for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.6 General ON/OFF time mask for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.3.1.7 General ON/OFF time mask for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO |  | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3D.3.4 SRS time mask for UL MIMO |  | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b) |  | ON Power TBD |  |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.1 Relative power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.2 Relative power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.3 Relative power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.3.4 Relative power tolerance for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.5 Relative power tolerance for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.6 Relative power tolerance for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.3.7 Relative power tolerance for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.1 Aggregate power tolerance for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.2 Aggregate power tolerance for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.3 Aggregate power tolerance for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.3A.4.4.4 Aggregate power tolerance for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.5 Aggregate power tolerance for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.6 Aggregate power tolerance for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.3A.4.4.7 Aggregate power tolerance for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4.1 Frequency error |  | 0.005 ppm (NTC & ETC testing) |  | TT = 0.5 x MTSU |  |
| 6.4.2.1 Error vector magnitude |  | TBD |  | Minimum requirement + TT |  |
| 6.4.2.2 Carrier leakage |  | IFF (Max Device size ≤ 30 cm) FR2a: ±3.54 dB (BW ≤ 400MHz) FR2b: ±3.62 dB (BW ≤ 400MHz) |  | TT = 0.65 x MTSUIFF |  |
| 6.4.2.3 In-band emissions |  | TBD |  |  |  |
| 6.4.2.4 EVM equalizer spectrum flatness |  | TBD |  |  |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation |  | TBD |  |  |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.4A.1.4 Frequency error for CA (5UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.5 Frequency error for CA (6UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.6 Frequency error for CA (7UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.1.7 Frequency error for CA (8UL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) |  | TBD |  |  |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) |  | TBD |  |  |  |
| 6.5.1 Occupied bandwidth |  | 0 kHz |  | Minimum requirement + TT |  |
| 6.5.2.1 Spectrum Emission Mask |  | IFF (Max Device size ≤ 30 cm) 3.21 dB (FR2a) 3.46 dB (FR2b) |  | TT = 0.65 x MTSUIFF |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio |  | Absolute requirement 0 dB Relative requirement IFF (Max Device size ≤ 30 cm) FR2a: ±4.66 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) FR2b: ±4.96 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) |  | TT = max(R, ΔSNRmr+0.65 x (MTSUIFF-1.0)) -R + TT due to metric change TT due to metric change : 1.0 dB R: Relaxation needed to limit influence of TE noise to 1 dB (specified in clause 6.5.2.3.5) ΔSNRmr: Systematic offset due to noise when measuring ACP at minimum requirement level |  |
| 6.5.3.1 Transmitter Spurious emissions |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.2 Spurious emission band UE co-existence |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5.3.3 Additional spurious emission |  | 0 dB |  | Minimum requirement + TT |  |
| 6.5A.1.1 Occupied bandwidth for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.2 Occupied bandwidth for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.3 Occupied bandwidth for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.1.4 Occupied bandwidth for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.1.5 Occupied bandwidth for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.1.6 Occupied bandwidth for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.1.7 Occupied bandwidth for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.1 Spectrum Emission Mask for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.2 Spectrum Emission Mask for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.3 Spectrum Emission Mask for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.2.1.4 Spectrum Emission Mask for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.5 Spectrum Emission Mask for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.6 Spectrum Emission Mask for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.1.7 Spectrum Emission Mask for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.1 Adjacent channel leakage ratio for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.2 Adjacent channel leakage ratio for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.3 Adjacent channel leakage ratio for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.2.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  | TT = 0.65 x MTSUIFF + TT due to metric change TT due to metric change : 1.0 dB |  |
| 6.5A.2.2.4 Adjacent channel leakage ratio for CA (5UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.5 Adjacent channel leakage ratio for CA (6UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.6 Adjacent channel leakage ratio for CA (7UL CA) |  | TBD |  |  |  |
| 6.5A.2.2.7 Adjacent channel leakage ratio for CA (8UL CA) |  | TBD |  |  |  |
| 6.5A.3.1.1 Transmitter Spurious emissions for CA (2UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.2 Transmitter Spurious emissions for CA (3UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5A.3.1.3 Transmitter Spurious emissions for CA (4UL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.5.3.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 6.5D.2.1 Spectrum Emission Mask for UL MIMO |  | Same as 6.5.2.1 |  |  |  |
| 6.5D.2.2 Adjacent channel leakage ratio for UL MIMO |  | Same as 6.5.2.3 |  |  |  |
| 6.6.1 Beam correspondence - EIRP |  | PC3 1.26 dB (FR2a, FR2b) |  | PC3 TT = 0.60 x (MTSUIFF - ΔSNRmr) ΔSNRmr: Systematic offset due to noise when measuring at minimum requirement level |  |
| 6.6.2 Enhanced Beam correspondence - EIRP |  | TBD |  |  |  |
| NOTE 1: FR2a: 23.45GHz ≤ f ≤ 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz FR2c: 40.8GHz ≤ f ≤ 44.3GHz |  |  |  |  |  |

F.3.3	Measurement of receiver

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |
| --- | --- | --- | --- | --- | --- |
| 7.3.2 Reference sensitivity power level |  | PC3 IFF (Max Device size ≤ 30 cm) 2.34 dB (FR2a, FR2b, NTC) 2.45 dB (FR2a, FR2b, ETC) |  | TT = 0.45 x MTSUIFF |  |
| 7.3.4 EIS spherical coverage |  | IFF (Max Device size ≤ 30 cm, FR2a, FR2b) 2.21 dB |  | PC3 TT = 0.45 x MTSUIFF |  |
| 7.3A.2.1 Reference sensitivity power level for CA (2DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.2 Reference sensitivity power level for CA (3DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.3 Reference sensitivity power level for CA (4DL CA) |  | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 7.3.2 for each component carrier Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |  |  |
| 7.3A.2.4 Reference sensitivity power level for CA (5DL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 7.3A.2.5 Reference sensitivity power level for CA (6DL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 7.3A.2.6 Reference sensitivity power level for CA (7DL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 7.3A.2.7 Reference sensitivity power level for CA (8DL CA) |  | Intra-band contiguous CA TBD |  |  |  |
| 7.3A.3.1 EIS spherical coverage for CA (2DL CA) |  | TBD |  |  |  |
| 7.3A.3.2 EIS spherical coverage for CA (3DL CA) |  | TBD |  |  |  |
| 7.3A.3.3 EIS spherical coverage for CA (4DL CA) |  | TBD |  |  |  |
| 7.3A.3.4 EIS spherical coverage for CA (5DL CA) |  | TBD |  |  |  |
| 7.3A.3.5 EIS spherical coverage for CA (6DL CA) |  | TBD |  |  |  |
| 7.3A.3.6 EIS spherical coverage for CA (7DL CA) |  | TBD |  |  |  |
| 7.3A.3.7 EIS spherical coverage for CA (8DL CA) |  | TBD |  |  |  |
| 7.4 Maximum input level |  | TBD |  |  |  |
| 7.4A.1 Maximum input level for CA (2DL CA) |  | TBD |  |  |  |
| 7.4A.2 Maximum input level for CA (3DL CA) |  | TBD |  |  |  |
| 7.4A.3 Maximum input level for CA (4DL CA) |  | TBD |  |  |  |
| 7.4A.4 Maximum input level for CA (5DL CA) |  | TBD |  |  |  |
| 7.4A.5 Maximum input level for CA (6DL CA) |  | TBD |  |  |  |
| 7.4A.6 Maximum input level for CA (7DL CA) |  | TBD |  |  |  |
| 7.4A.7 Maximum input level for CA ((DL CA) |  | TBD |  |  |  |
| 7.5 Adjacent channel selectivity |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.5A.1 Adjacent channel selectivity for CA (2UL CA) |  | TBD |  |  |  |
| 7.5A.2 Adjacent channel selectivity for CA (3UL CA) |  | TBD |  |  |  |
| 7.5A.3 Adjacent channel selectivity for CA (4UL CA) |  | TBD |  |  |  |
| 7.5A.4 Adjacent channel selectivity for CA (5UL CA) |  | TBD |  |  |  |
| 7.5A.5 Adjacent channel selectivity for CA (6UL CA) |  | TBD |  |  |  |
| 7.5A.6 Adjacent channel selectivity for CA (7UL CA) |  | TBD |  |  |  |
| 7.5A.7 Adjacent channel selectivity for CA (8UL CA) |  | TBD |  |  |  |
| 7.6.2 In-band blocking |  | 0 dB |  | Wanted signal power + TT T-put limit unchanged |  |
| 7.6A.2.1 In-band blocking for CA (2UL CA) |  | TBD |  |  |  |
| 7.6A.2.2 In-band blocking for CA (3UL CA) |  | TBD |  |  |  |
| 7.6A.2.3 In-band blocking for CA (4UL CA) |  | TBD |  |  |  |
| 7.6A.2.4 In-band blocking for CA (5UL CA) |  | TBD |  |  |  |
| 7.6A.2.5 In-band blocking for CA (6UL CA) |  | TBD |  |  |  |
| 7.6A.2.6 In-band blocking for CA (7UL CA) |  | TBD |  |  |  |
| 7.6A.2.7 In-band blocking for CA (8UL CA) |  | TBD |  |  |  |
| 7.9 Spurious emissions |  | 0 dB |  | Minimum requirement + TT T-put limit unchanged |  |
| NOTE 1: FR2a, FR2b and FR2c are specified in Table F.3.2-1. |  |  |  |  |  |

<<End of change>>