3GPP TSG-RAN5 Meeting #95-e	R5-222545r1

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1675 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of BCS in test configuration of CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In some CA test cases the specification of channel bandwidth is ambiguous. It is not clear whether the test has to be performed for each BCS of a CA configuration or whether highest and lowest aggregated channel bandwidth is determined for all BCS supported by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For several TCs the test configuration has been clarified similar to the wording in the Refsens CA test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain unclear resulting in inconsistent testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3C.3.2, 7.4A.1.4.1, 7.4A.2.4.1, 7.4A.3.4.1, 7.5A.1.4.1, 7.5A.1.5, 7.5A.2.4.1, 7.5A.3.4.1, 7.6A.2.1.2, 7.6A.2.1.4.1, 7.6A.2.1.5.1, 7.6A.2.2.4.1, 7.6A.2.3.4.1, 7.6A.3.1.4.1, 7.6A.3.2.4.1, 7.6A.3.3.4.1, 7.6A.4.1.4.1, 7.6A.4.2.4.1, 7.6A.4.3.4.1, 7.8A.2.1.4.1, 7.8A.2.2.4.1, 7.8A.2.3.4.1; 7.9A.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: - removed the updates to the headers of Table 7.6A.4.2.4.1-1 and Table 7.6A.4.3.4.1-1 in order to resolve overlap with CRs R5-222332 and R5-222333. - clarified test configuration for TCs 7.4A.x, 7.8A.x, 7.9A.1. - updated table headers by replacing 2CA by 2DL CA, etc. - updated applicability of TCs 7.3C.3 and 7.6A.2.1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.3C.3	Reference sensitivity power level for SUL (3CC)

Editor’s Note:

- No test points defined for Reference sensitivity power level testing for SUL with DL CA. This test case is covered by 7.3.2 and 7.3C.2.

7.3C.3.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under SUL and 2 DL CA operation and conditions of low signal level, ideal propagation and no added noise.

7.3C.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports SUL operation on the SUL bands with2DL CA.

< Unchanged sections omitted >

### 7.4A.1	Maximum input level for CA (2DL CA)

7.4A.1.1	Test purpose

The same test purpose as defined in 7.4.1.

7.4A.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2DL CA.

7.4A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

7.4A.1.4	Test description

7.4A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.1.4.1-1 or 7.4A.1.4.1-2. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.1.4.1-1: Test configuration table for Intra-band contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

Table 7.4A.1.4.1-2: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1, NOTE 3 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 4 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-2. Only test points verifying non-exceptional REFSENS requirements are used for maximum input level. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 7.4A.1.4.1-3: Test configuration table for Intra-band non-contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-3. Only test points verifying non-exceptional REFSENS requirements are used for maximum input level. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

< Unchanged sections omitted >

### 7.4A.2	Maximum input level for CA (3DL CA)

7.4A.2.1	Test purpose

The same test purpose as defined in 7.4.1.

7.4A.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 3DL CA.

7.4A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

7.4A.2.4	Test description

7.4A.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.2.4.1-1, 7.4A.2.4.1-2 or 7.4A.2.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.2.4.1-1: Test Configuration Table for 3DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Intra-band contiguous: Mid range for PCC and SCCs Inter-band: NOTE 1, NOTE 5 Intra-band contiguous + Inter-band: NOTE 1, NOTE 5 Intra-band non-contiguous + Inter-band: NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.2.4.1-1 (NOTE 5) |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 6 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |
| Network signalling value |  |  | NS_01 by default |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.2.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for maximum input level testing. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.4A.3	Maximum input level for CA (4DL CA)

7.4A.3.1	Test purpose

The same test purpose as defined in 7.4.1.

7.4A.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 4DL CA.

7.4A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

7.4A.3.4	Test description

7.4A.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 5 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  | NS_01 by default |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intraband non-contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for maximum input level testing. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.5A.1	Adjacent channel selectivity for CA (2DL CA)

7.5A.1.1	Test Purpose

Adjacent channel selectivity for 2DL CA verifies the receiver's ability to receive a wanted 2DL carrier aggregated at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

7.5A.1.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 2DL CA.

7.5A.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.5A.0.

7.5A.1.4	Test Description

7.5A.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.5A.1.4.1-1, Table 7.5A.1.4.1-2 or Table 7.5A.1.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.5A.1.4.1-1: Test Configuration Table for intra-band contiguous 2DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Lowest NRB_agg, Highest NRB_agg NOTE 3 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation |  | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested |  |  |  |  |  |  |

Table 7.5A.1.4.1-2: Test Configuration Table for inter-band 2DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1, NOTE 3 |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 4 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation |  | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-2. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |

Table 7.5A.1.4.1-3: Test Configuration Table for intra-band non-conguous 2 DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 3 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation |  | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 |  | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-3. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |

< Unchanged sections omitted >

7.5A.1.5	Test Requirement

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5A.1.5-2, 7.5A.1.5-2a, 7.5A.1.5-3 and 7.5A.1.5-3a.

Table 7.5A.1.5-1: ACS for intra-band contiguous 2DL CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | CA Bandwidth Class |  |
| --- | --- | --- | --- |
| Rx Parameter | Units | B | C |
| ACS | dB | 26.0 | 33.0 |

Table 7.5A.1.5-1a: ACS for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

|  |  | CA Bandwidth Class |  |
| --- | --- | --- | --- |
| Rx Parameter | Units | B | C |
| ACS | dB | 20.0 | 17.0 |

Table 7.5A.1.5-2: Test parameters for intra-band contiguous 2DL CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 24.5 dB | Aggregated power + 31.5 dB |
| BWInterferer | MHz | 20 | BWchannel CA |
| FInterferer (offset) | MHz | 10 + Foffset / -10 -Foffset | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4.3. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Table 7.5A.1.5-2a: Test parameters for intra-band contiguous CA with FDL_low<2700 MHz and FUL_low<2700 MHz, case 1

| Rx Parameter | Units | CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 18.5 dB | Aggregated power + 15.5 dB |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Table 7.5A.1.5-3: Test parameters for intra-band contiguous 2DL CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -49.5 + 10log(NRB,c/NRB_agg) | -56.5 |
| PInterferer | dBm | -25 | -25 |
| BWInterferer | MHz | 20 | BWchannel CA |
| FInterferer (offset) | MHz | 10 + Foffset / -10 -Foffset | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4.3. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

Table 7.5A.1.5-3a: Test parameters for intra-band contiguous CA with FDL_low <2700 MHz and FUL_low<2700 MHz, case 2

| Rx Parameter | Units | CA Bandwidth Class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -43.5+ 10log(NRB,c/NRB_agg) | -40.5+ 10log(NRB,c/NRB_agg) |
| PInterferer | dBm | -25 | -25 |
| BWInterferer | MHz | 5 | 5 |
| FInterferer (offset) | MHz | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |

For NR SCC of inter-band CA with FDL_high < 2700 MHz and FUL_high < 2700 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1)] with parameters specified in Tables 7.5A.1.5-5 and 7.5A.1.5-6.

Table 7.5A.1.5-4: ACS for NR band with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| ACS | dB | 33 | 33 | 30 | 27 | 26 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| ACS | dB | 25.5 | 24 | 23 | 22.5 | 21 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| ACS | dB | 20.5 | 20 |  |  |  |

Table 7.5A.1.5-5: Test parameters for NR inter-band CA with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 42.5 dB | REFSENS for SCC + 39.5 dB | REFSENS for SCC + 38.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 38 dB | REFSENS for SCC + 36.5 dB | REFSENS for SCC + 35.5 dB | REFSENS for SCC + 35 dB | REFSENS for SCC + 33.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 33 dB | REFSENS for SCC + 32.5 dB |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset from SCC) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.5A.1.5-6: Test parameters for NR inter-band CA with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 | -56.5 | -53.5 | -50.5 | -49.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -49 | -47 | -46.5 | -46 | -44.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -44 | -43.5 |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset from SCC) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

For NR SCC of inter-band CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in [Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1)] with parameters specified in Tables 7.5A.1.5-8 and 7.5A.1.5-9.

Table 7.5A.1.5-7: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| ACS | dB | 33 | 33 | 33 | 33 | 33 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| ACS | dB | 33 | 33 | 33 | 33 |  |

Table 7.5A.1.5-8: Test parameters for NR inter-band CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset from SCC) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset from SCC) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.5A.1.5-9: Test parameters for NR inter-band CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset from SCC) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 | -25 | -25 |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset from SCC) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

For NR SCC of intra-band non-contiguous CA with FDL_high < 2700 MHz and FUL_high < 2700 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 with parameters specified in Tables 7.5A.1.5-11 and 7.5A.1.5-12.

Table 7.5A.1.5-10: ACS for NR band with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| ACS | dB | 33 | 33 | 30 | 27 | 26 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| ACS | dB | 25.5 | 24 | 23 | 22.5 | 21 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| ACS | dB | 20.5 | 20 |  |  |  |

Table 7.5A.1.5-11: Test parameters for NR intra-band non-contiguous CA with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 42.5 dB | REFSENS for SCC + 39.5 dB | REFSENS for SCC + 38.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 38 dB | REFSENS for SCC + 36.5 dB | REFSENS for SCC + 35.5 dB | REFSENS for SCC + 35 dB | REFSENS for SCC + 33.5 dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 33 dB | REFSENS for SCC + 32.5 dB |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset from SCC) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.5A.1.5-12: Test parameters for NR intra-band non-contiguous CA with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 | -56.5 | -53.5 | -50.5 | -49.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -49 | -47 | -46.5 | -46 | -44.5 |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset from SCC) | MHz | 17.5 / -17.5 | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 | 42.5 / -42.5 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 90 MHz | 100 MHz |  |  |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -44 | -43.5 |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 |  |  |  |
| Finterferer (offset from SCC) | MHz | 47.5 / -47.5 | 52.5 / -52.5 |  |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

For NR SCC of intra-band non-contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, the throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 with parameters specified in Tables 7.5A.1.5-14 and 7.5A.1.5-15.

Table 7.5A.1.5-13: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| ACS | dB | 33 | 33 | 33 | 33 | 33 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| ACS | dB | 33 | 33 | 33 | 33 |  |

Table 7.5A.1.5-14: Test parameters for NR intra-band non-contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset from SCC) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB | REFSENS for SCC + 45.5 dB |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset from SCC) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.5A.1.5-15: Test parameters for NR intra-band non-contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 10 | 15 | 20 | 40 | 50 |
| Finterferer (offset from SCC) | MHz | 10 / -10 | 15 / -15 | 20 / -20 | 40 / -40 | 50 / -50 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -56.5 |  |  |  |  |
| Pinterferer | dBm | -25 | -25 | -25 | -25 |  |
| BWinterferer | MHz | 60 | 80 | 90 | 100 |  |
| Finterferer (offset from SCC) | MHz | 60 / -60 | 80 / -80 | 90 / -90 | 100 / -100 |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.5A.2	Adjacent channel selectivity for 3DL CA

7.5A.2.1	Test Purpose

Adjacent channel selectivity for 3DL CA verifies the receiver's ability to receive a wanted 3DL carrier aggregated at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

7.5A.2.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 3DL CA.

7.5A.2.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.5A.0.

7.5A.2.4	Test Description

7.5A.2.4.1	Initial Conditions

Same as in clause 7.5A.1.4.1 with following exceptions:

-	Instead of Table 7.5A.1.4.1-1 use Table 7.5A.2.4.1-1.

-	Instead of Table 7.5A.1.4.1-2 use Table 7.5A.2.4.1-1.

-	Instead of Table 7.5A.1.4.1-3 use Table 7.5A.2.4.1-1.

Table 7.5A.2.4.1-1: Test Configuration Table for 3DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Intra-band contiguous: Mid range for PCC and SCCs Inter-band CA: NOTE 1, NOTE 5 Inter-band + Intra-band contiguous : NOTE 1, NOTE 5 Inter-band + Intra-band non-contiguous : NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.2.4.1-1(NOTE 5) |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Intra-band contiguous: Lowest NRB_agg, Highest NRB_agg Inter-band: Highest NRB_agg Inter-band + Intra-band contiguous : Highest NRB_agg Inter-band + Intra-band non-contiguous : Highest NRB_agg NOTE 6 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |
| Network signalling value |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.2.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for ACS. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8 NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.5A.3	Adjacent channel selectivity for 4DL CA

Editor’s note:

- part content of Table 7.5A.3.4.2-1 is FFS.

7.5A.3.1	Test Purpose

Adjacent channel selectivity for 4DL CA verifies the receiver's ability to receive a wanted 4DL carrier aggregated at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

7.5A.3.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support 4DL CA.

7.5A.3.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.5A.0.

7.5A.3.4	Test Description

7.5A.3.4.1	Initial Conditions

Same as in clause 7.5A.1.4.1 with following exceptions:

-	Instead of Table 7.5A.1.4.1-1 use Table 7.5A.3.4.1-1.

-	Instead of Table 7.5A.1.4.1-2 use Table 7.5A.3.4.1-2.

-	Instead of Table 7.5A.1.4.1-3 use Table 7.5A.3.4.1-3.

Table 7.5A.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Intra-band contiguous: Mid range for PCC and SCCs Inter-band CA: NOTE 1, NOTE 5 Inter-band + Intra-band contiguous : NOTE 1, NOTE 5 Inter-band + Intra-band non-contiguous : NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.3.4.1-1(NOTE 5) |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Intra-band contiguous: Lowest NRB_agg, Highest NRB_agg Inter-band: Highest NRB_agg Inter-band + Intra-band contiguous : Highest NRB_agg Inter-band + Intra-band non-contiguous : Highest NRB_agg NOTE 6 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation |  | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (intra-band contiguous) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA-nVA Configuration (Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYB-nZA, CA_nXA-nYA-nZB, CA_nXD-nYA, CA_nXC-nYB, CA_nXC-nYC and CA_nXB-nYB Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZ(2A), CA_nXA-nY(3A) and CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(4A) Configuration intra-band non-contiguous) |  |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for ACS. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z,V correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n7A-n28A, X=1, Y=3, Z=7,V=28; Intra-band contiguous + Inter-band: X,Y,Z correspond to the different bands in the CA Configuration, E.g. for CA_n3A-n7B-n28A, X=1,Y=3,z=28; Intra-band non-contiguous + Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n2A-n66A-n77(2A), X=2, Y=66,Z=77； NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

Table 7.5A.3.4.1-2: Void

Table 7.5A.3.4.1-3: Void

< Unchanged sections omitted >

#### 7.6A.2.1	In-band blocking for CA (2DL CA)

7.6A.2.1.1	Test purpose

Inband blocking is defined for an unwanted interfering signal falling into the range from 15 MHz below to 15 MHz above the UE receive band, with FDL_high < 2700 MHz and FUL_high < 2700 MHz, or into an immediately adjacent frequency range up 3CBW below or above the UE receive band, with FDL_high ≥ 3300 MHz and FUL_high ≥ 3300 MHz, at which the relative throughput shall meet or exceed the requirement for the specified measurement channel.

7.6A.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support 2DL CA.

7.6A.2.1.3	Minimum conformance requirements

Minimum requirements are defined in clause 7.6A.2.0.

7.6A.2.1.4	Test description

7.6A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.6A.2.1.4.1-1, 7.6A.2.1.4.1-2 or 7.6A.2.1.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2..

Table 7.6A.2.1.4.1-1: Test configuration table for Intra-band contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg, NOTE 1, NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested |  |  |  |  |  |

Table 7.6A.2.1.4.1-2: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1, NOTE 3 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 1, NOTE 4 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-2. Only test points verifying non-exceptional REFSENS requirements are used for in-band blocking. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 7.6A.2.1.4.1-3: Test configuration table for Intraband non-contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 1, NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-3. Only test points verifying non-exceptional REFSENS requirements are used for in-band blocking. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

< Unchanged sections omitted >

7.6A.2.1.5	Test requirement

7.6A.2.1.5.1	Intra-band contiguous 2DL CA

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters defined in 7.6A.2.1.5.1-1a and 7.6A.2.1.5.1-2a. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.6A.2.1.5.1-1: In-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |  |
| --- | --- | --- | --- | --- |
|  |  | B | C |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA bandwidth class specific value below |  |  |
|  |  | 10.0 | 6 |  |
| BWInterferer | MHz | 20 | BWchannel CA |  |
| FIoffset, case 1 | MHz | 30 | BWchannel CA+ BWchannel CA/2 |  |
| FIoffset, case 2 | MHz | 50 | BWInterferer + FIoffset, case 1 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |

Table 7.6A.2.1.5.1-1a: In-band blocking parameters for intra-band contiguous CA with FDL_low < 2700 MHz and FUL_low < 2700 MHz

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | B | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + NR CA bandwidth class specific value below | REFSENS + NR CA bandwidth class specific value below |
|  |  | 16.0 | 19.0 |
| BWInterferer | MHz | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the Reference measurement channel specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |

Table 7.6A.2.1.5.1-2: In-band blocking for intra-band contiguous CA  with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 3BWchannel CA to FDL_high + 3BWchannel CA |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal |  |  |  |  |

Table 7.6A.2.1.5.1-2a: In-band blocking for intra-band contiguous CA with FDL_low  < 2700 MHz and FUL_low  < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |  |
| n66 n41 n484 | Finterferer (offset) | MHz | -BWchannel CA/2 –FIoffset, case 1 and BWchannel CA/2 +FIoffset, case 1 | ≤ -BWchannel CA/2 –FIoffset, case 2 and ≥ BWchannel CA/2 +FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWchannel CA/2 – FIoffset, case 1; b: BWchannel CA/2 + FIoffset, case 1 NOTE 3: BWchannel CA denotes the aggregated channel bandwidth of the wanted signal NOTE 4: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1A |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.2.2	In-band Blocking for CA (3DL CA)

7.6A.2.2.1	Test purpose

Same test purpose as in clause 7.6A.2.1.

7.6A.2.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support 3DL CA.

7.6A.2.2.3	Minimum conformance requirements

Minimum requirements are defined in clause 7.6A.2.0.

7.6A.2.2.4	Test description

7.6A.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.6A.2.2.4.1-1, 7.6A.2.2.4.1-2 or 7.6A.2.2.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.2.2.4.1-1: Test Configuration Table for 3DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Intra-band contiguous: Mid range for all CCs Inter-band: NOTE 1, NOTE 5 Intra-band contiguous + Inter-band: NOTE 1, NOTE 5 Intra-band non-contiguous + Inter-band: NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.2.4.1-1 (NOTE 5) |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg, NOTE 1, NOTE 6 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |
| Network signalling value |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous CA) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA，CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.2.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for in-band blocking testing. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8 NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.2.3	In-band Blocking for CA (4DL CA)

7.6A.2.3.1	Test purpose

Same test purpose as in clause 7.6A.2.1.

7.6A.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support 4DL CA.

7.6A.2.3.3	Minimum conformance requirements

Minimum requirements are defined in clause 7.6A.2.0.

7.6A.2.3.4	Test description

7.6A.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.6A.2.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.2.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg, NOTE 1, NOTE 5 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (Intra-band contiguous CA) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA-ZA and CA_nXB-nYA-ZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA-ZA Configurations (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nY(2A) Configurations (Intra-band non-contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for in-band blocking testing. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8 NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.3.1	Out-of-band blocking for CA (2DL CA)

7.6A.3.1.1	Test purpose

Out-of-band band blocking for CA is defined for an unwanted CW interfering signal falling more than 15 MHz or 3*BWChannel_CA below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz or 3* BWChannel_CA below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5A and sub-clause 7.6A.2 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2DL CA.

7.6A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6A.3.0.

7.6A.3.1.4	Test description

7.6A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.6A.3.1.4.1-1, 7.6A.3.1.4.1-2 or 7.6A.3.1.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.3.1.4.1-1: Test configuration table for Intra-band contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

Table 7.6A.3.1.4.1-2: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 4 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 5 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. NOTE 3: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 4: Test frequency is set to Mid Range PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1 For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 7.6A.3.1.4.1-3: Test configuration table for Intra-band non-contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 3 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest NRB_agg for PCC and SCC, NOTE 1 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-3. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.3.2	Out-of-band blocking for CA (3DL CA)

7.6A.3.2.1	Test purpose

Out-of-band band blocking for CA is defined for an unwanted CW interfering signal falling more than 15 MHz or 3*BWChannel_CA below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz or 3*BWChannel_CA below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5A and sub-clause 7.6A.2 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 3DL CA.

7.6A.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6A.3.0.

7.6A.3.2.4	Test description

7.6A.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.6A.3.2.4.1-1 or 7.6A.3.2.4.1-2. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.3.2.4.1-1: Test configuration table for Intra-band contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCCs RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

Table 7.6A.3.2.4.1-2: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Inter-band : NOTE 5 Intra-band contiguous + Inter-band: NOTE 5 Intra-band non-contiguous + Inter-band: Max WGap for Intra-band non-contiguous NOTE 5 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCCs RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test frequency is set to Mid Range PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.3.3	Out-of-band blocking for CA (4DL CA)

7.6A.3.3.1	Test purpose

Out-of-band band blocking for CA is defined for an unwanted CW interfering signal falling more than 15 MHz or 3*BWChannel_CA below or above the UE receive band, at which a given average throughput shall meet or exceed the requirement for the specified measurement channels.

For the first 15 MHz or 3*BWChannel_CA below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in sub-clause 7.5A and sub-clause 7.6A.2 shall be applied.

The lack of out-of-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.3.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 4DL CA.

7.6A.3.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6A.3.0.

7.6A.3.3.4	Test description

7.6A.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.6A.3.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.3.3.4.1-1: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Inter-band : NOTE 5 Intra-band contiguous + Inter-band: NOTE 5 Intra-band non-contiguous + Inter-band: MaxWGap for Intra-band non-contiguous (NOTE 5) Intra-band non-contiguous + Intra-band non-contiguous: MaxWGap for Intra-band non-contiguous (NOTE 5) |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCCs RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test frequency is set to Mid Range PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.4.1	Narrow band blocking for CA (2DL CA)

7.6A.4,1.1	Test purpose

Verifies a receiver's ability to receive an NR signal at its assigned CA channel frequencies in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The lack of narrow-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.4.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2DL CA.

7.6A.4.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6A.4.0.

7.6A.4.1.4	Test description

7.6A.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.6A.4.1.4.1-1, 7.6A.4.1.4.1-2 or 7.6A.4.1.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.4.1.4.1-1: Test configuration table for Intra-band contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

Table 7.6A.4.1.4.1-2: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 4 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 5 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 4: Test frequency is set to Mid Range PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 7.6A.4.1.4.1-3: Test configuration table for Intra-band non-contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 1, NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-3. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.4.2	Narrow band blocking for CA (3DL CA)

7.6A.4.2.1	Test purpose

Verifies a receiver's ability to receive an NR signal at its assigned CA channel frequencies in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The lack of narrow-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.4.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 3DL CA.

7.6A.4.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6A.4.0.

7.6A.4.2.4	Test description

7.6A.4.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.6A.4.2.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.4.2.4.1-1: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Inter-band : NOTE 5 Intra-band contiguous + Inter-band: NOTE 5 Intra-band non-contiguous + Inter-band: MaxWGap for Intra-band non-contiguous (NOTE 5) |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCCs RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test frequency is set to Mid Range PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.6A.4.3	Narrow band blocking for CA (4DL CA)

7.6A.4.3.1	Test purpose

Verifies a receiver's ability to receive an NR signal at its assigned CA channel frequencies in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The lack of narrow-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.4.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 4DL CA.

7.6A.4.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6A.4.0.

7.6A.4.3.4	Test description

7.6A.4.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.6A.4.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6A.4.3.4.1-1: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Inter-band : NOTE 5 Intra-band contiguous + Inter-band: NOTE 5 Intra-band non-contiguous + Inter-band: MaxWGap for Intra-band non-contiguous (NOTE 5) Intra-band non-contiguous + Intra-band non-contiguous: MaxWGap for Intra-band non-contiguous (NOTE 5) |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCCs RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test frequency is set to Mid Range PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.8A.2.1	Wide band Intermodulation for CA (2DL CA)

7.8A.2.1.1	Test purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8A.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2DL CA.

7.8A.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.8A.2.0.

7.8A.2.1.4	Test description

7.8A.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.8A.2.1.4.1-1, 7.8A.2.1.4.1-2 or 7.8A.2.1.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.8A.2.1.4.1-1: Test configuration table for Intra-band contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg (NOTE 3) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

Table 7.8A.2.1.4.1-2: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 4 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 5 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 4: Test frequency is set to Mid Range PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 5: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

Table 7.8A.1.4.1-3: Test configuration table for Intraband non-contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg, NOTE 1, NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-3. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.8A.2.2	Wide band Intermodulation for CA (3DL CA)

7.8A.2.2.1	Test purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8A.2.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 3DL CA.

7.8A.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.8A.2.0.

7.8A.2.2.4	Test description

7.8A.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.8A.2.2.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.8A.2.2.4.1-1: Test configuration table for CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Intra-band contiguous: Mid range Inter-band: NOTE 3 Intra-band contiguous + Inter-band: NOTE 3 Intra-band non-contiguous + Inter-band: NOTE 3 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg (NOTE 4) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCCs RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous) |  |  |  |  |  |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| Default Test Settings for a CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: The specific test frequencies for PCC and SCCs and Wgap for intra-band non-contiguous are defined in Table 7.3A.2.4.1-1. Only test points verifying non-exceptional REFSENS requirements are used for wide band blocking testing. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 5: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. |  |  |  |  |  |

< Unchanged sections omitted >

#### 7.8A.2.3	Wide band Intermodulation for CA (4DL CA)

7.8A.2.3.1	Test purpose

Intermodulation response tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when two or more interfering signals exist which have a specific frequency relationship to the wanted signal.

7.8A.2.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 4DL CA.

7.8A.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.8A.2.0.

7.8A.2.3.4	Test description

7.8A.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 7.8A.2.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.8A.2.3.4.1-1: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Inter-band : Mid range for PCC and SCCs (NOTE 5) Intra-band contiguous + Inter-band: Mid range for PCC and SCCs (NOTE 5) Intra-band non-contiguous + Inter-band: Mid range for PCC and SCCs with maxWGap for Intra-band non-contiguous (NOTE 5) |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg (NOTE 3) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCCs RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM QPSK | Full RB1 | Full RB1 | DFT-s-OFDM QPSK | REFSENS2 |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |  |

< Unchanged sections omitted >

### 7.9A.1	Spurious emissions for CA (2DL CA)

7.9A.1.1	Test Purpose

Test verifies the UE's spurious emissions meet the requirements described in clause 7.9A.1.3.

Excess spurious emissions increase the interference to other systems.

7.9A.1.2	Test Applicability

This test case applies to all types of NR UE release 15 and forward that support inter-band 2DL CA with a DL-only band.

7.9A.1.3	Minimum Conformance Requirements

The minimum conformance requirements are defined in clause 7.9A.0.

7.9A.1.4	Test Description

7.9A.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR CA bands specified in Table 5.5A.3-1. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in Table 7.9A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.9A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, Mid range, High range (NOTE 3) |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Highest NRB_agg NOTE 4 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Highest |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |
| Ch Configuration / NRB_agg |  | Downlink Configuration |  |  | Uplink Configuration |  |
| PCC NRB | SCCs NRB | Mod'n | PCC & SCC RB allocation |  | Mod'n | PCC RB allocation |
| 100 | 100 | N/A | 0 | 0 | N/A | 0 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3.2.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested |  |  |  |  |  |  |

< End of changes >