3GPP TSG-RAN5 Meeting #95-e	R5-222553r1
Electronic Meeting, 9th May – 20th May 2022

Agenda item:	5.4.17

Source:	Keysight Technologies

Title:	On QoQZ for 40cm QZ

Document for:	Discussion and Endorsement

# Introduction

This contribution discusses the QoQZ MU for a 40cm QZ to support larger devices. Given the various adjustments of virtual meeting dates and submission deadlines, the measurement campaign scheduled a few months ago could not be pulled in and did not complete on time. More complete results will be provided in a revision of this contribution.

# Discussion

This contribution is based on an EIRP&TRP QoQZ MU campaign with 166 measurement points; the full set of 238 measurements has not been completed yet given some debugging of the measurement results at 23.45 GHz. The quiet zone size evaluated in this contribution is a 40cm sphere and the procedure is following the procedure and requirements outlined in [1] while taking the re-positioning concept into account, the default for MU definition purposes. The reference AUT positions, other than P1, are all displaced by the centre of the QZ by R=20 cm, as tabulated in Table 1.

Table 1: Reference AUT Measurement Coordinates

| Position | x | y | z |
| --- | --- | --- | --- |
| P1 | 0 | 0 | 0 |
| P2 | 20cm | 0 | 0 |
| P3 | -20cm | 0 | 0 |
| P4 | 0 | 20cm | 0 |
| P5 | 0 | -20cm | 0 |
| P6 | 0 | 0 | 20cm |
| P7 | 0 | 0 | -20cm |

The measured EIRP/TRP QoQZ standard deviations/MUs from this QoQZ campaign are tabulated in Table 2.

Table 2: EIRP and TRP std. deviations from the 40 cm QoQZ scan with 166 out of 238 test points

| Frequency [GHz] | Standard Deviation [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | EIRP (P1 only) | EIRP (P1-P7) | TRP (P1 only) | TRP (P1-P7) |
| 23.45 | 0.59 | 0.69 | 0.45 | 0.50 |
| 32.125 | 0.36 | 0.58 | 0.41 | 0.41 |
| 40.8 | 0.37 | 0.42 | 0.34 | 0.34 |
| 44.3 | 0.18 | 0.45 | 0.29 | 0.29 |
| 49.0 | 0.44 | 0.59 | 0.36 | 0.36 |

Table 3: EIRP and TRP std. deviations from the full QoQZ scan (238 test points) for a 30cm QZ [taken from [2])

| Frequency [GHz] | Standard Deviation [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | EIRP (P1 only) | EIRP (P1-P7) | TRP (P1 only) | TRP (P1-P7) |
| 23.45 | 0.36 | 0.46 | 0.24 | 0.34 |
| 32.125 | 0.23 | 0.47 | 0.15 | 0.39 |
| 40.8 | 0.32 | 0.34 | 0.22 | 0.27 |
| 44.3 | 0.25 | 0.44 | 0.18 | 0.35 |
| 49.0 | 0.29 | 0.46 | 0.13 | 0.34 |

The 40 cm QoQZ performance (P1 and P1-P7) at 23.45 GHz is worse than the performance at the other test frequencies up to 49 GHz which is currently being investigated. Except for the performance at 23.45 GHz, the 40 cm QoQZ performance (P1 and P1-P7) is similar to that of the 30 cm QoQZ presented in [2] and tabulated in Table 3 which continues to make us believe that no degradation in QoQZ MU might be required when extending the QZ from 30 cm to 40 cm.

Observation 1: The 40 cm QoQZ performance at 23.45 GHz shows unexpectedly high MUs (expected to be an outlier)

Observation 2: The 40 cm QoQZ performance at 32.125 GHz through 49 GHz is similar to that of the 30 cm QoQZ performance

Observation 3: It is expected that no additional degradation in QoQZ MU is required when extending the QZ from 30 cm to 40 cm, but more time is needed to confirm this expectation.

Proposal 1: Defer the 40 cm QoQZ definition by one meeting cycle.

The XPDs were evaluated according to the approach outlined in [3], specifically B.2.1.10, and tabulated in Table 4. These results show some degradation of the XPDs for when the QZ is extended from 30 cm (previously agreed XPD MU of 0.01 dB) to 40 cm, especially for frequencies beyond 40.8 GHz.

Table 4: Measured XPD MUs for 40cm QZ

| XPD MU [dB] vs f [GHz] | Measured XPD MU [dB] | Proposed XPD MU [dB] |
| --- | --- | --- |
| 23.45 | 0.02 | [0.03] |
| 32.125 | 0.03 |  |
| 40.8 | 0.02 |  |
| 44.3 | 0.07 | [0.07] |
| 49 | 0.07 |  |

It is therefore proposed to relax the XPD MUs for the 40 cm QZ according as indicated in Table 4, i.e., XPD MU of [0.03] dB for frequencies up to 40.8 GHz and [0.07] dB for frequencies beyond 40.8 GHz.

Proposal 2: Relax the XPD MUs for the 40 cm QZ, i.e., define an XPD MU of [0.03] dB for frequencies up to 40.8 GHz and [0.07] dB for frequencies beyond 40.8 GHz.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The 40 cm QoQZ performance at 23.45 GHz shows unexpectedly high MUs (expected to be an outlier)

Observation 2: The 40 cm QoQZ performance at 32.125 GHz through 49 GHz is similar to that of the 30 cm QoQZ performance

Observation 3: It is expected that no additional degradation in QoQZ MU is required when extending the QZ from 30 cm to 40 cm, but more time is needed to confirm this expectation.

Proposal 1: Defer the 40 cm QoQZ definition by one meeting cycle.

Proposal 2: Relax the XPD MUs for the 40 cm QZ, i.e., define an XPD MU of [0.03] dB for frequencies up to 40.8 GHz and [0.07] dB for frequencies beyond 40.8 GHz.

# References

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.11.0 (2022-03)

- R5-211192, On n259 QoQZ and XPD MU, Keysight Technologies, 3GPP TSG-RAN5 Meeting #90-e, February 2022

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, V16.11.0 (2022-03)