3GPP TSG-RAN5 Meeting #95-e	R5-222584r1

# Electronic Meeting, 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0518 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to FR2 CA normal PDSCH test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SNR values are still in [] and TT analysis is pending for FR2 CA normal PDSCH test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed [] for SNR values since RAN4 has removed them Updated the testability of test requirements table to include CA test points Updated Annex F for MU/TT Removed editor’s note |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1_1, 7.2A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: segregated the testability of the FR2 CA case into BWagg <= 200 and and >200MHz |  |  |  |  |  |  |  |  |

### 7.1.1_1	Applicability of test requirements due to maximum achievable SNR

Editor’s note: The applicability of test requirements with a modulation above 64 QAM is FFS.

Table 7.1.1_1-1 specifies the current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Max device size ≤ 30 cm under fading conditions.

Table 7.1.1_1-1: maximum testable SNRBB under fading conditions for modulations up to 64 QAM

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | 30.6 | 27.5 | 24.4 |
| n258 mid | 30.6 | 27.5 | 24.4 |
| n259 mid | 20.4 | 17.2 | 14.1 |
| n260 mid | 24.4 | 21.2 | 18.2 |
| n261 mid | 30.6 | 27.5 | 24.4 |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 7.1.1_1-2 for indirect farfield (IFF), PC3, Max device size size ≤ 30 cm under fading conditions.

Table 7.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | x | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | x | x | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | x | x | x |
|  | 2-4 | 200 | Yes | 15.8 | x | x | - | x | x |
|  | 2-5 | 50 | Yes | 16 | x | x | x | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | - | x | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | - | x | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22.0 | TBD | TBD | TBD | TBD | TBD |
| 7.2.2.2.2_1 | 1-1 | 100 | Yes | TBD | TBD | TBD | TBD | TBD | TBD |
| 7.2.2.2.3_1 | 1-1 | 100 | Yes | 3.1 | x | x | x | x | X |
| 7.2A.2.1 | 1,2 | BWagg ≤ 200 | Yes | 12.0 | x | x | x | x | x |
|  | 3,4 | BWagg ≥ 200 | Yes | 12.0 | TBD | TBD | TBD | TBD | TBD |
| 7.2A.2.2 | 1,2 | BWagg ≤ 200 | Yes | 12.0 | x | x | x | x | x |
|  | 3,4 | BWagg ≥ 200 | Yes | 12.0 | TBD | TBD | TBD | TBD | TBD |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | x | x | x |

### <several sections skipped>

### 7.2A.2	2RX requirements

7.2A.2.0	Minimum conformance requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 7.2A.2.0-3-3 based on the single carrier requirements for different bandwidth specified in Table 7.2A.2.0-2, with the parameters in Table 7.2A.2.0-1 and the downlink physical channel setup according to Annex C.2.2. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 7.2A.2.0-1: Test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| TDD UL-DL pattern |  |  | 120kHz SCS: FR2.120-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

Table 7.2A.2.0-2: Single carrier performance for TDD 120 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 | R.PDSCH.5-9.1 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.4 |
| 100 | R.PDSCH.5-9.2 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.2 |
| 200 | R.PDSCH.5-9.3 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.3 |
| 400 | R.PDSCH.5-9.4 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 10.3 |

Table 7.2A.2.0-3: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | TDD 120 kHz + TDD 120 kHz | As defined in Table 7.2A.2.1_0-2 |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 7.1.1.5. |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 7.2A.2.1

#### 7.2A.2.1	2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (2DLCA)

7.2A.2.1.1	Test Purpose

Verify the PDSCH mapping Type A normal performance with different channel models, MCSs and number of MIMO layers.

7.2A.2.1.2	Test Applicability

This test applies to all types of NR UE release 15 and forward that support NR 2DL CA.

7.2A.2.1.3	Test Description

7.2A.2.1.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-2.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

CA capability to be tested: Any one of the supported CA capability as per the selection criteria specified in Table 7.1.1.5.2-2.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

1.	Connection between SS, the faders, AWGN noise source and the UE is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 7.2-1, 7.2A.2.0-1 and Table 7.2A.2.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.2A.2.1.3.3.

7.2A.2.1.3.2	Test Procedure

1.	Set the UE in a direction that satisfies the 3 normative criteria specified in Annex H.0. If no direction found, mark the test as inconclusive.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.2A.2.1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table  7.2A.2.1.4-1

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Tables G.1.5-1 in Annex G.

5.	Repeat steps from 1 to 4 for each subtest in Table 7.2A.2.1.4-1.

7.2A.2.1.3.3	Message Contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1 and 5.4.2

7.2A.2.1.4	Test Requirements

Table 7.2A.2.1.4-1: Single carrier performance for TDD 120 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 | R.PDSCH.5-9.1 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.1 |
| 100 | R.PDSCH.5-9.2 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 11.9 |
| 200 | R.PDSCH.5-9.3 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |
| 400 | R.PDSCH.5-9.4 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |

#### 7.2A.2.2	2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (3DLCA)

7.2A.2.2.1	Test Purpose

Same as 7.2A.2.1.1

7.2A.2.2.2	Test Applicability

This test applies to all types of NR UE release 15 and forward that support NR 3DL CA.

7.2A.2.2.3	Test Description

7.2A.2.2.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-2.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

CA capability to be tested: Any one of the supported CA capability as per the selection criteria specified in Table 7.1.1.5.2-2.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

1.	Connection between SS, the faders, AWGN noise source and the UE is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 7.2-1, 7.2A.2.0-1 and Table 7.2A.2.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.2A.2.1.3.3.

7.2A.2.2.3.2	Test Procedure

1.	Set the UE in a direction that satisfies the 3 normative criteria specified in Annex H.0. If no direction found, mark the test as inconclusive.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.2A.2.2.4-1 on each CC. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 7.2A.2.2.4-1.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Tables G.1.5-1 in Annex G.

5.	Repeat steps from 1 to 4 for each subtest in Table 7.2A.2.2.4-1.

7.2A.2.2.3.3	Message Contents

Same as 7.2A.2.1.3.3

7.2A.2.2.4	Test Requirements

Table 7.2A.2.2.4-1: Single carrier performance for TDD 120 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 50 | R.PDSCH.5-9.1 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.1 |
| 100 | R.PDSCH.5-9.2 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 11.9 |
| 200 | R.PDSCH.5-9.3 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |
| 400 | R.PDSCH.5-9.4 TDD | 16QAM, 0.33 | TDLA30-75 | 2x2, ULA Low | 70 | 12.0 |

### <several sections skipped>

Table F.2.1.2-2: Maximum test system uncertainty for FR2 demodulation performance test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.2.2.2.1_1 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for SA and NSA | 2Tx, Rank 1: ± 1.82 dB for Doppler < 100 Hz ± 1.78 dB for Doppler ≥100 Hz 2Tx, Rank 2: ± 1.67 dB for Doppler < 100Hz ± 1.63 dB for Doppler ≥100 Hz | Overall system uncertainty for fading conditions comprises four quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time 5. Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator SNR Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2 ) + Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB AWGN flatness and signal flatness ±3.6 dB SNR uncertainty due to finite test time ±0.3 dB for doppler < 100Hz, otherwise 0 dB Impact on non-ideal isolation between branches for the wireless cable mode 0.60 dB for Rank1, 0.45 dB for Rank2 |
| 7.2.2.2.1_2 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with enhanced type 1 receiver for SA and NSA | 2Tx, Rank 2: ± 1.67 dB for Doppler < 100Hz ± 1.63 dB for Doppler ≥ 100Hz | Same as 7.2.2.2.1_1 |
| 7.2.2.2.1_3 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with 256QAM for SA and NSA (Rel-16 and forward) | 2Tx, Rank 1: ± 1.82 dB for Doppler < 100 Hz | Same as 7.2.2.2.1_1 |
| 7.2A.2.1 2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (2DLCA) | 2Tx, Rank 2: ± 1.67 dB for Doppler < 100Hz | Overall system uncertainty for fading conditions comprises four quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time 5. Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator SNR Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2 ) + Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB AWGN flatness and signal flatness ±3.6 dB SNR uncertainty due to finite test time ±0.3 dB for doppler < 100Hz, otherwise 0 dB Impact on non-ideal isolation between branches for the wireless cable mode 0.45 dB for Rank2 |
| 7.2A.2.2 2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (3DLCA) | Same as 7.2A.2.1 | Same as 7.2A.2.1 |
| 7.3.2.2.1 2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA | 1Tx, rank1: ± 1.74 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time 5. Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator SNR Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2 ) + Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for 1Tx, ±0.7 dB for 2Tx AWGN flatness and signal flatness ±3.6 dB SNR uncertainty due to finite test time ±0.4 dB Impact on non-ideal isolation between branches for the wireless cable mode 0.6 for Rank1 and 0.45 for rank2 |
| 7.3.2.2.2 2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA | 2Tx, rank1: ± 1.84 dB | Same as 7.3.2.2.1 |
| 7.3.2.2.3 2Rx TDD FR2 PDCCH 1 Tx antenna performance for power saving | 1Tx, rank1: ± 1.74 dB | Same as 7.3.2.2.1 |
| 9.4B.1.2 Sustained downlink data rate performance for EN-DC including FR2 NR carrier | Downlink absolute power uncertainty (including beam peak search , averaged over BWConfig ± 5.19 dB | Downlink absolute power uncertainty (including beam peak search error) is one of the factors used to determine the max testable SNR for a given Test System as listed in sheet “Mode2 100MHz” in 38.521-4 Spreadsheet - Demod SNR range calculator V3.xlsx of TR 38.903 |

### <several sections skipped>

### F.2.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 7 is defined in Table F.2.3.2-1.

Table F.2.3.2-1: Derivation of Test Requirements (FR2 demodulation performance tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 7.2.2.2.1_1 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for SA and NSA | SNRs as specified | 2Tx, Rank 1: 1.8 dB 2Tx, Rank 2: 1.7 dB for doppler < 100Hz 1.6 dB otherwise | Formula: SNR + TT T-put limit unchanged |
| 7.2.2.2.1_2 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with enhanced type 1 receiver for SA and NSA | SNRs as specified | 2Tx, Rank 2: 1.7 dB for doppler < 100Hz 1.6 dB otherwise | Formula: SNR + TT T-put limit unchanged |
| 7.2.2.2.1_3 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with 256QAM for SA and NSA (Rel-16 and forward) | SNRs as specified | 2Tx, Rank 1: 1.8 dB | Formula: SNR + TT T-put limit unchanged |
| 7.2A.2.1 2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (2DLCA) | SNRs as specified | 2Tx, Rank 2: 1.7 dB | Formula: SNR + TT T-put limit unchanged |
| 7.2A.2.2 2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (3DLCA) | Same as 7.2A.2.1 | Same as 7.2A.2.1 | Same as 7.2A.2.1 |
| 7.3.2.2.1 2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 1Tx, rank1: 1.7 dB | Formula: SNR + TT T-put limit unchanged |
| 7.3.2.2.2 2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 2Tx, rank1: 1.8 dB | Formula: SNR + TT T-put limit unchanged |
| 7.3.2.2.3 2Rx TDD FR2 PDCCH 1 Tx antenna performance for power saving | SNRs as specified | 1Tx, rank1: 1.7 dB | Formula: SNR + TT T-put limit unchanged |
| 9.4B.1.2 Sustained downlink data rate performance for EN-DC including FR2 NR carrier | Indirect far field (IFF) with 30cm QZ, PC3, 100MHz CHBW Downlink power n257, n261: -79.5 dBm/120 kHz n258: -79.2 dBm/120 kHz | No test tolerance applied | T-put limit unchanged |