3GPP TSG-RAN5 Meeting #95-e	R5-222586r1

# Electronic meeting , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0520 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to FR1 CA CQI reporting test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Connection diagram for CA CQI reporting test case is currently missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added reference to connection diagram |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1.1, 6.2A.3.1.2, 6.2A.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependent on R5-222917 which introduces the 1x2 nDLCA connection diagram |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: editorial correction in 6.2A.3.1.1.1.3.4 |  |  |  |  |  |  |  |  |

##### 6.2A.3.1.1	2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA)

6.2A.3.1.1.1	Test Purpose

To verify that the CQI is correctly reported in accordance with the CQI definition given in TS 38.214 [12] for each CC with multiple cells configured for periodic reporting.

6.2A.3.1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA.

6.2A.3.1.1.3	Test description

6.2A.3.1.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

CA capability to be tested: test any one of the supported CA capabilities with largest aggregated CA bandwidth combination, as specified in 6.1.1.5.2.

CA configuration to be tested: For the selected CA capability, test any one of the supported CA configurations with largest aggregated CA bandwidth combination, as specified in 6.1.1.5.2.

Table 6.2A.3.1.1.3.1-1: Test point selection table

| Test number | CA duplex mode | Configuration | PCell CC configuration |
| --- | --- | --- | --- |
| 1 | FDD 15 kHz + TDD 30 kHz | As defined in Table 6.2A.3.1.0-1 to Table 6.2A.3.1.0-3 | TDD CC if supported, otherwise FDD CC |
| 2 (Note 2) | FDD 15 kHz + FDD 15 kHz | As defined in Table 6.2A.3.1.0-1 to Table 6.2A.3.1.0-2 | Any of CCs |
| 3 | TDD 30 kHz + TDD 30 kHz | As defined in Table 6.2A.3.1.0-1 and Table 6.2A.3.1.0-3 | Any of CCs |
| NOTE 1: The test coverage can be considered fulfilled if UE passes one of the CC as PCell in Test 1. NOTE 2: These scenarios are only tested for UEs which are not verified with Test 1. |  |  |  |

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A Figure A.3.1.7.2A for TE diagram, Figure A.3.2.3 for UE supporting only 2Rx RF bands on all CC. Annex A, Figure A.3.2.5 for UE supporting 4Rx on some or all the CCs.

2.	The parameter settings for the cell are set up according to Table 6.1.2-1, and Table 6.2A.3.1.0-1 to Table 6.2A.3.1.0-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2A.3.1.1.3.3.

6.2A.3.1.1.1.3.2	Test Procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 6.2A.3.1.1.1.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[19], clause9.3).

4.	Set the parameters of bandwidth, reference channel, propagation condition and antenna configuration according to Table 6.2A.3.1.0-1 to Table 6.2A.3.1.0-3 as appropriate. Set the SNR according to Table 6.2A.3.1.0-4 as appropriate for PCC and SCC (For UE supporting 4Rx antenna ports on a given CC, the SNR is reduced by 3dB for that CC).

5.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

6.	The SS shall start gathering CQI reports, and will continue gathering CQI reports until 2000 wideband CQI reports have been gathered for each PCC and SCC. For each CSI report calculate the respective difference CQIP-S = wideband CQIPCell – wideband CQISCell.

7.	If more than 1800 values of CQIP-S are ≥ 2 pass the UE. Otherwise fail the UE.

8.	Repeat steps from 1 to 7 for each test point in Table 6.2A.3.1.1.3.1-1 as appropriate.

6.2A.3.1.1.1.3.3	Message contents

Message contents are according to TS 38.508 [6] clause 5.4.2 with the following exceptions:

6.2A.2.1.1.1.3.3_1	Message exceptions for SA

Table 6.2A.2.1.1.1.3.3_1-1: CSI-RS-ResourceMapping

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.4-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row2 | 0000 1000 0000 | k0= 4, row2 |  |
| } |  |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2A.2.1.1.1.3.3_1-2: NZP CSI-ResourcePeriodicityAndOffset

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.4-2a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  | For FDD CC |
| slot10 | 1 |  | For TDD CC |
| } |  |  |  |

Table 6.2A.2.1.1.1.3.3_1-3: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.4-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| periodicityAndOffset SEQUENCE { |  |  |  |
| slots5 | 1 |  | For FDD CC |
| slots10 | 1 |  | For TDD CC |
| } |  |  |  |
| } |  |  |  |

Table 6.2A.2.1.1.1.3.3_1-4: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.4-12 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |
| subbandSize | 8 | For the CC with FDD 15kHz SCS 5MHz and 10MHz CHBW; For the CC with 30kHz SCS 10MHz, 15MHz, 20MHz and 25MHz CHBW. |
|  | 16 | For the CC with FDD 15kHz SCS 15MHz, 20MHz and 25MHz CHBW; For the CC with 30kHz SCS 30MHz, 40MHz and 50MHz CHBW. |
|  | 32 | For the CC with FDD 15kHz SCS 30MHz, 35MHz, 40MHz, 45MHz and 50MHz CHBW; For the CC with 30kHz SCS 60MHz, 80MHz, 90MHz and 100MHz CHBW. |
| } |  |  |

6.2A.2.1.1.1.3.3_2	Message exceptions for NSA

Same as specified in 6.2A.2.1.1.1.3.3_2.

6.2A.3.1.1.1.3.4	Test Requirements

The pass fail decision is as specified in the test procedure in clause 6.2A.3.1.1.1.3.2.

##### There are no parameters in the test setup or measurement process whose variation impacts the results so there are no applicable test tolerances for this test.

##### 6.2A.3.1.2	2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA)

6.2A.3.1.2.1	Test Purpose

Same with 6.2A.3.1.1.1.

6.2A.3.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 3DL CA.

6.2A.3.1.2.3	Test description

6.2A.3.1.2.3.1	Initial conditions

Same with 6.2A.3.1.1.3.1.

6.2A.3.1.1.2.3.2	Test Procedure

1.	Configure SCCs according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 6.2A.3.1.1.2.3.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[19], clause9.3).

4.	Set the parameters of bandwidth, reference channel, propagation condition and antenna configuration according to Table 6.2A.3.1.0-1 to Table 6.2A.3.1.0-3 as appropriate. Set the SNR according to Table 6.2A.3.1.0-5 as appropriate for PCC and SCCs (For a UE supporting 4Rx antenna ports on a given CC, the SNR is reduced by 3dB for that CC).

5.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

6.	The SS shall start gathering CQI reports, and will continue gathering CQI reports until 2000 wideband CQI reports have been gathered for each PCC and SCCs. For each CSI report calculate the respective difference CQIP-S1 = wideband CQIPCell – wideband CQISCell1 and the respective difference CQIS1-S2 = wideband CQISCell1 – wideband CQISCell2.

7.	If more than 1800 values of CQIP-S1 are ≥ 2 and more than 1800 values of CQI S1-S2 are ≥ 2, pass the UE. Otherwise fail the UE.

8.	Repeat steps from 1 to 7 for each test point in Table 6.2A.3.1.0-6 as appropriate.

6.2A.3.1.1.2.3.3	Message contents

Same with 6.2A.3.1.1.1.3.3.

6.2A.3.1.1.2.3.4	Test Requirements

The pass fail decision is as specified in the test procedure in clause 6.2A.3.1.1.2.3.2.

There are no parameters in the test setup or measurement process whose variation impacts the results so there are no applicable test tolerances for this test.

##### 6.2A.3.1.3	2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA)

6.2A.3.1.3.1	Test Purpose

Same with 6.2A.3.1.1.1.

6.2A.3.1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 4DL CA.

6.2A.3.1.3.3	Test description

6.2A.3.1.3.3.1	Initial conditions

Same with 6.2A.3.1.1.3.1.

6.2A.3.1.1.3.3.2	Test Procedure

1.	Configure SCCs according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 6.2A.3.1.1.3.3.3.

3.	SS activates SCCs by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[19], clause9.3).

4.	Set the parameters of bandwidth, reference channel, propagation condition and antenna configuration according to Table 6.2A.3.1.0-1 to Table 6.2A.3.1.0-3 as appropriate. Set the SNR according to Table 6.2A.3.1.0-5 as appropriate for PCC and SCCs (For a UE supporting 4Rx antenna ports on a given CC, the SNR is reduced by 3dB for that CC).

5.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC on both PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

6.	The SS shall start gathering CQI reports, and will continue gathering CQI reports until 2000 wideband CQI reports have been gathered for each PCC and SCCs. For each CSI report calculate the respective difference CQIP-S1 = wideband CQIPCell – wideband CQISCell1, the respective difference CQIS1-S2 = wideband CQISCell1 – wideband CQISCell2 and the respective difference CQIS1-S3 = wideband CQISCell1 – wideband CQISCell3.

7.	If more than 1800 values of CQIP-S1 are ≥ 2, more than 1800 values of CQI S1-S2 are ≥ 2 and more than 1800 values of CQI S1-S3 are ≥ 2, pass the UE. Otherwise fail the UE.

8.	Repeat steps from 1 to 7 for each test point defined in Table 6.2A.3.1.0-6 as appropriate.

6.2A.3.1.1.3.3.3	Message contents

Same with 6.2A.3.1.1.1.3.3.

6.2A.3.1.1.3.3.4	Test Requirements

The pass fail decision is as specified in the test procedure in clause 6.2A.3.1.1.3.3.2.

There are no parameters in the test setup or measurement process whose variation impacts the results so there are no applicable test tolerances for this test.