3GPP TSG-RAN5 Meeting #95-e	R5-222666r1

# Online, , 9th May 2022 – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1684 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Spurious emission for UE co-existence TC for CA_n24-n48 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The spurious emission for UE coexistence test requirements and test cases for CA_n24A-n48A are absent in clause 6.5A.3.2 in TS 38.521-1 and needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Tables 6.5A.3.2.0.3-3, 6.5A.3.2.1.4.1-1 and 6.5A.3.2.1.5-1b with values related to CA_n24A_n48A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing for spurious emission UE coexistence for CA_n24A-n48A will not be possible |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.2.0, 6.5A.3.2.1.4.1, 6.5A.3.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 608 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revisions: i) Addresses title inconsistency between 3GU and CR |  |  |  |  |  |  |  |  |

##### 6.5A.3.2.0	Minimum conformance requirements

<<Unchanged Text Skipped>>

Table 6.5A.3.2.0.3-3: Requirements for uplink inter-band carrier aggregation (two bands) Rel-17

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n39 | E-UTRA Band 1, 34, 40, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n8-n78 | E-UTRA Band 1,8, 11, 20, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n24-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n26-n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 43, 47, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 53 NR band 77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n70 | E-UTRA Band 2, 5, 10, 12, 13, 14, 17, 24, 25, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 8 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4, 8 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n48-n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n48-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5A.3.2.

<<Unchanged Text Skipped>>

6.5A.3.2.1.4	Test description

6.5A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS for default test points. CA configuration specific test points: Refer to “PCC NRB@SCS”and “SCC NRB@SCS ” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  |  |  | CA Configuration / NRB_agg (Note 4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  |  |  |  |  | UL Allocation (Note 2,3) |  |  |  |  |  |  |
|  |  |  |  | CA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC NRB @SCS |  |  |  | SCC NRB@SCS |  |  | CC MOD |  |  |  | PCC & SCC RB allocation |  |  |  |  | CC MOD |  | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
|  |  |  |  | PCC |  |  |  |  |  |  |  | SCC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC | SCC |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Band |  |  |  | Range |  |  |  | Band |  |  |  | Range |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XA-YA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  |  |  | Low |  |  |  | Y |  |  |  | Low |  |  |  | Highest NRB@SCS |  |  | Highest NRB@SCS |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  |  | 1@0 |  |
| 2 |  | X |  |  |  | High |  |  |  | Y |  |  |  | High |  |  |  | Highest NRB@SCS |  |  | Highest NRB@SCS |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@RBmax |  |  |  | 1@RBmax |  |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 |  |  |  | High |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  |  | 1@272 |
| 2 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | 3455MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 3 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | 3475MHz |  |  |  | 106@15kHz |  |  |  |  |  | 273@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 1@0 |
| 4 | n1 |  |  |  | High |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 106@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@105 |  |  | 106@0 |
| 5 | n1 |  |  |  | Low |  |  |  | n78 |  |  |  | High |  |  |  | 106@15kHz |  |  |  |  |  | 106@30kHz |  |  | CP-OFDM QPSK |  |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  |  | 1@0 |  |  | 106@0 |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 160@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@160 |  |  | 1@270 |  |  |
| 2 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 78@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@78 |  |  | 1@273 |  |  |
| 3 |  |  | n3 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 38@60KHz |  |  | 135@60KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@38 |  |  | 1@135 |  |  |
| Test Settings for CA_n24A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n24 |  |  |  | High |  |  |  | n48 |  |  |  | Mid |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@136 |  |  |
| 2 |  |  | n24 |  |  |  | High |  |  |  | n48 |  |  |  | High |  |  |  | 52@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 2 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 270@15kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@106 |  |  | 1@270 |  |  |
| 3 |  |  | n8 |  |  |  | Low |  |  |  | n78 |  |  |  | Low |  |  |  | 51@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@273 |  |  |
| 4 |  |  | n8 |  |  |  | Mid |  |  |  | n78 |  |  |  | Mid |  |  |  | 51@30KHz |  |  | 273@30KHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@51 |  |  | 1@273 |  |  |
| Test Settings for CA_n28A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n28 |  |  |  | High |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@105 |  |  | 1@272 |  |  |
| 2 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | Mid |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| 3 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@272 |  |  |
| 4 |  |  | n28 |  |  |  | Low |  |  |  | n41 |  |  |  | High |  |  |  | 106@15kHz |  |  | 273@30kHz |  |  |  | CP-OFDM QPSK |  |  | NA |  |  |  |  | CP-OFDM QPSK |  |  | 1@0 |  |  | 1@0 |  |  |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4870MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@0 |  |  |
| 2 |  |  | n41 |  |  |  | Mid |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@136 |  |  | 1@136 |  |  |
| 3 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@272 |  |  |
| 4 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4450MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| 5 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | 4500MHz |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |
| 6 |  |  | n41 |  |  |  | Mid |  |  |  | n79 |  |  |  | Low |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@136 |  |  | 1@0 |  |  |
| 7 |  |  | n41 |  |  |  | High |  |  |  | n79 |  |  |  | Mid |  |  |  | 273@30KHz |  |  | 273@30KHz |  |  |  | QPSK |  |  | NA |  |  |  |  | QPSK / CP-OFDM QPSK |  |  | 1@272 |  |  | 1@136 |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [6] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.2.1.4.3.

6.5A.3.2.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, and Annex C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.5A.3.2.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.2.1.4.1-1 on both PCC and SCC. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5.3.2.3-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5.3.2.3-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

6.5A.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.5A.3.2.1.4.3-1: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Inter-band CA |

6.5A.3.2.1.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements. The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Tables 6.5A.3.2.1.5-1, 6.5A.3.2.1.5-1a (Rel-16) and 6.5A.3.2.1.5-1 (Rel-17). The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

<<Unchanged Text Skipped>>

Table 6.5A.3.2.1.5-1b: Requirements for uplink inter-band carrier aggregation (two bands) Rel-17

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band |  | Frequency range (Mhz) |  |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) |  | NOTE |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high |  |  | -50+TT |  | 1 |  |
|  | Frequency range |  | 1880 |  | - |  | 1895 |  |  | -40+TT |  | 1 | 4, 6 |
|  | Frequency range |  | 1895 |  | - |  | 1915 |  |  | -15.5+TT |  | 5 | 4, 6, 7 |
|  | Frequency range |  | 1915 |  | - |  | 1920 |  |  | +1.6+TT |  | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low |  | - |  | FDL_high |  |  | -50+TT |  | 1 |  |  |
|  | Frequency range | 1880 |  | - |  | 1895 |  |  | -40+TT |  | 1 |  | 4, 6 |
|  | Frequency range | 1895 |  | - |  | 1915 |  |  | -15.5+TT |  | 5 |  | 4, 6, 7 |
|  | Frequency range | 1915 |  | - |  | 1920 |  |  | +1.6+TT |  | 5 |  | 4, 6, 7 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n8-n39 | E-UTRA Band 1, 34, 40, 50, 51, 74 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 4 |
| CA_n8-n78 | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 3, 7,41 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n24-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
| CA_n26-n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 43, 47, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 41, 42, 48, 53 NR band 77 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n26-n70 | E-UTRA Band 2, 5, 10, 12, 13, 14, 17, 24, 25, 29, 30, 48, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 41, 53 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41+TT |  |  | 0.3 |  | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | E-UTRA Band 18, 19 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 11 |
|  | E-UTRA Band 1 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 11, 15 |
|  | E-UTRA Band 11, 21 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 11, 12 |
|  | Frequency range |  | 470 |  | - |  | 694 | -42 |  |  | 8 |  | 4, 14 |
|  | Frequency range |  | 470 |  | - |  | 710 | -26.2 |  |  | 6 |  | 13 |
|  | Frequency range |  | 662 |  | - |  | 694 | -26.2 |  |  | 6 |  | 4 |
|  | Frequency range |  | 758 |  | - |  | 773 | -32 |  |  | 1 |  | 4 |
|  | Frequency range |  | 773 |  | - |  | 803 | -50 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41 |  |  | 0.3 |  | 3, 11 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  |  |  | 1855 | -40 |  |  | 1 |  | 8 |
|  | Frequency range |  | 1855 |  |  |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n77, n78, n79 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  | - |  | 1855 | -40 |  |  | 1 |  | 4 |
|  | Frequency range |  | 1855 |  | - |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | NR Band n78 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  | 2 |
|  | Frequency range |  | 1805 |  | - |  | 1855 | -40 |  |  | 1 |  | 4, 8 |
|  | Frequency range |  | 1855 |  | - |  | 1880 | -15.5 |  |  | 5 |  | 4, 7, 8 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 |  | FDL_low |  | - |  | FDL_high | -50 |  |  | 1 |  |  |
|  | Frequency range |  | 1884.5 |  | - |  | 1915.7 | -41 |  |  | 0.3 |  | 3 |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
| CA_n48-n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 41 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
| CA_n48-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high | -38+TT |  |  | 1 |  | 15 |
|  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 15 |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high | -38+TT |  |  | 1 |  | 4 |
|  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 4 |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 |  | FDL_low |  | - |  | FDL_high | -50+TT |  |  | 1 |  | 2 |
|  | E-UTRA Band 29 |  | FDL_low |  | - |  | FDL_high | -38+TT |  |  | 1 |  | 4 |
|  | E-UTRA Band 71 |  | FDL_low |  | - |  | FDL_high | -38+TT |  |  | 1 |  | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 from the edge of the channel bandwidth. NOTE 5: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart> 3. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5A.3.2.1.5-2: Test Tolerance for uplink inter-band carrier aggregation (two bands)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than ΔfOOB (MHz) in Table 6.5.3.2.3-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.5.3.2.3-1 apply for all transmitter band configurations (NRB) and channel bandwidths for all CC combinations.

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------