3GPP TSG-RAN5 Meeting #95-e	R5-222764r1

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0397 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 7.12-MO Voice Call without preconditions at MT UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The description that steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed, is missing between step 7 and 8. But the corresponding behavior has been implemented in TTCN code. - RRCReconfigurationComplete message is missing after steps 1A-1F. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add steps 7A-7C corresponding to steps 10-12 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1. - Add Table 7.12.3.2-2 for RRCReconfigurationComplete message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of test body is not aligned with the implemetation of TTCN code. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | 1st revision: - Fix editorial issue after step 2. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.12	MTSI MO Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS

7.12.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions }

ensure that {

when { UE is being made to initiate a voice call }

then { UE sends INVITE for voice call with preconditions }

}

(2)

with { UE having sent INVITE with preconditions }

ensure that {

when { UE receives 183 Session Progress without preconditions }

then { UE sends PRACK for 183 Session Progress }

}

(3)

with { UE having sent PRACK }

ensure that {

when { UE receives 200 OK for PRACK followed by 180 Ringing followed by 200 OK for INVITE }

then { UE sends ACK }

}

7.12.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.3.1]:

During the session initiation, if the originating UE indicated the support for the precondition mechanism in the initial INVITE request and:

a)	the received response with an SDP body includes a Require header field with "precondition" option-tag, the originating UE shall include a Require header field with the "precondition" option-tag:

-	in subsequent requests that include an SDP body, that the originating UE sends in the same dialog as the response is received from; and

-	in responses with an SDP body to subsequent requests that include an SDP body and include "precondition" option-tag in Supported header field or Require header field received in-dialog; or

b)	the received response with an SDP body does not include the "precondition" option-tag in the Require header field,

-	in subsequent requests that include an SDP body, the originating UE shall not include a Require or Supported header field with "precondition" option-tag in the same dialog;

-	in responses with an SDP body to subsequent requests with an SDP body but without "precondition" option-tag in the Require or Supported header field, the originating UE shall not include a Require or Supported header field with "precondition" option-tag in the same dialog; and

-	in responses with an SDP body to subsequent requests with an SDP body and with "precondition" option-tag in the Require or Supported header field, the originating UE shall include a Require header field with "precondition" option-tag in the same dialog.

7.12.3	Test description

7.12.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

Preamble:

-	UE is in state 1N-A and registered to IMS

7.12.3.2	Test procedure sequence

Table 7.12.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call. | - | - | - | - |
| 2 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel with Step 3, parallel behaviour defined in table 7.12.3.2-2 takes place | - | - | - | - |
| 3 | Step 1 of Annex A.4.1 happens (Check: does the UE send INVITE for voice call with preconditions?) | --> | INVITE | 1 | P |
| 4 | Step 2 of Annex A.4.1 happens | <-- | 100 Trying | - | - |
| 5 | Step 3 of Annex A.4.2 happens (Note: the SS sends 183 Session Progress without attributes for preconditions in the SDP body.) | <-- | 183 Session Progress | - | - |
| 6 | Step 4 of Annex A.4.2 happens (Check: does the UE send PRACK?) | --> | PRACK | 2 | P |
| 7 | Step 5 of Annex A.4.2 happens | <-- | 200 OK | - | - |
| 7A-7C | Steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 8 | Step 6 of Annex A.4.2 happens | <-- | 180 Ringing | - | - |
| 9 | Step 7 of Annex A.4.2 happens | <-- | 200 OK | - | - |
| 10 | Step 8 of Annex A.4.2 happens (Check: does the UE send ACK?) | --> | ACK | 3 | P |

Table 7.12.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

7.12.3.3	Specific message contents

None as fully described in Annex A.4.1 and A.4.2.

<End of modified section 1>