3GPP TSG-RAN5 Meeting #95-e	R5-222835r1

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2353 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Combinations of system information blocks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose for 8.1.1.2.4 , Test purpose is to verify Rel-15 UE by sending the extended field in system information that it does not comprehent . for this test purpose SIB11-r16 is sent along with SIB2 as per the table 8.1.1.2.4.3.3-2: System Information. For SS to schedule SIB11-r16, it is also mandatory that SIB1 si_SchedulingInfo schedulingInfoList sib_MappingInfo should contain the SIB11-r16 entry |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal to add new system information combination NR-18 which contains Scheduling of system information combination SIB2, SIB11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-222835r1 : Instead of using NR-18, used NR-19 to avoid overlap and conflicts with other CRs. |  |  |  |  |  |  |  |  |

#### 4.4.3.1	Common configurations of system information blocks

##### 4.4.3.1.1	Combinations of system information blocks for E-UTRA standalone, EN-DC and NGEN-DC

The combination of system information blocks for standalone E-UTRA, EN-DC and NGEN-DC network scenarios are specified in TS 36.508 [2] clause 4.4.3.1.

For EN-DC and NGEN-DC network scenarios the SS shall in addition to broadcasting the E-UTRA system information blocks also broadcast the NR MIB on the NR cell(s).

##### 4.4.3.1.2	Combinations of system information blocks for NR standalone and NE-DC

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, several combinations of system information blocks are defined.

Regardless of the combination of system information blocks indicated as being used by a test case, the SS shall broadcast only the NR MIB on the NR Cell(s) configured on an SDL band.

Combination NR-1 is the default combination which applies to the following test case scenarios:

-	NR FDD single cell scenario except RRM test case scenarios

-	NR TDD single cell scenario except RRM test case scenarios

-	NR FDD inter-band DC component carriers cell scenario

-	NR TDD inter-band DC component carriers cell scenario

-	NR FDD and NR TDD inter-band DC component carriers cell scenario

Combination NR-2 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario

-	NR TDD intra-frequency multi cell scenario

-	NR FDD and NR TDD dual mode multi cell roaming scenario

-	NR FDD single cell RRM test case scenario

-	NR TDD single cell RRM test case scenario

Combination NR-3 applies to the following test case scenarios:

-	NR FDD intra-frequency multi cell scenario with neighbouring cell related information

-	NR TDD intra-frequency multi cell scenario with neighbouring cell related information

Combination NR-4 applies to the following test case scenarios:

-	NR FDD inter-frequency multi cell scenario

-	NR TDD inter-frequency multi cell scenario

-	NR FDD inter-band multi cell scenario

-	NR TDD inter-band multi cell scenario

-	NR FDD and NR TDD dual mode multi cell non-roaming scenario

-	NR FDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD inter-band carrier aggregation component carriers cell scenario

-	NR TDD intra-band carrier aggregation component carriers cell scenario

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario

Combination NR-5 applies to the following test case scenarios:

-	NR FDD intra-band carrier aggregation component carriers cell scenario + NR FDD intra-frequency neighbour.

-	NR FDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR TDD intra-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

-	NR FDD and NR TDD inter-band carrier aggregation component carriers cell scenario+ NR FDD intra-frequency neighbour.

Combination NR-6 applies to the following test case scenarios:

-	3GPP inter-RAT NR FDD + E-UTRA FDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA TDD multi cell scenario

-	3GPP inter-RAT NR TDD + E-UTRA FDD multi cell scenario

Combination NR-7 applies to the following test case scenarios:

-	NR FDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

-	NR TDD inter-frequency + 3GPP inter-RAT E-UTRA multi-cell scenario

Combination NR-8 applies to the following test case scenarios:

-	NR FDD ETWS single cell scenario

-	NR TDD ETWS single cell scenario

Combination NR-9 applies to the following test case scenarios:

-	3GPP NR FDD + CMAS single cell scenario

-	3GPP NR TDD + CMAS single cell scenario

Combination NR-10 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS primary notification single cell scenario

-	3GPP NR TDD + ETWS primary notification single cell scenario

Combination NR-11 applies to the following test case scenarios:

-	3GPP NR FDD + ETWS secondary notification single cell scenario

-	3GPP NR TDD + ETWS secondary notification single cell scenario

Combination NR-12 applies to the following test case scenarios:

-	3GPP NR FDD + SNPN only single cell scenario

-	3GPP NR TDD + SNPN only single cell scenario

Combination NR-13 applies to the following test case scenarios:

-	3GPP NR FDD + CAG cell  multi cell scenario

-	3GPP NR TDD + CAG cell  multi cell scenario

Combination NR-14 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD single cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD intra-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-frequency multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR FDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

-	3GPP NR TDD inter-band multi cell scenario + NR sidelink communication with or without network scheduling.

Combination NR-15 applies to the following test case scenarios:

-	3GPP NR FDD single cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD single cell scenario + positioning test case which require posSIBs.

Combination NR-16 applies to the following test case scenarios:

-	3GPP NR FDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD intra-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-17 applies to the following test case scenarios:

-	3GPP NR FDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

-	3GPP NR TDD inter-frequency multi cell scenario + positioning test case which require posSIBs.

Combination NR-19 applies to the following test case scenarios:

-	3GPP NR FDD + Extended field in SI scenario

-	3GPP NR TDD + Extended field in SI scenario

Table 4.4.3.1.2-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB1 | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 | SIB14 | posSIBs |
| NR-1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-2 | X | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-3 | X | X | X |  |  |  |  |  |  |  |  |  |  |  |  |
| NR-4 | X | X |  | X |  |  |  |  |  |  |  |  |  |  |  |
| NR-5 | X | X | X | X |  |  |  |  |  |  |  |  |  |  |  |
| NR-6 | X | X |  |  | X |  |  |  |  |  |  |  |  |  |  |
| NR-7 | X | X |  | X | X |  |  |  |  |  |  |  |  |  |  |
| NR-8 | X |  |  |  |  | X | X |  |  |  |  |  |  |  |  |
| NR-9 | X |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| NR-10 | X |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| NR-11 | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| NR-12 | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| NR-13 | X | X |  |  |  |  |  |  |  | X |  |  |  |  |  |
| NR-14 | X | X |  | X |  |  |  |  |  |  |  | X |  |  |  |
| NR-15 | X |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-16 | X | X |  |  |  |  |  |  |  |  |  |  |  |  | X |
| NR-17 | X | X |  | X |  |  |  |  |  |  |  |  |  |  | X |
| NR-19 | X | X |  |  |  |  |  |  |  |  | X |  |  |  |  |

##### 4.4.3.1.3	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables. There is no scheduling information for combination NR-1.

Table 4.4.3.1.3-1: Scheduling for combination NR-2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |

Table 4.4.3.1.3-2: Scheduling for combination NR-3

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB3 |

Table 4.4.3.1.3-3: Scheduling for combination NR-4

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |

Table 4.4.3.1.3-4: Scheduling for combination NR-5

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB3 |
| 3 | 64 | SIB4 |

Table 4.4.3.1.3-5: Scheduling for combination NR-6

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB5 |

Table 4.4.3.1.3-6: Scheduling for combination NR-7

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4, SIB5 |

Table 4.4.3.1.3-7: Scheduling for combination NR-8

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB6 |
| 2 | 32 | SIB7 |

Table 4.4.3.1.3-8: Scheduling for combination NR-9

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB8 |

Table 4.4.3.1.3-9: Scheduling for combination NR-10

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
|  | 32 | SIB6 |

Table 4.4.3.1.3-10: Scheduling for combination NR-11

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB7 |

Table 4.4.3.1.3-11: Scheduling for combination NR-12

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 64 | SIB10 |

Table 4.4.3.1.3-12: Scheduling for combination NR-13

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB10 |

Table 4.4.3.1.3-13: Scheduling for combination NR-14

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2 |
| 2 | 64 | SIB4 |
| 3 | 64 | SIB12 |

Table 4.4.3.1.3-18: Scheduling for combination NR-19

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 32 | SIB2, SIB11 |