3GPP TSG-RAN5 Meeting #95-e	R5-222870r1

# Electronic Meeting, May 9 - May 20, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0170 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Jumbo Applicability CR for NR_RF_TxD WI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Bureau Veritas, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Applicability for TC 6.2.2 MPR, TC 6.2.3 A-MPR and TC 6.5.2.4.1 ACLR need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicability for TC 6.2.2 MPR, TC 6.2.3 A-MPR and TC 6.5.2.4.1 ACLR have been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 6.2.2 MPR, TC 6.2.3 A-MPR and TC 6.5.2.4.1 ACLR can not be implemented correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The pending CR is R5-223163, R5-222227, R5-222228, R5-222229, R5-222230. “XX” in Table A.4.0-1 need to be allocated with hard numbers. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Typo of “xDiversity-r16” has been revised as “txDiversity-r16”; Pending CR information has been updated. C001e and C001f have been added. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The tested bands selection criteria given in Table 4.0-2 apply. The tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2 and Table 4.0-3 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-1: Applicability of conformance test cases conditions

C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C001a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7/3 THEN R ELSE N/A

C001b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C001c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.1-2/2e OR A.4.3.1-2/12) THEN R ELSE N/A

C001d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/QQ) THEN R ELSE N/A

C001e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 NOT A.4.3.2-1/XX) THEN R ELSE N/A

C001f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND ( (A.4.3.1-7/2 AND A.4.3.2-1/XX) OR (A.4.3.1-7/5) ) ) THEN R ELSE N/A

C002 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C003 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C003b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) AND (A.4.3.2-1/58 OR A.4.3.2-1/59 OR A.4.3.2-1/60) THEN R ELSE N/A

C003a IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001e | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 | Not required to be tested for Power Class 2 if UE reports txDiversity-r16. |
| 6.2.2 | UE maximum output power reduction | Rel-15 | C001e | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.4.1 is executed. Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.3 or 6.5B.2.3.3.1 has been executed. Not required to be tested for Power Class 2 if UE reports txDiversity-r16. |
| 6.2.3 | UE additional maximum output power reduction | Rel-15 | C001e | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed. Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.3 has been executed or TS 38.521-3 TCs 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 have been executed. Not required to be tested for Power Class 2 if UE reports txDiversity-r16. |
| << UNCHANGED PARTS SKIPPED >> |  |  |  |  |  |  |  |
| 6.2D.2 | UE maximum output power reduction for UL MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL MIMO | D001 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.4.1 is executed. Not required to be tested for Power Class 2 if UE reports txDiversity-r16. |
| << UNCHANGED PARTS SKIPPED >> |  |  |  |  |  |  |  |
| 6.5.2.4.1 | NR ACLR | Rel-15 | C001e | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 | Skip TC 6.5.2.4.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.3.1 has been executed. Not required to be tested for Power Class 2 if UE reports txDiversity-r16. |
| << UNCHANGED PARTS SKIPPED >> |  |  |  |  |  |  |  |
| 6.2G | Transmitter power for Tx Diversity |  |  |  |  |  |  |
| 6.2G.1 | UE maximum output power for Tx Diversity | Rel-15 | C001f | UEs supporting 5GS FR1 | D001 | PC1.5 PC2 | Required to be tested for Power Class 2 if UE reports txDiversity-r16. |
| 6.2G.2 | UE maximum output power reduction for Tx Diversity | Rel-15 | C001f | UEs supporting 5GS FR1 | D001 | PC1.5 PC2 | Required to be tested for Power Class 2 if UE reports txDiversity-r16. |
| 6.2G.3 | UE additional maximum output power reduction | Rel-15 | C001f | UEs supporting 5GS FR1 | D001 | PC1.5 PC2 | Required to be tested for Power Class 2 if UE reports txDiversity-r16. |
| << UNCHANGED PARTS SKIPPED >> |  |  |  |  |  |  |  |
| 6.5G.2.3 | Adjacent channel leakage ratio | Rel-15 | C001f | UEs supporting 5GS FR1 | D001 | PC1.5 PC2 | Required to be tested for Power Class 2 if UE reports txDiversity-r16. |

<< END OF CHANGES >>