3GPP TSG RAN5 Meeting #95-e	                   R5-222891r1

Electronic Meeting, May 9 – 20, 2022

Agenda item:	5.3.15.6

Source:	Huawei, HiSilicon

Title: 	Discussion on minimum test time for 1% residual BLER

Document for:	DISCUSSION AND ENDORSEMENT

1	Background

The minimum test time for PDSCH demodulation test cases have been specified in TS 38.521-4 for various fading channel and RMCs [1-3]. Most test cases have the minimum requirement of 70% or 30% throughput.

In Rel-16, RAN4 specified a new type of PDSCH demodulation test cases with minimum requirement of 1% residual BLER. The relevant test cases include following:

5.2.2.1.6	Minimum requirements for PDSCH repetitions over multiple slots

5.2.2.1.14	Minimum requirements for PDSCH with single-DCI based Inter-slot TDM scheme

The minimum test time for legacy test cases can’t apply to 1% residual BLER test cases. This discussion paper provides some simulation result and consideration on proper minimum test time.

2	Discussion

In RAN4#94 meeting, some initial simulation results were reviewed and discussed[4]. It showed that residual BLER doesn’t converge to the BLER corridor of 1% +/- 0.1% for all the 30 seeds with the simulation time of 70s. The group suggested to further extend the simulation time to see the maximum time needed for the UE to converge.

Based on the discussion, new simulation was run for following two test cases. 30 seeds were run for each test cases, and the simulation time is 200s for each seed

|  | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | SNR (dB) | Simulation time | # of Seeds |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.2.1.6 | R.PDSCH.1-11.1 FDD | 10 / 15 | 16QAM, 0.54 | - | TDLA30-10 | 2x2, ULA Low | 1.6 | 200s | 30 |
| 5.2.2.1.14 | R.PDSCH.1-11.2 FDD | 10 / 15 | 16QAM, 0.54 | - | TDLA30-10 | 2x2, ULA Low | 2.9 | 200s | 30 |

The convergence time is acquired when the throughput curve enters the convergence corridor. Four convergence corridors are considered: 1%+/-0.1% ~ 1%+-0.5%.

1%+0.1%

1%+0.2%

1%+0.3%

1%+0.5%

Below figures summarize the simulation result.

Table 1 Simulation result for test case 5.2.2.1.6

| BLER Convergence corridor | Maximum converge time | No. of not converged seeds |
| --- | --- | --- |
| 1% +/- 0.1% | 200s | 16 |
| 1% +/- 0.2% | 132s | 11 |
| 1% +/- 0.3% | 85s | 8 |
| 1% +/- 0.5% | 200s (NOTE 1) | 3 |
| NOTE 1: There are two seeds that converge to 1% +/- 0.5% by 190~200s, which couldn’t converge at all for narrower corridors. If these two seeds are taken out, the maximum converge time is 29s. |  |  |

Table 2 Simulation result for test case 5.2.2.1.14

| BLER Convergence corridor | Maximum converge time | No. of not converged seeds |
| --- | --- | --- |
| 1% +/- 0.1% | 190s | 17 |
| 1% +/- 0.2% | 184s | 10 |
| 1% +/- 0.3% | 80s | 6 |
| 1% +/- 0.5% | 70s | 2 |

Based on the simulation result, following observations could be made:

Observation 1: With the BLER convergence corridor selected as 1%+/-0.1, more than half of the seeds couldn’t converge.

Observation 2: Only with BLER convergence corridor selected as 1%+/-0.5%, most of the seeds could converge.

Even with the BLER convergence corridor of 1%+/-0.5%, not all the seeds could converge. Those outstanding seeds would converge to a relatively far BLER point. Below figure shows the converged BLER point for each seed at the end of 200s.

Based on the simulation results, it seems the minimum test time should be decided as at least 200s, which could allow most seeds converge into the 0.5% corridor.

Proposal 1: The minimum test time is decided as 200s with the assumption of 1%+/-0.5% convergence corridor.

Considering the converged BLER point would be up to 1.5%, which might lead to mistakenly failing a conformant DUT, some tolerance needs to be considered.

Option 1: Directly apply 0.5% tolerance to the BLER requirement.

Option 2: Apply a SNR offset to make sure most seeds could converge to <1%. The delta SNR between 1% and 1.5% BLER could be considered, which is 0.3dB for 5.2.2.1.6 and 0.4dB for 5.2.2.1.14 according to simulation.

Proposal 2: RAN5 to decide a tolerance from Option 1 and 2.

3	Summary

This document provides further simulation result for 1% residual BLER test cases. Following proposals are provided for RAN5 to consider:

Proposal 1: The minimum test time is decided as 200s with the assumption of 1%+/-0.5% convergence corridor.

Proposal 2: RAN5 to decide a tolerance from Option 1 and 2.

Option 1: Directly apply 0.5% tolerance to the BLER requirement.

Option 2: Apply a SNR offset to make sure most seeds could converge to 1%. The delta SNR between 1% and 1.5% BLER could be considered, which is 0.3dB for 5.2.2.1.6 and 0.4dB for 5.2.2.1.14 according to simulation.

4	References

[1]	R5-191250, Discussion paper on statistical tput calculation for demod test cases, Qualcomm

[2]	R5-194978, Discussion on minimum test time for PDSCH and PDCCH with more simulation results, Qualcomm

[3]	R5-204738	FR2 Demod MU due to finite test time

[4]	R5-220811	Discussion on minimum test time for 1% residual BLER, RAN5#94