3GPP TSG-RAN5 Meeting #95-e	R5-222892r1

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0537 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Annex G for minimum test time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The minimum test time for 1% residual BLER is still FFS. In this meeting a discussion paper R5-222891 is submitted to address this issue. This CR is to capture the output of discussion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding minimum test time for 1% residual BLER test cases as [200s] based on the output of discussion of R5-223631. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The minimum test time remain unfinalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Adding minimum test time for 1% residual BLER test cases as [200s] based on the output of discussion of R5-223631. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

G.1.5	Minimum Test time

Editor's Note: Simulation method to derive minimum test time for FR2 needs to be evaluated.

If a pass fail decision in clause G.1.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

The tables below contain the minimum number of slots for FDD and TDD.

By simulations the minimum number of active subframes (carrying DL payload) was derived (MNAS), then adding inactive subframes to the active ones. (for TDD additional subframes contain no DL payload), then rounding up to full thousand.

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30 % or 70 % of the max. throughput) the preliminary throughput versus time converges towards the final throughput. The allowance of ± 0.2 dB around the above mentioned level is predefined by RAN5 to find the minimum test time. The allowance of ±0.2 dB maps through the function "final throughput versus level" into a throughput corridor. The minimum test time is achieved when the preliminary throughput escapes the corridor the last time. The two functions "final throughput versus level" and "preliminary throughput versus time" are simulation results, which are done individual for each demodulation scenario.

Figure G.1.5-1: Simulation method to derive minimum test time

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern |  | Reference Channel |  | Propagation condition |  | Demodulation scenario (doppler speed) |  | Minimum number of active subframes (MNAS) |  | MNAS to MNS Scaling factor (Note 3) |  | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NA |  | R.PDSCH.1-8.1 FDD |  | HST-750 |  | 750 Hz |  | 6000 (Note 1) |  | 1.0526 |  | 6400 |  |
| NA |  | R.PDSCH.1-8.2 FDD |  | HST-972 |  | 972 Hz |  | 6000 (Note 1) |  | 1.0526 |  | 6400 |  |
| NA |  | R.PDSCH.1-8.1 FDD |  | TDLC300-600 |  | 600 Hz |  | 8000 (Note 1) |  | 1.0526 |  | 9000 |  |
| NA |  | R.PDSCH.1-1.1 FDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.0526 |  | 11000 |  |
| NA |  | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.0526 |  | 22000 |  |
| FR1.15-1 |  | R.PDSCH.1-1.1 TDD, R.PDSCH.1-1.2 TDD |  | TBD |  | 10 Hz |  | 75000 (Note 2) |  | 2.8571 |  | 215000 |  |
| NA |  | R.PDSCH.1-1.3 FDD, R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-2.5 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-3.2 FDD, R.PDSCH.1-3.3 FDD, R.PDSCH.1-3.4 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-12.1 FDD, R.PDSCH.2-1.1 FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.0526 |  | 79000 |  |
| NA |  | R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.25 |  | 94000 |  |
| NA |  | R.PDSCH.1-8.3 FDD |  | HST-SFN |  | 870 Hz |  | 30000 (Note 1) |  | 1.0526 |  | 32000 |  |
| NA |  | R.PDSCH.1-8.4 FDD, R.PDSCH.1-8.4 FDD |  | HST-DPS |  | 870 Hz |  | 30000 (Note 1) |  | 1.0526 |  | 32000 |  |
| FR1.15-1 |  | R.PDSCH.1-1.1 TDD, R.PDSCH.1-1.2 TDD |  | TDLA30-10 |  | 10Hz |  | 75000 (Note 1) |  | 2.8571 |  | 215000 |  |
| FR1.30-1A |  | R.PDSCH.2-1.1 TDD |  | TDLB100-400 |  | 400 Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-1 |  | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD |  | TDLC300-100 |  | 100 Hz |  | 20000 (Note 1) |  | 1.2903 |  | 26000 |  |
| FR1.30-1 |  | R.PDSCH.2-2.2 TDD, R.PDSCH.2-2.3 TDD, R.PDSCH.2-2.4 TDD, R.PDSCH.2-2.5 TDD, R.PDSCH.2-3.1 TDD, R.PDSCH.2-3.2 TDD, R.PDSCH.2-3.3 TDD, R.PDSCH.2-3.4 TDD, R.PDSCH.2-4.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-1 |  | R.PDSCH.2-1.3 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.4815 |  | 112000 |  |
| FR1.30-2 |  | R.PDSCH.2-5.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-2 |  | R.PDSCH.2-17.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 5 |  | 375000 |  |
| FR1.30-3 |  | R.PDSCH.2-6.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.4815 |  | 112000 |  |
| FR1.30-4 |  | R.PDSCH.2-9.1 TDD |  | TDLA30-10 |  | 10 Hz |  | 75000 (Note 1) |  | 1.2903 |  | 97000 |  |
| FR1.30-5 |  | R.PDSCH.2-11.1 TDD |  | TDLB100-400 |  | 400Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-6 |  | R.PDSCH.2-12.1 TDD |  | TDLB100-400 |  | 400Hz |  | 10000 (Note 1) |  | 1.2903 |  | 13000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.1 TDD |  | HST-1000 |  | 1000 Hz |  | 15000 (Note 1) |  | 1.4815 |  | 23000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.1 TDD |  | HST-1667 |  | 1667 Hz |  | 15000 (Note 1) |  | 1.4815 |  | 23000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.4 TDD |  | HST-SFN |  | 1667 Hz |  | 30000 (Note 1) |  | 1.4815 |  | 45000 |  |
| FR1.30-1 |  | R.PDSCH.2-10.5 TDD |  | HST-DPS |  | 1667 Hz |  | 30000 (Note 1) |  | 1.4815 |  | 45000 |  |
| FR2.60-1 |  | R.PDSCH.4-1.1 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.33 |  | 27000 |  |
| FR2.120-1A |  | R.PDSCH.5-1.1 TDD |  | TDLC60-300 |  | 300 Hz |  | 10000 (Note 2) |  | 1.25 |  | 13000 |  |
| FR2.120-1 |  | R.PDSCH.5-2.1 TDD, R.PDSCH.5-2.2 TDD, R.PDSCH.5-2.3 TDD, R.PDSCH.5-3.1 TDD |  | TDLA30-300 |  | 300 Hz |  | 10000 (Note 2) |  | 1.25 |  | 13000 |  |
| FR2.120-1 |  | R.PDSCH.5-1.2 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.25 |  | 25000 |  |
| FR2.120-2 |  | R.PDSCH.5-4.1 TDD, R.PDSCH.5-5.1 TDD, R.PDSCH.5-5.2 TDD, R.PDSCH.5-6.1 TDD |  | TDLA30-75 |  | 75 Hz |  | 20000 (Note 2) |  | 1.33 |  | 27000 |  |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table G.1.5-1a: Minimum Test time for PDSCH demodulation with 1% BLER

| TDD UL-DL pattern | Reference Channel | Propagation condition | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- | --- |
| NA | R.PDSCH.1-11.1 FDD R.PDSCH.1-11.2 FDD | TDLA30-10 | 10 Hz | [200000] (Note 1) | 1.1111 | [223000] |
| FR1.30-1 | R.PDSCH.2-16.1 TDD R.PDSCH.2-16.2 TDD | TDLA30-10 | 10 Hz | [200000] (Note 1) | 1.6667 | [334000] |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed). Note 3: MNS/MNAS ratio decided by scheduling pattern (how much time is required to collect required number of active DL SFs). |  |  |  |  |  |  |

Table G.1.5-2: Minimum Test time for PDCCH demodulation

| Reference Channel | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- |
| R.PDCCH 1-1.1 FDD, R.PDCCH.1-1.3 FDD, R.PDCCH.1-2.1 FDD, R.PDCCH.1-2.2 FDD, R.PDCCH.1-2.3 FDD, R.PDCCH.1-2.4 FDD, R.PDCCH.1-2.5 FDD, R.PDCCH.1-2.6 FDD | 10, 100, 400 Hz | 100000 (Note 1) | 1.0526 | 106000 |
| R.PDCCH.2-1.1 TDD, R.PDCCH.2-1.2 TDD, R.PDCCH.2-2.1 TDD, R.PDCCH.2-1.3 TDD | 10, 100, 400 Hz | 100000 (Note 1) | 1.2903 | 130000 |
| R.PDCCH.5-1.1 TDD, R.PDCCH.5-1.2 TDD, R.PDCCH.5-1.3 TDD, R.PDCCH.5-2.1 TDD | 75, 300 Hz | 100000 (Note 2) | 1.25 | 130000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. |  |  |  |  |

<End of Changes >