3GPP TSG-RAN5 Meeting #95-e	R5-222905r3

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2371 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of RedCap default test channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The maximum channel bandwidth supported by the RedCap UE is 20MHz. Current default test channel bandwidth for non-RedCap UE might exceed this limit. It’s required to specified appropriated Low, Mid, High test channel bandwidth for RedCap UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding separate table to capture the default Low, Mid, High test channel bandwidth for RedCap UE. The bandwidths only applicable to UL or DL are not considered. The bandwidths only applicable for SCell in CA/DC configuration are not considered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test channel bandwidth might exceed the RedCap UE’s capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0A, 4.3.1.0B, 4.3.1.0C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Adding ‘non-redcap’ to existing FR2 table Remove redundant notes from the redcap table. Correcting middle channel bandwidth for n79 Updating maximum CHBW of n70 to 15UL/20DL Updating mid CHBW of n71 to 10MHz Updating mid CHBW of n66 to 10MHz. R2: Merging RedCap CHBW into the existing table for better readability. R3: Updating the table headers. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1 and Table 4.3.1.0A-2 for FR1 and FR2 respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz] | RedCap UE Mid Test Channel bandwidth [MHz] |
| --- | --- | --- |
| n1 | 156, 257 | 15 |
| n2 | 15 | 15 |
| n3 | 20 | 15 |
| n5 | 15 | 15 |
| n7 | 156, 257 | 15 |
| n8 | 15 | 15 |
| n12 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 156, 209 | 15 |
| n26 | 108 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 10 | 10 |
| n38 | 156, 209 | 15 |
| n39 | 20 | 15 |
| n40 | 30 | 15 |
| n41 | 60 | 15 |
| n48 | 204, 405 | 15 |
| n50 | 20 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 15 | 15 |
| n66 | 20 | 108 |
| n70 | 15 | 10 |
| n71 | 10 | 10 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 60 | 20 |
| n80 | 203 | 15 |
| n81 | 153 | 15 |
| n82 | 153 | 15 |
| n83 | 153 | 15 |
| n84 | 153, 6, 257 | 15 |
| n86 | 203 | 15 |
| n95 | 103 | 10 |
| n97 | 50 | 15 |
| n99 | 103 | 10 |
| Note 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 UEs. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: This Mid test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 20MHz. Note 7: This Mid test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 50MHz. Note 8: This Mid test channel bandwidth is chosen since it is more commonly used. Note 9: This Mid test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 40MHz. |  |  |

Table 4.3.1.0A-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 200 |
| n258 | 200 |
| n260 | 200 |
| n261 | 200 |
| Note 1: For UEs with limited UE channel bandwidth capability, if mid channel BW is not supported by the UE, select the closest channel BW to the average channel BW of all supported channel bandwidths among all SCSs. If there are two channel bandwidths that have same distance to the mathematical center, the higher one is selected. This shall apply only for Rel 15 UEs. |  |

4.3.1.0B	Low test channel bandwidth

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1 and Table 4.3.1.0B-2 for FR1 and FR2 respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR Band |  | UE Low Test Channel bandwidth [MHz] |  | RedCap UE Low Test Channel bandwidth [MHz] |  |
| --- | --- | --- | --- | --- | --- |
| n1 |  | 5 |  | 5 |  |
| n2 |  | 5 |  | 5 |  |
| n3 |  | 5 |  | 5 |  |
| n5 |  | 5 |  | 5 |  |
| n7 |  | 5 |  | 5 |  |
| n8 |  | 5 |  | 5 |  |
| n12 |  | 5 |  | 5 |  |
| n14 |  | 5 |  | 5 |  |
| n20 |  | 5 |  | 5 |  |
| n24 |  | 5 |  | 5 |  |
| n25 |  | 5 |  | 5 |  |
| n26 |  | 5 |  | 5 |  |
| n28 |  | 5 |  | 5 |  |
| n29 |  | 52 |  | N/A |  |
| n30 |  | 5 |  | 5 |  |
| n34 |  | 5 |  | 5 |  |
| n38 |  | 5 |  | 5 |  |
| n39 |  | 5 |  | 5 |  |
| n40 |  | 54,105 |  | 10 |  |
| n41 |  | 10 |  | 10 |  |
| n48 |  | 54, 105 |  | 10 |  |
| n50 |  | 54,105 |  | 10 |  |
| n51 |  | 5 |  | 5 |  |
| n53 |  | 5 |  | 5 |  |
| n65 |  | 5 |  | 5 |  |
| n66 |  | 5 |  | 5 |  |
| n70 |  | 5 |  | 5 |  |
| n71 |  | 5 |  | 5 |  |
| n74 |  | 5 |  | 5 |  |
| n75 |  | 52 |  | N/A |  |
| n76 |  | 52 |  | N/A |  |
| n77 |  | 10 |  | 10 |  |
| n78 |  | 10 |  | 10 |  |
| n79 |  | 40 |  | 10 |  |
| n80 |  | 53 |  | 5 |  |
| n81 |  | 53 |  | 5 |  |
| n82 |  | 53 |  | 5 |  |
| n83 |  | 53 |  | 5 |  |
| n84 |  | 53 |  | 5 |  |
| n86 |  | 53 |  | 5 |  |
| n95 |  | 53 |  | 5 |  |
| n97 |  | 5 |  | 5 |  |
| n99 |  | 53 |  | 5 |  |
| Note 1: For UEs with limited UE channel bandwidth capability, if the above defined low channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel.15 UEs. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable for use as SCell in CA or SCell in DC configuration. Note 5: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. |  |  |

Table 4.3.1.0B-2: Low Test Channel bandwidths for each NR band, FR2

| NR Band | UE Low Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 50 |
| n258 | 50 |
| n260 | 50 |
| n261 | 50 |
| Note 1: For UEs with limited UE channel bandwidth capability, if the above defined low channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel.15 UEs.. |  |

4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1 and Table 4.3.1.0C-2 for FR1 and FR2 respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz] | RedCap UE High Test Channel bandwidth [MHz] |
| --- | --- | --- |
| n1 |  | 206, 507 |  | 20 |  |
| n2 |  | 20 |  | 20 |  |
| n3 |  | 308, 409 |  | 20 |  |
| n5 |  | 20 |  | 20 |  |
| n7 |  | 206, 507 |  | 20 |  |
| n8 |  | 20 |  | 20 |  |
| n12 |  | 15 |  | 15 |  |
| n14 |  | 10 |  | 10 |  |
| n20 |  | 20 |  | 20 |  |
| n24 |  | 10 |  | 10 |  |
| n25 |  | 206, 409 |  | 20 |  |
| n26 |  | 20 |  | 20 |  |
| n28 |  | 206, 308 |  | 20 |  |
| n29 |  | 102 |  | N/A |  |
| n30 |  | 10 |  | 10 |  |
| n34 |  | 15 |  | 15 |  |
| n38 |  | 206, 409 |  | 15 |  |
| n39 |  | 40 |  | 20 |  |
| n40 |  | 80 |  | 20 |  |
| n41 |  | 100 |  | 20 |  |
| n48 |  | 403, 1004 |  | 20 |  |
| n50 |  | 80 |  | 20 |  |
| n51 |  | 5 |  | 5 |  |
| n53 |  | 10 |  | 10 |  |
| n65 |  | 20 |  | 20 |  |
| n66 |  | 40 |  | 20 |  |
| n70 |  | 151/252 |  | 151/202 |  |
| n71 |  | 20 |  | 20 |  |
| n74 |  | 20 |  | 20 |  |
| n75 |  | 202 |  | N/A |  |
| n76 |  | 52 |  | N/A |  |
| n77 |  | 100 |  | 20 |  |
| n78 |  | 100 |  | 20 |  |
| n79 |  | 100 |  | 20 |  |
| n80 |  | 301 |  | 20 |  |
| n81 |  | 201 |  | 20 |  |
| n82 |  | 201 |  | 20 |  |
| n83 |  | 201, 308 |  | 20 |  |
| n84 |  | 201, 507 |  | 20 |  |
| n86 |  | 401 |  | 20 |  |
| n95 |  | 151 |  | 15 |  |
| n97 |  | 100 |  | 20 |  |
| n99 |  | 101 |  | 10 |  |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: For UEs with limited UE channel bandwidth capability, if the above defined high channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel-15 UEs. Note 6: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 20MHz. Note 7: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 50MHz. Note 8: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 30MHz. Note 9: This High test channel bandwidth is applicable to UEs supporting maximum channel bandwidth 40MHz. |  |  |

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR Band | UE High Test Channel bandwidth [MHz] |
| --- | --- |
| n257 | 400 |
| n258 | 400 |
| n260 | 400 |
| n261 | 400 |
| Note 1: For UEs with limited UE channel bandwidth capability, if the above defined high channel bandwidth is not supported by the UE, select the closest channel bandwidth in both DL and UL. This shall apply only for Rel-15 UEs. |  |

<End of Changes >