3GPP TSG-RAN5 Meeting #95-e	R5-222916r1

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2521 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR IIoT: Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_IioT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Conformance testing of NR IIoT feature “PDCP packet duplication enhancements” requires a new PDCP test model with support of 2 activated SCells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new Test Model for covering NR PDCP Carrier aggregation with 3CC. - New pixit defined for 3CC band combinations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR IIoT signalling conformance test cases cannot be implemented in TTCN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.2.2.2.4(new), 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Additional corrections to figure 5.2.2.2.4 have been done: Corrected direction of the link between the MAC entity of SCell1 to the RLC entity of the PCell. Test model only applicable for AM RLC entities. Minor editorial corrections. |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ASP	Abstract Service Primitive

ATS	Abstract Test Suite

AWGN	Additive White Gaussian Noise

CA	Carrier Aggregation

CBRA	Contention Based Random Access

CC	Component Carriers

CCE	Control Channel Element

CFRA	Contention Free Random Access

CORESET	Control Resource Set

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FR1	Frequency Range 1

FR2	Frequency Range 2

HO	Handover

H-SFN	Hyper SFN

ICS	Implementation Conformance Statement

IUT	Implementation Under Test

IXIT	Implementation eXtra Information for Testing

LSB	Least Significant Bit

MCG	Master Cell Group

MN	Master Node

MSB	Most Significant Bit

NE-DC	NR-E-UTRA Dual Connectivity

NR	NR Radio Access

NR-DC	NR-NR Dual Connectivity

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCell	Primary SCG Cell

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RA	Random Access

RACH	Random Access Channel

RAR	Random Access Response

RAT	Radio Access Technology

RMSI	Remaining Minimum SI

SCell	Secondary Cell

SCG	Secondary Cell Group

SFN	System Frame Number

SN	Secondary Node

SRS	Sounding Reference Signal

SS	System Simulator

SSB	Synchronization Signal and PBCH block

SSS	Secondary Synchronisation Signal

TC	Test Case

UL	Uplink

UL-SCH	Uplink Shared Channel

UT	Upper Tester

VNG	Virtual Noise Generator

<SECTIONS SKIPPED>

##### 5.2.2.2.3	NR-E-UTRA dual connectivity

Figure 5.2.2.2.3-1: Test model for NE-DC PDCP testing (split DRB)

The UE is configured in Test Loop Mode, to loop back the user domain data above NR PDCP layer.

On UE side Ciphering is enabled as null algorithm and ROHC is not configured.

Test Loop Mode can be active on split DRB as shown in Figure 5.2.2.2.3-1.

On the SS NR, SRBs 0,1 and 2 are configured as per Layer3 test model. The Layer1, MAC and RLC of the MCG bearer are configured in normal operation. The PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

On the SS E-UTRA the Layer1, MAC and RLC of the SCG bearer are configured in normal operation. The PDCP layer of the SCG leg is configured as Proxy mode, TTCN shall configure E-UTRA for NE-DC PDCP testing only when a Test Loop Mode is active on a split DRB.

The SS shall route DL PDCP PDUs from TTCN to PCell and/or PSCell and SS shall indicate that the UL PDCP PDU is received from PCell or PSCell.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

##### 5.2.2.2.4	NR carrier aggregation

Figure 5.2.2.2.4-1: Test model for NR CA PDCP testing

The UE is configured in Test Loop Mode, to loop back the user domain data above PDCP layer.

On UE side Ciphering is enabled as null algorithm and ROHC is not configured.

The SS configures in the PCell, Layer 1, MAC and RLC in the normal operation. The NR PDCP is configured in a special mode, where SS does not add any PDCP headers in DL and/or not remove any PDCP headers in UL directions respectively at DRB port on the NR PTC. The TTCN maintains sequence numbers and state variables for the PDCP layer.

The SS configures DRB j, DRB j+1 and DRB j+2 on the PCell, every DRB is connected to an RLC entity. The RLC entity configured on DRB j is linked to the MAC entity on the PCell, the RLC entity configured on DRB j+1 is linked to the MAC entity on the SCell 1 and the DRB j+2 is linked to the MAC entity on the SCell 2.

The SS configures SCell 1 with Layer 1, MAC in normal operation. The MAC entity of each SCell is linked to the RLC entity on DRB j+1 of the PCell.

The SS configures SCell 2 with Layer 1, MAC in normal operation. The MAC entity of each SCell is linked to the RLC entity on DRB j+2 of the PCell.

The NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception over system indication port, if configured.

#### 5.2.2.3	RLC

<SECTIONS SKIPPED>

## 9.2	E-UTRA and NR PIXIT

Table 9.2-1: E-UTRA and NR PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_ENDC_BandCombination | ENDC_BandCombination_Type | DC_1A_n28A |  | EN-DC Band Combination |
| px_ENDC_CA_BandCombination | ENDC_CA_BandCombination_Type | DC_1A_n28A_n78A |  | EN-DC CA Band Combination |
| px_ENDC_SecondaryBandCombination | ENDC_BandCombination_Type | DC_1A_n77A |  | Secondary EN-DC Band Combination |
| px_IPv4_Address_Other | charstring |  |  | Other IPv4 Address (see NOTE 4) |
| px_IPv6_Address_Other | charstring |  |  | Other IPv6 Address (see NOTE 4) |
| px_IPv4_Address4_NW | charstring |  |  | IPv4 Gateway Address in PDN4 |
| px_IPv6_Address4_NW | charstring |  |  | IPv6 Gateway Address in PDN4 |
| px_IPv4_Address4_UE | charstring |  |  | IPv4 Address connected to PDN4 |
| px_IPv6_Address4_UE | charstring |  |  | IPv6 Address connected to PDN4 |
| px_IPv4_Address5_NW | charstring |  |  | IPv4 Gateway Address in PDN5 |
| px_IPv6_Address5_NW | charstring |  |  | IPv6 Gateway Address in PDN5 |
| px_IPv4_Address5_UE | charstring |  |  | IPv4 Address connected to PDN5 |
| px_IPv6_Address5_UE | charstring |  |  | IPv6 Address connected to PDN5 |
| px_IPv4_Address6_NW | charstring |  |  | IPv4 Gateway Address in PDN6 |
| px_IPv6_Address6_NW | charstring |  |  | IPv6 Gateway Address in PDN6 |
| px_IPv4_Address6_UE | charstring |  |  | IPv4 Address connected to PDN6 |
| px_IPv6_Address6_UE | charstring |  |  | IPv6 Address connected to PDN6 |
| px_NEDC_BandCombination | NEDC_BandCombination_Type | DC_n1A_28A |  | NE-DC Band Combination |
| px_NR_CA_BandCombination | NR_CA_BandCombination_Type | CA_n3A_n77A |  | NR CA Band Combination with 2CC |
| px_NR_CA_3CC_BandCombination | FFS |  |  | NR CA Band Combination with 3CC |
| px_NR_DC_BandCombination | NR_DC_BandCombination_Type | DC_n78A_n257A |  | NR-DC Band Combination |
| px_NR_DC_CA_BandCombination | NR_DC_CA_BandCombination_Type | DC_n78A_n257G |  | NR-DC CA Band Combination |
| px_NR_CipheringAlgorithm | CipheringAlgorithm | nea2 |  | Ciphering Algorithm (see Note 1) |
| px_NR_IntegrityProtAlgorithm | IntegrityProtAlgorithm | nia2 |  | Integrity Algorithm (see Note 1) |
| px_NR_OverlappingNotSupportedBand_MFBI | integer | 1 |  | A not supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_PrimaryBand | integer | 1 |  | NR primary band |
| px_NR_SecondaryBand | integer | 2 |  | NR secondary band. Applied to inter-band and SUL test cases. |
| px_NR_SidelinkBand | integer | 47 |  | NR sidelink band |
| px_NR_RATComb_Tested | NR_RATComb_Tested_Type | NR_UTRA | NR_UTRA, NR_GERAN | This parameter represents the network RAT capability / preference and indicates which, if any is supported, RAT combination is to be tested. |
| NOTE 1: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. NOTE 2: Void. NOTE 3: Void. NOTE 4: IP address used for specific IP-address-related test requirements. The only requirements applying to this IP address is that its value shall be different from the values of all other IP address PIXITs (see TS 36.523-3 [12] clause 9) and it shall not be used within more than one PDN (PDU session) within a test case. In the present version of this specification, it is only used within PDN1 NW. |  |  |  |  |