3GPP TSG-RAN5 Meeting #95-e	R5-223003r1

# Electronic Meeting, 09th May – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1826 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 5.6.6.3 EN-DC FR2 L1-SINR measurement including Test Tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test case 5.6.6.3 was missing in TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test case 5.6.6.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP would not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.6.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.903 CR 0315 TS 38.533 CR 1823 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: Adding the affected CR to refer to the minimum requirements. |  |  |  |  |  |  |  |  |

<Start of Changes>

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX

Editor's Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test case is incomplete for extreme conditions

5.6.6.3.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.3.

5.6.6.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC FR2 and L1-SINR measurement.

5.6.6.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.6.6.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.5.6.6.3.

5.6.6.3.4	Test description

5.6.6.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.6.6.3.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 5.6.6.3.4.1-2. Test environment parameters are given in Table 5.6.6.3.4.1-3.

Table 5.6.6.3.4.1-1: Applicable NR configurations for EN-DC FR2 CSI-RS based CMR and CSI-IM based IMR L1-SINR measurement

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 5.6.6.3.4.1-2: General test parameters for FR2 L1-SINR test with CMR and dedicated IMR

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD |
| SSB configuration | 1~2 |  | SSB.1 FR2 |
| CSI-RS configuration | 1~2 |  | CSI-RS.3.3 TDD |
| CSI-IM configuration | 1~2 |  | CSI-IM.3.2 TDD |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | Off |
| reportConfigType | 1~2 |  | aperiodic |
| reportQuantity-r16 | 1~2 |  | cri-SINR-r16 |
| Number of reported RS | 1~2 |  | 2 |
| qcl-Info | 1~2 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~2 |  | 26 |
| T1 | 1~2 | s | 5 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 5.6.6.3.4.1-3: Test Environment parameters for FR2 L1-SINR test with CMR and dedicated IMR

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA, 7.2.3 for NR FR2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 5.6.6.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 5.6.6.3.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9. The UE RX Beam Peak direction has been obtained previously using one of the search procedures as described in Annex I.

5.6.6.3.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. UE is also configured to measure L1-SINR based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in Table 5.6.6.3.4.1-2.

- Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

- Set the parameters according to T1 in Table 5.6.6.3.5-1. T1 starts.

- After 480ms from the start of the test the SS transmits the DCI trigger in slot 8.

- The SS shall check following requirements:

- R1:	the UE shall send L1-SINR report at slot 26 from the reception of DCI trigger. If the report is received at slot 26 from the reception of DCI trigger, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the L1-SINR value of CSI-RS#1 reported by the UE is compared to the expected L1-SINR value for CSI-RS#1. If the resulting value is outside the limits in Table 5.6.6.3.5-2 or the UE fails to report the measurement value for CSI-RS #1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

-R3:	The DIFF SINR value of CSI-RS#0 reported by the UE is compared to the expected DIFF SINR value. If the resulting value is outside the limits in Table 5.6.6.3.5-3 or the UE fails to report the measurement value for CSI-RS 0, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

- The SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

6.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

7	If any the reconfiguration fails, switch off and on the UE and ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.6.6.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 5.6.6.3.4.3-1: Common Exception messages for EN-DC FR2 CSI-RS based CMR and CSI-IM based IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions APERIODIC and CSI-SINR and CSI-IM_IMR Table H.3.6A-2 with conditions CSI-RS and APERIODIC Table H.3.6A-4 with conditions APERIODIC Table H.3.4-1 |

Table 5.6.6.3.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

5.6.6.3.5	Test requirement

Table 5.6.6.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 5.6.6.3.5-1: CSI-RS specific test parameters for EN-DC FR2 CSI-RS based CMR and CSI-IM based IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.9 |  |
| Assumption for UE beamsNote 3 | 1~2 |  | Rough |  |
| Note1 | 1~2 | dBm/15kHz | -105 |  |
| Note1 | 1~2 | dBm/SSB SCS | -95.97 |  |
|  | 1~2 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1~2 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1~2 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1~2 | dB | 0 | 9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

After 480ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR. Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 5.6.6.3.5-2 and the corresponding relative accuracy requirements in Table 5.6.6.3.5-3. The reported L1-SINR value shall consider the Rx antenna gain in the range of [-10 ~ +20] dB when calculated.

Table 5.6.6.3.5-2: L1-SINR absolute accuracy requirements for the reported values

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 53 |
| Highest reported value (CSI-RS #1) | 72 |

Table 5.6.6.3.5-3: L1-SINR relative accuracy requirements for the reported values

| Normal Conditions | T1 |
| --- | --- |
| Lowest DIFF SINR reported (CSI-RS #0) | 5 |
| Highest DIFF SINR reported (CSI-RS #0) | 12 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Changes>