3GPP TSG RAN Meeting #95-e	                          R5-223037r1

Electronic Meeting, 9th May – 22nd May 2022

Agenda item:	5.3.19.8

Source:   Apple Portugal, [Keysight]

Title:       Discussion on FR2 Beam Correspondence test structure

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

RAN4 introduced Enhanced Beam Correspondence requirements as part of the Release 16 FR2 RF enhancements WID. The corresponding RAN5 WID is adding test procedures and requirements for this feature. There are still open items related to the test case structure and applicability that needs to be addressed.

This paper proposes a test structure that would help ensure all the enhanced beam correspondence requirements defined in Rel.16 TS 38.101-2 are accurately covered in RAN5 test specifications.

# 2	Discussion

## 2.1     Summary of BC requirements to be tested across Rel15 and Rel16.

Observation 1: In the original test structure for Beam Correspondence, clause 6.6.1 in TS 38.521-2 was added to test Rel15 UEs that do not support beam correspondence without UL beam sweeping.

Observation 2: For UEs that do not support UL beam sweeping, the existing Min Peak EIRP and Spherical coverage requirements are tested within 6.2.1.1 and 6.2.1.2 respectively with Annex K.1.1 appropriately updated to account for UE capability reported via beamCorrespondenceWithoutUL-BeamSweeping capability bit

The Rel16 Enhanced beam correspondence requirements however cover a more advanced set of side conditions and UE capabilities. The below table summarizes the Rel15/Rel16 Enhanced beam correspondence requirements along with the associated UE capability. For convenience the mapping to core requirements as well as a proposed test case coverage within RAN5 test specifications is also included.

Table 1

| Core Requirement to cover following UE Capabilities |  |  |  |  |
| --- | --- | --- | --- | --- |
| beamCorrespondenceWithoutUL-BeamSweeping | beamCorrespondenceSSB-based-r16 | beamCorrespondenceCSI-RS-based-r16 | 38.101-2 Core Requirements Coverage | 38.521-2 Test Requirement/Test Case Coverage |
| Yes | No | No | 1) 6.2.1 Min Peak EIRP and Spherical Cov with autonomously chosen UL beam and without UL beam sweeping | 6.2.1.1/6.2.1.2 (with Annex K updates) already added for Rel15 Beam Corr (autonomous) |
| Not Present | No | No | 1) 6.2.1 Min Peak EIRP and Spherical Cov with UL beam sweeping 2) 6.6.4.2 Beam Correspondence Tolerance Supports uplink beam management per TS 38.306 | 6.2.1.1/6.2.1.2 (with Annex K updates) and 6.6.1 for BCorr Tolerance requirement already added |
| Yes | Yes | No | 1) 6.2.1 Min Peak EIRP and Spherical Cov with autonomously chosen UL beam and without UL beam sweeping. Use 6.6.4.3.2 SSB-based Enh BCorr side conditions | 6.2.1.1/6.2.1.2 (with Annex K updates) already added for Rel15 Beam Corr (autonomous) Add 6.2.1.1_1 and 6.2.1.2_1 to cover Enh Beam correspondence requirements for UES not using UL beam sweeping. Annex K has to be updated |
| Yes | No | Yes | 1) 6.2.1 Min Peak EIRP and Spherical Cov with autonomously chosen UL beam and without UL beam sweeping. Use 6.6.4.3.3 CSI-RS -based Enh BCorr side conditions | 6.2.1.1/6.2.1.2 (with Annex K updates) already added for Rel15 Beam Corr (autonomous) Add 6.2.1.1_1 and 6.2.1.2_1 to cover Enh Beam correspondence requirements for UES not using UL beam sweeping. Annex K has to be updated |
| Not Present | Yes | No | 1) 6.2.1 Min Peak EIRP and Spherical Cov with UL beam sweeping Use 6.6.4.3.2 SSB-based Enh BCorr side conditions 2) 6.6.4.2 Beam Correspondence Tolerance Supports uplink beam management per TS 38.306 | To be added under 6.6.2 with Rel16 side conditions for SSB-Based Beam Corr |
| Not Present | No | Yes | 1) 6.2.1 Min Peak EIRP and Spherical Cov with UL beam sweeping Use 6.6.4.3.3 CSIRS—based Enh BCorr side conditions 2) 6.6.4.2 Beam Correspondence Tolerance Supports uplink beam management per TS 38.306 | To be added under 6.6.2 with Rel16 side conditions for CSI-RS Based Beam Corr |
| Yes | Yes | Yes | 1) SSB-based enhance beam Corr requirements in 3. 2) Either: A) Minimum peak EIRP requirement as defined in clause 6.2.1.3 using the SSB+CSI-RS based side conditions in clause 6.6.4.3.3 (5 dB lower power than 6.6.4.3.2) using Tx Beam Peak obtained using SSB side conditions; if this does not PASS B) CSI-RS based Enh Beam correspondence requirements in 4) | 6.2.1.1/6.2.1.2 (with Annex K updates) already added for Rel15 Beam Corr (autonomous) Add 6.2.1.1_1 and 6.2.1.2_1 to cover Enh Beam correspondence requirements for UES not using UL beam sweeping. Annex K has to be updated |
| Not Present | Yes | Yes | 1) SSB-based enhance beam Corr requirements in 3. 2) Either: A) Minimum peak EIRP requirement as defined in clause 6.2.1.3 using the SSB+CSI-RS based side conditions in clause 6.6.4.3.3 (5 dB lower power than 6.6.4.3.2) using Tx Beam Peak obtained using SSB side conditions; if this does not PASS B) CSI-RS based Enh Beam correspondence requirements in 4) | To be added under 6.6.2 with Rel16 side conditions for CSI-RS Based Beam Corr. Further analysis needed on implementation of EIRP requirements |

The initial approach to adding Enhanced Beam Correspondence test coverage within clause 6.6.2 was based on the assumption that all the core requirements that need to be tested can be covered under this clause. However, based on email discussions during RAN5#94e, it seems that this option would make the applicability statements and mapping to specific core requirements, more complex.

Observation 3: An option 1 to cover enhanced beam correspondence requirements is the existing approach to add all necessary test coverage under clause 6.6.2 of TS 38.521-2 but there are concerns around applicability statement and core requirement mapping complexity. An Option 2 separates out the BC requirements among Min Peak EIRP, Spherical Coverage and Beam Correspondence with uplink beam sweeping tests.

Option 1: New clause for enhanced beam correspondence test covers all the scenarios listed in Table 1 for Rel16 UEs supporting beam correspondence with or without uplink beam sweeping. This will include test procedures for each of the cases listed in Table 1 while accounting for overlapping test procedure steps as well as updates to Annex K to include every to include every use case listed in Table 1

Option 2: Follow the existing approach where only enhanced beam correspondence tolerance test requirements (UE does not support beamCorrespondenceWithoutUL-BeamSweeping) from Table 1 are tested within clause 6.6.2 which will make it in line with clause 6.6.1. Separate out 6.2.1_1 and 6.2.2_1 for covering Rel16 UEs which do not support uplink beam sweeping. Include Annex K updates to account for changes that will still be needed to factor in all the new Rel16 UE capabilities

Table 2

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | - Creates clear separation for Rel16 enhanced Beam Correspondence requirements - All side conditions are covered within the same test case - All modifications occur within same clause/test case | - Overlap among test cases is expected - Difficult to map to RAN4 core requirements since approach is different from Rel15 - Annex K changes likely to be more complex - Applicability becomes complex due to various UE capabilities covered under the same test. |
| Option 2 | - Similar approach as 6.6.1 - Test structure changes but has precedence in RAN5 (LTE new release tests have been added this way with “_1” suffix) - Annex K changes are easier to handle as it can be decoupled based on individual test case - Applicability statement is easier to handle and implement | - Need to carve out Rel16 tests from clause 6.2.1.1 and 6.2.1.2 which are well established tests (however Rel15 will remain untouched as mitigation) - Scenarios with both CSI and SSB based beam correspondence supported needs analysis - Separate Annex K content will be needed |

While there are challenges to both approaches, the impact on applicability and long-term test structure stability (accounting for future changes coming in Rel17 and Rel18 Beam Correspondence), makes option 2 a better and more robust choice with better scalability.

Proposal 1: Adopt Option 2 for Rel16 Enhanced Beam Correspondence test structure and coverage in TS 38.521-2

One specific aspect that was analyzed further pertains to UEs that support both SSB and CSI-RS based enhanced beam correspondence. A very specific applicability is provided in clause 6.6.4.4 of TS 38.101-2 for this type of UEs.

-	For a UE supporting both SSB based and CSI-RS based enhanced beam correspondence, the UE shall meet both SSB based and CSI-RS based enhanced beam correspondence requirements and the following applicability rules for verifying the requirements apply:

-	The enhanced beam correspondence requirements shall be verified with the SSB based enhanced beam correspondence side conditions in clause 6.6.4.3.2.	If the UE meets the SSB based enhanced beam correspondence requirements using the side conditions in clause 6.6.4.3.2 and meets the minimum peak EIRP requirement as defined in clasue 6.2.1.3 using the CSI-RS based side conditions in clause 6.6.4.3.3, where the link direction is determined in the SSB based enhanced beam correspondence test, the UE is considered to have met both the SSB based and CSI-RS based enhanced beam correspondence requirements.

-	Otherwise, if UE does not meet the minimum peak EIRP requirement as defined in clasue 6.2.1.3 using the CSI-RS based side conditions in clause 6.6.4.3.3, the enhanced beam correspondence requirements shall be further verified for the UE with the CSI-RS based enhanced beam correspondence side conditions in clause 6.6.4.3.3.

An open question that arises from the above applicability rule is whether spherical coverage test needs to be covered for this type of devices. The RAN4 Way forward in [4] that is copied below some gives some idea on this. It is seen that RAN4 looked at various options for the specific test point that is needed and while spherical coverage was one of the options, the eventual agreement was on the option which verifies only peak EIRP using CSI-RS side conditions. This is adopted in the proposed test procedures.

Observation 4: For UEs supporting SSB-Based and CSI-RS based enhanced beam correspondence, core specification guidance is that only peak EIRP with CSI-RS side conditions is sufficient to measure for such UEs.

## 2.1     Applicability

The proposed solution will require associated applicability updates in TS 38.522

Proposal 2: Align applicability statements in TS 38.521-2 and TS 38.522 with the Option 2 approach to enhanced beam correspondence testing.

# 3	Summary

Observation 1: In the original test structure for Beam Correspondence, clause 6.6.1 in TS 38.521-2 was added to test Rel15 UEs that do not support beam correspondence without UL beam sweeping.

Observation 2: For UEs that do not support UL beam sweeping, the existing Min Peak EIRP and Spherical coverage requirements are tested within 6.2.1.1 and 6.2.1.2 respectively with Annex K.1.1 appropriately updated to account for UE capability reported via beamCorrespondenceWithoutUL-BeamSweeping capability bit

Observation 3: An option 1 to cover enhanced beam correspondence requirements is the existing approach to add all necessary test coverage under clause 6.6.2 of TS 38.521-2 but there are concerns around applicability statement and core requirement mapping complexity. An Option 2 separates out the BC requirements among Min Peak EIRP, Spherical Coverage and Beam Correspondence with uplink beam sweeping tests.

Option 1: New clause for enhanced beam correspondence test covers all the scenarios listed in Table 1 for Rel16 UEs supporting beam correspondence with or without uplink beam sweeping. This will include test procedures for each of the cases listed in Table 1 while accounting for overlapping test procedure steps as well as updates to Annex K to include every use case listed in Table 1

Option 2: Follow the existing approach where only enhanced beam correspondence tolerance test requirements (UE does not support beamCorrespondenceWithoutUL-BeamSweeping) from Table 1 are tested within clause 6.6.2 which will make it in line with clause 6.6.1. Separate out 6.2.1_1 and 6.2.2_1 for covering Rel16 UEs which do not support uplink beam sweeping. Include Annex K updates to account for changes that will still be needed to factor in all the new Rel16 UE capabilities

Proposal 1: Adopt Option 2 for Rel16 Enhanced Beam Correspondence test structure and coverage in TS 38.521-2

Proposal 2: Align applicability statements in TS 38.521-2 and TS 38.522 with the Option 2 approach to enhanced beam correspondence testing.

Observation 4: For UEs supporting SSB-Based and CSI-RS based enhanced beam correspondence, core specification guidance is that only peak EIRP with CSI-RS side conditions is sufficient to measure for such UEs.

# 4	References

TS 38.521-2 – User Equipment radio transmission and reception; Part 2, Range 2 standalone

R4-2014923, Remaining issues with beam correspondence enhancements, Apple, 3GPP RAN WG4 Meeting #97-e

R4-2016821, eBC capability alignment with 38.306, Vivo, 3GPP TSG-RAN4 Meeting #97-e

R4-2016820, WF on beam correspondence, Apple, 3GPP TSG-RAN4 Meeting #97-e