3GPP TSG-RAN5 Meeting #95-e	R5-223058r1

# Online, , 9th May 2022 - 20th May 2022

Title:		Handling of FR2 Power Class 1 in RAN5

Source:			NTT DOCOMO, INC.

Agenda Item:		5.4.17

Document for:		Discussion and Endorsement

Introduction

The purpose of this contribution is to define WF for handling FR2 Power Class 1 in terms of time plan and priority.

Discussion

Necessity of FR2 Power Class 1 requirement

TS 38.101-2 currently defines devices into 7 Power Classes based on certain specific architectures in Table 6.2.1.0-1 as follows:

Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |
| Note: RedCap variants of non-RedCap UEs are not precluded |  |

Power Class 3 is the default Power Class.

Considering Japanese situation below, standardization of FR2 Power Class 1 is highly required immediately.

Observation 1	: In order to introduce FR2 Power Class 1 (Band n257) in Japan, it is required to specify the upper limits of MU and Relaxation (due to testability issue) by Aug-2022 at the latest.

- Japanese regulation for Band n257 Power Class 1 will be defined at CY2022.Q3 at the earliest. Since there are much correlation between national regulation and 3GPP standardization in terms of test requirement, it is highly required to specify the upper limits of MU and Relaxation by Aug-2022.

Time plan

RAN5 situation and meeting schedule can be summarized as below.

RAN5

- MU, Relaxation and TT are not completed in all TCs for FR2 Power Class 1.

- Completion target to specify the upper limits of MU and Relaxation is RAN5#96 (Aug-2022), and 2 meetings are
left including this May meeting.

Prioritization

In the existing FR2 work, transmitter and receiver test cases are categorized as P1 considering whether regulatory item. It is proposed that same priority to be applied for FR2 Power Class 1 work.

- Priority 1 (P1) test cases

  - MOP (EIRP, TRP), REFSENS, OFF power, Frequency error, OBW, ACLR, SEM, General Spurious, Spurious co-existence, Inband blocking, ACS, Rx Spurious

- Priority 2 (P2) test cases

  - All other test cases

Considering discussions in section 2.1, 2.2 and 2.3, following time plan can be proposed in order to complete FR2 Power Class 1 work on time.

Proposal 1	: Define following time plan for FR2 Power Class 1 work.

| Meeting | RAN5 time plan |
| --- | --- |
| RAN5#96 (Aug-2022) | To define the upper limits of MU and Relaxation for P1 test cases. |
| RAN5#97 (Nov-2022) | To define all MU, Relaxation and TT for P1 test cases. |
| RAN5#98 (Feb-2023) | To define MU, Relaxation and TT for P2 test cases. |

Proposal

Proposal 1	: Define following time plan for FR2 Power Class 1 work.

- Reference

[1] TS 38.101-2		: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (v17.5.0)”

[1] TS 38.521-2		: “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone (v16.11.0)”